Requirements Document Request for Tender for the provision of; Soils Analysis and Sampling. Procedure; Open. eTenders RFT ID; 8219093. Issue Date; 22/05/2026. Closing Date for Queries; 30/06/2026 at 2pm. Contact for Queries; Via eTenders messaging facility. Closing Date / Time for receipt of Completed Questionnaires; 09/07/2026 at 2pm.

SECTION A: REQUIREMENTS

- Introduction

Teagasc – the Agriculture and Food Development Authority – is the national body providing integrated research, advisory and training services to the agriculture and food industry and rural communities. It is a semi-state organisation established under legislation enacted by the Irish government. Teagasc is a client-based organisation and operates in partnership with all sectors of the agriculture and food industry and with rural development agencies.

- Requirements

Tenderers are required to submit proposals for both soil analysis services and soil sampling services.

Under this tender, Teagasc intends to move from its current model of separately procured soil sampling and laboratory analysis services to an integrated service delivery model operated by the successful Tenderer. In this proposed arrangement, the Tenderer shall provide a fully integrated soil sampling and analysis service, accessible through a digital platform that interfaces with Teagasc systems. Teagasc Advisors will initiate sampling requests via an online system by identifying the client, specifying sampling locations (e.g. LPIS parcels or geo-coordinates), and indicating timing requirements. These requests will be transmitted to the Tenderer, who will manage the end-to-end service including scheduling, sample collection, registration, analysis, and reporting. The Tenderer shall ensure that all associated data, including sampling metadata and laboratory results, are securely transmitted and integrated into Teagasc systems, in a format compatible with NMP Online, where results will be made available for advisor use in nutrient management planning. In this model, Teagasc will not directly procure, invoice, or collect payment for sampling or analysis services; instead, farmers or clients will engage directly with the Tenderer as the recommended provider for these services when sought by Teagasc farmer clients.

2.a Soil Sampling Requirements

- Overview

The successful Tenderer shall provide a professional soil sampling service to support the delivery of soil analysis and nutrient management advice to Teagasc clients.

All soil samples collected must be representative of the relevant field or sampling area and suitable for laboratory analysis in accordance with the soil analysis methods specified under this tender.

Accurate soil sampling is a critical component of the soil testing service. The quality and representativeness of samples directly influence the accuracy of laboratory results and the reliability of nutrient management recommendations.

The service shall support two service delivery models:

• Full Service Sampling – where the Tenderer is responsible for arranging and undertaking the collection of soil samples from the farm and delivering these for laboratory analysis; and

• Assisted / Self-Sampling – where soil samples are collected by the farmer or client, and the Tenderer is responsible for arranging collection, transport, registration, laboratory analysis, and reporting of those samples.

The Tenderer shall ensure that both service delivery models are fully supported within the proposed operational, logistics, and IT systems, including the ability for Teagasc Advisors to select the appropriate service option at the point of request.

The Tenderer shall ensure that:

• soil samples are collected using recognised and standardised soil sampling procedures;

• sampling personnel are appropriately trained, competent and supervised;

• appropriate systems are in place for the identification, handling, storage and transport of samples;

• robust quality assurance and traceability procedures are implemented throughout the sampling process; and

• equivalent standards of quality, traceability, and data integrity are maintained across both service delivery models.

The service must support Teagasc operations across all regions of the Republic of Ireland, including all Teagasc Advisory Offices and Clinic locations.

Tenderers MUST confirm and provide full details of their proposed soil sampling service.

- Programme Planning and Delivery

The Tenderer MUST describe the proposed approach for delivering the soil sampling programme.

The submission MUST include, but is not limited to, the following.

- Programme Planning

- Farmer Engagement

- Field Operations

- Receipt of sampling instructions (via designated portal or process) from Teagasc Advisor

- Sample Tracking and Logistics

The Tenderer MUST demonstrate sufficient personnel and operational capacity to deliver the soil sampling service to all areas in the Republic of Ireland for the full duration of the contract.

The Tenderer must demonstrate the capacity to provide services across all Teagasc advisory regions, including during peak sampling periods (typically autumn, winter and early spring).

Tenderers MUST confirm and provide full details.

- Sampler Competency, Qualifications and Training

The Tenderer MUST ensure that personnel responsible for soil sampling possess the knowledge, skills and experience required to perform sampling in accordance with recognised agricultural best practice and the requirements of this tender.

Sampling personnel MUST be competent in:

- soil sampling principles and objectives;

- correct sampling depth and methodology;

- use and maintenance of sampling equipment;

- composite sample preparation, identification and labelling;

- sample handling, packaging and transport; and

- recording and reporting sampling information.

The Tenderer MUST provide details of training programmes and operational procedures and demonstrate how quality assurance is managed within their proposed soil sampling processes.

Documented training records must be maintained and made available to Teagasc upon request.

Training may be delivered by the Tenderer and/or by Teagasc, as required. The Tenderer must ensure that all personnel engaged in sampling have received appropriate training prior to commencing work.

Teagasc reserves the right to attend, deliver, or require participation in training sessions where necessary.

Tenderers must confirm and provide full details.

- Soil Sampling Protocols

Soil sampling must be carried out using standardised procedures designed to produce representative samples for laboratory analysis and in accordance with recognised agricultural best practice.  The Tenderer MUST ensure that all soil sampling activities comply with applicable health and safety legislation.

Tenderers MUST provide details of soil sampling protocols.  The sampling protocol MUST include:

- sample areas not exceeding 4 hectares, except where soil type and cropping history have been consistent for the previous five years, in which case areas up to 5 hectares may be acceptable; (or alternative soil sampling area requirements as may be required by regulations and schemes within the time-frame of this tender)

- number of soil cores collected per composite sample;

- required sampling depth;

- sampling pattern across the sampling area; and

- procedures to avoid atypical or non-representative areas.

Sampling locations must avoid areas such as:

- gateways and headlands;

- areas recently fertilised or limed;

- dung or urine patches;

- areas near hedgerows, drains or watercourses; and

- areas subject to abnormal soil disturbance.

- Sample Identification, Handling and Transport

Each soil sample MUST be clearly identified to ensure accurate linkage with the corresponding sample record as identified by the Teagasc Advisor.

Sample identification MUST include:

- unique sample identifier;

- other information required for programme administration.

The Tenderer MUST ensure that:

- sample identifiers are correctly applied to containers;

- samples are securely sealed;

- labels remain legible throughout handling and transport;

- samples are protected from contamination; and

- samples are protected from damage during storage and transport.

Samples must be transported promptly to the designated laboratory or collection point using procedures that minimise risks of contamination, loss, damage or misidentification.

Tenderers MUST confirm and provide full details.

- Sample Registration and Data Entry in a LIMS Portal

All soil samples collected under this contract must be registered in a Laboratory Information Management System (LIMS) or equivalent system provided by the Tenderer.

The system must support sample tracking, data capture, and reporting requirements as specified by Teagasc, and must provide appropriate access to Teagasc for monitoring and oversight purposes. The Tenderer is responsible for logging each sample and ensuring that all associated site metadata are accurately recorded.

Each sample logged MUST include the following:

- farmer or client identifier;

  - Teagasc client code

  - Name

  - Company/partnership name is applicable

  - Address

  - DAFM identifier

  - Farmer County

- unique sample identifier as specified by advisor;

- Advisor details-

  - Name,

  - Code,

  - Teagasc Business unit assigned to- Regional Unit, programme

- Sample registration information-

  - Sampled by, Logged By and associated dates

- Sample information

  - Crop to Fertilise

  - Field Name/NMP Number

  - LPIS Parcel Number and/or GPS Location

  - Enterprise Type

  - Soil type

  - Previous Crop

  - Crop History Years and Crop History System

  - Soil service required

- other information required for programme administration.

The successful tenderer shall be required to adopt any revised soil sampling protocols, procedures, standards, or methodologies arising during the term of this contract to meet evolving legislation, policy, or industry best practice.

The Tenderer MUST ensure that the digital sample record corresponds with the physical sample submitted for laboratory analysis.

Procedures MUST ensure:

- accuracy and completeness of all mandatory data entry;

- correct linkage between physical samples and digital records;

- timely entry of sampling data; and

- secure handling of farmer and field information.

Tenderers MUST confirm and provide full details.

- Sample Transport and Delivery Timelines

All soil samples MUST be received by the Tenderer or the approved laboratory within 96 hours of collection on farm.

Tenderers MUST describe:

- transport arrangements from field to laboratory;

- packaging and handling procedures; and

- measures to ensure sample integrity and traceability during transport.

Tenderers MUST confirm and provide full details.

- Sampling Quality Assurance and Record Keeping

The Tenderer MUST implement quality assurance procedures to ensure consistent and compliant soil sampling.

The sampling process must provide full traceability from sample collection to laboratory analysis.

Records must be securely maintained and made available to Teagasc upon request.

Tenderers MUST describe procedures for:

- reporting delays or incidents including irregularities in sample received.

- escalation procedures; and

- corrective actions.

Tenderers MUST confirm and provide full details.

2.b Soil Analysis Methods & Procedures

This section sets out the methods of analysis required under this tender. Soil analysis must be carried out by a Department of Agriculture, Food and the Marine (DAFM) approved soil testing lab which is accredited to ISO 17025 from soil analysis1. Data generated in the analysis of field trial samples by these methods has been used in the production of Teagasc’s Soil Index System. The results of soils tested under contract will be used in conjunction with Teagasc’s Soil Index System to determine major and micro nutrient advice for crops2. As such the methods of analyses described below are the only methods permitted to be used for the analysis of Teagasc samples. During this contract the Tenderer may be required to change the methods of analysis in line with best practice or legislative requirements. In the event of any updates to the prescribed analysis methods, Teagasc will reserve the right to amend the analysis required in consultation with the successful Tenderer. Changes to accredited methods will only be made with the prior agreement of DAFM. Such agreement will allow laboratories time to extend their scope of accreditation to cover the new analysis methodology. Accredited laboratories may be able to submit data using the current method or new method for a defined period, until ISO 17025 accreditation is attained for the new method.

- Soil Sample Preparation (for analysis)

Soil Sample Reception

On receipt of soil samples, the following should be recorded:-

- The time and date of reception of samples at the laboratory and include with the analysis results as per IT requirements (Section 2C).

- Any anomalies with regard to the samples prior to analysis (e.g. sample and paperwork mismatch, sample paperwork missing etc.) should be recorded

Drying

Soil samples must be dried at 40oC ± 3oC for a sufficient time (typically 24-72 hrs) to allow soil to be sieved easily.  The soil does not need to be brought to standard moisture content.

Sieving

Dried soil must be broken up and sieved through 2 mm mesh/sieve and collected in a suitable container. No attempt should be made to grind the sample so that the entire sample (including stones and plant debris) passes through a 2 mm mesh/sieve. Stone and plant debris must be removed by sieving prior to analysis. Extreme care must also be taken to avoid sample contamination. Samples should be mixed thoroughly after sieving to obtain a homogenous sample.

Storage

Samples must be stored in a suitable environment for a minimum of 3 months after all analyses have been completed and results have been submitted to Teagasc. Teagasc clients may request a re-check of sample results during this period. Re-checks should be performed free of charge (FOC).

Teagasc may at any time request the return of samples for comparison analysis.

- Phosphorus (P), Potassium (K) and Magnesium (Mg) using Morgan’s Extracting Solution 3, 4

[Morgan’s extracting solution is the current agronomically preferred (and legislatively required in the case of phosphorus) method of analysis to estimate plant availability of these nutrients. Any future development or change regarding the preferred agronomic or legislatively required test for nutrients may need to be adopted within the lifetime of this tender in the event that such a need should arise.]

Morgan’s extracting solution is used to predict crop nutrient requirements for P, K and Mg in Irish soils. Morgan’s extracting solution is 10% sodium acetate in 3% acetic acid buffered at pH 4.80 ± 0.02.

Sample Extraction

A volume of dried sieved soil is extracted with a volume of Morgan's extracting solution (1:5 v/v ratio) by shaking at room temperature for 30 min at 180 rpm. The suspension is filtered through Whatman No. 2 filter paper and the filtrate is sent for analysis.

Phosphorus (P) Analysis

Phosphorus (P) in Morgan’s soil extracts must be analysed colorimetrically (typically at 675 nm) using the chemical reaction between P and ammonium molybdate. All standards are prepared in Morgan’s extracting solution and the results are reported as mg/l P in soil.  Teagasc Soil Index system for phosphorus nutrient management is based on this colorimetric assay. Analysis of Morgan’s P by non-colorimetric techniques (e.g. ICP-OES) is not acceptable under this contract.

Potassium (K) and Magnesium (Mg) Analysis

Potassium and Magnesium levels in Morgan’s soil extracts can be determined using the following techniques; Flame Emission Spectrometry (FES), Atomic Absorption Spectrometry (AAS), Inductively Coupled Plasma Optical Emission Spectrometry (ICP-OES/ICP-AES), Inductively Coupled Plasma Mass Spectrometry ICP-MS, or an agreed alternative. All standards are prepared in Morgan’s extracting solution and results are reported as mg/l K and mg/l Mg in soil.

- Soil pH

A 10 ml volume of dried sieved soil is mixed with 20 ml of deionised water in a suitable container and stirred vigorously for a fixed period (until a stable pH is reached) prior to pH determination.

- SMP Buffer pH 5, 6, 7, 5

[SMP Buffer pH is the current agronomically preferred (and legislatively required) method of analysis to estimate lime requirement. Any future development or change regarding the preferred agronomic or legislatively required test for lime requirement may need to be adopted within the lifetime of this tender in the event that such a need should arise.]

SMP buffer (pH7.50 ±0.02) is used to calculate the lime requirement for a soil. The change in buffer pH following the addition of soil is a measure of the lime requirement of that soil. Teagasc’s Soil Index System for the lime requirement of Irish soils is based on field trials carried out using SMP buffer.

Teagasc is seeking approval from DAFM to change the buffer method used for lime requirement determination from the SMP Buffer Method to the Modifed Mehlich Method9. For a defined grace period (1 year) DAFM will allow the laboratory’s INAB accreditation for the SMP buffer method to extend to cover the Modifed Mehlich Method. Laboratories must have attained ISO 17025 accreditation for the Modified Mehlich method before the end of this grace period. During the grace period, and subject to agreement with Teagasc, the laboratory may be allowed to submit data for lime requirement determination using the SMP Buffer method or the Modified Mehlich method.

Sample Extraction & Analysis

A 10 ml volume of dried sieved soil is extracted with 20 ml of SMP buffer solution by shaking at room temperature for 30 minutes at 180 rpm.  The suspension is then filtered through Whatman No. 2 filter paper and its pH determined.

Safety Note: - SMP buffer contains P-nitro phenol and potassium chromate and should be handled with care and with reference to associated safety data sheets (SDS).

- Copper (Cu), Zinc (Zn) & easily reducible (ER) Manganese (Mn) by EDTA – Quinol Method.

[EDTA-Quinol Method is the current agronomically preferred method of analysis to estimate plant availability of these nutrients. Any future development or change regarding the preferred agronomic test for nutrients may need to be adopted within the lifetime of this tender in the event that such a need should arise.]

0.05M EDTA + 0.01% Hydroquinone (Quinol) Extracting Solution (pH 7.00 ± 0.05) is used for the determination of available Cu, Zn and Mn in soil. The EDTA-Quinol method is a Teagasc Johnstown Castle Laboratory in-house method. Teagasc’s Soil Index System for Cu, Zn and easily reducible Mn is based on the results of field trial samples extracted using this method.

Sample Preparation & Analysis

A volume of dried sieved soil is extracted with a volume of 0.05M EDTA + 0.01% Hydroquinone (Quinol) Extracting Solution, pH 7.00 ± 0.05 (pH adjusted with ammonia solution) in a 1:5 v/v ratio by shaking at room temperature for 30 minutes at 180 rpm. The suspension is then filtered through Whatman No. 2 filter paper and the filtrate is sent forward for analysis.

The filtrate can be analysed for Cu, Zn and Mn by Flame Emission Spectroscopy (FES), Atomic Absorption Spectroscopy (AAS), Inductively Coupled Plasma Optical Emission Spectrometry (ICP-OES/ICP-AES), Inductively Coupled Plasma Mass Spectrometry (ICP-MS). All standards are prepared in 0.05M EDTA (pH 7.00 ± 0.05) and results are expressed as mg/l Cu, Zn and ER-Mn in soil. Hydroquinine (Quinol) is not added to the standard matrix as its omission increases the standard’s shelf-life.

6. 	Boron in Soil 8

[Extraction using Hot Water is the current agronomically preferred method of analysis to estimate plant availability of Boron. Any future development or change regarding the preferred agronomic test for Boron may need to be adopted within the lifetime of this tender in the event that such a need should arise.]

Contamination can be a problem in analysing for boron.  Care must be taken in the lab with regard to glassware used, sample preparation and storage. Adhesives used on soil boxes often contain boron and Pyrex glassware normally used in the laboratory is made of borosilicate glass.  Soda glass must be used for boron analysis.

Sample Preparation & Analysis

A portion of dried, sieved soil together with deionised water (1:3 w/v ratio) and a pinch or micro spatula of activated carbon (to remove colour from extract) is heated at 120 oC for 10 minutes. The extract is filtered through Whatman No. 2 filter paper and sent forward for analysis. If the solution is still coloured after filtration, additional activated carbon should be added and the solution should be re-filtered. The boron content in the soil extract can be measured colorimetrically at 430 nm following reaction with Azomethine-H or by another agreed analysis method. Results are expressed as mg/kg B in soil.

- Determination of ‘Total’ Cobalt (Co) and Manganese (Mn) in Soil 9,

[Extraction using Aqua Regia is the current agronomically preferred method of analysis to estimate plant availability of these nutrients. Any future development or change regarding the preferred agronomic test for nutrients may need to be adopted within the lifetime of this tender in the event that such a need should arise.]

A portion of dried sieved soil is extracted with aqua regia by standing for 16 hrs (or overnight) at room temperature followed by boiling under reflux for 2hrs. Alternatively, samples can be digested in a closed system (e.g. microwave digestion) using aqua-regia, reverse aqua-regia or another approved method e.g. U.S. EPA Method 3051A  The digest is filtered and made up to volume typically with nitric acid before being sent forward for analysis. Sample digests can be analysed by Flame Emission Spectroscopy (FES), Atomic Absorption Spectroscopy (AAS), Inductively Coupled Plasma Optical Emission Spectrometry (ICP-OES/ICP-AES), Inductively Coupled Plasma Mass Spectrometry (ICP-MS). Results for ‘total’ Co and ‘total’ Mn are expressed as mg/kg in soil.

- % Organic Matter in Soil 1

Organic Matter is measured by loss on ignition following combustion of the soil sample at 500 oC in a muffle furnace. Results are reported as % Organic Matter in soil.

- Quality Control Requirements

Quality Control Soil Samples (QC)

All sample runs must include at least 1 x QC soil sample. For runs greater than 10 samples, QC samples should be run at a rate of 1 x QC soil sample per 10 unknown samples as a minimum. QC soil samples should be prepared and stored under the same conditions as test samples as outlined in Section 1 above.

Result Acceptance Criteria

For all test parameters under contract, intra-laboratory quality control test data (i.e. internal method soil QC data) should deviate by ≤10% from the mean result with the exception of pH tests (soil pH and SMP buffer pH) where deviations from the mean result of no greater than ± 0.2 of a pH unit are required.

For all test parameters under contract, inter-laboratory test data (i.e. comparison of data from the successful tenderer with 1 or more laboratories)should deviate by ≤20% from the mean result with the exception of pH tests (soil pH and SMP buffer pH) where deviations from the mean result of no greater than ± 0.2 of a pH unit are required

Method Limits of Detection & Detection Ranges

Table 9 below gives details of the limits of quantification (LOQ) and the typical range of detection required under this tender

Table 9: Method limits of detections

[The methods included in the following table are the current agronomically preferred (and legislatively required in the case of phosphorus and lime requirement) methods of analysis to estimate plant availability of these nutrients. Any future development or change regarding the preferred agronomic or legislatively required test for nutrients may need to be adopted within the lifetime of this tender in the event that such a need should arise. In the event of alternate methods being introduced, the successful tenderer will be required to agree LOQ and typical range of detection requirements for any new or additional tests.]

| Parameter | LOQ Required | Typical Ranges Required |
| --- | --- | --- |
| Morgan’s P | ≤ 1 mg/l in soil | 1- 20 mg/l |
| Morgan’s K | ≤ 25 mg/l in soil | 25 - 300 mg/l |
| Morgan’s Mg | ≤ 25 mg/l in soil | 25 - 200 mg/l |
| EDTA Cu | ≤ 0.5 mg/l in soil | 0.5 - 20 mg/l |
| EDTA Zn | ≤ 0.5 mg/l in soil | 0.5 - 10 mg/l |
| EDTA Mn | ≤ 10 mg/l in soil | 10 - 200 mg/l |
| ‘Total’ Mn | ≤ 1 mg/kg in soil | 1 - 600 mg/kg |
| ‘Total’ Co | ≤0.5 mg/kg in soil | 0.5 - 20 mg/kg |
| Boron | ≤0.5 mg/kg in soil | 0.5 – 5 mg/kg |
| Organic Matter | n/a | 5-15% |
| Soil pH | pH 2 | pH 2-10 |
| SMP Buffer pH | pH 4 | pH 4-8 |

- References

1 S.I. No. 588/2025 - European Union (Good Agricultural Practice for Protection of Waters) Regulations 2025, Schedule 1

2 Teagasc Johnstown Castle Environment Research Centre Wexford, “Major & Micro Nutrient Advice for Productive Agricultural Crops” 5th Edition 2020, ISBN No. 1-84170-666-3

3 Peach, M. and English, L “Rapid micro chemical soil tests” Soil 	Science, 57, 16, 1944.

4 Byrne, E. 1979. Chemical Analysis of Agricultural Materials. An Foras Taluntais, Dublin, p.194

5 P.F. Pratt and F.L. Blair, “Buffer method for estimating lime and sulphur applications for pH control of soils,” Soil Science 93, 329,1963,.

6  H.E. Shoemaker, E.O. McLean and P.F. Pratt. “Buffer methods for determining lime requirements of soils with appreciable amounts of extractable aluminium,” Soil Sci. Soc. Amer. Proc. 25: 274 - 277, 1961.

7 A. Mehlich., New buffer pH method for rapid estimation of exchangeable acidity and lime requirement of soils, Communications in Soil Science and Plant Analysis, 1976, 7, 637-652.

8 Benjamin, W. 1971. The determination of boron in soil extracts, plant materials, composts, manures, water and nutrient solutions.  Soil Science and Plant Analysis 2(5): p. 363-374.

9 Soil Quality Part 3: Chemical Methods-Section 3.9 Extraction of trace elements soluble in aqua regia. BS 7755-3.9:1995, ISO 11466:1995

10 U.S. EPA. 2007. “Method 3051A (SW-846): Microwave Assisted Acid Digestion of Sediments, Sludges, and Oils,” Revision 1. Washington, DC

Appendix B: Soil analysis services

Table 10: Description of soil analysis service

[The methods included in the following table are the current agronomically preferred (and legislatively required in the case of phosphorus and lime requirement) methods of analysis. Any future development or change regarding the preferred agronomic or legislatively required test for nutrients that may need to be adopted within the lifetime of this tender in the event will require agreement with Teagasc to update the range of soil analysis suites available.]

| Analysis Service Number | Elements Tested | Average (3 yr) % of soil samples per suite. |
| --- | --- | --- |
| Soil 1/S1 Standard | LR, pH, P, K | 94.9% |
| Soil 2/S2 Livestock Grazing | LR, pH, P, K, Mg, Co and Total Mn | 0.49% |
| Soil 3/S3 Sheep - Tillage | LR, pH, P, K, Mg, Co, Cu, Zn, easily reducible Mn and Total Mn | 0.21% |
| Soil 4/S4 Tillage | LR, pH, P, K, Mg, Cu, Zn and easily reducible Mn | 2.18% |
| Soil 5/S5 Horticulture | LR, pH, P, K, B, Mg, Cu, Zn and easily reducible Mn, | 0.25% |
| Soil 6 /S6 (S1 + Mg) Tillage / Grassland | LR, pH, P, K, Mg | 1.42% |
| Soil 7/S7 Organic Matter | Soil organic matter % | 0.01% |
| Soil 8/S8 (S6 + Organic Matter) Tillage / Grassland / OM | LR, pH, P, K, Mg and soil organic matter % | 0.22% |
| Soil 9/S9 (S4 + Organic Matter) Tillage / OM | LR, pH, P, K, Mg, Cu, Zn, easily reducible Mn and soil organic matter % | 0.19% |
| Soil 10/S10 (S5 + Organic Matter) Horticulture / OM | LR, pH, P, K, Mg, B, Cu, Zn, easily reducible Mn and soil organic matter % | 0.13% |

2.c ICT and Data Integration Requirements

### 1. Overview

The successful Tenderer will be required to support secure electronic integration between the laboratory information management system (LIMS) used by the Tenderer and Teagasc systems used for soil sample registration, results management and nutrient planning.

The system shall provide the Teagasc Advisor the capability to initiate the soil sampling process by selecting a farm and defining sampling locations using LPIS parcels and/or geo-coordinates. The Advisor must be able to log in securely, select a registered farm/client, identify sampling areas through either LPIS parcel selection or with geo-coordinate and assign the task to a Soil Sampler. The system shall validate inputs, store the sampling request, and trigger downstream processes.

The integration must support the secure transfer, validation and processing of soil sample data and analysis results so that they can be incorporated back into Teagasc internal systems, including the national soil database and nutrient planning tools. A production and test environments will have to be provided.

The ICT integration solution must support the end-to-end workflow including:

- initiation of the soil sampling request in the vendor portal by Teagasc Advisor

- assignment of soil sampling task to appropriate local contract soil sampler

- registration of soil samples by contract soil samplers

- registration of soil samples by Teagasc advisor for soil samples taken by farmer for collection service option

- receipt and tracking of samples by the laboratory and capacity of sample status to be monitored by Teagasc staff in the vendor portal electronic transfer of validated data and analysis results in a format compatible with the Teagasc IT system for reporting and nutrient planning

- the receipt of electronic files which detail the summary of daily transfers received and files that fail Teagasc validation checks

- annual user access review

- that all soil sampling, client, and laboratory data remain secure, accurate, and accessible, in compliance with organisational policies and regulatory requirements (e.g., GDPR).

Laboratory data will be transferred electronically and processed through Teagasc systems before being made available to advisors and clients. Soil test results ultimately support nutrient advice and fertiliser planning through Teagasc systems including NMP Online.

### 2. Integration with Teagasc Systems

The successful Tenderer must ensure that its ICT systems support integration with the Teagasc Soil Registration Database.

The Tenderer must provide an electronic mechanism (SFTP) to transfer soil sample status information and authorised soil analysis results to Teagasc systems in the format specified in this tender.

### 3. System Overview

The solution integrates with the following systems:

- Laboratory Portal

- Laboratory System

- Laboratory File Transfer Server (SFTP)

- Teagasc File Transfer Server (SFTP)

- Soil Registration Database

### 4. Data Exchange Mechanism

#### 4.1 Soil sampling initiation via Advisor Portal

The system shall allow a Teagasc Advisor:

- Create a job that will be linked to advisor profile and code

- assign unique identifier at sample level to track job

- assign farm/client- search and select from registered records

- Select parcel locations and include an upload facility to attach information if required i.e. maps

- Register the soil service required at sample level

- Assign a timeline of service delivery

- Ability to save draft or submit sampling request

- Validate required data (LPIS or coordinates, within farm boundary, appropriate to assigned area)

- Receive notifications regarding status of job

- Receive provisional quote for order

#### 4.2 Secure File Transfer

The Tenderer must provide and maintain a secure electronic file transfer mechanism for publishing laboratory data files.

Requirements include:

- necessary ISO and Cyber Essential certifications

- data breach protocol in place

- secure file transfer using SFTP

- access restricted to authorised Teagasc systems

- laboratory data files must be transferred on 4 hourly intervals from 8am-12am

- a reliable internet connection sufficient to support file transfer operations.

Electronic files must be accessible to Teagasc systems continuously to allow automated retrieval and processing.

The Tenderer must ensure that files are published promptly following receipt of samples and authorisation of laboratory results in accordance with TAT requirements.

### 5. Electronic Data File Requirements

#### 5.1 File Format

All electronic data files must comply with the following requirements:

- File format: CSV

- One header row containing column names

- One or more rows containing sample data

- No blank rows

Any additional columns other than those specified in the file specification, as required due to changes in DAFM legislation over the duration of the contract. The file layout, column names and formats must conform to the specifications outlined in the electronic results file requirements.

#### 5.2 Soil sample report

The Tender is to provide a report of the soil sample results in a pdf format, mirroring the current reports generated and shared with Teagasc’s clients.  The successful tenderer shall be required to adopt and implement any new or revised output requirements arising during the term of this contract to meet evolving legislation, policy, or industry best practice.

#### 5.3 Folder structure

Four folders are to be created on the Tender’s SFTP server where Teagasc will pull the CSV soil results and the pdf soil reports from:

- /Home/SoilRegistration-Teagasc

- /Home/SoilRegistration-Errors

- /Home/SoilRegistration-Duplicates

- /Home/SoilRegistration-Summary

### 6. Data Standards and Validation

All laboratory data must comply with defined data standards to ensure successful integration with Teagasc systems.

Data provided must include mandatory identifiers linking the sample record to the corresponding Teagasc submission record.

Mandatory data elements include, but are not limited to:

- Job identifier

- sample identifier

- Sample location

- Advisor and Client details

- laboratory receive date

- authorised date analysis results.

Numeric values, date fields and identifiers must conform to the formats specified in the electronic file specification.

Incoming data must be subject to validation checks including:

- verification of mandatory fields

- validation of numeric and date formats

- verification of unique identifiers

- validation of analytical result values

Data failing validation checks will be rejected and an error csv file will be sent to the vendors SFTP server.

Current ingestion processes validate incoming data and prevent duplicate records prior to transfer to internal Teagasc systems. Duplicate records will not be processed by the Teagasc system and will be return to the vendor system.

### 7. Error Handling and Data Quality

The Tenderer must ensure appropriate procedures are in place for identifying and resolving data transfer errors.

The system must support:

- identification of invalid records

- generation of error logs

- correction and resubmission of data where required.

Where errors occur in transmitted files, the Tenderer must cooperate with Teagasc technical staff to resolve the issue within agreed support timelines.

### 8. Data Retention and Archiving

The Tenderer must implement procedures to archive electronic data files generated under this contract.

Requirements include:

- removal of files from the active file transfer location after the data transfer to Teagasc SFTP server has completed

- secure storage of archived files

- retention of archived files for a minimum of 5 years from date of creation.

Archived files must be retrievable if required for operational or audit purposes.

### 9. Integration Testing and Implementation

ICT integration will be tested as part of the project implementation process in the test systems.

Testing will include:

- verification of secure file transfer mechanisms

- validation of electronic file structure and format

- testing of automated data ingestion

- end-to-end system testing with Teagasc systems.

Testing will take place during the build and test phase of the project prior to go-live in the production systems.

### 10. ICT Support

The Tenderer must provide ICT support to ensure the continued operation of the integration solution.

Minimum support requirements include:

- technical support during working hours (Monday–Friday, 09:00–17:00 Irish time)

- support during integration testing and system transition periods

- defined escalation procedures for technical incidents

- resolution of integration issues within agreed response times.

### 11. Support and Maintenance Service Performance

The Tender must provide a service level where:

- the system is available 99% of the time

- service level resolution rates met 100% of the time

- users shouldn't experience delays when accessing soil reports on the Tender’s system

- provide a way for Teagasc to communicate with the vendor when required

- provide a mechanism to raising of support tickets

Appendix A – Teagasc Locations