Project description:

# Outline speciﬁcation

Proposed scheme is for Monaghan County Council (MCC) delivered by the Competitive Dialogue/Partnership Development Agreement form of procurement.

Housing Types:

Housing Types should be in accordance with Design Manual for Quality Housing, prepared by the Department of Housing Local Government and Heritage, section 5.9 Standard Internal Layouts and houses sizes. Dialougue to be had with Monaghan County Council in relation o the housing need in the area and what house types would be considered. The housing NEED in Monaghan at present are for 1, 2 and 3 bedroom accommodation units.

Dwellings external:

Structure:

- Timber frame U value to be driven by part L compliance (0.18w/m2k). Attic trusses included.

## External walls:

- 100mm blockwork with self-coloured render ﬁnish (Colour to be agreed with MCC/Planning)

- Facing brick detailing as per planning drawings – Sample panel to be agreed with MCC.

- Precast concrete windowsills & threshold block.

## Roof:

- Black concrete roof tiles

- Matching dry ridge, verge and valley detail

- Roof vents positioned to the rear of the roof where possible

## Fascia:

- PVC

## Rainwater goods:

- Seamless rolled aluminium gutter.

- 68mm round black PVC down pipe.

- Shoes directed to PVC gulley in path.

Windows:

- Aluminium double glazed to meet part L compliance minimum 30-year life span.

- Side or top hung depending on location demands.

- Handle heights to be between 800mm and 1200mm.

- No trickle vents in windows as ventilation is managed using through-wall vents as dictated by a centralised ventilation system.

## Doors:

- Composite high security front doors.

- Glazed aluminium rear doors.

- Energy performance to be compliant with Part L calculation.

- Drainage Channel to run the full width of the opening (including sidelight if present).

- Door designs as per approved planning drawings.

## Front Entrance Canopy:

- GRP ﬂat roof type.

- Lead ﬂashing built into structure.

- Sample to be approved by client.

Canopy example

## Dwellings internal:

Ground ﬂoor:

- Mixture of PCC slab with In-situ concrete screed ﬁnish and

- Ground bearing slab with In-situ concrete screed ﬁnish.

## Upper ﬂoors:

- Engineered joists to allow service runs.

- OSB ﬂooring ﬁnish covered with Temporary protection membrane during construction.

## Stores:

- In addition to the free area of the hot press, one of the stores is to be provided with 800mm length of 600mm deep slatted shelving in 45x19mm red deal slats on red deal support brackets. Shelves to be provided at 5 levels with the highest shelf at 1.8 m above ﬂoor level.

- Built in wardrobes - shall contain 450mm width of 6no. shelves and 2 no. 30 mm deep proprietary chromed steel hanging rails and brackets.

## Internal walls:

- Timber stud. Acoustic quilt between wc / bathroom and habitable rooms. Moisture resistant plasterboard to bathroom and shower room walls.

- Plasterboard thickness as per timber frame drawing details.

- Sockets and switches ﬁre protected as per timber frame supplier details.

## Doors:

- Door sets. 44m thick with 6mm lipping to all edges. Fire doors where required.

- All sharp edges sanded back.

- 10mm clearance below door unless otherwise instructed by MCC.

## Ironmongery, ﬁxtures and ﬁttings:

- As per approved MCC requirements.

Example of door used at Sli Lorcain

## Floor ﬁnishes:

- Kitchen & dining area (when open plan) - 2mm vinyl sheet with timber skirting.

- WC & bathrooms - 2mm vinyl sheet (with appropriate slip resistance). Fully welded joints. Vinyl coved 100mm up walls.

- Vinyl turned up behind bath panel as photo below.

- Flooring materials to be as per MCC requirements.

## Ceiling ﬁnishes:

- Plasterboard thickness to timber frame supplier details to comply with Fire Regulations.

- Joints taped, ﬁlled and skimmed.

- Moisture resistant plasterboard to bathroom and shower rooms.

- Painted ﬁnish.

## Kitchen:

- Carcass - 18mm MFC core board, melamine faced.

- Doors faced with high pressure laminate on MR MDF core, with melamine veneer to the inside door face. Doors to be 19/20mm thickness

- Drawer units to be metal sided, soft closing.

- Worktop to be laminate on 38mm particle board.

- Kitchen sink located in front of window where possible.

- Cooker positioned on external wall where possible.

- Under cupboard lighting provided to main run of cupboards.

- Kitchen extract ducting to be run through the engineered joists (sloping to external wall).

- TF supplier to form openings in panels where ducts and sewer pipes pass through. 30min FR ﬁre collar

required at ceiling junction.

## Sanitaryware:

- Matching white coloured suite throughout

- Toilets to be close coupled dual ﬂush.

- Bath to be ﬁtted with integral shower connection.

- Bath curtain rail ﬁtted by developer, curtain supplied by tenant.

- Shower hose mounted as high as possible.

- Wash hand basin provided with single mixer tap. Suites to be matching.

- Splash back tiling to WHB to be agreed with client. Mirror and tiling arrangement will be the same as photo below.

- Floor former shower tray to be provided to GF showers.

- Bath panel detail to be as per photo in ﬂoor ﬁnish section above (as per MCC requirements).

## Joinery:

- Skirting - 100mmx19 mm square proﬁle MDF.

- Architrave - 70mm x19 mm square proﬁle MDF.

- Window Board – MR MDF 25mm thick – bullnose proﬁle to edge.

- Alternative proﬁle option for skirting/architrave can be agreed with client.

## Internal painting:

- Walls – 3 coats of emulsion. MCC to choose colour.

- Ceilings - 3 coats of white emulsion.

- Skirting and architrave – primer, undercoat and ﬁnish coat.

## Stairs:

- Speciﬁcation as per MCC requirements.

- Finished with 3 coats of clear satin varnish or painted on completion.

## Wall tiling:

- 150x150 white ﬂat ceramic glazed tiles with white grout throughout.

- Bath – tiled to height of 2m meters above ﬂoor level to 3 sides.

- Accessible shower – tiled to height of 2m above ﬂoor level to 2 sides

- Splash backs to whb’s to be 300mm x 600mm or agreed on site.

- Any sink with window above will have tiled sill.

- Kitchens – tiled to height of 450mm above work tops.

- Cooker space to be fully tiled. Tiling taken up to underside of cooker hood.

- All exposed edges will be ﬁnished with white pvc quadrant trim.

## Electrical:

- Installation to meet all relevant standards.

- All switch, socket faceplates and pendant ﬁttings are to be in contract quality white plastic.

- Lighting:-

  - positions to be agreed on site.

  - Spot/down lights in the kitchen, lounge, and hallway,

  - Bulkhead type lights to wet rooms.

  - Pendant light on the landing, bedrooms, in the lounge and above kitchen table.

  - Bulkhead lights provided to front and back doors.

  - Stores not designed as wardrobes will have a single pendant light.

- USB ports provision in kitchen and bedrooms.

- Smoke alarms double socket and light provided to attic space.

- Link to be provided between ESB meter box / Open Eir box for broadband.

- Arc fault detection devices to entire house.

- Consumer unit located close to front door.

- Fire alarm test switch kept to the external wall opposite the stairs.

- Containment for future EV provision externally.

## Mechanical:

- Installation to meet all relevant standards.

- Heat Source - ASHP’s providing space heating and hot water. Manufactured by Mitsubishi.

- Radiators will provide space heating both up and down stairs.

- Gravity fed system – storage tank positioned in attic.

- Shower, sink and WHB ﬂows to be of standard as per MCC requirements. Established Schematic and pipe speciﬁcations to be followed to ensure compliance.

## Pipework and ﬁttings schematic:

Schematic ﬁle available on request

- House ventilation to be centralised mechanical extract system.

- Ventilation will be provided in the kitchen, bathroom and WC

- Control unit to be positioned in the attic.

- Joule InvaVent Mechanical Extraction Ozeo E Extract or equal approved.

- Passive ventilation to be provided to habitable rooms with manually controllable through wall ducted vents.

## Boundary treatments:

- Rear and sides of dwelling - Generally, 2m fence with Concrete H post, gravel board to low level and timber panels

- Fronts – Generally no fence – pin kerbs to identify boundary

- Fronts where levels are different between plots (>600mm) - Concrete ‘H’ post; Gravel boards at GL; timber boarded fence above GL

- Screen walls at sides of properties – 2.0m high blockwork masonry-built wall with PC coping; Self-coloured render ﬁnish.

- Segmental retaining walls are used where required to overcome signiﬁcant changes in height.

## Site Finishes Generally:

- Road surfacing - 80mm AC 32 HDM base,60mm AC 20 HDM binder course and 40mm SMA 10 surface wearing course.

- Private driveways - 30mm AC 20 dense bitmac binder coarse and 30mm SMA 10 surface course.

- Footpaths both private and public will receive a brushed concrete ﬁnish.