PRELIMINARY SAFETY & HEALTH PLAN (DESIGN STAGE)

PROJECT:

Demolition and Building of new offices at Belmont park X91 NCX7

DATE:

2nd June 2026

File Ref:__ PPE26__________

This Preliminary Safety and Health Plan has been prepared in accordance with

Regulation 12 (1) (a) of SI No. 291, the Safety, Health and Welfare at Work (Construction) Regulations 2013 at the planning stage to provide the appropriate information to facilitate the Project Supervisor Construction Stage in the preparation of the Safety and Health Plan required to control the construction activities on site.

CORLANN and all its Contractors will comply with all statutory requirements as defined by the following:

- Safety, Health &Welfare at Work Act, 2005

- Safety, Health & Welfare at Work (General Application) Regulations 2007

- Safety, Health & Welfare at Work (Construction) Regulations 2013

- All other applicable statutory legislation covering this project

The information contained in this Preliminary Safety and Health Plan has been prepared prior to the commencement of work on site. It does not take into account any matters or information which may come to light after the signed date of this document.

General Description of the Project

The following is a general information description of the project, its location, principal parties involved and customer interaction.

| Client | CORLANN South East Region Belmont Park, Ferrybank, Waterford, |
| --- | --- |

| Architects | N/A |
| --- | --- |

| Project Supervisor Design Process | CORLANN |
| --- | --- |

| General Project Description | Demolition and Building of new offices at Belmont park |
| --- | --- |

| Works Items | Yes | No |
| --- | --- | --- |
| Refurbishment of house |  | X |
| Replacement/Repairs of windows / doors | X |  |
| Blockwork/Plaster | X |  |
| Roofing/Repairs/Scaffolding | X |  |
| Energy upgrade works | X |  |
| Works Items | Yes | No |
| Extension | X | X |
| Plumbing and electrical works | X |  |
| Fit new kitchen units |  | X |
| Fit new multi-fuel insert stove |  | X |
| Painting and tiling | X |  |
| Cleaning | X |  |
| Fit new oil boiler |  | X |
| Fit new air to water heat pump | X |  |
| Fit new/Repair boundary fence / wall |  | X |
| Install stairlift |  | X |
| Bathroom adaptation |  | X |
| Fit new flooring | X |  |
| Install handrails to rear door |  | X |
| Install concrete pathway to house perimeter | X |  |
| Repave driveway |  | X |
| Install WC | X | X |

Site Location

Belmont Park, Ferrybank. X91 NCX7

Drawings / Specifications

PROJECT DRAINGS AND SCOPE OF WORKS DOC

| Relevant adjoining Land use or Industry activity to site |  |
| --- | --- |
| Front | Private carpark |
| Rear | Attached building |
| Right Facing | Private carpark leading to private road |
| Left Facing | Attached building |

Particular Safety Requirements of Client

Contractor to notify CORLANN of works at least one week before works are carried out.

Contractor to maintain supervision on site at all times that employees and sub-contractors are working on site.

Contractor to co-ordinate works so that the works are completed with least disruption to adjoining home and business owners.

Contractor to carry out site risk assessment prior to works to identify the most appropriate method for all working at height activities and identify same in their Safety & Health Plan for the works.

Care to be taken to ensure that work complies with SI 386 Safety, Health & Welfare at Work (Exposure to Asbestos) Regulations 2006 and SI 589 Safety, Health & Welfare at Work (Exposure to Asbestos) (Amendments) Regulations 2010. Refer to 2016 Code of Practice for Safety, Health and welfare at Work (Chemical Agents) Regulations 2001 for guidance on best practices.

Restrictions on Working Hours

No work to be carried out outside the hours of 08.00 – 18.00 Mon. to Sat.

Site Restrictions

Access to the site is by the existing entrance only.

Contractor to co-ordinate works so that the works are safely completed with the least disruption to neighbouring residents.

Road adjacent to site access to be kept free of all obstructions and debris at all times from vehicles leaving site.

The Contractor shall not restrict local traffic or pedestrians by concentration of parked vehicles or skips on the roadside or footpath outside of the site.

Skips, containers etc shall be located within the curtilage of the site and the public roadway shall be kept clean from any material from the site.

Adequate signage shall be provided at the entrance of the site to warn motorists and pedestrians.

| Other Contractors which may affect the work |  |
| --- | --- |
| Not Applicable |  |
| Intended Contract Commencement Date | July 2026 |
| Intended Contract Completion Date | November 2026 |

# Other Work Activities and Site Information

The following activities, not related to or included within the scope of the project, will be carried out on or adjacent to the site during the construction phase.

Existing Building Activities

Administration and Training for Corlann Staff

| Installation of Services |  |
| --- | --- |
| Telephone | Not applicable |
| Electricity Supply | yes |
| Gas | Not applicable |
| Wastewater | yes |
| Mains Water | yes |
| Public Lighting | Not applicable |

Installation of Special Water/Sewerage Treatment Equipment by Contractor/Sub Contractor

Not Applicable

Installation of furniture, equipment, fixtures and fittings or machinery. (Fit out work)

Not Applicable

External work such as Land Drainage, Landscaping, Public Utility Services, work by Local Authority.

Yes, parking and sewer connection in existing MH’s

Existing house to remain occupied by tenant(s) during the works

The adjacent buildings will be occupied during the works.

Site Access and Site Security:  All personnel entering the site shall report to the Contractor. Appropriate warning signs etc. are to be erected on site and at access points. The site shall be made secure at the end of each day’s work and at weekends.

Site security will be the sole responsibility of the Contractor.

The access locations will be adequately signed with adequate visibility to prevent creating a traffic hazard.

Permit to Work System:  The Contractor shall operate a permit to work  system. Copies of certificates for Safe Pass, Construction Skills Certification Scheme, and similar training required by the Regulations must be provided before work commences. The same applies to subsequent re-training, seminars or other training rendered to personnel employed by the Contractor on site.

All persons employed on the site will be required to undergo site induction training covering safety and health procedures. It is expected that every employee on the site will be inducted on the first day or as soon as it is practical to do so. Details of all persons who have carried out the site induction shall be recorded.

# Existing Services

This section refers to all existing services, which may impact on the activities of the project and increase the potential risk to safety.

The Contractor is to ensure that the integrity of any site services is maintained.

The PSDP recommends that for all excavation works the Contractor adhere to the “Code of Practice for Avoiding Danger from Underground Services” as issued by the Health and Safety Authority (HSA) and all works in close proximity to overhead cables shall be carried out in accordance with the “Code of Practice for Avoiding Danger from Overhead Electricity Lines” as issued by ESB Networks.

The Contractor is responsible for confirming the existence, nature and location of all existing services which could be potentially affected by the proposed works and upholding same. The location of the services is subject to confirmation on the site by safe methods.

| Water Mains | Present on site? |  | Yes |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  | Watermain expected to be in the front carpark |  |
| Please contact Irish Water |  |  |  |

| Electric Cables | Present on site? |  | Yes |
| --- | --- | --- | --- |
|  | Applicable to works? |  | YES |
| Provide details of locations if applicable. |  | Electric cables are surface in the existing store room, to be disconnected as part of the enabling works. |  |
| Please contact ESB Networks and check for power within walls/ceilings/floors before starting works. |  |  |  |

| Telecommunications Cables | Present on site? |  | Yes |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  | Telecom cables are expected to be Underground. Some data cables to adjacent buildings are outside and above the structure to be demolished |  |
| Please contact Eir |  |  |  |

| Gas Mains | Present on site? |  | yes |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  |  |  |
| Please contact GNI |  |  |  |

| Liquids or Gases under pressure | Present on site? |  | No |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  |  |  |
| Please contact N/A |  |  |  |

| Wastewater | Present on site? |  | Yes |
| --- | --- | --- | --- |
|  | Applicable to works? |  | Yes |
| Provide details of locations if applicable. |  | Manholes adjacent to the works. |  |
| Please contact N/A |  |  |  |

| Overground Services | Present on site? |  | No |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  |  |  |
| Please contact owner/industry user |  |  |  |

| Other Underground Services | Present on site? |  | No |
| --- | --- | --- | --- |
|  | Applicable to works? |  | No |
| Provide details of locations if applicable. |  |  |  |
| Please contact owner/industry user |  |  |  |

# Particular Risks

The following is a non-exhaustive list of particular risks to the safety and health of persons as set out in Schedule 1 of SI 291 of 2013 on what elements of the works may fall within each particular risk category. It should be noted that many of the risks on the project may arise out of working methods which are at the discretion of the Contractor and as such cannot be determined by the Project Supervisor for Design Stage. Materials and substances used during the construction stage could also present health and safety hazards. The Contractor will be required to carry out risk assessments as required by the Regulations, and to introduce appropriate control measures. The materials specified by the Design team are deemed to be within the normal experience of a competent contractor and have therefore not been specifically identified.

It is considered that the risks listed below should be capable of safe management and control by a competent contractor using safe systems of work and the appropriate levels of resources and equipment.

1. Work which puts persons at work at risk of falling from a height, burial under earthfalls, or engulfment in swampland, where the risk is particularly aggravated by the nature of the work or processes used or by the environment at the place of work or construction site.

- Falling from a height:

Work at height is work in any place, including a place at, above or below ground level, where person could be injured if they fell from that place.   Access and egress to a place of work can also be work at height.

| Risk anticipated by PSDP at site |  |
| --- | --- |
| yes |  |
| Hazard description | Roofing to new structure. |
| Proposed controls | Erect access and work scaffold, suitably anchored propped in position. Fall arrest system in place when fitting roof slabs |

Details:

Works where there is a risk associated with falling from heights, which may be aggravated by the presence of another significant hazard.

  - For example, works to or on roofs or from scaffolding. All works to roofs should incorporate a suitably anchored safety restraint / harness in areas where:

  - There is not suitable scaffolding surrounding the building at eaves level to all sides of the building

  - Work is on or adjacent to fragile roofing materials, e.g. opes in roofs, skylights etc. This is particularly applicable to any demolition element of the project.

  - Operations where the nature of the work or other restrictions make it impracticable to provide standard scaffolding or other suitable and sufficient means of support.

(Contractors are advised to carefully read the Safety, Health and Welfare at Work (Work at Height) Regulations 2006 (S.I. No. 318 of 2006).)

- Burial under earthfalls:

Ensure proper Earthwork Support as per Safety Guidelines on working in Excavations and Trenches.

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

The Contractor is to ensure safe work methods with particular reference to access and egress, edge protection, cuttings support, storage of materials and reinstatement are in place before work commences.

  - For example; deep foundations, deep cuttings, works in excavation which are in or adjacent to unstable ground, hazardous underground services or where there is a risk of undermining existing foundations.

  - Work in excavated wells, underground earthworks, tunnels or underpinning.

  - Any trenching / excavations, should be suitably shored as determined by a risk assessment.

  - All material storage areas

Contractors are advised to carefully read the Safety, Health and Welfare at Work (Constructions) Regulations 2013 Part 5 – Excavations, Shafts, Earthworks, Underground Works and Tunnels.

AF3 forms are to be completed by a competent person inspecting excavation works.

- Engulfment in swampland:

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

Works associated with soft ground excavation or adjacent to soft ground / swampland.

Contractors shall clearly identify boundaries of soft ground / swampland within or adjacent to the site and fence them off appropriately.

- Works which puts persons at work at risk from chemical or biological substances constituting a particular danger to the safety and health of such persons or involving a statutory requirement for health monitoring.

| Risk anticipated by PSDP at site |  |
| --- | --- |
| YES |  |
| Hazard description | - Risks associated with chemical substances used during construction. - Cutting timber, plasterboard etc, airborne particles. - Risk of project personnel contacting Weil’s disease. |
| Proposed controls | 1. All chemical substances chosen by Contractor to be used on site are to be properly assessed and safe methods for handling, storage and use to be put in place. Hazard sheets relating to these materials and locations were used to be included in the Safety File. 2. The Contractor should ensure that adequate protective clothing and gloves are provided to all operatives handling or working with lead. - A safe system of work must be put in place to minimise risks and only competent persons must carry out works on site. - Use appropriate PPE when carrying out all works including but not limited to disposable latex gloves. There must also be hand washing facilities on site. The Contractor should also remain vigilant for evidence of contamination (e.g. discolouring, bad smells, etc.). - No eating, drinking or smoking on site. Operatives are to wash their hands prior to taking any breaks. 6. It is considered that the risks associated with working with chemical or biological substances on the project should be capable of safe management and control by a competent Contractor by providing adequate welfare facilities and using safe systems of work and the appropriate levels of resources and equipment. |

Details:

Interaction which would put the safety and health of persons at work at risk or may require health monitoring in regard to legal requirements.

  - Where certain materials or substances are to be used, or materials in existing buildings are to be modified, a particular risk in relation to chemical or biological substance may exist.

  - The circumstances of the use of certain materials or substances may give rise to particular risks based on their toxicological or physio-chemical properties.

  - Work involving the use of substances which must be labelled with the risk phrases R45 “may cause cancer”, R49 “may cause cancer by inhalation" and where a risk assessment in accordance with

Regulation 4 (b) of the Safety, Health and Welfare at Work (Carcinogens) Regulations 2001 (SI No. 78 of 2001) as amended by the Safety, Health and Welfare at Work (Carcinogens)

(Amendment) Regulations 2015 (SI No. 622 of 2015) reveals a risk to safety or health.

  - Work involving the use of substances or preparations, which must be labelled with the risk phrase R42 “may cause sensitisation by inhalation” and where a risk assessment in accordance with Regulation 4 of the Safety, Health and Welfare at Work (Chemical

Agents) Regulations 2001 (SI No. 619 of 2001) as amended by the Safety, Health and Welfare at Work (Chemical Agents (Amendment) Regulations 2015 (SI No. 623 of 2015) reveals a risk to health.  Also refer to the "Code of Practice for the Chemical Agents Regulation" published by the HSA.

  - The presence or potential presence of explosive atmospheres (including the potential for dust explosions).

Materials Safety Data Sheets are to be maintained on site.

Proper storage facilities are to be provided and maintained for all Chemical and Biological Substances as per the Regulations and manufacturer's requirements.

- Work with ionising radiation requiring the designation of

controlled or supervised areas as defined in Directive

96/29/Euratom

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

  - Risks associated from the use of a Nuclear Density Meter. These are usually used for measurement of compaction of materials in road construction. Safe work method to be put in place in line with the manufacturer’s guidelines and the most current safety practices if using a Nuclear Density Meter. All operators to be fully trained in the proper use of the machine and the precautions to be undertaken.

- Work near high voltage power lines

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

  - Risk of contact with uncharted underground high voltage power lines;

  - Risk of contact with existing overhead high voltage power lines;

  - Risk of electrocution during installation of services;

  - Risk of electrocution at high voltage cables supplying fixed plant where a significant risk of breaching the cable insulation exists.

There are underground electricity cables in the vicinity of the site.

The PSDP recommends that for all excavation works the Contractor adhere to the “Code of Practice for Avoiding Danger from Underground Services” as issued by the Health and Safety Authority (HSA) and all works in close proximity to overhead cables shall be carried out in accordance with the “Code of Practice for Avoiding Danger from Overhead Electricity Lines” as issued by ESB Networks.

The Contractor is responsible for confirming the existence, nature and location of all existing services which could be potentially affected by the proposed works and upholding same. The location of the services is subject to confirmation on the site by safe methods.

The Contractor is to ensure a permit to work system is in place when working in close proximity to high voltage electrical cables, to mitigate against the risks mentioned above.

- Work exposing persons at work to the risk of drowning

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

  - Risk of water following heavy rainfall filling up excavations, trenches creating a drowning hazard;

  - Work over or near water where there is a risk of persons falling into the water and drowning;

  - Work carried out significantly below the water table;

  - Work on or below bridges;

  - Work in culverts;

  - Work on or near the sea, lakes, ponds, reservoirs;

  - Work where there is a risk of falling into and drowning in product, vats, slurry tanks or grain silos;

The Contractor is to ensure good site drainage during the works and have dewatering pumps available if necessary. Any existing watercourses or drowning hazards are to be fenced off from the works area.

- Work on wells, underground earthworks and tunnels

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

- Work carried out by divers at work having a system of air supply

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

- Work carried out in a caisson with a compressed-air

atmosphere

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

- Work involving the use of explosives

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

  - Rock blasting

  - Demolition

- Work involving the assembly or dismantling of heavy prefabricated components

| Risk anticipated by PSDP at site |  |
| --- | --- |
| YES |  |
| Hazard description | - Risk of injury while moving and installing doors and/or windows. - Installing and/or removing of shower trays / shower doors / toilets / sinks / baths and all other associated sanitary ware. - Fitting of roof slabs and joisting |
| Proposed controls | - The Contractor is to ensure safe work methods are in place with particular reference to providing lifting equipment and a detailed lift plan for handling heavy prefabricated components. - All persons carrying out such works must have carried out manual handling training and machinery will be operated only by those qualified to do so. Mechanical aids should be used where required and a safe system of works must be implemented at all times. - It is considered that these risks should be capable of safe management and control by a competent contractor using safe systems of work and the appropriate levels of resources and equipment. |

Details:

  - Risks associated with assembling of prefabricated materials such as roof trusses;

  - Risks associated with installation of a wastewater treatment unit

  - Risks associated with the dismantling of prefabricated materials such as roof trusses, canopies, doors & windows etc.

The Contractor is to ensure safe work methods are in place with particular reference to providing lifting equipment and a detailed lift plan for handling heavy prefabricated components.

All persons carrying out such works must have carried out manual handling training and machinery will be operated only by those qualified to do so. Mechanical aids should be used where required and a safe system of works must be implemented at all times.

The risks below are additional to those identified above in accordance with Schedule 1 of the Safety Health and Welfare (Construction) Regulations 2013.

- Work exposing persons to asbestos

| Risk anticipated by PSDP at site |  |
| --- | --- |
| NO |  |
| Hazard description |  |
| Proposed controls |  |

Details:

Work which could lead to possible inhalation of asbestos dust. This would include:

  - The removal of material, which contains or is likely to contain asbestos where the “exposure limit value” as defined in the Safety, Health and Welfare at Work (Exposure to Asbestos) Regulations

2006 (SI No. 386 of 2006) as amended by the Safety, Health and Welfare at Work (Exposure to Asbestos) (Amendment) Regulations 2010 (SI No. 589 of 2010) is likely to be reached or exceeded.

  - The breaking or mechanical drilling and sawing of asbestos cement products where the above mentioned “exposure limit value” is likely to be reached or exceeded. This is more likely to occur when removing old or existing asbestos cement products rather than installing such products when new.

Where asbestos is present, it shall only be removed by persons competent to do so and shall be disposed of in accordance with the Regulations.

Asbestos may be present in materials such as roof sheeting, drainage pipes, insulation, floor or ceiling tiles, lagging products etc.

- Work exposing persons to lead

| Risk anticipated by PSDP at site |  |
| --- | --- |
| no |  |
| Hazard description |  |
| Proposed controls |  |

Details:

Any interaction with lead which could result in possible injury to persons at work. This would include:

  - Demolition involving the flame cutting of painted steel where the paint is likely to contain lead and could result in exceeding of the limits listed in Schedules 1 and 2 of the Safety, Health and Welfare at Works (Chemical Agents) Regulations 2001 as amended by the Safety, Health and Welfare at Work (Chemical Agents (Amendment) Regulations 2015 (SI No. 623 of 2015).

Lead may be present in Roof Flashing, Water Pipes, Paint Products

- Work carried out in confined spaces

| Risk anticipated by PSDP at site |  |
| --- | --- |
| No |  |
| Hazard description | . |
| Proposed controls |  |

Details:

Work where the activity exposes persons to additional hazards. This would include the following areas or activities:

  - Where there is a possibility of a deficiency of oxygen.

  - Where there is the presence of dangerous gases or vapours

  - Where there are energy sources which can be activated from outside the confined space without the persons knowledge.

This applies to Manholes, Septic Tanks (Precast and Built), Sewer Pipes and other confined spaces.

- Work involving reversing mobile plant

| Risk anticipated by PSDP at site |  |
| --- | --- |
| Yes |  |
| Hazard description | Risk of injury due to contact with delivery vehicles |
| Proposed controls | Contractor to ensure all deliveries have assistance of a spotter on maneuvers on site Spotter |

Details:

The works will of necessity involve reversing plant. Due regard to be taken of construction personnel, road-users and members of the public in the vicinity of the site.

- Works involving interaction with road traffic

| Risk anticipated by PSDP at site |  |
| --- | --- |
| YES |  |
| Hazard description | Collision between construction traffic and traffic on public and private road resulting in material damage, injury or death |
| Proposed controls | Contractor to ensure all construction traffic exiting site does so with due care and attention |

Details:

Due regard shall be taken of the safety of road users and construction personnel. The roadway fronting the dwelling is a public road. The contractor is advised not to obstruct the roadway.

- Work exposing persons to underground services

| Risk anticipated by PSDP at site |  |
| --- | --- |
| YES |  |
| Hazard description | Risk of injury or death due to contact with uncharted services |
| Proposed controls | - Contractor to confirm location of known services before works begin. - Contractor to prepare method statement for excavation works in the vicinity of known services |

Details:

  - Risk of contact with uncharted services causing injury;

  - Risk of contact with uncharted electrical services causing electrocution;

The Contractor is to ensure that the locations of all existing services are identified prior to works being carried out in any area by way of such techniques as (but not limited to) test trenching by hand, ground radar, or non-invasive testing and to excavate with all due care and diligence. The Contractor shall also ensure that all project personnel are made aware of the presence of all known services.

The Contractor is reminded that this is a built-up area where there are many underground services potentially ranging from ESB Networks, Gas Networks Ireland, Eir and Irish Water in the vicinity of the site.

- Covid 19

| Risk anticipated by PSDP at site |  |
| --- | --- |
| YES |  |
| Hazard description | Risk of infection from Covid 19. |
| Proposed controls | - Contractor to confirm measures being taken to minimise possibility of infection of site personnel or members of the public. - Contractor to comply with guidance issued by health authorities and CIF in relation to Covid 19 in the workplace. 2. Contractor to implement above measures and inform Employer if any site personnel exhibit symptoms of Covid 19. |

Details:

  - Risk of infection of site personnel;

  - Risk of infection of members of public;

The Contractor is to put in place measures and controls to minimise the possibility of infection arising from Covid 19.

The Contractor shall notify the Employer and relevant health authorities if there are any cases of Covid 19 among site personnel or members of the public within the vicinity of the site.

# General Principles of Prevention

The following General Principles of Prevention will apply at all times

General principles of are:

- The works have been designed taking into account the general principles of prevention.

- Where it has not been possible to eliminate a hazard, it has been either mitigated in design or is considered capable of mitigation in execution so as to render the works capable of safe management and control by a competent contractor using safe systems of work and the appropriate levels of resources and equipment.

- There will be an ongoing requirement for co-ordination between the designers, the PSDP, the PSCS and the Contractor (as appropriate) to deal with issues such as design changes, temporary works design, work permits, adherence to site rules and so on.

- The actual construction sequencing, which is in the control of the Contractor/PSCS, will also have to take into account the general principles of prevention.

- Similarly, the entire construction process including methodologies, sequencing, site co-ordination, use of sub-contractors, traffic management, co-ordination of the Works, implementation of temporary speed limits, phasing of the Works, temporary works design and the construction stage safety and health plan will all have to take into account the general principles of prevention.

# The location of Electricity, Water and Sewage Connections, where appropriate, to facilitate adequate Welfare Facilities

The Contractor is to ensure that the integrity of any site services is maintained and is to liaise with ESB, Eir and Irish Water  to arrange any connections required to his site compound.

The Contractor shall identify and confirm the location of all existing services prior to commencing work. Particular care should be exercised at all times during excavation for and construction of the works.

# Continuing Liaison

In the course of the works, there may be additional Design work carried out by any of the Designers in the project.

Regulation 15 states the following duties for all Designers:

(3) In carrying out work related to the design of a particular project, a designer shall promptly provide in writing to the project supervisor for the design process or for the construction stage, whichever is appropriate, all information—

- about the project that is known to the designer regarding particular risks to the safety, health and welfare of persons at work, including but not limited to the risks referred to in Schedule 1,

- Regarding the nature and scope of the project to the extent necessary to enable the project supervisor to comply with these Regulations,

- About the project that is necessary for that project supervisor to prepare the safety file, and

- that is known to that person and is necessary to ensure, so far as is reasonably practicable, the safe construction of the design for the project.

Therefore it is a standing duty of all designers to provide the PSDP and/or the PSCS (whichever is appropriate) with the information necessary to enable them to comply with the Construction Regulations.

Particular attention is hereby drawn to the requirement of the Designers of Temporary Works to co-operate with the PSDP and provide the PSDP with all of the information required to allow them to carry out their duties under the Regulations.

If the necessity for a significant design change arises during the construction phase, the respective designer will carry out their duties in accordance with Regulation 15 and pass on all the necessary information to the PSDP/PSCS in a timely manner, to allow them to carry out their duties in accordance with the Construction Regulations.

# Safety File

It is the responsibility of the PSDP to assemble the completed Safety File.

The Regulations require the Designers, Contractors and the Project Supervisor for the Construction Stage to co-operate with and provide information to the PSDP to enable the Safety File to be prepared.

The file must be reviewed and adjusted during the progress of the works to take account of any changes that have occurred.

The following information will be included in the Safety File by the PSDP where relevant:

Smaller Projects (e.g. bathroom adaptation, rewiring of house):

- Electrical certificate

- Boiler passport

- Certification for stove installation

Larger Projects (e.g. extensions or schemes for etenders):

- As-built drawings where applicable;

- Technical data sheets/O&M manuals for all new elements such as but not limited to:

- New oil/gas boilers, passports

- All types of heating systems

- Pumps

- All types of percolation systems

- Alarms

- Mechanical vents

- Insulation

- New windows/doors

- Kitchen units

- Paints

- Electrical/Pump showers

- Electrical certificate

- Boiler passport

- Certification for stove installation

- Details of the location and nature of utilities and services

- Disposal records for materials removed by the Contractor during the works;

- Any residual hazards (maintenance or future alteration) which may remain on site after completion of works.

The relevant parties (Contractor/ PSCS/ Designers, etc.) should note that they will be required to provide all of the above information in a timely and professional manner in electronic format.

The PSCS will note his duties as listed in the Regulations as follows:

Duties of the project supervisor for the construction stage, safety file.

21. (1) The project supervisor for the construction stage of a project shall –

- coordinate arrangements among contractors to ensure the provision of relevant information, in writing, necessary for the project supervisor for the design process to complete the safety file referred to in Regulation 13, monitor the implementation of the arrangements and take any necessary corrective action, as set out in Regulation 20, and

- provide in writing to the project supervisor for the design process all relevant information necessary for that project supervisor to complete the safety file referred to in Regulation 13.

The PSCS will comply with the requirements as noted above.

Preliminary Safety & Health Plan prepared by:

Signe

d

:

__james Brennan_____

Date:                02/06/2026

# Project Supervisor Design Process

For and on behalf of CORLANN (Project Supervisor for the Design Process

James Brennan

Signed:  ___james brennan____________________

02/06/ 2026

Date: