OPEN PROCEUDRE

REQUEST FOR TENDER FOR A CONTRACT

| Scope of Contract |  |
| --- | --- |
| Supply of Battery Operated RTC Equipment for Fire Service Use |  |
| Procedure |  |
| Open Procedure – National Level |  |
| Key Dates |  |
| Issue Date | 8-6-2026 |
| Closing Date for Queries | 16:00, 27-7-2026 |
| Closing Date for Tender Submissions | 16:00, 31-7-2026 |
| Contact for Queries |  |
| Questions and Answers facility on www.etenders.gov.ie |  |
| Format for submission of tenders |  |
| Via www.etenders.gov.ie only |  |
| Please note that information relating to this Request for Tender, including clarifications and changes, will be published on the Irish Government Procurement Opportunities Portal www.etenders.gov.ie. Registration is free of charge and there is no charge for documents. Please note that the Contracting Authority accepts no responsibility for information relayed (or not relayed) via third parties. |  |

CONTENTS

1.	ABOUT THE CONTRACTING AUTHORITY	3

1.1	The Contracting Authority	3

1.2	Small and Medium Enterprise Participation	3

2.	SCOPE OF THE CONTRACT	4

2.1	Overview of the Requirement	4

2.2	Details of Options	4

2.3	Anticipated Timeline	4

2.4	Termination of Contract	4

3.	DETAILED SPECIFICATION OF REQUIREMENTS	5

A 1.3.2 Tool Requirements	6

A 1.3.3 Battery Requirements	9

A 1.3.4 Tool Specifications	10

A 1.5 Diagnostics Equipment	15

3.8.5 Delivery Lead-Time	15

3.9	Duration	16

3.10	Indicative budget	16

3.11	Contract Management	16

3.12	Compliance with the Terms and Conditions	16

3.13	Award to Runner Up	16

4.	SELECTION CRITERIA	18

4.1	 Relying on the Standing of Other Entities	18

4.2	General, Legal and Financial Requirements	18

4.3	Technical Capacity Requirements	19

5.	RFT AWARD CRITERIA	21

5.1	Methodology for Calculating the Cost Score	26

5.2	Methodology for Calculating Scoring of Qualitative Criteria	26

5.3	Clarification / Verification Meetings	27

5.4	Clarification of Abnormally Low Tenders	27

INSTRUCTIONS FOR TENDERERS	28

GENERAL INFORMATION RELEVANT TO SUCCESSFUL TENDERERS	32

# 1.	ABOUT THE CONTRACTING AUTHORITY

## 1.1	The Contracting Authority

Cork County Council, herein after referred to as the Contracting Authority, is the authority responsible for this procurement.

Further information is available at our corporate website www.corkcoco.ie

## 1.2	Small and Medium Enterprise Participation

It is the policy of the Contracting Authority to promote participation by Small and Medium Enterprises (SMEs) on a fair and equal basis.

SMEs are encouraged to explore the possibilities of forming relationships with other SMEs or with larger enterprises to meet the financial, economic or technical capacity requirements of the competition, if required.

Tenderers may include individuals, partnerships, limited companies, groupings or any combination of the foregoing with or without legal personality. However, a grouping if successful will be required to establish legal personality to enter the contract.

Tenderers are reminded that they may rely on the resources of other entities to establish the requirements on condition that they can prove to the satisfaction of the Contracting Authority that they will have these resources at their disposal when necessary.

If the tender is from a consortium / joint venture, Tenderers must ensure that all the relevant information is provided and where necessary, provide the information requested separately for each party.  Relevant information relates to where a tenderer is relying on the resources to qualify (e.g. turnover, manpower, previous experience) and/or to deliver the contract.   The consortium must appoint a single point of contact who will assume overall responsibility for delivery, and who is authorised to sign the contract on behalf of all consortia members. The Contracting Authority will not act as an arbitrator between members of consortia.

# 2.	SCOPE OF THE CONTRACT

## 2.1	Overview of the Requirement

Cork County Council is seeking tenders for the supply of the following  Battery RTC

The Contracting Authority reserves the right not to award a contract as a result of this competition.

Tenderers must satisfy the Contracting Authority that have at least 3 years experience as a supplier of Fire Service Extrication or RTC  Equipment . Tenderers must provide evidence of the successful performance of at least 3 contracts over the last 3 years.

## 2.2	Details of Options

There are no options on this tender

## 2.3	Anticipated Timeline

The following indicative timeline is envisaged for this procurement:

| Issue RFT | 8-6-2026 |
| --- | --- |
| Closing date for Queries | 16:00 27-7-2026 |
| Closing date for Receipt of Tenders | 16:00 31-7-2026 |
| Clarification meetings (if anticipated) | None Anticipated |
| Award decision | Anticipated August 2026 |
| Contract Commencement | Anticipated September 2026 |

The dates provided above are estimates at the time of publication of the Request for Tender.   The Contracting Authority will endeavor to run the process to this timetable, but this cannot be guaranteed.

## 2.4	Termination of Contract

The Contracting Authority reserves the right at its sole discretion to terminate any contract where, due to matters outside its control, including but not limited to, increased costs arising from any changes in the Customs Union, which render the commercial arrangement uncompetitive.

# 3.	DETAILED SPECIFICATION OF REQUIREMENTS

The Contracting Authority proposes to engage in a competitive process for the award of a contract as specified in accordance with the Appendix 1 “Supply of Battery Operated Rescue Tools for Fire Service Use -Requirements and Specification” as outlined below

Requirement

Warranty: The tools and ancillary equipment including batteries shall have a minimum warranty of 2 years.

Samples for Field Trials: The tenderer shall, upon request, provide a sample of each rescue tool being tendered. These samples must be: - Supplied within 15 working days of the request, and - Made available for a minimum period of 10 working days to facilitate field trials and operational evaluation.

Documentation: A full set of documentation shall be provided to include as a minimum: - Operating and safety manuals; - Cleaning, servicing, maintenance, and repair instructions; and - Relevant risk assessments. This documentation must be supplied in either Irish or English, or both, and may be provided in hard copy or electronic format.

Certification: The tenderer shall provide certification documentation with the tender demonstrating the equipment complies with EN 13204:2025 (and any subsequent amendments).

Requirement

Training: The tenderer shall provide full details on how training will be delivered at both station level and for Brigade maintenance personnel. A minimum of a half-day training session shall be provided for all operational personnel at each of 20 stations. Additionally, a minimum of a half-day inspection and light maintenance training shall be provided for Brigade maintenance personnel. All costs associated with the training including labour, travel, and subsistence must be included in the overall supply of the equipment and shall not incur any additional charges.

Parts & Servicing: The tenderer shall provide a detailed breakdown of all annual and multiannual replacement parts required for servicing, as recommended by the manufacturer. The tenderer shall include the current cost of these parts, along with the estimated total cost of annual servicing over a four-year period. This estimate must include all associated costs, including labour, travel, and subsistence, for servicing the tools at the stations where they will located – details to be provided.

### A 1.3.2 Tool Requirements

Requirement

Operating Life: All tools and ancillary equipment must have a minimum operating life of 15 years.

Requirement

Ergonomics: The tools shall be designed to deliver a high power-to-weight ratio, ensuring ease of manual handling and operation by a diverse workforce. The tools must incorporate: - Ergonomically designed handles that allow for secure grip and control in multiple operating positions; - Capability of operation by one or two personnel; - Usability across a range of working heights, including: - Calf/knee level – e.g. car sill level; - Waist level – e.g. base of car at A and C post level; and - Chest level – e.g. top of car at A, B, and C post level. These design features shall facilitate efficient and safe operation during extrication tasks in varied and challenging environments.

CE Marking: All tools and ancillary equipment must be individually CE marked.

Serial Number: All tools and ancillary equipment, including batteries, must be individually marked with a unique traceable serial number.

LED lighting: All tools must have integral LED lighting to facilitate use in restricted light conditions with an on/ off switch, the lights must be powered by the main battery.

Requirement

Control Handle and Actuator: The rescue tools shall be equipped with a manual control actuator, which must: - Be easily accessible in all operating positions; and - Feature an automatic return to neutral position for enhanced safety and control. The control handle shall: - Have a minimum length of 75 mm to ensure a full and secure grip for the operator’s hand; - Be designed for operation from all angles by a single operator, including in confined spaces such as those encountered inside vehicles during road traffic collisions; and - Support use in varied body positions and orientations, ensuring ergonomic and safe handling in challenging environments.

IP Ratings: Each tool and its associated battery must be suitable for underwater use and meet the following minimum Ingress Protection (IP) ratings: - Tools: Minimum protection rating of IP57, ensuring protection against dust ingress and temporary immersion in water. - Batteries: Minimum protection rating of IP54, ensuring protection against limited dust ingress and water spray from any direction.

Noise: Under full load conditions, the noise emission levels of each tool shall not exceed: - 75 dB(A) at a distance of 1 metre; and - 66 dB(A) at a distance of 4 metres

Requirement

Auto Stop/Start: Each tool shall be equipped with an auto start/stop function to optimise battery efficiency, to reduce wear on components and support efficient and operational safety. The motor/drive system must automatically stop when: - The tool is not in use, or - The tool has reached its maximum operating pressure. The tool must automatically restart when the control handle is turned again, allowing seamless reactivation by the operator without additional steps.

### A 1.3.3 Battery Requirements

Requirement

Requirement

Continuous Charging: The equipment must be designed to remain on continuous charge without risk of damage to either the battery or the charger.

Supply Voltage: Battery chargers must be available for use with a standard mains socket supply (220–240V AC). Chargers must provide clear indication of the battery’s state of health both visually on the battery unit, and via diagnostic software, enabling detailed performance monitoring and maintenance planning.

Requirement

On-board charging: The on-board charger shall meet the following criteria: - Operate at 24V DC; - Include a battery level indicator for real-time monitoring; - Be supplied with all necessary ancillary equipment, including a sufficient length of charging cable; - Be suitable for mounting within a fire appliance; - Be capable of charging a minimum of one battery; - Include suitable inline surge protection fuses that are easily accessible for replacement, or alternatively, be fitted with a resettable circuit breaker that is easily accessible for resetting.

Availability: Replacement batteries must be available for 15 years.

### A 1.3.4 Tool Specifications

#### A 1.3.4.1 Cutter – Type A

Requirement

Opening: The cutter must have a minimum jaw opening of 180mm, as classified under EN 13204:2025 (or any amendments to same)

Performance: The rescue cutter must meet a minimum cutting performance classification of 1K-2K-3K-4K-5K in accordance with EN 13204:2025 (or any amendments to same).

Weight: The cutter, including the connected battery, shall have a maximum total weight of 25.0 kg

Requirement

General: The design and materials used in the manufacture of the cutting blades must ensure optimum cutting performance is maintained over repeated use, while also providing high durability and resistance to wear. Blades should be engineered to withstand the demands of frequent operational deployment without significant loss of effectiveness.

#### A 1.3.4.2 Cutter – Type B

Requirement

Opening: The cutter must have a minimum jaw opening of 200mm, as classified under EN 13204:2025 (or any amendments to same)

Performance: The rescue cutter must meet a minimum cutting performance classification of 1K-2K-3K-4K-5K in accordance with EN 13204:2025 (or any amendments to same).

Weight: The cutter, including the connected battery, shall have a maximum total weight of 28.0 kg

General: The design and materials used in the manufacture of the cutting blades must ensure optimum cutting performance is maintained over repeated use, while also providing high durability and resistance to wear. Blades should be engineered to withstand the demands of frequent operational deployment without significant loss of effectiveness.

#### A 1.3.4.3 Spreader – Type A

Requirement

Opening: The spreader must have a minimum opening width of 725mm, as classified under EN 13204:2025 (or any amendments to same)

Performance: The spreader must meet or exceed the following performance criteria, in accordance with EN 13204:2025 (or any amendments to same): - Spreading Force: Greater than 40 kN, measured 25 mm from the tips and greater than 320 kN at the steel tips; - Pulling Force: Greater than 50 kN; - Pulling Distance: Greater than 610 mm; and - Squeezing Force: Greater than 100 kN.

Weight: The spreader, including the connected battery, shall have a maximum total weight of 22.0 kg

General: The spreader must be designed to meet the following operational and functional criteria: - Accessory Compatibility: The tool must be designed to allow for the secure attachment and use of accessories such as chains or other approved fittings, enhancing its versatility in rescue operations. - Tip Grip Performance: The spreader tips must be engineered to provide effective grip on a wide range of surfaces during spreading operations, ensuring stability and control under load. - Tip Profile for Access: The tip design must be suitably fine and tapered to allow insertion into narrow gaps enabling effective initial access in confined or obstructed spaces.

#### A 1.3.4.4 Spreader – Type B

Requirement

Opening: The spreader must have a minimum opening width of 800mm, as classified under EN 13204:2025 (or any amendments to same)

Performance: The spreader must meet or exceed the following performance criteria, in accordance with EN 13204:2025 (or any amendments to same): - Spreading Force: Greater than 60 kN, measured 25 mm from the tips and greater than 450 kN at the steel tips; - Pulling Force: Greater than 60 kN; - Pulling Distance: Greater than 600 mm; and - Squeezing Force: Greater than 110 kN.

Weight: The spreader, including the connected battery, shall have a maximum total weight of 25.0 kg

General: The spreader must be designed to meet the following operational and functional criteria: - Accessory Compatibility: The tool must be designed to allow for the secure attachment and use of accessories such as chains or other approved fittings, enhancing its versatility in rescue operations. - Tip Grip Performance: The spreader tips must be engineered to provide effective grip on a wide range of surfaces during spreading operations, ensuring stability and control under load. - Tip Profile for Access: The tip design must be suitably fine and tapered to allow insertion into narrow gaps enabling effective initial access in confined or obstructed spaces.

A1.3.4.5 Combination Tool

Requirement

Opening: Must have a minimum spreader opening of greater than 350 mm and a minimum cutting opening greater than 300mm under EN 13204:2025 (or any amendments to same)

Performance: The combination tool shall meet the following minimum performance criteria in accordance with EN 13204:2025 (or any amendments to same): - Spreading Force: Greater than 40 kN, measured 25 mm from the tips. - Cutting Force: The combination tool must achieve a minimum cutting performance classification of Class ‘J’ in accordance with EN 13204:2025 (or any amendments to same). - Squeezing Force: Greater than 75 kN.

Weight: The combination tool, including the connected battery, shall have a maximum total weight of 25.0 kg

#### A 1.3.4.6 Telescopic Ram

Requirement

Stroke and Dimensions: The ram shall meet the following minimum performance criteria in accordance with EN 13204:2025 (or any amendments to same): - Total stroke: Greater than 700 mm - Retracted length: Less than 600 mm and less than the maximum opening of the proposed spreader. - Extended length: Greater than 1200 mm (This may be achieved using an extension poles.)

Requirement

Performance: The ram shall meet the following minimum performance criteria in accordance with EN 13204:2025 (or any amendments to same): - Spreading Force: - First stage: Minimum 100 kN - Second stage: Minimum 60 kN

General: The ram must be designed for safe and effective use in conjunction with compatible accessories, including but not limited to ram supports and sill clamps.

Weight: The ram including the connected battery, shall have a maximum total weight of 22.0 kg

#### A 1.3.4.7 Mini Cutter

Requirement

Opening: Must have a minimum cutter opening of greater than 50 mm under EN 13204:2025 (or any amendments to same).

Performance: The cutter shall meet the following minimum performance criteria in accordance with EN 13204:2025 (or any amendments to same): - A minimum cutting performance classification of 1C-2C-3B in accordance with EN 13204:2025 (or any amendments to same).

Accessories: The Mini Cutter must be supplied with a suitable battery, mains charger and charging lead if battery powered.

Requirement

Weight: The combination tool, including any connected battery / drive unit, shall have a maximum total weight of 12.0 kg.

Battery Charge: If battery powered, the battery must be capable of at least 50 cuts.

#### A 1.3.4.8 Ram Support

Requirement

Performance: Each ram support must be specifically designed to maximise and evenly distribute the load exerted by the ram, particularly during cross-ramming techniques.

Size: Each ram support shall have a maximum footprint of 500 mm x 500 mm with a maximum height/depth of 200 mm.

Weight: Each ram support should weigh no more than 14.0 kg

Gripping surface: Each ram support must be fitted with a suitable gripping surface to ensure secure anchoring and stability during operation.

Material & Finishing: Each ram support must be manufactured from a suitable high-strength metal or alloy to ensure structural integrity under load and treated for corrosion resistance, preferably by zinc coating. If painting is used as the protective method, the supports must be rust-proofed prior to painting to ensure long-term durability.

## A 1.5 Diagnostics Equipment

The tenderer shall provide a quotation for the supply of a diagnostic system via software that enables maintenance personnel to monitor and assess the operational status of the battery and associated tools. The diagnostic software shall meet the following minimum requirements:

- Provide detailed information on the battery’s operating state, including:

  - The number of charge cycles completed;

  - The state of health of the battery; and

  - Any potential faults or anomalies detected.

- Offer diagnostic capabilities for the tools themselves, including:

  - A health status report on tool performance and operational capability; and

  - Integration with the annual service process to assess tool condition

- Generate a written or electronic record of each tool’s performance and status following its annual service: and

- Ensure the Brigade:

  - Has direct access to these service records, or

  - Is provided with the records annually by the supplier or service provider.

The cost of the diagnostic software and any associated licensing or access fees shall be clearly itemised in the quotation

## 3.8.5 Delivery Lead-Time

Tenderers must state their delivery lead time (in weeks) for a complete solution for each chassis option.

The lead time shall be taken as the period from the date of the official order placement to the delivery on of the equipment.

State the delivery lead times (in weeks) in the Tender Response Document

## 3.9	Duration

The contract will be for a period of 1 year.  The works must be complete within 10 weeks of placement of an official order.

## 3.10	Indicative budget

The indicative budget for the contract is €800,000. exc VAT

## 3.11	Contract Management

The Contracting Authority requires tenderers to nominate a dedicated contract manager who will act as the main point of contact for the duration of the contract.  This person shall have the authority to deal with all matters in relation to contracts and be responsible for the satisfactory delivery of the supplies/services required.  The duties of the contract manager will include the following:

•	Overall responsibility for a good working relationship with the Contracting Authority;

•	Provide regular reports on performance as agreed with the Contracting Authority;

•	Meet as and when required to review and examine performance;

•	Deal with disputes, complaints or concerns that cannot be adequately resolved;

•	Proactively discuss with the Contracting Authority ways of improving efficiency regarding service delivery in general and providing suggestions for improvement and cost savings.

NOTE: Tenderers will note that contract management activities will be non-billable.

## 3.12	Compliance with the Terms and Conditions

Award of contract will be subject to the successful tenderer agreeing to the Contract Terms and Conditions as contained in Form of Tender in Tender Response Document.

## 3.13	Award to Runner Up

If for any reason, it is not possible to award the contract to the designated successful tenderer emerging from this competitive process, or if having awarded a contract, the successful tenderer fails to deliver the contract in accordance with the terms and conditions, the Contracting Authority reserves the right to award the contract to the next highest scoring tenderer based on the terms advertised at any time during the tender validity period. This shall be without prejudice to the right of the Contracting Authority to cancel this competitive process and/or initiate a new contract award procedure at its sole discretion.

# 4.	SELECTION CRITERIA

The Contracting Authority is using the open procedure for the award of this contract, therefore, while all interested parties may submit a tender, only those demonstrating that they have the required level of financial and technical capacity will have their tender considered. In order to demonstrate a tenderers’ qualifications, tenderers are required to provide the information set out below in the Tender Response Document (TRD) which is based on a self-declaration model, however tenderers are required to provide the minimum information required.

Alternatively, tenderers may have compiled a European Single Procurement Document (ESPD) which will be accepted as evidence of compliance with Section 4.1.1 (a).

## 4.1	 Relying on the Standing of Other Entities

Tenderers are reminded that they may rely on the resources of other entities to establish the requirements on condition that they can prove to the satisfaction of the Contracting Authority that they will have these resources at their disposal when necessary.

If the tender is from a consortium / joint venture Tenderers must ensure that all the relevant information is provided and where necessary, provide the information requested separately for each party. The consortium must appoint a single point of contact who will assume overall responsibility for delivery, and who is authorised to sign the contract on behalf of all consortia members. The Contracting Authority will not act as an arbitrator between members of consortia.

## 4.2	General, Legal and Financial Requirements

Tenderers are required to provide information on the following in the Tender Response Document.   The criteria and rules outlined below are assessed on a pass/fail basis.  Failure to comply with the requirements will result in your tender being considered inadmissible.

| General Information |  |
| --- | --- |
| Provide contact and general information on the tendering organisation - company name, address and contact details for individual responsible for this tender and company overview as well as information on sub-contractors and consortium members if applicable. |  |
| Declarations |  |
| - Complete the Declaration of Bona Fides as per Art. 57 of Directive 2014/24/EU as implemented by SI 2814 of May 2016 as contained in the Tender Response Document. - Complete the Declaration regarding compliance with relevant statutory obligations as contained in the Tender Response Document. Where tenderers are established and operating outside of Ireland compliance with equivalent legislation as applicable in the country of establishment / operation is required. |  |
| Financial |  |
| Tax | Confirmation that the tenderer / all parties associated with the tenderer are fully tax compliant in accordance with the rules of the Irish Revenue Commissioners. |
| Turnover | Confirmation that the tendering party turnover exceeded €2,000,000 during one of the last three years or pro-rata if more recently established firms are tendering – however the firm must have been in existence for at least 12 months. |
| Insurance | Confirmation of the following insurances being in place: - Employer’s Liability - €13 million - Public Liability - €6.5 million - Product Liability - €6.5 million - Professional Indemnity - €1.0 million |

## 4.3	Technical Capacity Requirements

Tenderers are required to provide information on the following in the Tender Response Document.   The criteria and rules outlined below are assessed on a pass/fail basis.  Failure to comply with the requirements will result in your tender being considered inadmissible.

| Manpower Levels |  |
| --- | --- |
| Tenderers must provide information which demonstrates access to the minimum number of skilled personnel as indicated below and outlined in the TRD. |  |
| Skillset Required | Minimum Number |
| Trained and certified Skilled Labour | 2 |
| Technical Resources |  |
| Tenderers must provide information which demonstrates access to the required level of technical resources as indicated below and outlined in the TRD. |  |
| Technical Resource Required | Minimum Requirement |
| Workshop, Garage Equipment & Electronics/ Diagnostics Equipment to maintain And repair the equipment supplied | Workshop, Garage Equipment & Electronics/ Diagnostics Equipment to maintain And repair the equipment supplied |
| Previous Contracts / Experience |  |
| Tenderers must provide information clearly demonstrating successful delivery of 3 previous comparable contracts / experience, involving the following features: |  |
| Provide details of a contract where a similar specified equipment was delivered |  |
| Health & Safety |  |
| Tenderers must provide information which demonstrates operation of health & safety systems and procedures in line with all relevant Safety Health & Welfare at Work legislation. Please complete the TRD. Evidence of compliance will be required as condition of contract award. |  |

# 5.	RFT AWARD CRITERIA

Only tenders which meet the Selection Criteria and are confirmed as valid and responsive to the specifications set out in this document will be evaluated against the award criteria. Tenderers should ensure that they have submitted sufficient relevant information to allow their tenders to be assessed under each of the award criteria set out below.

The contract will be awarded on the basis of the most economically advantageous compliant tender taking into account the following award criteria and weightings.

| Criterion A |  | Weighting | Maximum Marks | Minimum Marks Required |
| --- | --- | --- | --- | --- |
|  |  | Pass/ Fail | Pass | Pass |
| Title | Compliance with Technical Specifications and Minimum Warranty Requirements |  |  |  |
| Description | The Tenderer must detail in The Tender Response Document the compliance of the tendered equipment with the technical specifications and minimum warranty requirements set out in Appendix 1. Cork County Fire Service will review various proposed solutions along with variations and will determine if the solution proposed broadly meets their requirements as set out in the RFT. Only Tenders that receive a “Pass” mark for this award criterion will proceed to be evaluated under the remaining award criteria. Please provide all detail in the Tender Response Document |  |  |  |

| Criterion B |  | Weighting | Maximum Marks | Minimum Marks Required |
| --- | --- | --- | --- | --- |
|  |  | 50% | 500 | N/A |
| Title | THE COST CRITERION |  |  |  |
| Description | Cost for the providing Battery Operated RTC tools as per specification |  |  |  |

| Criterion C |  | Weighting | Maximum Marks | Minimum Marks – 50% |
| --- | --- | --- | --- | --- |
|  |  | 20% | 200 | 100 |
| Title | Field Trials |  |  |  |
| Description | Cork County Fire Service will carry out a practical assessment to test the operational effectiveness of the equipment. The methodology for this and the scoring system is outlined in Appendix 1 Requirements & Specification |  |  |  |

| Criterion D |  | Weighting | Maximum Marks | Minimum Marks – 50% |
| --- | --- | --- | --- | --- |
|  |  | 10% | 100 | 50 |
| Title | Aftersales Support & Technical Assistance |  |  |  |
| Description | The Tenderer must set out the quality of its proposal in relation to its capacity and capability for the provision of maintenance, repair and technical assistance from its maintenance facilities, and its capacity and capability for repair/maintenance services. Tenderers must provide details of maintenance facilities, if any, they have in the Republic of Ireland to carry out repairs. The Tenderer should also provide details of the process by which clients’ repair issues are managed from the time a fault is reported through to successful conclusion. If sub-contracting after sales support to another supplier, full details must be provided of that sub-contractor, including their maintenance facility details, network details, technical capabilities of their employees in relation to support and their qualifications for same, and details of stock they carry and where the stock is located. Please provide all detail in the Tender Response Document |  |  |  |
| Criterion E |  | Weighting | Maximum Marks | Minimum Marks – 50% |
|  |  | 10% | 100 | 50 |
| Title | Availability of Spare Parts |  |  |  |
| Description | The Tenderer must set out the quality of its proposal for ensuring the availability of all spare parts for the equipment and ancillary equipment for the minimum expected lifespan of the equipment (15 years). The Tenderer should include details of its process and lead times for obtaining spare parts; how it manages this supply chain to ensure parts are delivered on time; and how it manages instances of order delays and parts being out-of-stock. Please provide all detail in the Tender Response Document |  |  |  |
| Criterion F |  | Weighting | Maximum Marks | Minimum Marks – 50% |
|  |  | 5% | 50 | 25 |
| Title | Ordering & Delivery |  |  |  |
| Description | The Tenderer must provide a detailed narrative demonstrating its high-level process and timescales for the successful delivery of RTC Equipment - from the time of the official order through to full commissioning and handover of the appliance. The Tenderer should Include details of the tenderer’s methodology for ensuring that all appliances are supplied and delivered in accordance with the requirements specified by the client. Please provide all detail in the Tender Response Document |  |  |  |

| Criterion G |  | Weighting | Maximum Marks | Minimum Marks – 50% |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50 | 25 |
| Title | Environmental & Sustainabilitilty Efficiencies |  |  |  |
| Description | The Tenderer must provide a detailed narrative to demonstrate how it incorporates environmental and sustainability efficiencies/measures during the build process. For example: - Energy optimisation initiatives in the warehouse/premises including rain harvesting, use of PV Panels, fabric of the building including insulation, efficiency of lighting and heating, EV charging capabilities etc. - Use of recycled and/or recyclable material - Waste management/disposal/recycling - Minimisation of waste - Transport and logistics measures - Corporate Social Responsibility policies - Proposed Targets for Environmental Sustainability - Sustainable purchasing Please provide all detail in the Tender Response Document |  |  |  |

NOTE 1:	Tenderers should ensure in their tenders that they provide detailed information in respect of all aspects of the contract award criteria as stated above.  This will enable the awarding authority to assess fully the extent of their offers.

## 5.1	Methodology for Calculating the Cost Score

The following formula will be applied to the cost score:

The lowest cost tender that also meets all the minimum requirements of the qualitative award criteria will receive the maximum score achievable under this criterion.  The scores of the other valid tenders will be calculated using the following formula:

| Lowest Cost from a Bona Fide Tender | A |
| --- | --- |
| Cost for the tender being evaluated | B |
| Maximum Points available for Cost | 500 |
| Formula employed | 500 x A B |

## 5.2	Methodology for Calculating Scoring of Qualitative Criteria

| Score | Meaning | Interpretation |
| --- | --- | --- |
| 90 – 100% | Outstanding | A very comprehensive response demonstrating extensive understanding offering full assurance to client – fully supported with no reservations. |
| 80 – 89% | Excellent | An excellent response demonstrating excellent understanding offering assurance to client – strongly supported. |
| 70 – 79% | Very good | A very good response demonstrating very good understanding offering assurance to client – fully supported. |
| 60 – 69% | Good | A good response demonstrating good understanding offering assurance to client – well supported. |
| 50 – 59% | Acceptable | An acceptable response demonstrating a minimum understanding offering assurance to client - satisfactorily supported. |
| Less than 50% is unacceptable |  |  |
| 25 – 49% | Mediocre | Response demonstrates limited understanding with insufficient or no detail and a risk of non-delivery. This is unacceptable and a fail. |
| 1 – 24% | Poor | Response demonstrates very limited understanding of the requirements and has fundamental flaws and lacks credibility with a significant risk of non-delivery. This is unacceptable and a fail. |
| 0% | No response | Response completely fails to address the criterion under consideration. This is unacceptable and a fail. |

Marks in the score ranges outlined above can be awarded where responses so merit additional marks.

## 5.3	Clarification / Verification Meetings

Award of contract may be subject to attendance at a clarification and verification meeting. It would be essential that the key personnel assigned to this contract should be available and present at this meeting.    If required, tenderers will be notified of the date, time, agenda and format for such meetings as soon as possible.

A visit to the tenderer’s premises may be required to clarify any questions or queries regarding the tender offer.

Tenderers should note that the Contracting Authority reserves the right to confirm that the financial and technical capacity of the tenderer is valid and unchanged prior to the award of any contract.

## 5.4	Clarification of Abnormally Low Tenders

If the Contracting Authority considers the tender submission to be commercially unsustainable or otherwise problematic in light of the tendered price or any other financial matter (including proposed indicative hours), the tenderer shall be invited to provide clarification to the Contracting Authority in respect of all elements of the tender submission that the Contracting Authority deems relevant. Any failure to satisfactorily comply with such a request, or to satisfactorily address the Contracting Authority’s concerns, may, at the discretion of the Contracting Authority, result in the elimination of the tender in question on the basis of it being considered abnormally low.

# INSTRUCTIONS FOR TENDERERS

(a)	Submission of Tenders via www.etenders.gov.ie

The Contracting Authority is using the Tender Postbox facility and tenders must be submitted electronically via the etenders postbox facility on www.etenders.gov.ie only.  Only Tenders submitted to the electronic postbox will be accepted.  Tenders submitted by any other means (including but not limited to by email, fax, post or hand delivery) will not be accepted.

Tenderers must ensure that they give themselves sufficient time to upload and submit all required tender documentation before the Tender Deadline.  Tenderers should consider the fact that upload speeds vary.

To submit a document to the electronic postbox, please note that tenderers must click “Submit Response”.  After submitting tenderers can still modify and re-send their response up until the response deadline.  Tenderers should be aware that the ‘Submit Response’ button will be disabled automatically upon the expiration of the response deadline.

Tenderers not familiar with uploading on eTenders should ensure they familiarise themselves with the process prior to the submission deadline.

(b) 	Closing date for Tenders

| The closing date for tenders | is 16:00 31-7-2026 |
| --- | --- |

It is the responsibility of the tenderer to ensure that their tender is complete and is uploaded / submitted by the designated deadline.

(c)	Queries

All queries regarding this tender should be through the Questions and Answers facility on www.etenders.gov.ie, including any omissions which would prevent tenderers from submitting a comprehensive tender.  Please submit queries as soon as possible.

| The closing date for queries | is 16:00 27-7-2026 |
| --- | --- |
| Process for submitting queries | Via www.etenders.gov.ie only |

In circulating responses, queries will be edited to avoid disclosing the identity of the querist and will be circulated to all parties who have expressed an interest in the procurement on the eTenders website.

(d)	Extension of Tender Period

The Contracting Authority reserves the right, at its sole discretion, to extend the closing date for receipt of tenders by giving notice in writing (by post or electronic means) to all parties who have expressed an interest in the notice via eTenders no later than six days before the original closing date.

Tenderers will be responsible for any costs incurred by them in the event that they are required to attend clarification or other meetings or make a presentation of their proposals.

(e) 	Tender Validity Period

To allow sufficient time for Tender assessment a Tender Validity period of 12 months is required, this period commencing on the closing date by which the Tenders are to be returned.

(f)	Amendment of Tender Documentation

Tenderers are prohibited from amending any text or content of forms or declarations or templates provided as part of this tender competition in their tender responses.  Where amendments have been identified, the Contracting Authority may at its discretion eliminate the tenderer from further consideration.

(g)	Collusive Tendering

If any Tendering Party is found to have, at any time, offered to give or to have agreed to offer or give to any person, any bribe, gift, gratuity, commission or consideration of any kind as an inducement or reward for taking or forbearing to take any action in relation to the obtaining of its Tenders, or for showing or forbearing to show any favour or disfavour to any person in relation to its Tenders, the bid submitted by such Tendering Part shall be automatically disqualified and the circumstances surrounding such action shall be referred to the appropriate authority.

(h)	Confidentiality

The distribution of the tender documents is for the sole purpose of obtaining offers. The distribution does not grant permission or licence to use the documents for any other purpose. Tenderers are required to treat the details of all documents supplied in connection with the tender process as private and confidential.

(i) 	Conflict of Interest

Any conflict of interest involving a tenderer (or tenderers in the event of a consortium bid) must be fully disclosed to the Contracting Authority.  Any registrable interest involving the tenderer and the Contracting Authority or employees of the Contracting Authority or their relatives must be fully disclosed in the tender submission or should be communicated to the Contracting Authority immediately upon such information becoming known to the tenderer, in the event of this information only coming to their notice after the submission of a bid and prior to the award of the contract.  The terms ‘registrable interest' and 'relative' shall be interpreted as per Section 2 of the Ethics in Public Office Act, 1995.  Failure to disclose a conflict of interest may disqualify a tenderer or invalidate an award of contract, depending on when the conflict of interest comes to light.

(j) 	Anti-Competitive Conduct

Tenderers attention is drawn to the Competition Act 2002 (as amended, the “2002 Act”). The 2002 Act makes it a criminal offence for Tenderers to collude on prices or terms in a public procurement competition.

(k)	Freedom of Information Acts

Tenderers should be aware that, under the Freedom of Information Act 2014 and the European Communities (Access to Information on the Environment) Regulations 2007 to 2014, information provided by them during this Competition may be liable to be disclosed.

Tenderers are asked to consider if any of the information supplied by them in their Tender should not be disclosed because of its confidentiality or commercial sensitivity. If Tenderers consider that certain information is not to be disclosed because of its confidentiality or commercial sensitivity, Tenderers must, when providing such information, clearly identify the specific sections of their Tender containing such information and specify the reasons for its confidentiality or commercial sensitivity. For the avoidance of doubt Tenderers may not assert confidentiality or commercial sensitivity over the entire Tender but must clearly identify the specific section containing such information. If Tenderers do not identify information as confidential or commercially sensitive, it is liable to be released in response to a request under the above legislation without further notice to or consultation with the Tenderer. The Contracting Authority will, where possible, consult with Tenderers about confidential or commercially sensitive information so identified before making its decision on a request received. The Contracting Authority accepts no liability whatsoever in respect of any information provided which is subsequently released (irrespective of notification) or in respect of any consequential damage suffered as a result of such obligations.

(l)	Data Protection

“Data Protection Laws” means all applicable national and EU data protection laws, regulations and guidelines including but not limited to Regulation (EU) 2016/679 on the protection of natural persons with regard to the processing of personal data and on the free movement of such data, and repealing Directive 95/46/EC (the “General Data Protection Regulation”), the Data Protection Act, 2018 and any guidelines and codes of practice issued by the Data Protection Commission or other supervisory authority for data protection in Ireland from time to time.

The Contracting Authority will be a Controller (where Controller has the meaning given under the Data Protection Laws) in respect of any Personal Data (where Personal Data has the meaning given under the Data Protection Laws) required to be provided by the Tenderer in response to this Request for Tender.

The Tenderer, as Controller in respect of any Personal Data provided by it in its Tender, is required to confirm by way of statement in the “Declarations” section of the accompanying Tender Response Document that all Data Subjects (where Data Subject has the meaning given under the Data Protection Laws) whose Personal Data is provided by the Tenderer have consented to the processing of such Personal Data by the Tenderer, the Contracting Authority, the Evaluation Team and the supplier of the etenders.gov.ie website, for the purposes of the participation of the Tenderer in this Competition or that the Tenderer otherwise has a legal basis for providing such Personal Data to the Contracting Authority for the purposes of its participation in this Competition.

(m)	Publicity

Tenderers shall not undertake (or permit to be undertaken) at any time, whether at this stage or after the award of the contract, any publicity activity with any section of the media in relation to this tender/agreement other than with the prior written consent of the Contracting Authority.  Such consent shall extend to the content of any publicity.  For the purposes of this paragraph, the word “media” includes (but is not limited to) radio, television, newspapers, trade and specialist press, the Internet and e-mail accessible by the public at large and the representatives of such media.

The Contracting Authority will have the right to publicise or otherwise disclose to any third-party information regarding this process and the agreement.

(n)	Correction of Errors

Detailed pricing of all tenders will be examined for errors that might alter the tender pricing as determined from the figures on the tender form or as between the hard copy and electronic versions of the tender.  Where a discrepancy arises between any figure submitted on the pricing element of eTenders versus the content of the Tender Submission, the Tender Submission figures will be used in the assessment.

In the case of manifest errors - where there is a discrepancy between the unit price and the total amount derived from the multiplication of the unit price and the quantity, the unit price as quoted will normally govern.

The amount stated in the tender form will be adjusted by the Contracting Authority in accordance with the above procedure and, with the agreement of the tenderer, shall be considered as binding upon the tenderer. Without prejudice to the above, a tenderer not accepting the correction of their tender as outlined may have their tender rejected.

(o)	Notification of Tender Evaluations

All tenderers will be informed of the outcome of their tenders following tender evaluation and any necessary clarifications. The Contracting Authority will issue a Letter of Regret with the name of the winning tenderer(s) and the scores of the tenderer and the winning tenderer.

# GENERAL INFORMATION RELEVANT TO SUCCESSFUL TENDERERS

- Currency and Payments

The currency and invoices in which all prices and rates shall be tendered, and which payments under the contract will be paid, shall be Euros (€). All prices and rates quoted should be exclusive of VAT.

Invoices shall be submitted in accordance with the terms agreed with the Contracting Authority.

- Withholding Tax

Where applicable, payments shall be subject to Irish ‘Professional Services Withholding Tax’ at the prevailing rate (currently at 20%) as laid down by the Revenue Commissioners in Ireland. Non-residents may be able to reclaim such deducted Tax from the Office of the Revenue Commissioners in Ireland, International Claims Section located currently at Government Buildings, Nenagh, Co. Tipperary, Ireland (Tel: +353-67-63400).

- Irish Legislation and Law

Tenderers should be aware that national legislation applies in other matters such as Employment, Working Hours, Official Secrets, Data Protection and Health and Safety. Tenderers must have regard to statutory terms relating to minimum pay and to legally binding industrial or sectoral agreements in the Contracting Authority tenders and in delivering contracts awarded to them. The contract[s] awarded on foot of this tender process will be governed by Irish law.

- Dignity at Work

The successful tenderer(s) shall comply with all relevant legislation relating to dignity at work. As a public body and employer, the Contracting Authority is committed to a policy of equality of opportunity for all personnel.

In line with the Disability Act 2005, accessibility requirements should be clearly stated in request for tenders / quotations where applicable. Under Section 27 of the Act the Contracting Authority is required to ensure that both the goods supplied, and services provided to it are accessible to persons with disabilities.

- Change in Law

As a condition of award, it shall be the sole responsibility of the tenderer (in the event of success in this competition) to fulfill the obligations under the Contract, notwithstanding any changes in circulars, laws, regulations, taxation, duties or other factors which might arise following the withdrawal of the United Kingdom from membership of the EU.