Specification for Flood Rails – Wexford County Council.

Wexford County Council invite tenders for the supply of approximately 1,600 Meters of minimum 1000mm diameter Mono or Single rail system, flood rails in various sectional lengths as Follows:

- 120 Number 5M Lengths – Total length 600M

- 100 Number 10M Lengths - Total length 1000M

The flood rails shall typically be filled with air, manoeuvred into position and the air shall be replaced with water through a quick release valve. The units shall be so designed to enable rapid emptying and deflation of the units once no longer required. The units shall be equipped with carrying handles and tie off lugs.

The flood rails shall be of

- Polyester or similar type construction 1100 dtex, 900g/M²

- Suitable air inflation valve system

- Quick release valve compatible with fire service QR hose fittings for water filling.

- System for emptying the rails of water – large drain plug or zipper.

- System for interconnecting rails together in series.

- Repair/patch kit for each rail.

- Carrying handles

- Weight of each rail to be specified. (5M & 10M)

- Tie off points

- Protective carrier bag

- Rails to specified EN/CE standards.

Interested parties must provide a sample length of rail to the contracting authority for assessment as part of the tendering process. The sample will be checked for ease of use (deployment and recovery), quality, compliance with relevant codes and suitability for use as an emergency deployment system to Wexford County Council.

Figure 1 & 2: Example of Flood Rails in use at Wexford 2026.