# COMHAIRLE CONTAE LÚ

# LOUTH COUNTY COUNCIL

Bus Shelter & Bus Stop Civils contract 2026

PRELIMINARY HEALTH & SAFETY PLAN

DESIGN PROCESS

Prepared by: - Louth County Council

Date: - May 2026

PREAMBLE

1.	The attention of Tenderers is drawn to the obligations arising under the relevant clauses of the General Conditions of Contract consequent upon the Safety, Health and Welfare at Work Act 2005 and Safety, Health and Welfare at Work (Construction) Regulations 2013 ("the Regulations") and any amendment thereof.

2.	It is intended that for this contract the successful tenderer will be appointed to fulfil the duties of the Project Supervisor (Construction Stage) under the Regulations.

3.	The tenderer is to examine the Site and the information supplied regarding the nature and extent of the proposed works, so as to be satisfied as to what is required to be done to fulfil the duties of the Project Supervisor (Construction Stage) and also what measures are required to be taken to meet the requirements of the Regulations in the execution of the Works including compliance with the Health and Safety Plan.

Tenderers are required to include adequate provision in the overall tender sum for:-

i)	The execution of the project in compliance with all relevant statutory enactments and regulations including the safety and health code:

ii)	The acceptance of the role and duties of Project supervisor (Construction Stage). The Tenderer will be required at tender stage to nominate a competent person employed by them to fulfil the role of Project Supervisor (Construction Stage).

4.	It should be noted that where the Contractor or any Sub-contractors carry out any design (as defined in the Regulations) the duties set out in Regulation 15 of the Regulations apply.

- The competence of any tenderer to carry out the duties of the Project supervisor (Construction Stage) was considered at the time of establishing the framework.

The acceptance of a Tender will be subject to:-

a)	Submission of the proposed Safety Statements for the project

b)	Submission of the proposed Method Statements when requested.

c)	Confirmation that the Time Schedule is adequate to enable the safe construction of the project.

d)	Submission of the developed Safety Plan.

e)	Submission of a Traffic management Plan.

6.	The successful Tenderer will be required to appoint a Health and Safety Co-ordinator as envisaged by section 17(2) of the Regulations, to undertake on behalf of the Project Supervisor (Construction Stage) the duties specified in 16 to 22 inclusive of the Regulations. All subcontractors must comply with the Main Contractors Safety and Health Plan.

7.	Tenderers must demonstrate that their tender has adequately addressed Safety and Health issues in the project and in particular that they have adequately dealt with the risks specified in the Safety and Health Plan prepared on a preliminary basis for the project.

8.	A Time for Completion is proposed in the contract documents. If, in the opinion of the tenderer, having considered the project in relation to the Regulations, this time is not adequate, the tenderer must inform, in writing, the Senior Executive Engineer, Local Roads and Services Section, Louth County Council.

PRELIMINARY SAFETY AND HEALTH PLAN

- INTRODUCTION

This preliminary Safety and Health Plan has been prepared by the Project Supervisor for the Design Process in accordance with the Health, Safety and Welfare at Work (Construction) Regulations 2013.

## 2.0	GENERAL DESCRIPTION OF THE PROJECT

  - Title

The scheme shall be known as: Bus Shelter Civils Contract 2026

  - Client

Louth County Council,

County Hall,

St. Alphonsus Rd,

Dundalk,

Co. Louth.

2.3	Project Supervisor (Design Process)

Louth County Council,

County Hall,

St. Alphonsus Rd,

Dundalk,

Co. Louth

2.4	Site Locations (Bus Shelter’s)

The sites are all located in County Louth and they are as follows:

- Georges Street, Drogheda

- Crosslanes North, Drogheda

- Crosslanes South, Drogheda

- North Road West, Drogheda

- North Road East, Drogheda

- Mc Donalds, Dundalk

- Castlebellingham Southbound

- DKIT Dundalk

- Monasterboice

- Drogheda Hospital North

- Drogheda Hospital South

- Omeath Village

- Annaloughan

- Louth Hospital, Dundalk

Site Locations (Bus Stop’s)

The sites are all located in County Louth and they are as follows:

- Annagassan 1

- Annagassan 2

- Baltray

- Bush post primary

- Carlingford

- Clogherhead 1

- Clogherhead 2

- Dromiskin 1

- Dromiskin 2

- Dromiskin Cross

- Greenore

- Louth Village 1

- Louth Village 2

- Lordship

- Omeath Village

- Tallanstown Cross

- Tinure 1

- Tinure 2

- Termonfeckin 1

- Termonfeckin 2

- Tullyallen 1

- Tullyallen 2

- Riverstown

- Georges Street, Drogheda

- Crosslanes North, Drogheda

- Crosslanes South, Drogheda

- North Road West, Drogheda

- North Road East, Drogheda

- Mc Donalds, Dundalk

- Castlebellingham Southbound

- DKIT Dundalk

- Monasterboice

- Drogheda Hospital North

- Drogheda Hospital South

- Omeath Village

- Annaloughan

- Louth Hospital, Dundalk

These locations carry large volumes of residential and commercial traffic. There are many existing residential developments, retail units, churches, sports fields, sports buildings, schools, crèches, playgrounds, both primary and secondary schools and third level colleges located along these roads.

2.5	Project Description

The works shall consist of the following:

Excavate and place concrete foundation for Bus Shelter & Bus stop poles

Excavate and place concrete surround and transition area to Bus Shelter

Place Ducting for ESB supply to Bus Shelter.

- The erection of “Goalposts” if/where required adjacent to overhead wires.

Provision of kerbing, Kassel Kerbing and road markings in compliance with national standards if/where required.

Place Bus Stop, including retention socket and concrete surround and finish.

All signing, lighting & guarding at the roadworks in accordance with updated Traffic Signs Manual Chapter 8 “Temporary Traffic Measures & Signs for Roadworks”, 2010 as issued by the Department of Transport or any amendment thereof.

Working alongside live traffic.

An integral part of the project will be devising and implementing an effective site-specific traffic & pedestrian management programme. This programme will take cognisance of restrictions affecting the programming of the works.  These restrictions are to minimise traffic disruption in and adjacent to the proposed works areas. The traffic management programme must be in accordance with the Traffic Signs Manual Chapter 8 “Temporary Traffic Measures & Signs for Roadworks”, 2010 as issued by the Department of Transport or any amendment/update thereof. It is to be submitted with the tender documents.

The Contractor shall manage the works to minimise disruption to local businesses, schools and occupiers of dwellings. In particular, access shall be maintained to dwellings, schools, businesses and other premises. Temporary signage must be provided where necessary and the contractor is deemed to have provided for such in his rates.

The Contractor shall, in carrying out the works, comply with all National and EU protocols and Regulations. The attention of the Contractor is drawn to the requirement to comply with the Waste Management Act 1996 – 2003 or any amendment thereof.

The contractor must comply with Louth County Council’s Environmental Policy.

  - Drawings and Specifications

Drawings & Design Sketches are contained in the contract documents. Construction details are to be referred to where required.

  - Site Restrictions

The methodology of construction shall be in accordance with the specified constraints set out in the Specification, unless otherwise approved, which forms part of this Preliminary Safety & Health Plan.

Work areas are restricted by traffic on the various public roadways, private and business entrances all of which must be kept open to traffic. No item of plant or equipment (including stores or offices) shall be placed or parked on the roadway or its verges in a manner which will result in danger to the personnel working on the site.

  - Adjoining Land Uses

All the sites are within the Louth Engineering Area, both within the urban areas of Dundalk & Drogheda within the 50km/hr Speed Limits. Speed limits at Monasterboice is 60km/hr and Annaloughan has a speed limit of 100km/hr. The adjoining land uses are mainly housing, shops, commercial, retail etc. with both primary and secondary schools and third level within the vicinity of proposed works.

3.0	PROJECT TIMESCALE AND ACCESS RESTRICTIONS

#### Restrictions on Working Hours

The normal working hours shall be:

08.00–18.00 Monday–Friday and 08.00-13.00 Saturday.

  - Restrictions on Access

The Contractor shall ensure that access to and from all properties adjoining the site shall not be unreasonably restricted during the construction works. The maintenance of access shall be the responsibility of the Contractor.

  - Time for Completion

The contract works must be completed in a reasonable timeframe and within the period up until 1st December 2026 unless by agreement with the Senior Executive Engineer- Operations Section, Louth County Council.

- OTHER WORKS ACTIVITIES

4.1	Local Authority

Louth County Council may be carrying out maintenance work on roads and services on and/or adjacent to the site.

5.0	PARTICULAR RISKS

The following work headings, numbers 5.1 to 5.10 are a non-exhaustive list of particular risks to the safety and health of persons as set out in the First Schedule to SI 182 of 2013 together with the opinion of the Project Supervisor (Design Process) on what elements, if any, of the works may fall within each particular risk category. A further work heading, number 5.11, is a non-exhaustive list of non-scheduled risks, which in the opinion of the Project Supervisor (Design Process), should be considered by the Project Supervisor (Construction Stage). Work heading number 5.12 is a Non-Exhaustive List of Matters to be considered as regards the Application of the General Principles of Prevention to Construction Work as set out in the Second Schedule to SI 182 of 2013.

It should be noted that many of the risks on the project arise out of working methods which are at the discretion of the Contractor and as a result cannot be determined by the Project Supervisors Design Process.

5.1	Work which puts persons at risk of burial under earth falls, engulfment in swampland or falling from a height, where the risk is particularly aggravated by the nature of the work or processes used or by the environment at the place of work or site.

Burial due to deep excavations, excavations in trenches for services and associated chambers and connections, possible soil stockpile instability due to stockpiling at an unstable depth or along an existing slope.

Engulfment: in peat or other weak material which could loose strength due to traffic or vibration, in stockpiles of weak material.

Falling: into excavations, or over the edge of an embankment.

- Excavations for drainage. Excavations not expected to exceed 	0.5m in depth.

5.2	Works which put persons at risk from chemical or biological substances constituting a particular danger to the safety and health of such persons or involving a legal requirement for health monitoring.

- Preparation and use of bituminous and concrete materials may have safety and health implications. The inhalation of dust during excavation, drilling or cutting of materials. Works in confined areas should be minimised where any chemicals or agents are used.

- Underground gas mains. Contractor must liaise with & take advice from Bord Gais in relation to the existence of all gas services within & adjacent to the works areas.

- Tracing & marking out of all underground services prior to excavation is a requirement of the contract and personnel carrying out this task must hold the relevant CSCS qualification for Location of Underground Services as certified by FÁS.

- Hand digging is recommended for excavations which are in and/or close to the alleged location of the services and in locations where no prior service information could be obtained

5.3    Work with ionising radiation requiring the designation of controlled or supervised areas as defined in Article 20 of Directive 80/836/Euroatom.

- Testing the density of bituminous materials may be carried out using a Nuclear Density Meter.

5.4	Work near high voltage power lines/ESB lines.

- Overhead and underground ESB services. Goalposts must be erected at overhead wires where appropriate, alternatively a banksman can be named on the Construction Stage Health & Safety Plan, and his duties shall include overseeing work in the vicinity of the overhead wires.

- The successful Contractor must liaise with & take advice from ESB and Airtricity in relation to the existence of all ESB and Airtricity services within & adjacent to the works areas and must comply with the relevant Code of Practice in relation to overhead & underground services.

- Tracing & marking out of all underground services prior to excavation is a requirement of the contract and personnel carrying out this task must hold the relevant CSCS qualification for Location of Underground Services as certified by FÁS.

- Hand digging is recommended for excavations which are in and/or close to the alleged location of the services and in locations where no service information could be obtained

5.5	Work exposing persons to the risk of drowning.

- Nonevidential at Design Stage.

5.6	Work on wells, underground earthwork or tunnels.

- Nonevidential at Design Stage.

5.7	Work carried out by divers at work having a system of air supply.

- None.

5.8	Work carried out in a caisson with a compressed air atmosphere.

- None.

5.9	Work involving the use of explosives.

- None. Blasting is not permitted on the contract.

  - Work involving the assembly or dismantling of heavy prefabricated components.

Work on concrete box culverts, various diameter concrete culverts and pipes, precast concrete elements, chambers, formwork and shuttering

5.11	non-scheduled risks.

Works adjacent to live carriageways. Special care must be exercised for traffic management where traffic flows are to be maintained works are being carried out. Particular care is required in areas of existing poor horizontal and vertical road alignment, areas of no or restricted width footpath, restricted width, poor visibility, narrow verges and where there are a number of junctions and in cul-de-sacs. All sites within urban areas are 50 km/h The Remaining roads are within 60Km/hr & 100km/hr speed limit zone.

- Pedestrians and cyclists (including, but not restricted to, school children and shoppers): Special care must be exercised for pedestrian management for the entire duration of the works. No element of the works area is to be left open to pedestrian or cyclist activity. Pedestrians and cyclists are not permitted to enter, travel inside or cross the sites unless at clearly designated, safe crossing points. Several of the sites are in very close proximity to schools or busy shopping areas or are located along routes to schools. All the urban sites are mainly residential in nature/housing estates and may have a high level of children playing etc.

- Control of noise and vibrations. This applies to all areas.

- Careful excavation in the vicinity of existing live services.

The successful contractor is to obtain drawings from the relevant utility provider prior to the commencement of the works and to carry out visual inspections. The following table is a preliminary indication of the services likely to be present.

- Careful excavation is required near signalised crossings/junctions. Cutting, breaking and drilling concrete or bituminous bound materials by hand.

- Work in confined spaces, narrow roads and /or heavily trafficked roads and areas of limited or no footpath

- Ground personnel on site in the presence of swinging excavators and other moving construction vehicles.

- Overhead cables in the presence of swinging excavators, tipping trucks and other moving construction vehicles.

- Manual handling.

- Work in the presence of trees and their root systems.

- Work in close proximity to walls.

- Adverse weather conditions.

5.12	Non-Exhaustive List of Matters to be Considered in Particular as regards the Application of the General Principles of Prevention to Construction Work

- Keeping the construction site in good order and in a satisfactory state of cleanliness.

- Choosing the location of workstations bearing in mind how access to them is obtained and determining routes or areas for the passage and movement of equipment.

- The conditions under which various materials are handled.

- Technical maintenance, pre-commissioning checks and regular checks on installations and equipment with a view to correcting any faults which might affect the safety, health and welfare of persons at work.

- The demarcation and laying out of areas for the storage of various materials, in particular where dangerous materials or substances are concerned.

- The conditions under which the dangerous materials used are removed.

- The storage and disposal or removal of waste and debris.

- The adaptation, based on progress made on the construction site, of the actual time to be allocated for the various types of work or work stages.

- Co-operation between employers and self-employed persons.

- Interaction with industrial activities at the place within which or in the vicinity of which the construction site is located.

- Traffic Management

- The proposed works will be carried out at the various locations as indicated in paragraph 2.4. Locations of proposed works are in both urban and rural areas, and all works will involve working with & accommodating live vehicular traffic, cyclists & pedestrians.

- Local access must be provided to schools, residential, recreational and commercial and other properties during the course of the proposed works.

- All of the above must be taken into account when drawing up the programme of works and the traffic management programmes.

| Site 1 Road Number | Georges Street, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 7m. |
| Length of Works | Approx. 35m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located along the Georges Street which caters for high volumes of Pedestrian traffic in close proximity to the nearby shops & residential housing developments |
| Site 2 Road Number | Crosslanes North, Drogheda |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2.9m. |
| Length of Works | 35m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located in Drogheda Town which caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly near the proposed works and nearby shops, Hospital, medical centres, church, sports facilities & school. |
| Site 3 Road Number | Crosslanes South, Drogheda |
| Townlands(s) | Drogheda |
| Minimum Road Width | 5m |
| Maximum Road Width | 6m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 3.9m. |
| Length of Works | 35m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located within the town of Drogheda which caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops & Hospital, sports facilities. |

| Site 4 Road Number | North Road, West, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 5m |
| Maximum Road Width | 6m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 4.170m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located within the town of Drogheda which caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops & garages. |

| Site 5 Road Number | North Road East, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 3.1m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located on the outskirts of town on the Dublin Road (R132) adjacent to several garages/shops this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 6 Road Number | Mc Donalds, Dundalk |
| --- | --- |
| Townlands(s) | Dundalk |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 4.75m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located in the town on the Dublin Road (R132) adjacent to shopping centre/catering facility this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 7 Road Number | Castlebellingham Southbound |
| --- | --- |
| Townlands(s) | Castlebellingham |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2.2m. |
| Length of Works | 40m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located in the village on the Dublin Road (R132) adjacent to several shops/bars this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 8 Road Number | Donore Road, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2.54m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located on the outskirts of town on the adjacent to several garages/restaurant this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 9 Road Number | DKIT Dundalk |
| --- | --- |
| Townlands(s) | Dundalk |
| Minimum Road Width | 6m |
| Maximum Road Width | 10m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2.7m. |
| Length of Works | 30m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located on the outskirts of town on the Dublin Road (R132) adjacent to several garages/shops and third level education facility this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 10 Road Number | Monasterboice |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 8m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2.4m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 60kph |
| Other Relevant Information | The work site is located on the outskirts of town on the Dublin Road (R132) adjacent to several restaurants/ sporting facilitys this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 11 Road Number | Drogheda Hospital North, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 6m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 1.8m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located outside Drogheda Hospital this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 12 Road Number | Drogheda Hospital South, Drogheda |
| --- | --- |
| Townlands(s) | Drogheda |
| Minimum Road Width | 6m |
| Maximum Road Width | 6m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 1.77m. |
| Length of Works | 25m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located outside Drogheda Hospital this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 13 Road Number | Omeath Village |
| --- | --- |
| Townlands(s) | Omeath |
| Minimum Road Width | 6m |
| Maximum Road Width | 6m |
| Hard Shoulder | No |
| Footpaths/ Grass Verge | Yes, alongside the proposed works site. Widths: 2.2m. |
| Length of Works | 10m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located in the village adjacent to several garages/shops/take aways this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 14 Road Number | Annaloughan (Fitzpatricks) |
| --- | --- |
| Townlands(s) | Dundalk |
| Minimum Road Width | 6m |
| Maximum Road Width | 10m |
| Hard Shoulder | No |
| Footpaths/Grass Verge | Yes, alongside the proposed works site. Widths: 3.04m. |
| Length of Works | 10m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 100kph |
| Other Relevant Information | The work site is located on the outskirts of town on the Greenore road (R173) adjacent to Restaurant/Bar this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

| Site 15 Road Number | Louth Hospital, Dundalk |
| --- | --- |
| Townlands(s) | Dundalk |
| Minimum Road Width | 6m |
| Maximum Road Width | 10m |
| Hard Shoulder | No |
| Footpaths | Yes, alongside the proposed works site. Widths: 2m. |
| Length of Works | 10m |
| Number of Junctions | 0 |
| Peak Traffic Flows | 0800 – 1000; 1200 –1430; 1600 - 1800 |
| Speed Limit | 50kph |
| Other Relevant Information | The work site is located on the outskirts of town on the Dublin Road (R132) adjacent to several garages/shops/hospital this road caters for high volumes of vehicular traffic. There is also a high volume of pedestrian traffic particularly in close proximity to the nearby shops, garages. |

7.0	CONSTRUCTION MATERIALS

The following is a non-exhaustive list of materials that have been identified which will be a risk to safety and health during construction and commissioning:-

- Concrete

- Concrete additives/Curing agents

- Fuel

- Cement/Lime/Dust

- Acid

- Salt

- Tack coat bitumen spraying.

- Surfacing materials – capping layer, Clause 804 stone, binder course macadam, surface course HRA.

Procedures must be developed for recognising when these materials are brought to site, store and used and ensuring that the appropriate COSHH (Control of Substances Hazardous to Health) data sheets are obtained, risk assessments carried out and appropriate controls put in place.

It is assumed that the Contractor will be aware of any precautionary information for the materials used, that the supplier, by law, is required to provide.

8.0	SITE WIDE ELEMENTS

8.1	Site Accommodation:

Site accommodation required by the Contractor should be located to prevent the creation of a traffic hazard.

8.2	Offloading and Storage:

Location and layout of unloading and storage areas shall be designed by the Contractor to prevent the creation of a traffic hazard.

8.3	Traffic and Pedestrian Routes:

Access must be maintained. The movement of both and machinery across or along the site will also have to be accommodated. Provision must be allowed for the efficient traverse of the site for Emergency Services Vehicles.

8.4	Unauthorised Access:

Provision should be made for the prevention of unauthorised access of pedestrians and vehicles to the works.

9.0	OVERLAP WITH CLIENT’S UNDERTAKINGS

Due consideration must be given in co-ordination of the Client’s and all other contractors staff. The following is a list of other Contractors that may require access in the site:

###### 9.1      ESB and Airtricity

Routine maintenance works on or adjacent to the site.

###### 9.2      Eircom and other telecommunication providers

Routine maintenance works on or adjacent to the site.

9.3	Irish Water

Work necessitated by this contract and routine maintenance works adjacent to the site.

9.4	Operations Section, Louth County Council

General routine maintenance works adjacent to the site.

###### 9.5	Landscape

Works (if any) necessitated by this contract – carried out by nominated sub contractor

9.6	Public Lighting

Routine maintenance or repair works adjacent to the site

###### 8.7	Testing

Contractor employed for specialist testing as directed by the Engineer

###### 9.8	Private

Private developments are adjacent to the site.

9.9	Emergency Services

Access must be available at all times to members of the Emergency services. They must also be notified prior to the commencement of the works.

- SITE RULES

The following is a non-exhaustive list of specific site rules which are to be developed:

- Safe means of access to and egress from the site and within the site.

- Safe means of unloading and storing of materials and plant.

- Control of access to the site and the prevention of unauthorised entry.

- Arrangements for personal protection measures for employees, visitors and other contractors and sub-contractors.

- Arrangements for training of employees in health, safety and welfare at work.

- Arrangements to ensure that other contractors and sub-contractors provide evidence of their health and safety policy and procedures to ensure compliance with site rules.

- Arrangements for ensuring that all visitors to site comply with the site rules.

- Development of procedures and method statements, for compliance with safety health and welfare requirements, arising from hazards and risks set out in paragraphs 2 to 7 above.

- Standard and proof of training certificates for all personnel operating mobile plant and specialist’s equipment.

- Emergency procedures to deal with accidents.

11.0	CONTINUING LIAISON

Where unforeseen circumstances arise, the safety and health issues arising must be submitted to the Project Supervisor (Design Process) as soon as possible after the event.

Where unforeseen circumstances cause significant design changes the safety and health implications arising are to be submitted to the Project Supervisor (Design Process). Changes in design can only be implemented by the written approval of the Project Supervisor (Design Process) prior to commencement of any works affected by the change in design. The following records will be compiled by the Project Supervisor (Construction Stage) in consultation with the Safety Co-ordinator and other Contractors working on site.

- Project History

- Design Data

- As-Built Drawings

- Risk Assessment

- COSHH Register

- Emergency Procedures

- Operations and Maintenance Manuals

- Electrical Safety Certificates

- Mechanical Completion Certificates

- Lifting Equipment Test Certificates

- Welfare provisions

- Other Factors

- Testing Calendar

- Formal reporting system for unforeseen eventualities

- Formal procedure system for emergencies

- Construction sequence and methods

- Materials used and specifications

- Maintenance requirements

Health and Safety issues arising during the course of construction from design changes, control measures, unsafe practices, incidents and accidents, amendments to the Safety and Health Plan and information for the Safety and Health File will be examined at regular monthly site meetings.

Any significant alteration to the scope of the works, for whatever reason, must be vetted by the Project Supervisor (Design Process). Any hazard discovered but not removed during the construction period must be recorded and a copy of the records sent to the Project Supervisor (Design Process) as soon as possible and in any case before Practical Completion.

This information contained in this preliminary Safety & Health Plan has been prepared prior to the commencement of work on site. It does not take account of any matters or information which may come to light after time.

Signed for and on behalf of Louth County Council:

___________________________________________________________

(Project Supervisor for Design Process)