Bus Shelter & Bus Stop Civils Contract 2026

Louth County Council

Volume A – Works Requirements

MAY 2026

- Preliminary Particulars

- Name, Nature & Location of the Works

- The name of the scheme will be Bus Shelter Civils contract 2026. The scheme provides for Bus Shelter Civils works in County Louth. The works will include Bus Shelter Base, Footpaths, Ducting, Kerbs and Road markings.

- General Description of the Works

Refer to Volume c – Bill of Quantities

- Employer

The Employer is Louth County Council with address at Louth County Council, County Hall, Millennium Centre, Dundalk, Co. Louth

- The Employer’s Representative

The Designer for this project is Louth County Council with address at Louth County Council, County Hall, Millennium Centre, Dundalk, Co. Louth

- Consents

- All works are within the public carriageway.

- Planning permission is not required for the scheme.

- The Site

- Site Boundaries

The proposed works are at the locations identified on the Project Drawings.

- Means of Access

There are no restrictions to the means of access to each location.

- Contractor’s Compound

Due to the nature of the works a Contractor’s Compound is not deemed necessary for the project. If however, the Contractor feels that a Compound may be required for specific Locations within the overall project he may approach the Employer’s Representative regarding the possible location for a Contractor’s Compound.

The Contractor will be responsible for all costs associated with the establishment, fencing, security arrangements, and provision of services to the compound, if required.

The Contractor will be responsible for all costs associated with the establishment, fencing, security arrangements, and provision of services for the storage of plant or materials on site, if required.

The Contractor shall at his own expense be responsible for the acquisition of any further working areas or means of access to those outlined above required for the carrying out of works.

- Position of the Works

The Contractor shall meet and agree with the Employer’s Representative the setting out and positioning of the works within the site immediately before entering the site for the purpose of constructing the works.

- Limitations on Use

The limitations on use of the site have been developed as part of the restrictions and obligations for the works and are detailed in Sub-Section 7 below.

- Work on Site by Others

There may be work on site by others during the construction period.

- Facilities of Employer to be Operated and Maintained

The following facilities of the Employer will be required to be operated and maintained by the Contractor;

- The Contractor will be required to provide and maintain safe pedestrian & cycle access and designated safe crossing points throughout the length of the works at all times for the duration of the construction works. Pedestrians & cyclists are to be prevented form entering or crossing the works, whether a road closure is in place or not.

- The Contractor will be required to maintain vehicular access to and egress from all residential, commercial & other premises affected by the construction works.

- The Contractor will be required to provide vehicular access along the length of the Works at all times for the duration of the construction works to comply with the latest revision of “Traffic Signs Manual Chapter 8 Temporary Traffic Measures & Signs for Roadworks”, as issued by the Department of the Environment .The Contractor will be required to carry out regular cleaning of public roads and access roads.

- Specific Site Rules and Regulations

The specific rules and regulations for the site have been developed as part of the Preliminary Health & Safety Plan for the construction works and are included in the information Pack.

- Utilities

Prior to the commencement of works the contractor must obtain from the Employer and other “Public Utility/statutory bodies” i.e. ESB, Airtricity, Bord Gais, Eircom, broadband companies’ etc details of all underground services in the working. The horizontal and vertical profiles of all services are to be established by the Contractor through the excavation of any trial pits and slit trenches.

- Site Investigations

As this is an accessibility improvement project no site investigation was deemed necessary.

- Sectional Completion

- Grant Related Deadlines for Agreement of Final Accounts for Works funded by the Department of Transport and/or National Roads Authority.

The works, which are funded by the Department of Transport or National Roads Authority, must be completed by within 8 weeks of letter of agreement

- Deadlines for Agreement of Final Accounts for Works funded by Louth County Council directly.

None

- Site Specific Requirements for the completion of Works

    - Works to be completed at off-peak hours and/or outside normal working hours in order to minimise disruption to Road Users.

At design stage no works have been identified as warranting working during off-peak times and/or outside normal working hours, to minimise disruption to Road Users.

- Drawings Specifications, Schedules and Reports

In the event of a conflict between the Works Requirements, Drawings and Specifications, the contractor is required to liaise with the Senior Executive Engineer Local Roads & Services Section Louth County Council in order to determine the precedence.

- General Arrangements, Working & Construction Drawings

  - Materials and Workmanship Specifications

The materials and workmanship specifications for use in connection with the proposed construction works are the TII Specification for Road Works with the amendments, supplements and additions as listed below.

Note 1:	Where British Standards and Codes of Practice are referred to in the General Specification, they are to be deemed to refer to the latest edition of the Standard or in the case of a Standard which has been withdrawn, to the latest edition of the Standard that supersedes it.

Note 2:	Where reference is made in the General Specification or Particular Specification to the “Engineer” or to the “Resident Engineer”, it will be deemed to refer to the “Employer’s Representative”.

- Specialist Works

None

Restrictions and Obligations

- Limitations on Working Hours

The normal working hours applied to the Contract will be 08:00 to 18:00 from Monday to Friday and 08.00 to 13.00 on Saturdays.

- Sunday work is only to take place on the Employer’s Representative instructions and as approved by the Employer.

In addition, the limitations to working hours detailed in the table below will apply:

| Item | Work Areas Affected | Restrictions and Conditions |
| --- | --- | --- |
| Bank Holidays | All Areas | No work between 17:00 hrs Friday to 08:00 hrs on the following Tuesday. |

Night working is only to take place on the Employer’s Representative instructions and as approved by the Employer. In any event, night work is to be restricted to the hours of 20:00 to 06:00 and all excavations to be backfilled, reinstated and the road fully cleared by 06:00 each morning.

- Advertising

The Contractor shall not undertake or allow any bill posting or advertising upon the Works without prior written approval from the Employer’s Representative.

- Maintenance of Existing Services

The Contract involves the construction of connections and modifications to existing pipelines, structures and water courses as shown on the Drawings. The Contractor shall maintain the flows in the existing pipes, sewers and water courses disturbed in the course of construction of the works and shall provide all temporary works and pumping to do so. The Contractor shall consult with the relevant department of the Local Authority and submit schedules and methods of his work for approval by the Employer’s Representative prior to the execution of these connections.

It may be necessary to raise and relay some of the existing services during the course of the construction works. The Contractor shall, in this event, conduct his operations in such a manner that the minimum disruption possible is caused to the normal working of these services. The Contractor shall consult the Employer or responsible Authority and submit schedules and methods of operation for approval by the Employer’s Representative prior to the execution of all such works. The Contractor shall include in his tender for all additional costs involved in the execution of this work and in providing temporary diversions, over pumping and other such means to maintain the required level of service during the construction of the works.

It may be necessary to raise and relay some existing cables, ducts, chambers and similar belonging to the E.S.B., Eircom, Bord Gáis or other utility providers or any existing site power and control cables etc. in the path or adjacent to the line of the proposed pipelines during the course of the construction works. The Contractor shall consult the responsible authority prior to the commencement of excavations for pipe laying or similar operations and submit schedules and methods of operation for approval by the Employer’s Representative. No excavation work shall commence without prior approval by the Employer’s Representative of such work.

The Contractor shall be deemed to have allowed for all such investigation and possible delays in commencement and other allied works in his programme and consequential costs for such works in his tender.

The Contractor shall be responsible for dealing with all aspects of work affected by water arising from all natural sources and causes as well as due to leakages from water or sewer mains close to excavations of temporary and permanent works.

- Execution of Work in a Specific Order

The Contractor shall backfill and carry out temporary reinstatement, to one section of the works prior to commencing excavation of the next section.

- Road Closures and Alternative Route Restrictions

It will be necessary, as far as possible, to maintain the traffic flow on the existing roads during the construction of the works. Road closures (if permitted), temporary one-way systems, traffic controls, traffic diversions, etc. shall be agreed with the Employer’s Representative and Roads Department of the Local Authority 14 days prior to the commencement of construction works at any given location.

All alternative routes for vehicular traffic are to be identified and agreed with the Employer or Employer’s Representative and all necessary signs and vehicle actuated traffic lights at strategic locations shall be provided, erected and maintained by the Contractor.

All detour and other signs shall be erected at the commencement point and along the road of the detour traffic for the duration of the detour period and shall be erected and maintained, on a daily basis, as the construction work progresses. The signage required for traffic management shall be in accordance with the latest revision of “Traffic Signs Manual Chapter 8 Temporary Traffic Measures & Signs for Roadworks”, as issued by the Department of the Environment.

The Contractor shall liaise with all owners and occupiers regarding access to their premises along the road and generally wherever the Contractor is working. The Contractor shall at all times maintain means of access to these premises to the satisfaction of the Employer’s Representative.

The Contractor shall provide prior notice and carry out leaflet drops to all premises affected by the works as & when it is deemed necessary or on the instruction of the employer’s representative.

The Contractor is required to obtain a permit from the Local Authority to allow the use of off-road dumpers on public roads.

An off-road dumper is defined as “Vehicles (commonly known as off-road dumpers) exceeding 3 metres cubed in capacity, level loaded, designed and constructed primarily for use on sites of construction works (including road construction and house and other building works) for the purpose of conveying concrete, rubble, earth and other like materials and incapable by reason of their design and construction of exceeding a speed of 50 kilometres per hour on a level road under their own power and which are subject of special permits under Article 17 of the Road Traffic (Construction, Equipment and Use of Vehicles Regulations, 1963 (S.I. No. 190 of 1963)” or latest amendment thereof. A copy of such permits should be supplied to the Employer’s Representative prior to the use of such vehicles on the Contract.

The Contractor shall be deemed to have included in his tender for all additional costs involved in providing Road closures, temporary one-way systems, traffic controls, traffic diversions, etc. and to have allowed for all possible associated delays in commencement of his works in his work programme.

It should be noted that there is no guarantee that permits will be issued in all/any cases and the Contractor may have to alter arrangements if permits are not issued.

- Temporary Accommodation and Facilities

The Contractor shall be responsible for the temporary acquisition and demobilisation, clearing and reinstatement of disturbed surfaces and removal and refurbishment to its original condition or better of any temporary works on completion of his work of a secure compound area for storage of materials, if required.

The Contractor shall allow for this aspect of work in his programme and any consequential expenditure which shall be deemed to have been included in his prices for the permanent works.

No night accommodation shall be allowed on site except in the case where a night watchman is employed.

- Use of Explosives

The use of Explosives is not deemed necessary for this Contract.

- Public Relations

The perception of the general public is that the Contractor’s employees working on this project represent the Employer. Great care should be taken by the Contractor in his instructions to his employees to make sure that they are made aware of their responsibilities in respect to public relations.

The Contractor shall at all time carry out the construction works in such a manner as to cause minimum disturbance to the public. The movement of construction vehicles shall be organised to cause minimal disturbance to public use of streets, lanes and other areas.

The Contractor shall ensure that all householders and owners of businesses along the works are given adequate written notice of any disturbance to access to their houses as a result of these works.

The Contractor’s employees shall refer all queries from the general public to the Employer’s Representative, without delay, and shall not attempt to answer such queries themselves. There shall also be clear lines of communication established to keep the Employer’s Representative informed of all complaints received from the general public, business premises, house owners and landowners.

It may be necessary for the Contractor’s Representative to attend from time to time meetings arranged by the Employer’s Representative with the Representatives of local residents, chamber of commerce, Garda and or Fire and safety officers in respect of the work programme, Contractor’s method of work or any other aspects of the Contractor’s work, Except for any emergency situation, the Contractor shall be informed in good time of such meetings.

- Work in Proximity to River Sites

The construction works must be performed in compliance with the “Requirements for the Protection of Fisheries Habitats During Construction and Development Works at River Sites” published by the Eastern Regional Fisheries Board.

- Waste Management Plan

The Contractor is required to prepare and submit for approval a Waste Management Plan in compliance with the Waste Management Act 1996 to 2005 for the works, with particular reference to the transport and disposal to appropriate licensed disposal sites of spoil, planning’s and excavated material arising from the construction and/or demolition works associated with the contract. The Contractor will comply with the Louth County Council Environment Policy Statement and the criteria set out in Appendix 26 / 70 in respect to the execution of their Works.

- Steel Reinforcement Certification

The Contractor shall provide certification by an independent third party that the steel reinforcement used on the Contract is in compliance with BS 4449.

- Covering Up of Works

No work shall be covered up or put out of view without the consent of the Employer’s Representative. The Contractor shall give in writing at least 1 days notice of work which is about to be covered up or put out of view. No responsibility shall be accepted by the Employer’s Representative for the Contractor’s failure to give such notice.

- Water for the Purpose of Construction

Water for construction purposes may be obtained from the Local Authority pipelines provided water is available and the Authority grants permission. The Contractor shall ascertain at the time of tendering if permission will be forthcoming and if any restrictions will be made as regards quantity, rate of draw-off, etc. and what charges (if any) will be levied. Whether or not ample water is made available by the Local Authority the Contractor shall include in his tender rates for all water required from whatever source for construction, testing, sterilising and any other works under the Contract.

- Cable Ducting

All ducts shall be laid with nylon pull through ropes so that cables can be threaded through at a later stage. All power and control ducts shall be ESB Specification TL/DIS/160/94 and shall be laid complete with safety marking tape “Danger Electricity Cables”.

- Health & Safety

- Proposed Arrangement for Health & Safety

The Safety, Health and Welfare at Work Act 2005 came into effect on 1st September 2005. The Safety, Health and Welfare at Work (Construction) Regulations 2006 (“the regulations”) came into effect in 6th November 2006 and the General Application Regulations in November 2007.

It is the intention of the Employer that the company appointed as Contractor for the construction of the works shall undertake the duties of the Project Supervisor for Construction Stage of the project as set out in Articles 16 to 22 incl. of the Regulations. The Contractor will be required to appoint a Health and Safety Coordinator as envisaged by Article 17 (2) of the Regulations, to undertake on behalf of the Project Supervisor (Construction Stage) the duties specified in 16 to 22 of the Regulations. The appointment of the Contractor as Project Supervisor for Construction Stage is separate to, and independent of, the contract for the construction works.

The Health and Safety Plan has been prepared on a preliminary basis in accordance with Regulations 12 (1) (a) of the Safety, Health and Welfare at Work (Construction) Regulations, 2006. It has been prepared to provide information to the Project Supervisor for the Construction Stage for the development of the detailed Health and Safety Plan, which is to be developed by the Project Supervisor for the Construction Stage before the start of the construction work. The Contractor will be required to develop the plan in response to Regulation 16 of the above Regulations and to fulfil the duties of Project Supervisor for the Construction Stage of the Project.

- Contract

- Form of Contract

The Form of Contract to be used in connection with the proposed works is the ‘Short form of contract”

- Model Forms

The following Model Forms included in the tender documentation & information pack are to be completed and returned with each Tender:

- Form of Performance Bond.

- Form of Appointment of Project Supervisor for Construction Stage

- Form of Rates of Pay

- Form of Professional Indemnity

- Health & Safety Questionnaire

The Health and Safety questionnaire and Competency questionnaire must be completed and returned.
Works Proposals

- Documents to be Provided by the Contractor

- Programme of Works

The programme of works shall clearly show the period for:

- Mobilisation (which shall include erection of Temporary Accommodation/Site Compound, if required)

- Structural condition survey and topographic survey as required.

- Any other activities to locate existing services and to agree with the Employer’s Representative the locations and route of same including manholes, inspection chambers, valves, etc.

- Approval of materials to be used for temporary and permanent works.

- Works-related Health & Safety.

- Any temporary works required for construction works to commence including site investigation if required.

The programme of works shall give details of all major components, equipment, structures and services forming the Works. The following items shall be incorporated within the programme as a minimum requirement.

- Mobilisation on site

- Liaison with Utility Providers

- Liaison with Local Authority

- Traffic Management Submissions

- Advance Notification of Works to residents and other stake holders

- Erection of Temporary Roadworks Signage

- Earthworks

- Construction of pipelines

- Raising/lowering of Iron works

- Pavement Overlay Works

- Construction completion tests

- Erection of signage and completion of road markings

- Take over

The programme shall also contain precisely identified ‘milestones’ showing the key dates of important events including but not limited to;

- The dates for commencement of Works on Site

- Dates for achievement of activities necessary to determine payment entitlement

The programme shall clearly illustrate the ‘critical path’ for the Contract.

The programme shall be prepared using computer software of an approved type and shall be submitted in the form of a GANTT and PERT chart printed on A1 and A3 size paper. The computer software shall be compatible with ‘Microsoft Project’ software or an approved other, and if required by the Employer’s Representative, the Contractor shall submit a copy of the programme in compatible digital form.

- Method Statements

The Contractor shall prepare statements for his proposed methods, timing and sequence of construction, including the use and design of temporary works and equipment. The method statements shall contain sufficient information to enable the Employer’s Representative to assess any likely detriment to either the proposed or existing works. The Contractor shall programme the timely issue of method statements at least one week in advance of the relevant section of work.

A method statement shall be submitted by the Contractor for, at minimum, the following activities (this is not an exhaustive list):

- Cold Milling (Planing)

- Traffic Management

- Connection to existing live sewers (surface water)

- Service Crossings

- Construction within the public areas and highways

- Work in proximity to overhead and underground services

- Working with bituminous materials

- Operating heavy plant

- Working in the vicinity of heavy plant

- Road Marking

- Any other work if so directed by the Employer’s Representative

Each method statement shall include but not be limited to the following aspects of the relevant site operation:

- Health and Safety measures

- Extent of working areas and protective barriers

- Access to working areas, including confined spaces

- The implementation of relevant Statutory Regulations

- The design and construction of temporary works

- Plant requirements, siting and mode of operation

- Manpower requirements and supervision

- Delivery and storage of materials

- Provision of access by third parties

- Details of the construction sequence

- Details of working methods

- Requirements for shutting down or limiting any power supply, water supply, signalling or other live services

- Results of any consultation with relevant authorities

- Contingency plans in the event of unforeseen difficulties

- Contingency plans shall address the possibility of delayed completion of critical sections of the Works by the Contractor

  - Submittal Procedures

The Contractor shall present to the Employer’s Representative, not later than twenty days after receipt of the letter of acceptance, a programme for the construction works, setting out dates on which they propose to start and finish the various sections of the works and the resources in manpower and plant that he intends to use in the construction of the works.

The Contractor shall allow a period of 14 days following by the Employer’s Representative for the approval or rejection of method statements. Work shall not commence until the relevant method statement has been approved by the Employer’s Representative. Neither the Employer’s Representative nor the Employer shall be responsible for the implications of any delays or costs incurred as a result of the late submission of method statements.

The acceptance of any method statement shall not relieve the Contractor of his obligations under the Contract.

- reports

  - Site Meetings

Site Meetings will be held between the Contractor and the Employer’s Representative as required over the duration of the Contract.

A Progress Meeting shall be held at scheduled fortnightly intervals from the commencement of work on site to substantial completion. The Contractor and the Employer’s Representative shall attend all Progress Meetings. The Employer will be invited to attend any of these meetings as and when deemed necessary.

The Contractor shall report on the progress of construction and related activities at the Progress Meeting.

  - Progress Reporting

The Contractor shall submit written progress reports to the Employer’s Representatives before each scheduled progress meeting. The progress reports shall comprise of the following:

Programme:

A copy of the current approved Programme marked up to show progress and percentage completion on each activity to date.

Financial Statement:

Value of tendered work completed in month.

Cumulative value of tendered work completed.

Value and brief particulars of Change Orders.

Value and brief particulars of Compensation or Delay Events.

Narrative:

A brief narrative summarising progress and giving reasons for any delay and the Contractor’s proposals for rectifying them.

12. 	Confidential Matters

Section not used.

13. 	Quality Assurance & Testing

The Quality assurance and testing requirements for the works are included in the specification for the works.

14. 	Title

Section not used.

Numbered Appendices

(Contract Specific Information)

This document contains information relating to the Specification for Road Works in the form of numbered Appendices which provide the Contract Specific Information for the works required for roads, footpaths, earthworks, road drainage, foul and surface water drainage, watermains and ancillary works.

The Road Authorities responsible for the Public Roads to be resurfaced/reconstructed is Louth County Council & Louth County Council.

The Sanitary Authority is Louth County Council.

The Authorities responsible for exercising Building Control are Louth County Council & Louth County Council.

The Authorities responsible for exercising Planning Control are Louth County Council & Louth County Council.

The Authorities responsible for exercising Environmental Control are the Environmental Protection Agency & Louth County Council.

The Authorities responsible for exercising additional Environmental Controls are Louth County Council and the Health and Safety Authority.

The Engineer for the Works is Louth County Council. Where the title ‘Engineer’ is used it shall read ‘Employer’s Representative’.

| TABLE OF CONTENTS: |  |  |
| --- | --- | --- |
|  |  | INTRODUCTION |
|  |  | Preamble to the Works Requirements |
| Appendix | 0/1 | Contract-specific Additional, Substitute and Cancelled Clauses, Tables and Figures Included in the Contract |
| Appendix | 0/2 | Contract-specific Minor Alterations to Existing Clauses, Tables and Figures Included in the Contract |
| Appendix | 0/3 | List of Numbered Appendices Referred to in the Specification and Included in the Contract |
| Appendix | 0/4 | List of Drawings Included in the Contract |
|  |  | PRELIMINARIES |
| Appendix | 1/1 | Accommodation and Equipment for the Engineer |
| Appendix | 1/2 | Vehicles for the Engineer |
| Appendix | 1/3 | Communication System for the Engineer |
| Appendix | 1/4 | Working and Fabrication Drawings |
| Appendix | 1/5 | Testing to be Carried out by the Contractor |
| Appendix | 1/6 | Supply and Delivery of Samples to the Engineer |
| Appendix | 1/7 | Site Extent and Limitations on Use |
| Appendix | 1/8 | Operatives for the Engineer |
| Appendix | 1/9 | Control of Noise and Vibration |
| Appendix | 1/10 | Structures to be Designed by the Contractor |
| Appendix | 1/11 | Structural Elements and Other Features to be Designed by the Contractor |
| Appendix | 1/12 | Setting Out and Existing Ground Levels |
| Appendix | 1/13 | Programme of Works |
| Appendix | 1/14 | Monthly Statements |
| Appendix | 1/15 | Accommodation Works |
| Appendix | 1/16 | Privately and Publicly Owned Services and Supplies |
| Appendix | 1/17 | Traffic Safety and Management |
| Appendix | 1/18 | Temporary Diversions for Traffic |
| Appendix | 1/19 | Routing of Vehicles |
| Appendix | 1/20 | Recovery Vehicles for Breakdown |
| Appendix | 1/21 | Scheme Information Boards |
| Appendix | 1/22 | Progress Photographs |
| Appendix | 1/23 | Substances Hazardous to Health |
| Appendix | 1/24 | Quality Management Schemes |
| Appendix | 1/25 | Product Certification Schemes |
| Appendix | 1/26 | Irish Agreement Board Roads and Bridges Certificates |
|  |  | SITE CLEARANCE |
| Appendix | 2/1 | List of Buildings etc. to be Demolished |
| Appendix | 2/2 | Filling of Trenches & Pipes |
| Appendix | 2/3 | Retention of Material Arising from Site Clearance |
| Appendix | 2/4 | Explosives and Blasting |
| Appendix | 2/5 | Hazardous Materials |
| Appendix | 3/1 | Fencing, Gates and Stiles |
| Appendix | 3/2 | Fencing: NRA Road Construction Details |
|  |  | SAFETY FENCES, SAFETY BARRIERS AND PEDESTRIAN GUARDLINES |
| Appendix | 4/1 | Safety Fences and Safety Barriers |
| Appendix | 4/2 | Pedestrian Guardrails |
| Appendix | 4/3 | Safety Fences and Safety Barriers: Safety Fence Detail |
|  |  | DRAINAGE AND SERVICE DUCTS |
| Appendix | 5/1 | Drainage Requirements |
| Appendix | 5/2 | Service Duct Requirements |
| Appendix | 5/3 | Surface Water Channels and Drainage Channel Blocks |
| Appendix | 5/4 | Geotextiles for Filter Drains |
| Appendix | 5/5 | Combined Drainage and Kerb Systems |
| Appendix | 5/6 | Drainage and Service Ducts: NRA Road Construction Details |
|  |  | EARTHWORKS |
| Appendix | 6/1 | Requirements for Acceptability & Testing etc. of Earthworks Materials |
| Appendix | 6/2 | Requirements for Dealing with Class U2 Unacceptable Material |
| Appendix | 6/3 | Requirements for Excavation, Deposition, Compaction (Other then Dynamic Compaction) |
| Appendix | 6/4 | Not Used |
| Appendix | 6/5 | Geotextiles Used to Separate Earthworks Materials |
| Appendix | 6/6 | Fill to Structures & Fill Above Structural Foundations |
| Appendix | 6/7 | Sub-Formation & Capping & Preparation & Surface Treatment of Formation |
| Appendix | 6/8 | Topsoiling, Grass Seeding and Turfing |
| Appendix | 6/9 | Earthwork Environmental Bunds, Landscape Areas, Strengthened Embankments |
| Appendix | 6/10 | Ground Anchorages, Crib Walling and Gabions |
| Appendix | 6/11 | Swallow Holes and Other Naturally Occurring Cavities & Disused Mine Workings |
| Appendix | 6/12 | Instrumentation and Monitoring |
| Appendix | 6/13 | Ground Improvement |
|  |  | ROAD PAVEMENTS – GENERAL |
| Appendix | 7/1 | Permitted Pavement Options Sheets 1,2,& 3 |
| Appendix | 7/2 | Excavation and Reinstatement of Existing Surfaces |
| Appendix | 7/3 | Surface Dressing |
| Appendix | 7/4 | Bituminous Sprays |
| Appendix | 7/5 | Not used |
| Appendix | 7/6 | Breaking up or Perforation of Existing Pavement |
|  |  | KERBS, FOOTWAYS AND PAVED AREAS |
| Appendix | 11/1 | Kerbs, Footways and Paved Areas |
|  |  | TRAFFIC SIGNS |
| Appendix | 12/1 | Traffic Signs: General |
| Appendix | 12/2 | Traffic Signs: Reflective Markers |
| Appendix | 12/3 | Traffic Signs: Road Markings and Studs |
| Appendix | 12/4 | Traffic Signs: Cones, Cylinders, FTD’s and other traffic delineators |
| Appendix | 12/5 | Traffic Signs: Traffic Signals |
| Appendix | 12/6 | Traffic Signs: Special sign requirements on Gantries |
| Appendix | 12/7 | Traffic Signs: Preparation and Finish of Metal and other surfaces |
|  |  | ROAD LIGHTING COLUMNS AND BRACKETS |
| Appendix | 13/1 | Lighting Columns & Brackets: Information |
| Appendix | 13/2 | Column and Data Bracket Sheets |
| Appendix | 13/3 | Instructions for completion of column and bracket data sheets |
| Appendix | 13/4 | Certification For Lighting Columns |
| Appendix | 13/5 | Road Lighting Columns And Brackets: NRA Road Construction Details |
|  |  | # ELECTRICAL WORK FOR ROAD LIGHTING AND TRAFFIC SIGNS |
| Appendix | 14/1 | Site Records |
| Appendix | 14/2 | Lighting Design Parameters and Location of Lighting Units and Feeder Pillars |
| Appendix | 14/3 | # Temporary Lighting |
| Appendix | 14/4 | Electrical Equipment for Road Lighting |
| Appendix | 14/5 | # Electrical Equipment for Traffic Signs |
| Appendix | 14/6 | # Preparation and Finish of Metal and Other Surfaces |
|  |  | # MOTORWAY COMMUNICATIONS |
| Appendix | 15/1 | # Motorway Communications |
|  |  | # PILING AND DIAPHRAGM WALLING |
| Appendix | 16/1 | Requirements for Piling and Diaphragm Walling |
|  |  | # STRUCTURAL CONCRETE |
| Appendix | 17/1 | Concrete – Classification of Mixes |
| Appendix | 17/2 | Impregnation and Coating Schedule |
| Appendix | 17/3 | Concrete – Surface finishes. |
| Appendix | 17/4 | Concrete General |
|  |  | # STRUCTURAL CONCRETE |
| Appendix | 18/1 | Requirements for Structural Steelwork |
|  |  | # PROTECTION OF STEELWORK AGAINST CORROSION |
| Appendix | 19/1 | Requirements for Bridges, Parapets, and Other Highway Structures Except for Bearings and Lighting Columns |
| Appendix | 19/2 | Requirements for Bearings |
| Appendix | 19/3 | Requirements for lighting Columns and Bracket Arms |
| Appendix | 19/3** | (Alternative): Requirements for Lighting Columns And Bracket Arms |
| Appendix | 19/3** | (Alternative): G2b System for Flange Mounted Columns With A Plinth Built Over The Flange |
| Appendix | 19/4 | Requirements for Other Work |
| Appendix | 19/5 | Form BE/P2 (New Works) Paint System Sheet |
| Appendix | 19/6 | Form BE/P3 Paint Sample Despatch List |
| Appendix | 19/7 | Form BE/P4 Paint Sample Analysis Report |
| Appendix | 19/8 | Form BE/P5 Paint Data Sheet |
| Appendix | 19/9 | General Requirements |
|  |  | # WATERPROOFING FOR CONCRETE STRUCTURES |
| Appendix | 20/1 | Form PWS Proprietary Waterproofing System Data Sheet |
| Appendix | 20/2 | Waterproofing for Concrete Structures |
|  |  | BRIDGE BEARINGS |
| Appendix | 21/1 | Bridge Bearing Schedule |
|  |  | PARAPETS |
| Appendix | 22/1 | Parapet Schedule |
|  |  | BRIDGE EXPANSION JOINTS AND SEALING OF GAPS |
| Appendix | 23/1 | Bridge Deck Expansion Joint Schedule |
| Appendix | 23/2 | Sealing of Gaps Schedule |
|  |  | BRICKWORK, BLOCKWORK AND STONEWORK |
| Appendix | 24/1 | Brickwork, Blockwork and Stonework |
|  |  | SPECIAL STRUTURES |
| Appendix | 25/1 | Requirements for Corrugated Steel Buried Structures |
| Appendix | 25/2 | Requirements for Reinforced Earth and Anchored Earth Structures |
| Appendix | 25/3 | Requirements for Pocket Type Reinforced Brickwork Retaining Wall Structures |
|  |  | MISCELLANEOUS |
| Appendix | 26/1 | Ancillary Concrete |
| Appendix | 26/2 | Bedding Mortar |
| Appendix | 26/3 | Cored Thermoplastic Node Markers |
| Appendix | 26/4 | Criteria for Louth County Council re Waste Management Act 1996-2005 |
|  |  | ANNEXES |

# PREAMBLE TO THE Works Requirements

- The Specification referred to in the Contract shall be the 'Specification for Road Works', published by the National Roads Authority as Volume 1 of the Manual of Contract Documents for Road Works in March 2000 and including all amendments to the Standards prior to and including the latest revision dated April 2002 and as extended by the following:-

  - Appendix 0/1: Contract-specific Additional, Substitute and Cancelled Clauses, Tables and Figures;

  - Appendix 0/2: Contract-specific minor alterations to existing Clauses, Tables and Figures;

  - The Numbered Appendices listed in Appendix 0/3.

  - Appendix 0/4 containing a list of the Contract Drawings referred to in the Specification.

- An Additional Clause as indicated by a suffix 'AR' in Appendix 0/1 is a Contract-specific alteration.

- A Substitute Clause as indicated by a suffix 'SR' in Appendix 0/1 is a Contract-specific alteration.

- A Cancelled Clause indicated by a suffix 'CR' in Appendix 0/1 is a Contract-specific alteration.

- Insofar as any of the Numbered Appendices may conflict or be inconsistent with any provision of the Specification for Road Works the Numbered Appendices shall always prevail.

- Any reference in the Contract to a Clause number or Appendix shall be deemed to refer to the corresponding Substitute Clause number or Appendix listed in Appendix 0/1 or 0/2.

- Where a Clause is altered, any original Table/Figure referred to in the Clause shall apply unless the Table/Figure is also altered. Where a Table/Figure is altered, any reference in a Clause to the original Table/Figure shall apply to the altered Table/Figure.

- Where a Clause in the Specification relates to work goods or materials that are not required for the Works it shall be deemed not to apply.

- Any Appendix referred to in the Specification that is not used shall be deemed not to apply.

- References in the Specification to “NRA Road Construction Details” shall be taken to refer to “Road Construction Details” as published by the National Roads Authority in March 2000 and including all amendments to the Standards.

- Throughout the Specification and Appendices of the Specification the following interpretations shall apply:

"Construction" shall mean "execution and completion".

"Maintenance" shall include Defects correction in the Works.

"Plant" shall mean "Goods".

“Engineer” shall mean “Employer’s Representative”

“Date for Commencement of the Works” shall mean “date of commencement of the execution of the Works”

“Maintenance Certificate” shall mean “Defects Certificate”

“the Permanent Works” shall mean “the Works excluding temporary works for the Works”

# Appendix 0/1

Contract Specific Additional, Substitute and Cancelled Clauses, Tables and Figures Included in the Contract

List of Additional Clauses, Tables and Figures

| Clause No. (etc.) | Title | Written on Page No. following |
| --- | --- | --- |
| 170AR 171AR 172AR 173AR 174AR 175AR 176AR 177AR 178AR 179AR | Series 100 Preliminaries Safety and Health Approval from Authorities Use of Mains Water and Electricity Pollution of the Groundwater Site Security Public Roads and Traffic Control Safety and Health Policy Risk Assessment Fire Plan Procedure Deleterious Materials | 48 48 48 48 50 50 51 51 52 52 |
| 570AR | Series 500 Drainage and Service Ducts Protection of existing sewers, drains, ducts and services | 53 |
| 670AR 671AR | Series 600 Earthworks Trial Holes Disposal of Surplus Material | 53 53 |
| 770AR 771AR 772AR | Series 700 Road Pavements – General Working in adverse weather conditions Trafficking of Carriageway Surfaces Ramping of Inspection Covers, Manhole Covers, Sluice Valve Covers, Gullies and the like | 53 53 54 |

List of Substitute Clauses, Tables and Figures

| Clause No. (etc.) | Title | Rewritten on Page No. following |
| --- | --- | --- |

List of Cancelled Clauses, Tables and Figures

| Clause No. (etc.) | Title |
| --- | --- |

Additional Clauses, Tables and Figures

| Clause No. (etc.) | Title and written text |
| --- | --- |
| 170AR 171AR 172AR 173AR 174AR 175AR 176AR 177AR 178AR 179AR | Series 100 Preliminaries Safety and Health - The attention of the Contractor is directed to current legislation covering Safety and Health at Work; the Safety, Health and Welfare at Work Act, the General Application Regulations. the General Application Regulations and the Safety, Health and Welfare at Work (Construction) Regulations. In particular, he shall at all times maintain safe and healthy working conditions at the site, comply with the requirements of Codes of Practice that are applicable to the work being undertaken, and ensure that all employees and sub-contractors comply with this Clause. Compliance with the Clause shall not relieve the Contractor of any of his other obligations and liabilities under the Contract. Approval from Authorities Where the Contractor requires any approval from Authorities in addition to approval from the Employer’s Representative, such approval shall be obtained by the Contractor in writing and duplicate copies shall be submitted to the Employer’s Representative not less than fourteen days prior to the commencement of the work affected. Use of Mains Water and Electricity - If the Contractor uses water abstracted from the mains supply he shall be responsible for obtaining all necessary permissions in advance and he shall also be liable for payment of any charges incurred. - The Contractor shall make all necessary arrangements for the provision of adequate electrical supply sufficient for all his requirements, including temporary lighting, temporary traffic lights, etc. - The Contractor shall be liable for payment of any charges for electricity supply. Pollution of Groundwater - The Contractor shall organise and design his working methods so as to prevent any pollution whatsoever from entering the ground. - The Contractor shall not discharge effluent into or otherwise pollute any existing watercourse. He shall not without the written authority of the Employer’s Representative interfere with, reduce or terminate any streams or existing watercourses. The Contractor shall not abstract water from any stream or utilise ground water unless approved by the Employer’s Representative. - The Contractor will be required to carry out detailed assessments on all operations and provisions and demonstrate to the satisfaction of the Employer’s Representative and Environmental Protection Agency how risks of pollution incidents are to be prevented. - In order to prevent pollution, secure sealed leak-proof tanks with 20 years guaranteed life to the Employer’s Representative’s satisfaction, shall be installed within bunds which are required to be constructed for the following circumstances: - All fuel storage - Toilet and Canteen Facilities - Liquid Chemical Storage - Vehicle washing areas - Rubbish collection points - To any area where liquids or chemicals are likely to be stored or discharged. - The Contractor is required to provide a suitable leak-proof membrane, within compounds for storage of fuel, all to the satisfaction of the Employer’s Representative. - The Contractor shall provide a detailed list of all fuel, chemicals and liquids stored on the site to the Employer’s Representative. A copy of the list shall be made available at all times by the Contractor. - The Contractor’s proposals for fuel storage are to be agreed with the Employer’s Representative prior to the delivery of fuel, tanks or materials for the construction of the storage compound. - No petroleum spirit shall be stored on the Site or adjacent lands until the approval of the Employer’s Representative and the necessary licences have been obtained. - The arrangements for the storage of compressed gas containers shall conform with Statutory Regulations and with the requirements of the relevant Local Authority. - The Contractor is to particularly ensure that washings and any works including concrete/cement operations are undertaken in secured sealed tanks. - The Contractor shall ensure surface water run-off or other sources of water that may reach the surfaces within his works do not pollute the aquifer. - Liquids other than water shall be stored only in accordance with the following: - In tanks located within a bund, having a capacity of not less than 110% of the total capacity of the largest tank or, if interconnected, all the tanks. - In containers or drums located within a bund, having a capacity of not less than 25% of the total capacity of the largest container whichever is greater. - The floor and walls of the bund shall be impervious to the contents of the tanks, drums or containers. Inlet/outlet/vent pipes shall be directed downwards and, together with the gauges shall be within the bund. - The bunded area shall be inspected daily and kept empty of liquid. Any liquid removed from the bunded areas shall be disposed of to a suitably licensed site or, with the approval of Louth County Council to a foul sewer. - The Contractor is to submit to the Employer’s Representative for approval his proposals for the construction and location of collection points required for rubbish, etc., which shall be positioned within the appropriate bund. - All toxic, harmful or flammable liquids should be transported and stored in leak-proof containers. Regular inspections should ne maintained to detect leakage; any found should be cleared up immediately. The Contractor shall procure at his own expense a Contractor licence by the authorities to carry out any necessary disposal of toxic, harmful or flammable liquids. Records of all disposals, including the final disposal site, should be kept. - In no circumstances must toxic, harmful or flammable liquids be permitted to enter private or public drains or water courses and provision must be made to prevent any such liquids soaking into the ground. Should any accidental contamination occur, immediate remedial measures must be taken and any resulting waste disposed of as detailed previously. - The Contractor shall indemnify and hold harmless the Employer from and against all liabilities for damages, costs, expenses (including legal expenses) arising directly from the Contractor’s failure to comply with the foregoing provisions. Site Security The Contractor shall be fully responsible for the security of his Works and all his Equipment, materials, etc. Public Roads and Traffic Control - The Contractor shall not permit the use of vehicles or any equipment on public roads or bridges communicating with the Site that may cause damage to such roads and bridges. The Contractor shall co-operate with the relevant Local Authority and Garda Authority in the movement of abnormal loads or extraordinary traffic to and from the site. - If, notwithstanding the foregoing, any damage to any road or bridge shall occur the Contractor shall be solely liable for the settlement of any and all claims from any authority, member of the public or owner of property entitled to make such claims and the Contractor shall indemnify and hold harmless the Employer against all costs, charges or expenses (including legal expenses) in respect thereof or in relation thereto. - The Contract shall take any necessary measures to keep all public roads, pavings and hardstandings clean of any spillage or droppings from his own or sub-Contractors’ traffic travelling to and from the site. All such spillages and droppings shall be cleared immediately to the satisfaction of the Employer’s Representative. - The Contractor shall provide a wheel wash if so instructed by the Employer’s Representative and shall ensure that all vehicles associated with his works pass through the wheel wash facility before leaving the site. - Should the Contractor fail to keep the roads and hardstandings clean, the Employer’s Representative may arrange for the Local Authority or others to carry out such cleaning and the total cost incurred shall be charged to the Contractor. - The Contractor shall provide, maintain and keep clean all necessary temporary access and construction roads and paths required for the execution of the Work. The Contractor is to submit his proposed haulage/temporary road layout to the Employer’s Representative for approval, together with road construction details before commencing any works. - All existing roads are to remain in use for the duration of the contract. Manual traffic signalling will be in operation during the a.m. and p.m. peak hours. Safety and Health Policy - The Contractor will be required to provide details for the Employer’s Representative’s approval of his Safety and Health Policy, incorporating requirements of the current Safety, Health and Welfare at Work Act and the current Safety, Health and Welfare at Work (Construction) Regulations and General Application Regulations where applicable. - The Contractor should be aware that the Employer’s Representative will monitor Welfare and First Aid facilities for the site and also check that all staff and employees are properly trained for carrying out their designated roles. - All persons engaged on the works shall use proper sanitary conveniences and the Contractor shall prevent fouling of the Site or land owned by others. - Throughout the period of performance of the Works the Contractor shall maintain the whole area of his operations in a clean, tidy and safe condition by arranging material in an orderly manner. All rubbish, waste material, debris and the like shall systematically and regularly be cleared off the working areas as they accumulate and if not removed directly off the site shall be deposited at general collecting points. - Electricity, gas or oil heaters having exposed heating coils, elements or burners shall not be used on the Site. - The Contractor shall provide details of his methodology for the site, with particular attention to on site Traffic Management and safety of site personnel. Risk Assessment - The Contractor will be required to submit details of his Risk Assessments for monitoring, preparation and implementation of all the work to the Project Supervisor (Design Process) for approval prior to commencement, taking into account requirements of the current Safety, Health and Welfare at Work Act, the current Safety, Health and Welfare at Work (Construction) Regulations and General Application Regulations where applicable. Fire Plan Procedure The Contractor will be required to provide details for approval by the Employer’s Representative of the Company’s Project Fire Plan. Deleterious Materials The Contractor shall use all reasonable endeavours to ensure that none of the following materials is used in the Works:- - High alumina cement - Woodwool slabs in permanent formwork to concrete or in structural elements; - Calcium chloride in admixtures for use in reinforced concrete; - Asbestos or asbestos-containing products, as defined in The Asbestos Regulations 1969 or any statutory modification or re-enactment thereof; - Naturally occurring aggregates for use in reinforced concrete which do not comply with British Standard 8110:1985 or naturally occurring aggregates for use in concrete which do not comply with the provisions of British Standard 882:1992; - Lead or any materials containing lead which may be ingested, inhaled or absorbed except where copper alloy fittings containing leads are specifically required in drinking water pipework by any relevant statutory requirements and except for weathering or flashing details; - Urea formaldehyde foam or materials which may release formaldehyde in quantities which may be hazardous, unless within the limits set from time to time by the Health and Safety Authority; - Materials which are generally comprised of mineral fibres, either man-made or naturally occurring, which have an diameter of 3 microns or less and a length of 200 microns or less or which contain any fibres not sealed or otherwise stabilised to ensure that fibre migration is prevented; - Polyisocyanate or polyurethane foam; - Polytetrafluorethylene (PTFE) except when used as non-sticking valves or insulating tape; - Calcium silicate bricks; - Concrete which may be susceptible to alkali or silica reaction (HSR); - Slipbricks; - Timber which is not obtained from a managed and regulated sustainable source; - Insulation which uses chlorofluorocarbons (CFC’s); Any other substance not in accordance with applicable current Codes of Practice or British Standards or Irish Standards or otherwise generally considered to be deleterious or unacceptable to the Environmental Protection Agency at the time of construction. The Contractor shall arrange suitable tests and be responsible for obtaining all necessary approvals from the Environmental Protection Agency and Louth County Council Environmental Department for any materials that are believed by the Employer’s Representative to be of a deleterious nature. |
| 570AR | Series 500 Drainage and Service Ducts Protection of Existing Sewers, Drains, Ducts and Services - Protection of any existing sewers, drains, ducts and services shall be to the requirements as described in the Contract. - Before protection works are carried out the existing sewer, drain, duct or service shall be located by means of hand dug trial holes except where otherwise permitted by the Employer’s Representative. - Any damage to the sewer, drain, duct of service occurring during protection works shall be made good by the Contractor at his expense. - Protection of the sewer, drain, duct or service shall be carried out in accordance with the appropriate Specification Clauses. |
| 670AR 671AR | Series 600 Earthworks Trial Holes Trial Holes for the Employer’s Representative shall be dug as and when directed. Backfilling shall be with material acceptable to the Employer’s Representative and shall comply with Clause 505. Disposal of surplus material All provisions of the Waste Management Act 1996 to 2005 and the Waste Management (Permit) Regulations shall be adhered to in relation to disposal of material. All surplus excavated or other material shall be disposed of only at licensed tips or tips where a waste permit under the Waste Management (Permit) Regulations has been obtained. |
| 770AR 771AR 772AR | Series 700 Road Pavements – General Working in Adverse Weather Conditions Laying of bituminous materials shall be avoided as far as practicable during wet weather and shall be suspended when free standing water is present on the surface. If the wet weather does not ease then manufacture of bituminous materials shall be suspended. Trafficking of Carriageway Surfaces - Longitudinal joints between new and existing carriageway surfaces (i.e. joints parallel to the general direction of traffic flow) shall not be trafficked under any circumstances. - All transverse joints shall be ramped with the regulating material before traffic is allowed over them. The maximum ramp grading shall be 5% and the minimum length shall be 1 metre. - Planed full-width or half-width areas of carriageway shall not be trafficked for a period greater than 5 days. Ramping of Inspection Covers, Manhole Covers, Sluice Valve Covers, Gullies and the like. - Where any item, i.e. inspection cover, manhole cover, sluice valve cover, gully and the like and their associated frame has been left temporarily proud of any trafficked surface (including footways) it shall be ramped off prior to being exposed to traffic. - Ramping shall be carried out using a bituminous material that shall be approved by the Employer’s Representative. Ramps shall form a maximum gradient of 5%. - Areas where any cover, gully, etc., is in excess of 50mm above the level of the surrounding carriageway shall not be trafficked. - Inspection covers, manhole covers, gullies and the like shall NOT be left proud of any section of carriageway that is to be used as a site access where the wearing course is to be applied at a later date. The covers are to be raised at the time that the wearing course is placed. |

Substitute Clauses, Tables and Figures

| Clause No. (etc.) | Title and written text |
| --- | --- |

# Appendix 0/2

#### Contract specific minor alterations to existing Clauses, tables and figures included in the Contract

| Clause No. (etc.) | Alternations to be made |
| --- | --- |

# Appendix 0/3

List of numbered Appendices refered to in the specification and included in the Contract

Appendix 0/3 is comprised of two lists A and B of Numbered Appendices as follows:

List ‘A’ is a complete list of the Numbered Appendices referred to in the Specification for Road Works with those not adopted marked ‘Not Used’. Those identified by the letters T or C shall be completed by the Tenderer or Contractor respectively.

Guide to types of numbered appendices – who compiles/completes

Symbol

(E)	Engineer Compiles: Identified in the Notes for Guidance examples by the term ‘Sample’ included in their title.

(E/C)	Engineer partially compiles and Contractor completes and returns to Engineer

(E/T)	Engineer partially compiles and Tenderer completes and returns with Tender

(C) 	Contractor completes and returns to Engineer

(I)	For Contractors information only

(P)	This indicates the Appendix is a national proforma and format must not be altered

List “B” gives the list of Contract-specific Numbered Appendices devised for the Contract.

LIST A

| Volume No. | Completed By | Appendix | Used | Not Used |
| --- | --- | --- | --- | --- |
| VolumeA | (E) | 0/1 | ✓ |  |
|  | (E) | 0/2 |  | ✓ |
| VolumeA | (E) | 0/3 | ✓ |  |
| VolumeA | (E) | 0/4 | ✓ |  |
|  | (E) | 1/1(A) |  | ✓ |
|  | (E) | 1/1(B) |  | ✓ |
|  | (E) | 1/2 |  | ✓ |
|  | (E) | 1/3 |  | ✓ |
| VolumeA | (E) | 1/4 | ✓ |  |
| VolumeA | (E) | 1/5 | ✓ |  |
| VolumeA | (E) | 1/6 | ✓ |  |
| VolumeA | (E) | 1/7 | ✓ |  |
|  | (E) | 1/8 |  | ✓ |
| VolumeA | (E) | 1/9 | ✓ |  |
|  | (E) | 1/10 |  | ✓ |
|  | (E) | 1/11 |  | ✓ |
| VolumeA | (E) | 1/12 | ✓ |  |
| VolumeA | (E) | 1/13 | ✓ |  |
| VolumeA | (E) | 1/14 | ✓ |  |
| VolumeA | (E) | 1/15 |  | ✓ |
| VolumeA | (E) | 1/16 | ✓ |  |
| VolumeA | (E) | 1/17 | ✓ |  |
|  | (E) | 1/18 |  | ✓ |
| VolumeA | (E) | 1/19 | ✓ |  |
|  | (E) | 1/20 |  | ✓ |
|  | (E) | 1/21 |  | ✓ |
| VolumeA | (E) | 1/22 | ✓ |  |
|  | (E) | 1/23 |  | ✓ |
|  | (E) | 1/24 |  | ✓ |
|  | (E) | 1/25 |  | ✓ |
|  | (E) | 1/26 |  | ✓ |
|  | (E) | 2/1 |  | ✓ |
|  | (E) | 2/2 |  | ✓ |
|  | (E) | 2/3 |  | ✓ |
|  | (E) | 2/4 |  | ✓ |
| VolumeA | (E) | 2/5 | ✓ |  |
|  | (E) | 3/1 |  | ✓ |
|  | (E) | 3/2 |  | ✓ |
|  | (E) | 4/1 |  | ✓ |
|  | (E) | 4/2 |  | ✓ |
|  | (E) | 4/3 |  | ✓ |
| VolumeA | (E) | 5/1 | ✓ |  |
|  | (E) | 5/2 |  | ✓ |
|  | (E) | 5/3 |  | ✓ |
|  | (E) | 5/4 |  | ✓ |
|  | (E) | 5/5 |  | ✓ |
| VolumeA | (E) | 5/6 | ✓ |  |
|  | (E) | 6/1 |  | ✓ |
|  | (E) | 6/2 |  | ✓ |
|  | (E) | 6/3 |  | ✓ |
|  | (E) | 6/4 |  |  |
|  | (E) | 6/5 |  | ✓ |
|  | (E) | 6/6 |  | ✓ |
|  | (E) | 6/7 |  | ✓ |
|  | (E) | 6/8 |  | ✓ |
|  | (E) | 6/9 |  | ✓ |
|  | (E) | 6/10 |  | ✓ |
|  | (E) | 6/11 |  | ✓ |
|  | (E) | 6/12 |  | ✓ |
|  | (E) | 6/13 |  | ✓ |
| VolumeA | (E) | 7/1 | ✓ |  |
| VolumeA | (E) | 7/2 | ✓ |  |
|  | (E) | 7/3 |  | ✓ |
| VolumeA | (E) | 7/4 | ✓ |  |
|  | (E) | 7/5 |  | ✓ |
| VolumeA | (E) | 7/6 |  | ✓ |
| VolumeA | (E) | 11/1 | ✓ |  |
| VolumeA | (E) | 12/1 | ✓ |  |
|  | (E) | 12/2 |  | ✓ |
| VolumeA | (E) | 12/3 | ✓ |  |
|  | (E) | 12/4 |  | ✓ |
|  | (E) | 12/5 |  | ✓ |
|  | (E) | 12/6 |  | ✓ |
|  | (E) | 12/7 |  | ✓ |
|  | (E) | 13/1 |  | ✓ |
|  | (C) (P) | 13/2 |  | ✓ |
|  | (P) | 13/3 |  | ✓ |
|  | (E/C) (P) | 13/4 |  | ✓ |
|  | (E) | 13/5 |  | ✓ |
|  | (E) | 14/1 |  | ✓ |
|  | (E) | 14/2 |  | ✓ |
|  | (E) | 14/3 |  | ✓ |
|  | (E/C) | 14/4 |  | ✓ |
|  | (E) | 14/5 |  | ✓ |
|  | (E) | 14/6 |  | ✓ |
|  | (E) | 15/1 |  | ✓ |
|  | (E/C or E/T) | 16/1 |  | ✓ |
|  | (E) | 17/1 |  | ✓ |
|  | (E) | 17/2 |  | ✓ |
|  | (E) | 17/3 |  | ✓ |
|  | (E) | 17/4 |  | ✓ |
|  | (E) | 18/1 |  | ✓ |
|  | (E) | 19/1 |  | ✓ |
|  | (E) | 19/2 |  | ✓ |
|  | (E) | 19/3 |  | ✓ |
|  | (E) | 19/3** |  | ✓ |
|  | (E) | 19/3** |  | ✓ |
|  | (E) | 19/4 |  | ✓ |
|  | (E) | 19/5 |  | ✓ |
|  | (P) | 19/6 |  | ✓ |
|  | (I) (P) | 19/7 |  | ✓ |
|  | (C) (P) | 19/8 |  | ✓ |
|  | (E) | 19/9 |  | ✓ |
|  | (C) (P) | 20/1 |  | ✓ |
|  | (E) | 20/2 |  | ✓ |
|  | (E) | 21/1 |  | ✓ |
|  | (E) | 22/1 |  | ✓ |
|  | (E) | 23/1 |  | ✓ |
|  | (E) | 23/2 |  | ✓ |
|  | (E) | 24/1 |  | ✓ |
|  | (E) | 25/1 |  | ✓ |
|  | (E) | 25/2 |  | ✓ |
|  | (E) | 25/3 |  | ✓ |
|  | (E) | 26/1 |  | ✓ |
| VolumeA | (E) | 26/2 | ✓ |  |
|  | (E) | 26/3 |  | ✓ |
| VolumeA | (E) | 26/4 | ✓ |  |
| VolumeA | (E) | Annex A | ✓ |  |
|  | (E) |  |  |  |
|  | (E) |  | ✓ |  |

#### Notes to Compiler: ** These Appendices relate to alternatives in the Sample Appendices where the choice is to be made by the Contractor and all appropriate alternatives should be listed in List A.

LIST B

| Volume No. | Completed By | Appendix | Used | Not Used |
| --- | --- | --- | --- | --- |
| VolumeA | (E) | 26/70 | ✓ |  |

# Appendix 0/4

List of Drawings included in the Contract

1 Contract Specific Drawings supplied to each Tenderer

Refer to drawings folder on Volume A

2 Standard Drawings

| Drawing No. | Title | Volume No. |
| --- | --- | --- |

2 (ii) Inspected by Tenderers

The following drawings are made available for inspection by Tenderers at:…………...........…………………………………………………………………………………………...…………………………………………………………………………………………….

Between the following

dates……………………………………………………..………..………………………………..

and at the following

times………………………………………………………..………..……………………………..

One copy will be supplied to the contractor

| Drawing No. | Title | Volume No. |
| --- | --- | --- |

- (iii) Brought to Into the Contract by Reference

- Road Construction Details (RCD) published by National Roads Authority (NRA) as Volume 3 of the Manual of Contract Documents for Road Works.

- All relevant Standard Details of Telecommunications, electricity, gas, broadband and other statutory undertakers.

APPENDIX 1 / 4

working and fabrication drawings

As Constructed Drawings

The Contractor shall provide “As Constructed” Drawings on CD using AutoCAD (.dwg) format which shall indicate all revisions to the detailed design.

Fabrication Drawings

Drawings of proposed alternative designs / construction items shall be produced in AutoCAD format, Microsoft Word or Excel format as appropriate and issued for consideration and sign off by the Engineer.

APPENDIX 1 / 5

testing to be carried out by the contractor

- Tests carried out by the Contractor to demonstrate compliance with the Specification shall be carried out by a laboratory with ILAB accreditation and approved by the Engineer.

- Testing is to be carried out by the Contractor in accordance with the National Roads Authority Manuals of Contract Documents for Road Works, Specification for Road Works, Volume 1 and in accordance with the table below.

- Testing shall be carried out immediately after sampling and results shall be passed to the Engineer on completion of testing.

- Additional testing shall be carried out by the Contractor as and when instructed by the Engineer.

| Clause | Works, goods or materials | Test | Frequency of Testing | Comments |
| --- | --- | --- | --- | --- |
| Series 800: Clause 804 | Granular Material Type B | Grading 10% fines (IL) Flakiness Index (IL) | Monthly Monthly Monthly | Engineer to be notified of changes to batches, supplier |
| Series 900: Clause 901, 925 | Aggregates for bituminous materials Binders for bituminous materials | 10% fines (IL) Impact Value (IL) Soundness (IL) Sieve Test (IL) Flakiness Index (IL) (crushed rock gravel) PSV (IL) & AAV (IL) (coarse aggregate for wearing courses) Penetration (IL) Softening Point (IL) | Monthly Monthly 1 per source Monthly Monthly 1 per source 1 per 750 tonne 1 per 750 tonne | Engineer to be notified of changes to batches, supplier |
| Series 900: Clause 903 to 914, 916, 925, 930, 932 to 934 | Bituminous Mixtures | Grading (IL) Binder Content (IL) | 1 per 75 tonne (min. 2 per day) | Engineer to be notified of changes to batches, supplier |
| Series 900: Clause 915, 925 | Coated chippings | Grading (IL) Binder Content (IL) Flakiness Index (IL) PSV (IL) AAV (IL) Hot Sand test (IL) Rate of Spread | 1 per stockpile 1 per stockpile 1 per source 1 per source 1 per source 1 per source On site, several times daily as directed by the Engineer | Engineer to be notified of changes to batches, supplier |
| Series 900: Clause 921 | Surface texture of bituminous wearing course | Texture Depth (sand patch method) (IL) Texture Depth (TRL mini texture meter) | 1 per day (set of 10) |  |

APPENDIX 1 / 6

supply and delivery of samples to the engineer

The following samples are to be provided or made available by the Contractor for testing by the Engineer:

| Clause | Sample Description | Frequency of Samples | Delivery Location | Comments |
| --- | --- | --- | --- | --- |
| 903 to 914, 916, 925, 930, 932 to 934, 942 | Bituminous mixtures | On demand | On site, to the Engineer | Engineer to be notified of changes to batches, supplier |
| 915, 925 | Coated chippings | On demand | On site, to the Engineer | Engineer to be notified of changes to batches, supplier |

Notes

- Samples comparable to those specified in this Appendix will be necessary for any equivalent work, goods or materials proposed by the Contractor (See sub-Clause 105.4).

- Unless otherwise shown in this Appendix, samples of work, goods or materials as scheduled under any one Clause are required for all such work, goods or materials in the Works.

- Unless otherwise scheduled under Clause 2602 samples of concrete complying with that Clause are not required.

APPENDIX 1 / 7

site extent and limitations on use

# 1. 	Extent of the Site

1.1	The extent of the Site shall include inter alia:

- The lands made available by the Employer as shown on the Contract Drawings as listed in Appendix 0/4 of the Specification.

- Areas required for traffic management measures by the Contractor in compliance with Clauses 117 and 118 of the Specification.

- Areas required for the installation alteration and removal of plant for statutory or other bodies. The use of these areas will be limited by the terms of the way-leaves acquired by the statutory or other bodies for execution of the works.

- Lands necessary for the construction of Accommodation Works in accordance with the Contract that may not be made available by the Employer.

# 2.	Limitations on the Use of the Site

- Prior to entering parcels of land not acquired in their entirety, the Contractor shall erect suitable temporary/permanent fencing taking into account the requirements of adjacent land usage.

- Where public roads are included within the extent of the Site, the Contractor shall maintain adequate traffic routes across, within and through the Site, or temporary diversions for traffic in accordance with Appendix 1/17 & Appendix 1/18 of the Specification.

- The Contractor is limited in the use of public roads as a means of access to the Site as stated in Appendix 1/19.

- The Site shall be used solely for the execution of the Works.

- The Contractors attention is drawn to Appendix 1/9 of the Specification concerning limitations on noise and vibration. The normal working hours on the Site shall be Monday to Friday between 08.00 and 18.00 hours and Saturday between 08.00 and 13.00hours with no working on Sundays and public holidays, unless otherwise restricted to comply with traffic management restrictions in appendices 1/17, 1/18 and 1/19. Exceptionally, the Employer’s Representative’s consent for work outside these hours may be given after necessary consultation and written approval. A minimum of 14 days notice shall be required from the Contractor when seeking such consent.

- The Contractor shall note and have regard to the presence of other contractors on parts of and adjacent to the site.

- The Contractor shall not use areas of land with a temporary right of access for any purpose other than the execution and completion of the Works. The Contractor shall minimize the area of land occupied to that which is essential for the safe execution and completion of such part of the Works. The Contractor shall reinstate all areas of land which have been temporarily occupied to the satisfaction of the affected landowner, occupier and the relevant Authorities, Utilities Service Providers and Private Utility Services and other companies.

- Access for pedestrians, cyclists and vehicular traffic shall be maintained to all properties affected by the Works.

- Accommodation Works on lands not made available by the Employer shall be executed and completed as soon as possible so that inconvenience to the property owners and occupiers shall be minimized.

- Where the execution and completion of the Works affects lands and property the Contractor shall provide suitable temporary access having regard to the use of the lands and property.

- Notwithstanding the other provisions of the Contract the Contractor shall not have exclusive use of the Site and shall be aware that third parties shall be operating within the extent of the Site. Without limiting the previous sentence, those areas defined in Appendix 1/13 of the Specification and within this Appendix 1/7 shall not be for the sole use of the Contractor.

- The Contractor’s use of the Site shall be limited to the constraints and restrictions contained within this Appendix 1/7, Appendix 1/13, Appendix 1/17, and Appendix 1/19 of the Specification.

# 3.	Archaeological Investigations

- As this is a resurfacing/re-construction project there was no requirement for an Archaeological Investigation. No Archaeological exclusion Zones are located within this Contract. If a exclusion Zone is identified the Contractor shall not execute work in Archaeological exclusion Zones until all archaeological resolution works shall have been completed in these areas and shall take cognisance of this when programming the Works. Notwithstanding any other provision of this Contract, the Contractor shall consult and comply with the Project Archaeologist regarding the archaeological exclusion zones, until the necessary resolution works shall have been completed.

# 4.	Adjacent Works

The contractor shall co-operate with Louth County Council in relation to any adjacent works being undertaken by or on behalf of the Council.

# 5.	Licences

- In accordance with the Contract Drawings, the Road Opening Licence for each location will be deemed to have been applied for upon receipt of a site specific Traffic Management Plan prepared in accordance with the Traffic Signs Manual Chapter 8 “Temporary Traffic Measures & Signs for Roadworks”, October 2008 as issued by the Department of the Environment or any amendment thereof.

APPENDIX 1 / 9

control of noise and vibration

Noise

- The local authority has informally agreed that the following measures would be appropriate and these are given as a guide; however it is for the Contractor to decide whether to seek the Local Authority’s formal consent to his proposed methods of work and to the steps he proposes in order to minimise noise.

- The normal working hours within the site shall be Monday to Friday between 08.00 hours and 18.00 hours and Saturday between 08.00 and 13.00, with no working on public holidays. In exceptional cases, the Engineer’s consent for work outside these hours may be given after any necessary consultation. The Contractor shall give seven days notice when seeking such consent.

- The noise levels for periods outside of the normal working hours will only be permitted when consent has been given to exceptional working.

- The ambient noise level, Leq from all sources when measured 2.0 metres above the ground and at 1 metre from the façade of the nearest noise sensitive office, dwelling, etc. shall not exceed the level given in the table below.

| Period |  | Ambient Noise Level | Period over which Leq Applicable |
| --- | --- | --- | --- |
| Monday to Friday (0800 to 1900 Hrs) | Leq (15min) | 75 dB (A) | 1 hour |
| Monday to Friday (1900 to 2200 Hrs) | Leq (15min) | 60 dB (A) | 1 hour |
| All unattended plant outside normal working hours | Leq (15min) | 50 dB (A) | 18 hours |

Notes:

(i)    	Noise levels relate to free field conditions. Where noise control stations are located 1m from facades of buildings, the permitted noise levels can be increased by 3dB(A).

(ii)   	The ambient noise level, Leq, at a noise control station is the total Leq from all the noise sources in the vicinity over the specified period.

(iii)  	The existing ambient noise level, Leq, at a control station is the total Leq from all the noise sources in the vicinity over the specified period prior to the commencement of the Works.

(iv)   	Maximum sound level is the highest value indicated on a sound level meter which meets the requirements of BS 5969 Type 1 or 2 set to SLOW response, and frequency weighting A.

- The Contractor shall employ the best practical means to minimise noise produced by his operations including plant maintenance and shall comply with the recommendations in BS5228, 1997 (Noise Control on Construction and Open Sites)

- All vehicles and Contractor’s mechanical equipment used for the purpose of the works shall be fitted with effective exhaust silencers and shall be maintained in good and efficient working order.

- All compressors shall be “sound reduced” models fitted with properly lined and sealed acoustic covers and shall be kept closed whenever the machines are in use.

Vibration

- The Contractor shall select and utilise methods for working and items of plant so that the maximum measured ground vibrations do not exceed the following peak particle velocities:

  - 8.5mm per second at any occupied property.

  - 10mm per second at any other property.

  - 15mm per second at existing trunk watermains and gasmains traversing and in close proximity to the site.

- The Contractor shall provide instrumentation suitable for monitoring vibration when required by the Engineer. Such monitoring shall include locations outside the limits of the site. The Engineer is to be provided with copies of all vibration records.

Control of Dust

- The Contractor shall provide, use, maintain and keep available plant and equipment necessary to minimise the forming and accumulation of dust arising from the works, with particular reference to dry weather conditions.

APPENDIX 1 / 12

setting out and existing ground levels

- The Contractor shall set out the Works by reference to Ordinance Survey Datum (Malin Head), to which all levels quoted in the Drawings will refer. Horizontal alignment shall be determined in accordance with National Grid co-ordinates, to which all reference to Northings and Eastings in the Contract Documents will refer.

- The Contractor shall set out, mark and maintain until no longer required, all reference lines, bench marks and markers, necessary for the setting out and checking of the works.

- The Contractor shall not commence either general site clearance or fencing before obtaining the approval of the Engineer to the setting out of site boundaries.

- The design of the Works uses Computer Aided Design software. The Contractor should be capable of accepting information on CD and magnetic media to AutoCAD 2000 (.dwg) format, Adobe Reader 6 (.pdf) format and plot files in HPGL2 format.

- Preparation of some of the setting out information involved the use of AutoCAD Civil 3D and the Contractor should be capable of accepting this information in the form of electronic files.

- The position and level of Contract Bench Marks shall be agreed with the Engineer. Contract Bench Marks shall be within the works area.

- The Contractor shall not destroy any Permanent Bench Mark until a replacement subsidiary permanent control marker has been established in a position agreed with the Engineer.

APPENDIX 1 / 13

Programme of works

# The Contractor shall provide the programme in the form of a bar chart, produced as a result of a “Critical Path Analysis” and must abide by the constraints stated or implied in the Contract. It shall show the level of detail appropriate to each stage of the Works, and all activities and restraints, each of which shall be given a short title. All events shall be numbered and annotated with earliest and latest event dates. The programme must show plant and labour resources, as well as anticipated outputs for each activity and required each flows. The critical path analysis referred to above shall be provided to the Engineer.

# At the time of presentation of the programme the Contractor shall also provide a mass-haul diagram showing his intended earthworks movements and locations and capacities of anticipated plant and other resource input.

# Schedule of Stated Constraints:

  - Work to privately and publicly owned services and supplies

  - Traffic safety and management including notice requirements and restrictions arising from the use of substances hazardous to health.

  - Provision of environmental protection prior to the main construction operation.

  - Trials and demonstrations in advance of main construction.

  - Approvals by the Employer’s Representative of domestic sub-contractor nominations.

4.	The level of detail should be not less than the following:

## Level 1

Submitted in accordance with the Conditions of Contract Clause 4.9 and any subsequent revision.

- Roadworks – in lengths not exceeding 1.0km for main route and for each side road, link road and slip road:

  - Site Clearance.

  - Drainage (pre-earthworks and second stage).

  - Sub-base.

  - Subgrade improvement level.

  - Crack and Sealing of existing Concrete Slab Pavement.

  - Road-base or concrete paving.

  - Surfacing.

- Major privately and publicly owned services and supplies.

- Traffic management measures including operation of site accesses, plant

crossings and temporary diversions of traffic.

Level 2

At least four weeks before the commencement of any item of work:

- Roadworks

As for Level 1 but intervals not exceeding 200m and including lighting, signing, soiling and seeding, road-marking, cabling and communication equipment.

(iii)	All public alterations or additions to privately and publicly owned services and supplies.

Level 3

Further breakdown of items and other details as may be required.

APPENDIX 1 / 14

monthly statements

# Monthly Statement

The monthly statements submitted to the Engineer by the Contractor shall, in accordance with Clause 11 of the Conditions of Contract, whenever dealing with matters covered by the Pricing Document, be set out under part and section headings similar to those in the Pricing Document and shall separately identify each item and specify quantity, unit, rate and value.

APPENDIX 1 / 16

privately and publicly owned services and supplies

- Details of preliminary arrangements that have been made with Statutory Undertakers and others for the alteration of services affected by the Works are listed in this Appendix.

- The Contractor shall, prior to commencement of the works and during the progress of the Works, liaise with each statutory undertaker and obtain a copy of any special requirements or codes of practice from the individual statutory undertaker. The Contractor shall take all measures required by any statutory undertaker for the support and full protection of all services or supplies in accordance with the relevant codes of practice or special requirements. He shall keep the Engineer informed of such liaison and measures.

- The Contractor shall make arrangements with the Statutory Undertakers and others concerned, for the co-ordination of his work with all work which needs to be done by them or their contractors concurrently with the Works. Compliance with the periods of notice given in this Appendix does not relieve the Contractor of his obligations.

All cabling and jointing will be carried out by the Statutory Undertaker. The Contractor will carry out the following:

- Excavation for pipes and chambers.

- Construction of chambers, including covers.

- Backfilling of trenches and permanent reinstatement as appropriate.

- Bedding, laying and surrounding of pipes and ducts.

- Jointing of pipes and ducts.

- Private services to individual properties have not generally been listed or shown on the Drawings.  The Contractor shall make arrangements with the Statutory Undertakers and others concerned for the phasing of all necessary disconnections and diversion of private services affected by the Works.

- Disconnected apparatus shall be removed by the Contractor only with the prior consent of the Authority concerned.

- The names, addresses and telephone numbers of the authorities serving in the locality are listed below:

Schedule of Contacts for Private and Publicly Owned Services and Suppliers

| Name | Addresses and Tel No. | Contact |
| --- | --- | --- |
| Statutory Undertakers | Louth County Council, County Hall, Crowe St, Louth 042-9392900 ESB Networks, Long Avenue, Louth Airtricity , Louth, Sean Lynch 0872881648 BGE, Michael Gallagher, 087-7781228 |  |
| Other Authorities |  |  |

- Services and Supplies Affected by the Works

| Location | Description | Group* | Drawing No. | Notice Required to Commence | Time for Completion |
| --- | --- | --- | --- | --- | --- |
| Statutory Undertakers |  |  |  |  |  |
| Other Authorities/ Bodies/ Individuals |  |  |  |  |  |

*

A	Work expected to be completed before the commencement of the Works

B	Work required in the Contract which does not require prior work by the Contractor

C	Work required in the Contract which does require prior work by the Contractor

D	Work expected to be under construction at the Commencement of the Works

E	Work required to be carried out jointly by the Contractor (all civil works) and the Statutory Undertaker (Cabling Only) in close liaison and co-operation with each other.

F 	Work to be wholly undertaken by the Contractor (in liaison with the statutory undertaker)

- Other preliminary arrangements

APPENDIX 1 / 17

traffic safety and management

# Traffic Safety and Management Requirements

## The Contractor shall be responsible for the planning, design, implementation, maintenance and removal of traffic safety and management measures required in order to facilitate the work.

  - The Contractor shall comply at all times with the requirements of “Traffic Signs Manual Chapter 8 Temporary Traffic Measures & Signs for Roadworks”, October 2010 as issued by the Department of the Environment or any amendment thereof, and Guidance for the Control and Management of Traffic at Road Works (2010) prepared by the Local Government Management Services Board and any additional requirements detailed in the Design Manual for Roads and Bridges.

  - If the contractor decides to utilise the initial traffic management proposal as prescribed by the PSDP, he shall further develop it as set out herein under and finalise its inception through the PSCS. In the event that an alternative plan is proposed, it shall as a minimum meet the requirements as set out above and herein under. The PSDP shall be provided with a copy.

## The Contractor shall submit, three weeks prior to the commencement of each traffic management phase of the works, to the Employers Representative, detailed traffic management plans, including the following information as a minimum:

- Phasing of the works at each location;

- Drawings showing the traffic management layout, including:

(i)	Geometric Design

(ii)	Width of Lanes

(iii)	Working Areas

(iv)	Safety Zones

(v)	Access and exit locations for construction vehicles

(vi)	Barriers

(vii)	Signing

(viii)	Road markings

(ix)	Temporary lighting

(x)	Provision for pedestrians; including proposals to prevent pedestrians from entering, walking along and crossing the sites, and the provision of safe crossing areas.

(xi)	Provision for emergency services

- Timing of operations.

- Road lighting.

## Drawings showing the Contractor’s Traffic Management plans shall be to a scale not less than 1/2000, supplemented by drawings at 1/500 scales as necessary, or as required by the Employers Representative.

## The Contractor’s traffic management proposals shall take account of the work area required for paving and the location of construction joints.

## The Contractor shall be subject to the full statutory procedures outlined in the Temporary Closing of Roads Regulations.  The period of closure shall be the minimum to facilitate construction of the works.  All applications relating to road closures, lane occupations, signs or signals must be submitted to the Employers Representative in writing and require the following notice:

i)	For making road closures 		6 weeks

ii)	For non-prescribed signs 		4 weeks

- For temporary traffic signals 		4 weeks

A temporary road closure shall only be permitted in circumstances where the Contractor can demonstrate to the Employers Representative that such a closure is absolutely necessary for the execution of the works and in accordance with a risk assessment report.

## The Contractor shall be responsible for the maintenance of all public roads, site access roads and temporary diversions within the site until issue of the certificate of Substantial Completion. The Contractor shall constantly monitor the public roads while any Lane and/or Carriageway Occupations are in force during the execution of the Works.  Any defects identified shall be rectified immediately to the satisfaction of the Employers Representative.

## The Contractor shall keep a daily record of all defects, the times when they were identified or reported to him, the action taken to correct the defects, and the times when they were successfully corrected.  A copy of this record shall be forwarded to the Employers Representative on the following day, until the completion of those Works requiring the Lane or Carriageway Occupation.

## The Contractor shall provide and maintain access to all existing properties adjacent to the Works. The Contractor shall liaise with all owner/occupiers where access to/from their property may be affected by the works. This shall include (but not be limited to) consultation & agreement of suitable arrangements to be made with An Garda Síochána, Iarnród Éireaann & the management of Clarke train station, public bus services, and business/commercial premises affected by the works. Letter drops to businesses, residents and other parties affected by the works at each site shall be carried out prior to the works and, if necessary, during the works in order to keep all affected parties fully & properly informed.

## In the event of a traffic accident occurring adjacent to any of the Works, the Contractor shall immediately contact the Gardaí and the Employers Representative informing them of the following:

a)	Location of the accident

b)	The seriousness of the accident and whether any persons are trapped, whether the collision involves vehicles carrying inflammable, corrosive or hazardous substances, whether there is a possibility of ignition from leaking fuel or chemicals.

## Liaison with An Garda Síochána shall be through the following office:

Louth Garda Station, The Crescent, Louth

The Contractor shall provide telephone numbers of a minimum of 3 employees who can be contacted by the Garda Síochána and/or Employers Representative, both during and outside normal working hours, and who shall be responsible for initiating whatever action shall reasonably be required in the event of an emergency.

## All drivers including those delivering plant and materials must be given clear instructions regarding the traffic arrangements applicable at any particular time.

## The Contractor shall be responsible for maintaining the running traffic carriageway and any pedestrian routes adjacent to the Works in a clean and safe condition at all times: they shall be kept free of mud, debris & dust through regular road sweeping & cleaning as and when required and at the direction of the Engineer.

## The Contractor shall assist An Garda Síochána in moving wide/abnormal loads through the Works by modifying the signing/coning as necessary.  Signs/cones so moved shall be replaced immediately the abnormal loads have passed through the Works.

## Heavy Goods Vehicles used on Site by the Contractor, his Sub-Contractors or suppliers must be fitted with an audible reversing warning device.

## Suitable locations shall be agreed with the An Garda Síochána and the bus service operators for temporary bus stops where existing facilities are affected by the works.

# Minimum Provisions for Vehicular Traffic for Static Operations

  - General

Static operations are those carried out within a fixed work site area. The length of a fixed site should be kept to the minimum required to carry out the operation safely.

  - When designing temporary traffic measures the optimum lane width for all

classes of vehicles is 3.25m. This may be reduced to a minimum of 3.0m.

Below this, HGVs and buses must be marshalled past the works. The absolute minimum lane width, if only cars and light vehicles are present, is 2.5m.

  - The residual road space which forms the traffic lane for one-way traffic should be a minimum width of 3.25m and maximum lane width of 3.7m. In difficult circumstances, the single lane width may be reduced as noted above.

  - For two-way traffic the minimum road width should be 6.0m. Where there are two lanes of traffic travelling in the same direction the near-side lane should be a minimum of 3.25m and the off-side lane a minimum of 3.0m.

  - Where temporary traffic measures are likely to have a significant impact on the capacity of the road, the anticipated queue lengths and delays should be estimated by the Designer. Traffic management should be designed to

minimise delays. It is important that WK 001, Roadworks Ahead, and the

subsequent warning sign WK 062, Queues Likely, be positioned to ensure that, regardless of queue length, approaching traffic is forewarned of queues ahead.

  - Flagmen shall be used to provide variable ‘green’ time commensurate with actual demand at all times during working hours. Outside working hours temporary traffic signals may be considered, subject to a risk assessment. The Contractor’s proposal together with the risk assessment shall be submitted to the Employer’s Representative

  - Where a shuttle system is to be put in place, then the maximum length of work site should be kept to a minimum, with a recommended maximum of 500m. In urban areas it will be necessary to limit the work site to shorter lengths.

  - In the case where the capacity is not greatly reduced and the permanent

number of lanes is maintained, the length can be increased but only to facilitate continuous operations. Long lengths of restricted width traffic lanes, with no apparent activity being carried out, will tend to encourage drivers to disregard the signage and directions being given, with attendant risks to site operatives and drivers as a consequence of inappropriate driver behaviour. It is recommended that the maximum length should not exceed 5km.

  - The minimum length to the next section of roadworks on the same route should ideally be at least 10km, or as agreed with the Road Authority. This

requirement is to allow platoons of traffic to dissipate and to avoid continued

delays and driver stress. Designers of roadworks close to a Road Authority

boundary should consult with the Road Authority in the adjoining county in order to plan and co-ordinate roadworks.

  - Advance signs are used to alert the driver to an unexpected road layout or hazard ahead. To achieve the desired effect the use of signs must be

standardised and appropriate. It is important to ensure that the end of the

roadworks is signed.

  - If permanent road signs display a contradictory message to the roadworks signs, they must be covered for the duration of the works and the relevant signs uncovered or reinstated during the removal of the temporary traffic measures.

  - There will often be a requirement to implement different stages of temporary traffic measures as the works progress. Each subsequent stage will be governed by the same design parameters but could present an additional risk to the workforce and road user as the layout of the road may differ from what they had become used to. It is important for each stage to be properly

engineered to manage the traffic as necessary and to be marked and signed clearly to avoid confusion.

  - For short-term work sites on roads with speed limits of 60km/h or less, where the traffic lanes are shifting daily due to the nature of the works, then cones or temporary barriers with steady beacons can be used to channel the traffic. However, the temporary traffic measures must clearly define, over and above any existing markings, the route through the works, especially on un-lit roads during hours of darkness, or poor visibility. For a similar scenario on roads with a speed limit in excess of 60 km/h, where the use of cones may not impart a clear layout then temporary road markings or road studs will normally be required.

  - On an existing road with road studs, if temporary traffic measures are required during the hours of darkness or in poor visibility then temporary road studs should be provided. This is necessary as the visual impact on the driver is affected if a short section of road has no road studs and could possible lose their positioning.

  - Where there are two or more lanes of traffic travelling in the one direction through the works, it is recommended temporary road studs be used as it is important to ensure appropriate delineation is provided between the lanes. Any conflicting road studs or markings should be removed or obscured.

  - During the programme of the works consideration should be given to the

possibility of removing the traffic management measures in order to deal with:

- Particularly high traffic volumes due to a sporting or other event;

- Adverse weather conditions;

- Emergency access; or

- Times when work is not in progress.

  - Once the works are completed, the removal of traffic management measures should only be carried out if it is possible to ensure the road is safe and free from any obstructions, all road surfacing is completed and all permanent signs, road markings, safety barriers and other items are in place.

## At all times when one-way shuttle working is in operation, a sufficient number of suitable operatives are to be in attendance solely for traffic management duties.

# Pedestrian Traffic

## The Contractor shall programme the works with the aim of ensuring the safety of pedestrians/cyclists at all times while keeping disruption to pedestrians/cyclists to an absolute practical minimum and shall submit to the Employers Representative a method statement/Pedestrian management plan to demonstrate how he intends to achieve that aim. This shall include the contractor’s proposals to ensure that pedestrians and cyclists do not enter, travel inside or cross the sites and his proposals for designated safe crossing points.

# Details of Events that could have a Bearing of the Works

Race meetings, football fixtures, etc should not have a bearing on the works.

Roads including footpaths, cycle tracks and other traffic routes, described above or listed in Appendix 1/19 are the responsibility of:

| Authority | Louth County Council |
| --- | --- |
| Address: | County Hall, Millenium Centre, St. Alphonsus Road, Dundalk. Co. Louth |
| Tel No: | +353 (0)42 9392900 |

APPENDIX 1 / 19

routing of vehicles

- Permitted Access Routes to and From the Site

Access to the Site for the Works Traffic shall only be by means of public roads open for public use.

- The Use of the Permanent Works by Construction Traffic

Areas of the finished pavement forming part of the Permanent Works shall only be used to carry construction traffic when approved by the Engineer.

- Movement of Machinery and Plant Across Public Roads

No item of the Contractor’s Equipment or Vehicles used by the Contractor or his sub-contractors in carrying out the Works shall be operated on the public highway in such a manner that it would disrupt the normal flow of traffic.

- Temporary Structures for Construction Traffic Spanning Areas Used by the Public

None required.

APPENDIX 1 / 22

progress photographs

- The Contractor shall appoint a professional photographer to take and record the photographs of the Works. The photographer shall be approved by the Engineer. These photographs shall cover such extent of the Works as the Engineer may direct.

- The progress photographs shall include a set of photographs taken at each site location/works area prior to commencement of the works at that location, to stand as a record of the prior condition of that site location/works area.

- All prints shall be marked on the reverse side with the date of exposure, name and address of photographer, identification reference number and brief description of the Work, for future reference, including chainage and direction of view.

- The copyright of all photographs shall be vested in the Employer and the negatives and prints shall be delivered to the Engineer within four (4) weeks of the exposure. The photographs shall not be used for any purpose whatsoever without the Engineer’s approval.

APPENDIX 2 / 5

hazardous materials

- The appropriate measures must be taken to safeguard the health of the Contractor’s operatives and the general public.

- Hazardous materials arising from site clearance and/or excavations shall be disposed of only at sites licensed by Louth County Council or other licensing authorities.

- Contractor must comply with all relevant Irish & European Law in relation to hazardous materials.

APPENDIX 5 / 1

drainage requirements

- The Hydraulic Design of the surface water pipe system is based on the Modified Rational Method using a pipe roughness of 0.6mm. the run-off calculations are based on the Sustainable Urban Drainage System (SUDS).

- The surface water and foul drainage requirements are indicated on the contract drawings and include the following:

- Permitted pipe and bedding combinations.

- Trench construction details.

- Chamber and gully types including covers, gratings and frames.

- Details of connections to existing drains and sewers.

- Joints in surface water pipes shall be watertight.

- Chambers for foul sewers shall be tested for water tightness in accordance with the requirements of the Sanitary Services Department of Louth County Council.

- Plastic pipes shall have an ultimate pipe stiffness (STES) in excess of 1,400 N/m2, tested in accordance with BS4962. Resistance to impact shall comply with BS4962, except that the striker used in the test shall have a mass of 1kg and a 25mm hemispherical radius. The Contractor shall submit test certificates to the Engineer prior to the installation of any plastic pipes.

- The Contractor is responsible for ensuring that all pipelines, manholes, gullies, concrete channels, ACO drains, flow control, petrol interceptor, etc. are clear of obstructions and are sited as not to damage or obstruct access to or working of Utilities’ plant and services.

- All trenches beneath or within 1.5 metres of the carriageway shall be backfilled with selected fill material unless otherwise stated on the Contract Drawings. Selected fill whether from locally excavated material or imported, shall consist of readily compactable material, free from vegetable matter, building rubble and frozen material, or materials susceptible to spontaneous combustion, and excluding clay of Liquid Limit greater than 80 and/or Plastic Limit greater than 55 and materials of excessively high Moisture Content. Clay lumps and stones retained on 75mm and 37.5mm sieves respectively shall be excluded from the fill material.

- All manhole covers, gratings and frames shall be Grade A Ductile Iron to BS497.

Covers, gratings and frames to chambers of sizes which are not covered by BS497, manhole covers to be Class C in footpaths and Class D in roads shall generally comply with the requirements of BS497.

- Surface Water Drainage Requirement

| Type | Material | Depth | Standard | Extra Requirements |
| --- | --- | --- | --- | --- |
| 225mm diameter pipe | Concrete (with Portland Cement) | Not exceeding 2.0m with average of 1.4m | - BS5911 Part 1 and Part 3 | - Class H spigot and socket point - Condition of laying the pipes: - Under the grass verge/greenspace: Class B bedding shall be used. - Under the footpath/driveway: Class E bedding shall be used. - Under the road: Class F bedding shall be used. - Laying, grading and surrounding of pipes shall comply to Cl. 503.1, Cl. 503.3 (i) and (iii) - Granular type of materials for bedding and surround shall be to Cl. 804. |
| 150mm diameter pipe | Unplasticised polyvinyl chloride (uPVC) | Not exceeding 2.0m with average of 1.4m | - BS4660 - BS5481 - IS424 | - All pipes shall be laid in accordance to Type G: RCD/500/2 - Jointing of pipes shall comply to Cl. 504.3 (ii), (iii) and (v) |
| Drainage Channel Block Type B | Concrete | - | - Cl. 1101 - BS7533-3: 2005 | - All channel block shall comply with Cl. 1101 - All channel block shall be laid in accordance with BS7533-3: 2005. |
| ACO drain type N100K (or similar approved by Engineer) | Resin concrete | Not exceeding 150mm | Supplier Specification and Deliveries | - ACO drain or similar approved by Engineer shall be used in constant invert and level ground. The bedding and laying shall be in accordance to Supplier Specification and subject to Engineer approval. |
| - The drainage shall comply with Cl. 509 after completion of the whole works or otherwise stated by Engineer. - Extra requirements above are subject to Engineer approval. |  |  |  |  |

- The Table below are the requirements for Surface Water Manholes

| Size | Material | Depth | Standard | Extra Requirements |
| --- | --- | --- | --- | --- |
| 900mm diameter | Concrete (with Portland Cement) | Not exceeding 2.0m | - BS5911 - Cl. 507 | - Manholes are to be constructed to RCD/500/6 - The chamber rings shall be surrounded with C30 120 concrete of 150mm minimum thickness unless shown otherwise on the Drawings. - The foundation of the finished formwork shall be in accordance to Cl. 507.1 and Cl. 507.2. - The manhole steps shall comply with BS1247: Part 1 or Part 2. - All manholes on pipes 600mm diameter and over shall be provided with safety chains as per RCD/500/6. - Bases and benching shall be rendered in hydrophilic concrete 50mm thick finished smooth and shall slope towards the main channel at a slope of 1 in 10 or as per RCD/500/6. - All the conditions above are subject to Engineer Approval. |
| 1200mm diameter | Concrete (with Portland Cement) | Not exceeding 2.0m | - BS5911 - Cl. 507 |  |
| 1800mm diameter | Concrete (with Portland Cement) | Not exceeding 2.5m | - BS5911 - Cl. 507 |  |

9.	Flow Control for Attenuation – Not Required

1 -	Flow Control shall be formed of hydrobrake (Type 194 SH, 188 CH, 197 5xH, 120 5xH) or similar approved by Engineer.

2 - 	The Contractor shall install the Flow Control in accordance with standard installation details provided by approved supplier.

3 - 	Contractor shall bear the charges if they require assistance from the supplier representative(s).

10.	Petrol Interceptor – Not Required

1 -	Type of Petrol Interceptor shall be Klargester NSBD08 or equivalent approved by Engineer.

2 - 	The Contractor shall install the Petrol Interceptor in accordance with standard installation details provided by approved supplier.

3 - 	Contractor shall bear the charges if they require assistance from the supplier representative(s).

11. 	Attenuation Basin - Not Required

APPENDIX 5/2

service duct requirments

- Trench construction for ducts is shown on the Contract Drawings.

- Ducts for cabling to lighting columns shall be uPVC, 5mm wall thickness, and orange in colour with an internal diameter of 100mm, marked clearly indicating Street Lighting.

- The Contractor shall provide “As Built” Drawings suitably marked up to show the exact location of all ducts.

- Details of duct construction [503.5]

- A schedule of service duct requirements

- Details of permanent marker blocks and location posts required for service ducts [505.5]

APPENDIX 5/6

drainage and service ducts – nra road construction details

| Clause No. | Road Construction Detail Drawing No. |
| --- | --- |
| 501.8 | RCD/500/50 |
| 503.3 (i) & (iii) | RCD/500/02 |
| 508.1 | RCD/500/14, RCD/500/15 and RCD/500/16 |
| 509.9 | RCD/500/51 and RCD/500/52 |
| 517.4 | RCD/500/01 |

APPENDIX 7 / 1

Pedmitted Pavement Options

Sheet 1 – Flexible or Flexible Composite

- Location – all locations, as appropriate

- Grid for checking surface levels of pavement courses: as per 702.4

- Surface Regularity: to Clause 702.7

- Requirements for determination of compaction level – as per sub-clause 901.19 and 927.1

- Coated Chippings (different requirements of Clause 910.5, 911.8 and 915)

Nominal Size			20mm (HRA) & 14mm (SMA)

Minimum PSV			65

Maximum AAV		10

Maximum Flakiness Index	20

- Requirements for hardness, durability and cleanliness of aggregates – as per sub-clause 901.2

- Requirements for Regulating Course: Clause 907

|  | Clause | Material | Grade of Binder | Thickness (mm) | Special Requirements |
| --- | --- | --- | --- | --- | --- |
| Surface Course | 910 | Hot Rolled Asphalt (Recipe Mix) | 50 Pen Per Clause 910.2 | 45 | BS594: Part 1 Table Nos.: 6 Column Nos.: 6/4 Marshall Stability Range: 4-8 kN Marshall Flow: 5mm |
| Surface Course | TBC | *Stone Mastic Asphalt | 40/60 Pen with fibres or polymer modified bitumen | 45 | 14mm High PSV Binder content 5.7%+/- 0.3% |
| Surface Course | 912 | DBM | 100 Pen Or 200 Pen as per Clause 912 | 45 | PSV not less than 45 |
| Binder Course | 906 | DBM | 100 PEN | 55 | BS4987, Group 2 (20mm Nominal Size) |
| Base | 903 | DBM | 100 PEN | 150 | BS4987, Group 1 (28mm Nominal Size) |
| Sub-base | 804 | Type B |  | 150 |  |

* Reference “Stone Mastic Asphalt” published by Roadstone.

- Coloured macadam to be red in colour.

Sheet 2 – Rigid Construction

| 1. Location |  |
| --- | --- |
| 2. Grid for Checking surface levels of pavement courses, if different from the requirements of sub-Clause 702.4 |  |
|  | Longitudinal Dimension |
|  | Traverse Dimension |
| 3 Surface Regularity [702.7] |  |
|  | Category of road [A or B] |

| Slab Type | Slab Thickness | Max Traverse Joint Spacing (m) |  | Longitudinal Steel Reinforcement | Particular Requirements |
| --- | --- | --- | --- | --- | --- |
|  |  | Contraction | Expansion |  |  |
| Un-reinforced Concrete slabs (URS) |  |  |  | None |  |
| Jointed reinforced concrete surface slabs (JRC) |  |  |  | mm²/m width |  |
|  | Clause | Material |  | Thickness (mm) |  |
| Sub-Base |  |  |  |  |  |

APPENDIX 7 / 2

excavation and reinstatement of existing surfaces

- In general, the existing road pavement, where it falls within the area of the proposed new road re-surfacing/re-construction, is to be re-surfaced/re-constructed and the full new road surface is to be formed.

At the tie-ins of the new roadworks with the existing roadway, reference should be made to the schematic sections in RCD/700/3 and RCD/700/4.

Tie in of resurfaced roadways with existing roadways

The surface of the existing pavement to be planed over a length of 5 to 10 metres to a depth to allow for full depth of 45mm Surface/Wearing Course to be laid.

- Regulating course to Clause 907 shall be laid as required at tie-in locations.

- Trenches are to be cut through the retained road pavement for such services as:

  - Drainage runs and gully connections

  - Watermains

- All trench widths shall be as for Series 500. Prior to excavation, the route of any trench shall be agreed with the Engineer and marked out and the bounded pavement shall be cut using a saw along a line 75mm wider on each side than the required trench width in accordance with RCD/700/5.

- The drainage runs and gully connections trenches shall be backfilled, once the pipe is laid, in accordance with bed Type E, RCD/500/2.

- The watermains trenches shall be backfilled, once the service is laid, with concrete (ST2 mix) to within 100mm of the existing surface. The final 100mm is to be finished with a 55mm layer of Dense Bitumen Macadam Binder Course to Clause 903 and a 45mm layer of Hot Rolled Asphalt with chippings Surface Course to Clause 910 & 915.

- Where the existing road pavement is to be kept for use as an access road, the pavement is to be protected during excavations, and cleaned of dirt and other material that may have been stored there during the construction work.

APPENDIX 7 / 4

bituminous sprays

- Unless otherwise agreed by the Engineer, the Contractor shall apply a tack coat to all existing and new bituminous surfaces to receive bituminous overlays.

- In all cases, irrespective of time, there shall be a tack coat application between the binder course and surface course materials.

- Binder shall be cationic bitumen emulsion Class K1-70.

- Rate of spread shall be 0.4 to 0.6 litres/m2.

- Requirements for bituminous sprays – as per sub-clause 920 and BS4987-2: 2003 under Clause 5.

| Location | As Required |
| --- | --- |
| Type of binder [920.2] | Bitumen Emulsions |
| Rate of Spread | 0.4 – 0.6 l/m2 |
| Blinding Material [920.5] | N/A |

APPENDIX 11 / 1

kerbs, Footways and Paved Areas

- The height of the kerbface is generally to be 125mm, except at drop kerbs and where noted on the Drawings.

- Kerbs and channel block used to be to Clause 1101.

- Requirements for bedding to paving blocks to be sand: cement mix 3:1.

- Colour of paving blocks to match existing stone paving. Samples to be provided for Engineer’s approval.

- Garden kerbs shall be precast concrete and the size will be similar with what is shown in RCD/1100/4. Samples to be provided for Engineer’s approval.

- Tactile Pavement shall be in accordance to BS7997: 2003.

- In-situ concrete footpath to be to Clause 1106.

APPENDIX 12 / 1

Traffic signs: general

- The Contractor shall comply fully with the requirements of the latest revision of Chapters 4, 5, & 6 of the Traffic Signs Manual as published by the Department of the Environment and Local Government. Outlined as follows:

  - Chapter 4 – Other Information Signs

  - Chapter 5 – Regulatory Signs

  - Chapter 6 – Warning Signs.

- All Permanent Traffic signage is to comply with the above.

- A schedule of Warning, Supplementary and Regulatory Signs to be provided within the project are as follows.

Schedule of Warning, Supplementary and Regulatory Signs – None Required

| Sign Ref. | Drawing No. | Location | Description | Traffic Signs Manual | Size (mm) | Mounting Height (mm) | Foundations W x L x D (mm) | Post Length (mm) | Post Diameter (mm) | No. of Posts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Existing Sign mounting column to be removed from site by the Contractor and replaced with a new sign and column to the above criteria.

- The Contract and Employer’s Representative must agree on the location and the requirement of a cranked mounting pole, if required, prior to the installation of the signage.

APPENDIX 12 / 3

Traffic signs: Road markings and studs

The materials and workmanship for thermoplastic road markings shall comply with the requirements of "Guidelines and Tender Documentation for Road Marking Materials” published by the Department for Environment and Local Government, July 2000 (Referred to below as “The Guidelines”) except insofar as the provisions of this specification modify or supersedes the above, in which case the provisions of this specification shall prevail.

- Location and details of permanent road markings and indicated on the Contract Drawings.

- Permanent road markings shall be white or other colour as indicated on the drawings, thermoplastic screed material complying with BS3262: Parts 1 – 3: 1989.

- Temporary road markings which may be required shall be the responsibility of the Contractor to install to the satisfaction of the Engineer.

- Road markings shall be reflectorised in accordance with Clause 1212 and BS3262: Part 1 – 3: 1989.

Normal application of thermoplastic road markings.

- The quality of the material and the workmanship shall comply with the requirements of all relevant European Standards including EN 1423, EN 1424, EN 1436 and EN 1871.  Any aspects of the work and materials not covered by the European Standard shall comply with the specification as set out below.

- The Contractor shall apply the markings, both white and yellow to the following thicknesses:

    - the thickness of application shall be 4 + 0.5mm on road surfacings.

    - the thickness of application shall be 1.5 + 0.5 mm for temporary markings,

The contractor shall lay the appropriate thickness of marking following consultation, with, and as directed by the Engineer.

- The Contractor shall make available to the Engineer during the duration of the contract

(a)	A Micrometer suitable for measuring markings in accordance with Appendix 12/3/2.

(b)	5 number Steel Plates suitable for measuring markings in accordance with Appendix 12/3/2.

- The Contractor shall lay the appropriate thickness of marking as specified in the Contract or following consultation with and as directed by the Engineer.

  - Unless otherwise directed by the Engineer, all markings shall be reflectorised by the application of glass beads to the surface of the marking material.

  - In the case of reflectorised markings solid glass beads, 400 g/m2 to 500 g/m2, shall be applied by pressure concurrently with the laying of the line with sufficient velocity to ensure retention in the surface of the line. The solid glass beads so sprayed shall give uniform cover and immediate reflectivity over the whole surface of the marking. (On screed applied markings where pressure application is not practicable, gravity application will be permitted).

  - The Contractor shall note that the above mentioned solid glass beads which are to be applied to the surface of the markings are in addition to the solid glass bead content incorporated within the body of the mix for reflectorised markings.

  - The Contractor shall not carry out any work when the weather conditions or the road conditions (e.g. dampness, dust, cold) are such that the quality of the work might be adversely affected. In particular, application of markings shall not be carried out when the road temperature is less than 5oC. Where the Engineer is of the opinion that such adverse conditions exists, he/she may instruct the contractor to suspend the works, but this shall not relieve the contractor of the obligation to suspend the work in the first place where such adverse weather conditions or road conditions exist. In addition, the Contractor shall remove loose debris, mud and all other materials likely to affect adhesion between the thermoplastic road marking material and the road surface.

  - The Contractor shall ensure that the horizontal alignment of the applied marking is +/- 5mm and to the satisfaction of the Engineer in respect of straightness of the line and the smoothness of curvature. The contractor must, in advance of being awarded the contract, satisfy the Engineer as to his capability of carrying out the work as and from the starting date of the contract.

Thermoplastic Road Markings Under Guarantee

  - The minimum period of guarantee shall be for two years after completion of markings and shall apply to markings laid on all cement-bound and bituminous-bound road surfacings including surface dressings.

  - All markings in this Contract shall be laid under guarantee and shall be applied to the thickness specified in the Contract.

  - The following properties of the markings are required to be covered by the guarantee:-

- Durability

- Reflectivity

- Skid Resistance

- Luminance Factor

- Chromaticity

The average value of the above properties shall be established for each line adequate to guarantee that all markings are correct, minimum values are stated in clause 15.

  - During the two year period of guarantee the Contractor shall be required to replace or renew markings which fail to comply with one or more of the minimum values of the marking properties listed above. All costs associated with this testing, replacement or renewal of markings during the period of guarantee shall be borne by the Contractor. Testing will be in accordance with Appendix 1/5.

  - Notwithstanding the above verification tests, the Contractor shall be required to carry out verification tests at any time during the period of guarantee if the Engineer considers that the properties of the markings are less than the minimum values specified. The costs of such testing will be borne by the Contractor if the results of the tests show that the markings fail to comply with the prescribed minima. The costs of such testing shall be borne by the Louth County Council if the Contractor can demonstrate to the satisfaction of the Engineer that the requirements of the specification have been complied with in full.

  - On application and during the period of guarantee, these properties, when measured in situ in an approved manner, shall not be less than the values stated below.

Durability: At least 90 per cent of the marked areas shall contain a uniform coating of marking material through which the underlying surfacing has not penetrated. Discontinuities, due to wear, shall not be accepted in continuous lines and at least 75 per cent of each marked segment of a broken line shall remain intact.

Reflectivity:

Class R4:	RL  200 mcd/m2. 1x, on initial application

Class R2: 	RL  100 mcd/m2. 1x, for 12-24 months following application

when measured by the digital retrometer LTL. 2000. in accordance with Annex B of  IS EN 1436:1998

Skid Resistance: 50 B.P.N, when measured by the British portable pendulum apparatus, or

Class S2:	SRT>50 ,in accordance with Annex D of  IS EN 1436:1998

Luminance Factor:

White Material…Class B4:	  50

Yellow Material…Class B2:	  30

when measured in accordance with Annex C of  IS EN 1436:1998

Chromaticity: Refer to results in Table 6 of EN 1436: 1998; testing method is in Annex C & Table 3.3 of “The Guidelines”.

  - The procedures for carrying out the above tests shall be in accordance with those approved by The National Roads Authority or other agency approved by the Engineer.

  - The Contractor shall measure the values of the above properties on applied markings not less than twelve months after application. The tests shall be carried out in the presence of the Engineer or his/her representative. The sections on which such measurements are taken shall be randomly selected and representative of a marked length of road.

  - At least five observations/measurements shall be taken to establish average values of the above properties for the representative length of road. During the period of guarantee the Contractor shall replace or renew markings which fail to comply with one or more of the minimum values of the marking properties listed above. All costs associated with this testing, replacement or renewal of markings during the period of guarantee shall be borne by the Contractor.

  - The Contractor shall make available samples in accordance with Appendix 1/6, on demand by the Engineer or his representatives.

Screed Lines

  - All screed lines shall be 4.0 mm.

ROAD STUDS

Reflecting Road Studs

  - Permanent reflecting road studs shall be

Type 1 – embedded either of;

(i) 	Depressible self wiping, reflecting, complying with EN 1463 2000.  They shall be installed as recommended by the manufacturer and as approved by the Engineer. The depressible insert shall be manufactured from rubber.

(ii) 	Non-depressible reflector in a housing, complying with EN 1463 2000. They shall be installed as recommended by the manufacturer and as approved by the Engineer.

Photometric:

Class PRP1 is the minimum coefficient of luminance intensity R value,

Dimensions:

Class H0 for height,

Class HD0 for horizontal dimensions,

Colorimetric:

Class NCR1 for chromaticity,

for the stud types above as specified in EN 1463 2000.

Road test performance:

Studs must have an S1R1 or S1R2 performance certification in accordance with part 6 of EN 1463-2: 2000.

  - Temporary reflecting road studs shall be

Non-depressible reflecting stud, complying with EN 1463 2000. They shall be installed as recommended by the manufacturer and as approved by the Engineer.

Photometric:

Class PRT1 is the minimum coefficient of luminance intensity R value,

Dimensions:

Class H0 for height,

Class HD0 for horizontal dimensions,

Colorimetric:

Class NCR1 for chromaticity,

for the stud types above as specified in EN 1463 2000.

  - Where existing road studs are removed, they shall be reinstalled or replaced within 3 working days of their removal unless otherwise approved by the Engineer.

Performance Of Road Studs

  - Type1 Studs must remain in place and retain at least 50% of their reflective face over 24 months. Road Studs will be tested in accordance with Appendix 1/5. Road Studs failing the tests must be replaced within 3 weeks of notification by the Engineer.

| Schedule of Road Markings – Section 1: Pavement Overlay |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Marking Type | Colour | Width/Size | Mark | Gap | Location | Start Ch. | End Ch. | Length | Total | Traffic Signs |
|  |  | (mm) | (m) | (m) |  |  |  | (m) | Length | Manual Ref. |

| Schedule of Reflective Road Studs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Direction Type | Colour | Location | Start Ch. | End Ch. | Spacing | No. |

APPENDIX 26 / 2

Bedding Mortar

Where required by the Drawings; manhole covers, gully covers and the like shall be raised using spacers manufactured expressly for the purpose.

Installation of manhole covers, etc. using a thickness of mortar bedding greater than 12mm will not be permitted.

Appendix 26/70

Criteria for Louth COunty Council re Waste management Act 1996 – 2005

Waste is generally defined as anything which you intend to discard or discard. Waste must only be given to an authorised collector and brought to an authorised waste facility.

For jobs being carried out directly by Louth County Council and involving movement of waste off site, the requirement is that waste is brought to an authorised site. It is important to note that waste permit sites are only allowed to accept specific waste. For example, soil and stones cannot accept metals, wood, signs, plastic, etc.

For all local authority jobs involving the use of contractors where waste material will be produced and waste is being moved off site (includes movement of soil off site) by Contractor/Sub contractor the following requirements apply:

- Contractor or Sub contractor moving the waste must have a valid waste collection permit for County Louth.

- A full copy of the waste collection permit must be carried on board each vehicle carrying waste.

- Only vehicles listed on the waste collection permit can be used for carrying waste.

- Waste can only be moved to waste permitted sites as listed on the waste collection permit.

- Dockets are required for each load of waste moved.

- Only companies with valid waste collection permit for County Louth can be used on jobs.

- Details to be kept by person in charge of the job:

  - Name and address of waste collection permit holder, company name and waste collection permit number e.g. Neat and Tidy Ltd, MH2009/56B.

  - Type of waste to be collected i.e. domestic waste, Top soil, subsoil, septic tank cleaning, etc.

  - Quantity of material to be moved/ or moved.

  - Origin and destination of waste.

  - Office contact number and mobile number of company/person.

Note: Carriers licence for Northern Ireland does not allow for the collection of waste in County Louth.

Details can be checked with Pamela Dagg, Enforcement Section, Louth County Council before any job that will generate waste commences.

Using unauthorised collectors or moving waste to unauthorised sites is a breach of the Waste Management Act 1996 – 2005.

Your co-operation and assistance in this matter is greatly appreciated.

Please refer to Louth County Council’s Environmental Policy Statement.

ANNEX A

Not used