Invitation to Tender

National Parks and Wildlife Services

Refurbishment of Cottage No.16 (Mulligan’s Cottage)

- Introduction

Killarney National Park (KNP) invites competent building contractors, with experience in restoration of old buildings of historical and architectural merit, to tender for the full architectural/mechanical/electrical refurbishment of Cottage No.16 at Muckross, on the N71, Killarney Kenmare route

2.0 Cottage Description

Muckoss Cottage No 16 is an existing single storey semi-detached residential property, of architectural and heritage merit, although it is not a listed structure. It is located on the N71, close to the National Park Ranger/Farm and close to the main entrance to Muckross House and Gardens. The building was constructed in 1837. At some stage in the recent pass a single storey flat roof extension was constructed to the rear of the dwelling. There is a stand-alone small storage shed to the rear of the site. The shed is currently without a roof.

The building is of traditional construction methods. The external walls are of random rubble masonry construction and load bearing. The ground floor is concrete and is ground bearing. The first/attic floor is construction from timber joists overlaid with a T&G softwood board. The main roof is a traditional cut timber rafter supported on timber purlins, in turn supported on 3 main timber trusses. The roof is finished with a natural quarry slate. We understand that the building was re-slated in the recent past.

3.0 Scope of Works

The refurbishment works entrails the complete internal upgrade of the dwelling. In addition, some external site works by way of hardstanding for car parking, works to site boundary and works to the stand-alone store. There will be no new-build extensions. A detailed scope of works is provided in the tender drawings.