Call for Tender:  Fisheries Improvement Works within the Culdaff River (Culdaff_001) [Tender Ref: CfT2026/109]

SCOPE OF WORKS

Whilst every endeavour has been made to provide an accurate description of the requirements, bidders must form their own conclusions regarding the methods and resources needed to meet those requirements. Loughs Agency cannot accept responsibility for any bidder’s assessment of the requirements.

Introduction

Loughs Agency is an agency of the Foyle, Carlingford and Irish Lights Commission (FCILC), established as one of the North South Implementation Bodies under the Good Friday/Belfast Agreement, constituted under the North South Co-operation (Implementation Bodies) (Northern Ireland) Order 1999 and the British Irish Agreement Acts 1999 and 2002.

Our Mission

Sustainably manage, promote and develop the fisheries and resources of the Foyle and Carlingford Areas.

Overview of Requirement

Loughs Agency is responsible, through the Foyle Fisheries Act 1952 Section 11 (6), for the conservation, protection and improvement of the fisheries on the Culdaff River.  It is outside the scope of Loughs Agency to conduct this work. Expert Contractors are required to complete the proposed schemes.

Scope of Works

The Loughs Agency is proposing to carry out instream and riparian improvement works along the Culdaff River - 55°14'49.2"N 7°09'49.9"W to 55°15'08.5"N 7°10'26.3"W.  This document provides detailed specifications for the equipment to be installed in the proposed work area.

Below is a list of installations, including quantities, which comprise the proposed works:

SECTION A:  INSTREAM WORKS

Works Package 1:  Supply and Installation of Pinned Woody Material (360m)

- Woody material will be sourced by the contractor to the site - ensuring that biosecurity measures are followed to prevent the introduction of invasive species or diseases to the site.

- Woody material will be placed to ensure that the flow of the river is not blocked and that it will not collect significant additional debris from the river’s flow.

- The woody material will be securely fastened to the bank and/or the riverbed using a combination of posts and cables as anchors, depending on the part of the river being addressed.

- Figure 1 and Figure 2 give examples of pinned woody material.

Figure 1, example of pinned woody material installed.

Figure 2, example of pinned woody material installed.

Works Package 2:  Gravel Regrade (500m2)

- A long-armed digger equipped with a riddle bucket will be used to rake the exposed gravel and distribute it into the river. The plant machinery must remain out of the water as much as possible and the works must be carried out in a period of low flow.

- A protective bund must be put in place to protect against any accidental spills of fuel or hydraulic fluid from the digger entering the water course while carrying out this work.

- A buffer of 2m of gravel must be left from the area where the regrading is taking place to protect the bank of the river from any erosion associated with re-grading the gravel. A member of the Loughs Agency will advise on how to carry this operation out before work commences.

- An example of a digger regrading river gravel is shown in Figure 3.

Figure 3, Example of long arm digger working on river.

SECTION B:  RIPARIAN WORKS

Works Package 3:  Supply and Installation of Riparian Fencing (2,650m); removal and disposal of existing fencing

Prior to commencement of any works, a site walkover will be carried out with the appointed contractor and a member of the Loughs Agency to highlight where fencing is to be installed.

In areas where fencing is being replaced, before any new fencing is erected, the contractor will remove and dispose of the old fencing (the fencing can be offered to the landowner for salvage if desired).

Fencing Supply

- Only new fencing materials to be used.

- The minimum standard for this stock-proof fencing is galvanised six 2.5mmm twist line barb wire or sheep wire with 2-line barb at top.

- Straining posts must be equivalent in strength and durability to 180 mm top diameter round timber or 180 mm x 180 mm sawn timbers.

- Struts must be equivalent in strength and durability to 88 mm top diameter round timber or 88 mm x 88 mm sawn timber.

- Intermediate posts must be equivalent in strength and durability to 88 mm top diameter round timber or 88 mm x 88 mm sawn timber.

- Intermediate posts must be at least 1.83 m long.

- All posts must be free of bark.

- Posts must have a minimum 15-year life, evidenced by supporting documentation from the manufacturer.

- Where wooden posts have been treated with a preservative, this must have been applied by the manufacturer.

- Strands of galvanised 2.5 mm barbed wire and stock proof fencing must be used. (See Figure 4)

Fencing Installation

- Fencing must be erected to BS 1722-2:2006.

- The overall height of the fence must be at least 1.20 m from the ground to the top wire.

- Intermediate posts must be set at centres not exceeding 3.0m.

- Straining posts must be set at centres not exceeding 150 m or at each change in direction or gradient.

- Straining posts must be at least 2.10 m long when not set in concrete and at least 1.87 m long when set in concrete.

- Struts must be set at least 450 mm into the ground.

- Struts must be mortised into the straining post.

- All fencing must be properly strained and secured to posts with galvanised staples or appropriate fastenings (such as galvanised wire or bespoke fasteners).

- Fencing must be constructed in sections so that, if one section is hit by water and debris, it will not damage the tension or structure of adjacent lengths.

- In an area that is liable to be hit by water, e.g., at right angles to the direction of flow, then the fence should be split approximately every 50 metres. This can be extended where the danger is reduced such as parallel to the flow on a high bank.

- The method of splitting the fence into sections (shown below in Figures 1 and 2), uses two strainers and a fence post but only nailing rails to one strainer and the post. The bottom most plank must be 10cm from the ground to facilitate the movement of otters across the fence line.

- Fencing must not be installed on banks which are likely to be undermined by erosion from the watercourse.  If this happens, the contractor will be required to remove and reinstate the fencing two metres from the top of the bank.

- Fencing must not be placed so close to the top of the bank as to compromise bank stability.

- Livestock must not be able to cross over from the other side of the watercourse and graze the strip.

- To allow for future adjustments and to prevent damage to the galvanising, staples should be driven in at an angle, but not fully home.

- Fencing must not be attached to trees, hedgerows, or electricity poles.

- Fencing must not block or restrict rights of way.

Figure 4; Front view of fencing splitting with horizontal wire.

Figure 5; Rear view of fence splitting with horizontal wire.

Works Package 4:  Supply and Installation of Livestock Gates (1 no. 6ft, 1 no. 10ft, 2 no. 13 ft, 3 no. 14 ft)

- The livestock gates are to be supplied and installed by the contractor and will be installed at locations to be confirmed during the mandatory site visit.

- 6ft Gate - The gate will be 1,828.8mm long and 1,219mm high with 6 horizontal bars and 2 vertical bars.

- 10ft gate - The gate will be 3,048mm long and 1,219mm high with 6 horizontal bars and 2 vertical bars.

- 13ft gate - The gates will be 3,962.4mm long and 1,219mm high with 6 horizontal bars and 2 vertical bars.

- 14ft Gates - The gates will be 4,267.2mm long and 1,219mm high with 6 horizontal bars and 2 vertical bars.

- Top and bottom rails will be 41.5mm tube with middle rails 32mm tube.

- Gate will have a hot dip galvanised finish and box section hang and slam stiles.

- Fixed eye top hinge with adjustable bottom hinge.

- Padlock sliding latch.

- Gate will be hung so that it swings inwards to field.

- An example is shown in Figure 4 below.

Figure 6, example of gate to be installed.

Works Package 5:  Supply and Installation of Galvanised Double Step Stiles (18 no.)

- The contractor will need to supply and install galvanised stiles at locations to be confirmed during the mandatory site visit.

- Maximum distance from top step to top fence rail 450mm.

- Minimum width of stile 1200mm (between stile posts).

- Height of top rail from the ground Minimum 900mm to Maximum 1050mm.

- Handpost 500mm to 700mm above top rail.

- Maximum distance from handpost to centre of step 350mm - 450mm (for steps at 90 degrees to fence) or 500mm - 700mm for cross steps.

- Minimum length of step 900mm (1000mm for cross-step).

- Minimum width of step 200mm.

- Minimum thickness of step 50mm.

- Include Second post for two handles.

- Steps should be covered in non-slip paint before construction begins.

- May be able to incorporate stiles in areas where splitters have been installed.

Figure 7, example of stile to be installed.

Works Package 6:  Supply and Installation of Pipes (2 no.) and Gravel (5 tonnes)

Contractor is to supply and install two - 6m x 300mm pipes with 5 tonnes of gravel to cover

D rail to cover piped area 5 X 3.2m X 4 inch. At locations to be confirmed during the site visit.

Works Package 7:  Installation of Solar Powered Pumps (10 no.) and Supply of Stands

The contractor will install:

- Each solar powered pump (hotline model) provided by the Agency, upon a wooden stand as seen in Figure 8.

- The wooden stand is to be supplied by the contractor and will be installed at height to ensure that pumps do not get damaged during flood conditions.

- The exact location of the installation will be confirmed by the Loughs Agency representative and the landowner.

- Figure 8 shows an example of the required installation.

Figure 8: Example of low spec solar pump installation upon wooden stand.

Works Package 8:  Installation of Plastic Drinking Water Troughs (10 no.) and Supply of Stands

- Loughs Agency will supply the contractor with the plastic drinking water troughs for install.

- The contractor is to install 30 -gallon plastic drinking water troughs at locations which will be confirmed during the mandatory site visit.

- These are to be installed onto wooden stands, which shall be supplied and installed by the contractor.

- All drinking water troughs must be installed at least 20metres from the waterway, and located at a height, to ensure that units are not damaged during flood conditions.

- All drinking water troughs must be placed on hardstanding material, such as gravel, to reduce poaching around the area. The contractor must supply and install the hard standing material.

- Figure 9 shows an example of the required installation.

Figure 9: Example of how troughs should be mounted.

Works Package 9:  Supply and Installation of Concrete Drinking Water Troughs (3 no.) and Associated Materials

- The contractor is to supply and install 60 -gallon concrete drinking water troughs with associated piping, joints and fittings.

- The troughs are to be installed upon hard standing material at locations to be confirmed during the mandatory site visit (See Figure 10).

- All drinking water troughs must be installed at least 20metres from the waterway, and located at a height, to ensure that units are not damaged during flood conditions.

- All drinking water troughs must be placed on hardstanding material, such as gravel, to reduce poaching around the area. The contractor must supply and install the hard standing material.

- Figure 10 shows an example of the required installation.

Figure 10: Example of concrete trough to be supplied.

Works Package 10:  Supply 1000L Water Bowser (1 no.)

Contractor to supply 1000 Litre IBC Water Tank, must include:

- Strong Towable Trailer Chassis

- 50mm Ball Hitch Connection

- Adjustable Jockey Wheel

- ATV Wheels and Tyres

- Galvanised Self-Feeding Water Trough

- Adjustable Rear Lock Stands

- Complete Unit Ready to Use

Figure 8 Picture of water bowser to be supplied

Timeline for Delivery

Instream Elements of this project (Section A) must be completed by 30th September 2026.

Riparian elements of this project (Section B) must be completed by 30th November 2026.

It is the appointed contractor's responsibility to ensure that its work schedule can accommodate this deadline.

Pre-Contract Requirements

Prior to commencement of the contract, the appointed contractor will provide the following:

- Proof of insurance

- Organisation’s Safety Statement

- Job-specific Risk Assessment

Specific Requirements for Contract Delivery

- Pre- and post-works, the appointed contractor will attend site walkovers and meetings with Loughs Agency staff and landowners.

- Only tracked machines must be used within work area to avoid unnecessary disturbance to farmland.

- Machinery mats must be used in areas particularly likely to be affected by transit of plant machinery.

- If plant machinery must enter river to carry out work, then this MUST be done at a period of low flow.

- Plant machinery and personnel must use single track close to fence line to protect integrity of farmland.

- All access points and routes used for machinery will be agreed with relevant landowner and member of the Loughs Agency before commencing work on that section.

- A dedicated refuelling area for machinery, with appropriate sumps and bunding to contain any spills, must be set up and located at least 50m away from the watercourse.

- A wash area for machinery and personnel – using Virkon disinfectant, or equivalent - must be set up to avoid transmission of invasive species.