GENERAL SPECIFICATION

SHRP 2026 -03

Minor Temporary Building Works to 26 dwellings in three estates in BUNCRANA Co Donegal - Dun na nlarlai, Pairc Mor and Sliabh Sneachta

for

DONEGAL COUNTY COUNCIL

JULY 2026

###### CIO	DEMOLISHING STRUCTURES

To be read with Works Requirements.

GENERALLY

100	DEMOLITION GENERALLY:

Demolition generally shall be carried out in accordance with B.S.6187.

110	SAFETY OF OCCUPANTS:

When a building or terrace is to be partially demolished, take all necessary steps to ensure that the safety of any occupant of the remaining portion is not put at risk at any time by the process of demolition. lf necessary, occupants of remaining portions shall be evacuated.

120	PROTECTION:

Where works of demolition are likely to result in the discharge of materials or debris on the adjoining school property not being part of the demolition site, erect protective coverings, full height screens or fans or a temporary closure applied to the adjoining rooms. Fans shall be of adequate strength for the purpose  for which they are required and shall, if necessary, be waterproofed and laid to fall so that there is no likelihood of water drips falling upon persons using the adjoining rooms.

130	MINIMISING DUST:

Take all precautions to minimize dust arising from the demolition work.

140	SECURITY:

Ensure that any building that is partly demolishing is, as far as possible, properly secured or closed against entry at all times when demolition operations are not in progress, and that the building is left in a safe condition at the end of each day’s work.

150	EXPOSURE TO HAZARDOUS SUBSTANCES:

Take all possible steps to prevent exposure of persons to substances hazardous to health that are or could be present during demolition work. For demolition work on buildings where asbestos dust or fumes may be encountered, which may prejudice the health of any persons working or dwelling in the vicinity of the site, all work shall comply with the recommendations of the Health and Safety Inspectorate and shall be in accordance with the Control of Asbestos at Works Regulations (N.I.) 1998, the Health and Safety at Work (N.I.) Order 1978 and all other current legislation and relevant Codes of Practice.

170	TEMPORARY SUPPORT:

The removal of certain portions of a structure may result in other portions becoming unsafe and the contractor shall determine where temporary supports will be needed. If the Structure to be demolished abuts other buildings, the abutting shall be given such lateral support as is given by the structure to be demolished.

180	WARNING NOTICES:

Every working place of demolition and approach thereto shall be properly identified with warning notices and shall if necessary be properly illuminated.

190	OVERLOADING ON SACFFOLDING PLATFORMS:

Ensure that the load from any debris or materials collecting on the scaffold does not exceed the loading assumed for the design of the scaffold and means shall be taken to prevent debris or materials from being dislodged from the platforms.

200	OVERLOADING OF STRUCTURE:

Do not permit overloading of any part of a structure by debris or materials arising from demolition.

210	DANGER FROM FALLING DEBRIS:

Where materials or debris are to be lowered, ensure that this is done in a manner that will not create a danger to persons or property. Debris shall only be allowed to fall freely to the ground where it can cause no damage or danger and where the horizontal distance from the line of fall to a public highway or adjoining property is not less than 6 m or half the height from which the debris is dropped, a competent look-out man shall be posted to ensure safety generally.

OWNERSHIP OF PROPERTY ARISING FROM DEMOLITION:

Where materials are or are to become the property of one of the service authorities, the relevant authorities are io be given any necessary facility to disconnect and remove these from the site.

230	WEATHER CONDITIONS:

Before carrying out any part of the work, consider prevailing weather conditions and forecasts, paying particular attention to the effects of adverse wind. When mechanical plant is used demolition, ensure that no part of the machine comes into direct contact or in close proximity to electricity cables,

240	PLUMBING, ENGINEERING AND ELECTRICAL INSTALLAIONS:

The description of demolishing structures shall be deemed to include sealing and making safe the mains to the plumbing, engineering, and electrical installations. The descriptions of taking out equipment, appliances, ancillaries, fittings and accessories associated with plumbing, engineering and electrical installations shall be deemed to include disconnection from the associated gutterwork, pipework, ductwork, conduits, cables and the like.

###### C20	ALTERATIONS - SPOT ITEMS

To be read with Preliminaries/General conditions. GENERALLY

110		APPLICABILITY: The requirements of this section apply to all alteration work to existing building given spot items in various parts of the schedules/bills.

120	QUALITY OF WORK in spot items to be as described in the relevant other sections of the specification.

130	PRICING ON SITE: The Contractor shall be deemed to have visited the site and ascertained the full extent

And nature of the alterations,

150	SUPPORT existing structure as necessary during cutting of new openings or replacement of structural Parts. Do not remove supports until new work is strong enough to support the existing structure.

155	PROTECTION OF SURROUNDING OR NEIGHBOURING STRUCTURES: Carry out this work

To ensure, as far as possible, that no damage is occasioned to the surrounding or neighboring structures. The contractor shall, subject to Clause 21.2 of the Conditions of Contract, be responsible for the rectification of any such damage.

160		PROTECTION OF BUILDING INTERIORS: Protect building interiors from weather and dust during Course of alteration work with temporary screens of sufficient size to permit execution of new work and to the approval of CA.

165	REMOVING DEFECTIVE MASONRY: Carefully remove one brick/block at a time and ensure that no damage is caused to adjacent masonry which is to remain intact.

170		MAKING GOOD: Unless otherwise stated make good parts and/or finishes of existing structure Disturbed during alterations to match existing.

175		REMOVAL OF EXISTING COVERINGS, ETC: When removing existing coverings or finishing’s Such as timber boarding or roofing tiles, take due care not to damage the existing base or supports and

remove all protruding nails leaving existing base/supports prepared to receive new coverings or finishing’s. Contractor to make good any damage so caused at his own expense.

180		OVERLAPPING OPENINGS: When a new opening overlaps an existing opening the area of overlapping Stated is that which is contained within the nominal finished size of the new opening and excludes any overlap arising from the extra cutting that may be required to accommodate lintels, thresholds, the formation   of jambs and the making good of the structure around the openings, etc.

PRODUCTS MATERIALS ARISING

210		OWNERSHIP: Products and materials arising from the alterations work are to become the property of the Contractor except where otherwise stated. Remove from site as work proceeds. HARDCORE: Brick nibble or other hard materials arising from the work may be reused as hardcore, Subject to compliance with specification and written permission being obtained from C.A.

INFECTED TIMBER: Where instructed, remove timber affected by fungal/insect attack from the building in a way which will minimize the risk of infecting other parts of the building, and destroy as soon as possible.

| 280 | BURING ON SITE of materials arising from the work will not be permitted unless written Permission is obtained from CA. |
| --- | --- |
| 290 | SETTING ASIDE FOR MEASUREMENT: Where the removal of pipework is measured in the Bill of Quantities, set the pipework aside for measurement by the quantity surveyor before removal from site. |
| 300 | REUSE OF FACINGS: The reuse of facings arising from alterations shall only be allowed where permission is given by CA. |
| 400 | DISINFECTING EXISTING DWELLINGS |

Chemical preparations to be to satisfaction of CA. Thoroughly disinfect dwelling to satisfaction of CA.

Chemical preparation to be used strictly in accordance with manufacturer’s instructions.

500	REMOVING DEFECTIVE RENDER AND BLOCKWORK:

Strip off plaster from walls and take down blockwork by hand. The work to be carried out in sections (see measured work section). All material arising from the demolitions to be collected and removed to designated site provided by Donegal County Council.

Replastering and other builder’s work is measured in the appropriate sections.

###### F10	BRICK/ BLOCK WALLING

To be read with Works Requirements.

ADDITIONAL SPECIFICATION l CERTIFICATION / TESTING I AM SAMPLING FOR PRECAST CONCRETE BLOCKS

  - SPECIFICATION - GENERAL BACKGROUND.

  - BUILDING REGULATIONS.

The Technical Guidance Documents and Supporting Documentation on which these specifications are based on are as follows:

Part A - Structure (2012)

Part D - Materials and Workmanship (2013)

Where a more recent version of the above Technical Guidance Documents has been issued post the publication of the document, it shall supersede the related contents of this document. The specifications provided in this document are non-exhaustive and Contractors should refer to the above documents where specific build details are not covered.

  - MATERIAL AND WORKMANSHIP

Material and workmanship shall be in accordance with BUILDING REGULATIONS (PART D AMENDMENT) REGULATIONS

All material shall be ’proper materials’ as defined by the said regulations and:

- bear a CE Marking in accordance with the provisions of the Construction Products Regulation (EU) No. 3 05/ 2011 of the European Parliament and of the Council of 9th March 2011 laying down harmonized conditions for the marketing of construction products and repealing Council Directive 89 /1 06 /EE C .

- comply with an appropriate harmonized standard or European Technical Assessment in accordance with the provisions of the Construction Products Regulation; or

- comply with an appropriate Irish Standard or Irish Agreement Certificate or with an alternative national technical specification of any State which is a contracting party to the Agreement on the European Economic Area, which provides in use an equivalent level of safety and suitability.

Materials shall be:

  - Of a suitable nature and quality in relation to the purposes and conditions of their use, and

  - Adequately mixed or prepared, and

  - Applied, used or fixed so as to perform adequately the functions for which they are intended.

Certificates for all materials used shall be given to the Assigned Certifier/Client Representative.

  - AGGREGATE CONCRETE MASONRY UNITS (PRECAST CONCRETE BLOCKS)

  - STANDARDS

The most recent version of the following standards shall apply:

DONEGAL COUNTY COUNCIL STANOARD HOUSE TYPES

Page 1 of 6

Table F.

| STANDARD | TITLE |
| --- | --- |
| I.S. EN 771-3 2011+A1:2015 | SPECIFICATION FOR MASONRY UNITS - PART 3: AGGREGATE CONCRETE MASONRY UNITS (DENSE AND LIGHTWEIGHT AGGREGATES) |
| I.S. EN 1996-1-1 | EUROCODE 6 - DESIGN OF |
|  | MASONRY STRUCTURES - PART 1- |
|  | 1: GENERAL RULES FOR REINFORCED |
|  | AND UNREINFORCED MASONRY STRUCTURES |
|  | (INCLUOING IRISH NATIONAL ANNEX) |
| I.S. EN 1996•1-2 | General Rules. Structural fire design + National Annex; |
| I.S. EN 1996•2:2006 | Design considerations, selection of materials and |
|  | execution of masonry + National Annex; |
| S.R. 325:2013 | Recommendations for the Design of Masonry |
|  | Structures in Ireland to Eurocode 6 |
| I.S. EN 12620:2002+AI:2008 & S.R.16 | AGGREGATES FOR CONCRETE. |

  - AMENDMENTS & ADDITIONS TO THE SPECIFICATIONS.

Note; The criteria below are relevant to the particular project to which the overall specification refers.

    - The aggregate masonry units should be manufactured and conform to I.S. EN 771-3 2011-rA1:2015

    - Concrete blocks shall be group 1 masonry units conforming to IS EN 771 -3. The declared mean compressive strength shall be 13N/mm2.

    - The manufacturer is to be approved prior to placing an order. Similar applies to when the materials are being purchased through a distributer.

    - The supplier and certificates for all the raw and manufactured materials are to be approved prior to placing an order.

    - The location and source of both fine and coarse aggregates used in the manufacture of the precast concrete blocks is to be made known and approved prior to placing an order.

Likewise, in the event the location and source of the aggregate changes during the progression of the works this to be made known and also approved prior to use of that new source.

    - In relation to both coarse and fine aggregates used in the manufacture of the precast concrete blocks; only quarries certified by a Geologist and fully licensed are to be used.

    - In addition to the abovementioned standards the coarse and fine aggregate should not contain muscovite mica at a level of more than 1% of the aggregate quantity. Furthermore, the aggregate shall not contain harmful material in sufficient quantity to affect adversely the strength or durability of the concrete. Mica, shale or similar laminated materials, or soft particles, shall not be present in such a form or in such

DONEGAL COUNTY COUNCIL STANDARDHOUSE TYPE Page 2 of 6

quantity as to affect adversely the concrete. Coarse aggregates shall not be of a type that is liable to suffer from the action of frost.

Assurances will be required from a suitably qualified Geologist certifying the materials are free from harmful impurities that would adversely affect the structural integrity and durability of the manufactured precast concrete block.

The supplier is to demonstrate this conformity and to be agreed prior to placing an order.

    - “Random" sampling shall be carried out in accordance with Annex A of I.S. EN 771- 3 2011+A1:2015 to confirm the precast concrete blocks conform to the relevant standards. The independent “registered & certified" testing company is to be agreed between the Parties prior to placing the order.

The number of samples and tests in accordance Annex A is to satisfy the requirements set out therein.

Whenever possible the random sampling method shall be used in which every masonry unit in the consignment (load) has an equal chance of being selected for the sample. The appropriate number of units shall be selected at random from positions throughout the consignment without any consideration being given to the quality of those selected except that units damaged in transit shall not be selected.

There will be at least one requirement for random sampling for testing in accordance with Annex ZA for at least 1 in every 10-housing unit on each individual unit size (precast concrete block size). The purchaser may require additional random sampling/tests.

    - The delivery notes/invoices for the products are to be Marked in accordance with I.S. EN 771-3 2011+A1:2015 and to include.

      - the name, trademark or other means of identification of the manufacturer.

      - means of identifying the date of manufacture of the units.

      - means of identifying the units and relating them to their description and designation.

    - The relevant up to date signed and dated CE Marking should accompany the products and be available on request. This CE Marking should be accompanied with the Declaration of Performance setting out “all’ the essential characteristics contained in Annex ZA of I.S. EN 771-3 2011+A1:2015 including durability. The said DoP should indicate the Clauses in this European Standard related to essential characteristics and indicate the declared performance in compliance with the Standard.

The Declaration of Performance should also indicate the unit type (precast concrete block size), supplier, site and house number to which the product was supplied and relates to and the period which the particular unit was delivered to that site/house number.

TYPES OF WALLING

- CONCRETE COMMON BRICKWORX FOR INFILL WORK TO BLOCKWORK AND INSPECTION CHAMBERS

Bricks: To BS 6073-2:2008 and is IS EN 771-3:2011+A1:2015.

Category: Category 1. Type: solid.

Average compressive strength (minimum): 13 N/mm’. Work sizes: 215 x d5 x 100mm .

Mortar: As section Z21.

Mix: 1:1:5-6 group 3.

Bond: half lap stretcher .

- CONCRETE COMMON BRICKWORK FOR RETAINING WALLS Bricks: To BS 6073-2:2008 and IS EN 771-3:2011+A1:2015. Category: Category 1.

Type: solid.

Average compressive strength (minimum): 13 N/mm*. Work sizes: 215 x 65 x 100mm .

Mortar: As section Z21. Mix: 1:0.5:4.5 group 2 Bond: English garden wall .

350	CONCRETE COMMON BLOCKWORK TO CAVITY WALLS AND PARTITIONS. Blocks: To BS 6073-2:2008 and IS EN 771-3:2011+A1:2015.

Category: Category 1.

Manufacturer: to be approved before placing order . Type: solid

Average compressive strength (minimum):13 N/mm’, Work sizes: 440 x 215 x 100mm.

Mortar: As section Z21.

-	Mix: 1:1:5-6 group 1

-	Bond: half lap stretcher .

355	CONCRETE COMMON BLOCKWORK.

-	Blocks: To BS 6073-2:2008 and IS EN 771-3:2011+A1:2015.

-	Category: Category 1.

Manufacturer: to be approved before placing order . Type: solid .

Average compressive strength (minimum):13 N/mm’. Work sizes: 440 x 140 x 100mm .

Mortar: As section Z21. Mix: 1:1:5-6 group 1 Bond: half lap stretcher .

380	ENGINEERING BRICKWORK AS INDICATED ON THE DETAILS Engineering bricks: To BS EN 771-1 Engineering Class B. Category: Category 1.

Manufacturer: to approval before placing order . Mortar: As section Z21.

Mix: 1:3 .

Bond: as indicated on the detail drawings . Joints: Flush.

###### WORXMANSHIP GENERALLY

440	CONDITIONING OF CONCRETE BRICKS/ BLOCKS

Autoclaved concrete bricks/ blocks delivered warm from manufacturing process: Do not use. Age of non-autoclaved concrete bricks/ blocks: Do not use until at least four weeks old.

Avoidance of suction in concrete bricks/ blocks: Do not wet.

-	Use of water retaining mortar admixture: Submit details.

460	MORTAR GROUPS

-	Mix proportions: For a specified group select a mix design from the following:

Group	1	2	3

PC*:lime:sand with or without air entraining additive

1:0-0.25:3	1:0.5-4.5	1:1:5-6

4

1:2:8-9

Masonry cement:sand containing PC* and lime in approx ratio 1:1, and an air entraining additive

1:3	1:3.5-4 1:4.5

Masonry cement:sand containing PC” and inorganic materials other than lime and air entraining additive

-	1:2.5-3.5	1:4-5

PC“:sand and air entraining additive

1:5.5-6.5

1:3	1:3-4

PC“ = Portland cement

- Batching: Mix proportions by volume.

1:5-6

1:7-8

- Mortar type: Continuous throughout any one type of masonry work.

500	LAYING GENERALLY

- Mortar joints: Fill vertical joints. Lay bricks, solid and cellular blocks on a full bed.

- Bond where not specified: Half lap stretcher.

- Vertical joints in facework: Even widths. Plumb at every fifth cross joint.

520	ACCURACY

Courses: Level and true to line. Faces, angles and features: Plumb.

- Permissible deviations:

  - Position in plan of any point in relation to the specified building reference line and/ or point at

the same level	1 10 mm.

  - Straightness in any 5 m length	1 5 mm.

  - Verticality up to 3 m height	1 10 mm.

  - Verticality up to 7 m height	4 14 mm.

Overall thickness of walls	:t 10 mm.

  - Level of bed joints up to 5 m

(brick masonry)

  - Level of bed joints up to 5 m (block masonry)

z 11 mm.

:t 13 mm.

535		HEIGHT OF LIFTS IN WALLING USING CEMENT GAUGED OR HYDRAULIC LIME MORTAR

Quoins and advance work: Rack back.

Lift height (maximum): 1.2 m above any other part of work at any time. Daily lift height (maximum): 1,5 m for any one leaf.

545	LEVELLING OF SEPARATE LEAVES USING CEMENT GAUGED OR HYDRAULIC LIME MORTAR

Locations for equal levelling of cavity wall leaves: As follows:

- Every course containing vertical twist type ties or other rigid ties.

- Every third tie course for double triangle/ butterfly ties.

- Courses in which lintels are to be bedded.

560	COURSING BRICKWORK

-	Gauge: Four brick courses including bed joints to 300 mm.

561	COURSING BRICKWORK WITH EXISTING

-	Gauge: Line up with existing brick courses.

595	LINTELS

-	Bearing: Ensure full length masonry units occur immediately under lintel ends.

610	SUPPORT OF EXISTING WORK

Joint above inserted lintel or masonry: Fully consolidated with semidry mortar to support existing structure.

620	BLOCK BONDING NEW WALLS TO EXISTING

Pocket requirements: Formed as follows:

- Width: Full thickness of new wall.

- Depth (minimum): 100 mm.

- Vertical spacing:

Brick to brick: 4 courses high at 8 course centers. Block to block: Every other course,

Pocket joints: Fully filled with mortar.

635	JOINTING

Profile: Consistent in appearance.

645	ACCESSIBLE JOINTS NOT EXPOSEO TO VIEW

Jointing: Struck flush as work proceeds.

665	POINTING WHERE EXPOSED

Joint preparation: Remove debris. Dampen surface. Mortar: As section Z21.

-	Mix: 1:1:5-6 group 3 . Profile: flush .

671	FIRE STOPPING

Avoidance of fire and smoke penetration: Fit tightly between cavity barriers and masonry. Leave no gaps,

690	ADVERSE WEATHER

General: Do not use frozen materials or lay on frozen surfaces. Air temperature requirements: Do not lay bricks/ blocks:

- In cement gauged mortars when at or below 3°C and falling or unless it is at least 1•C and rising.

- In hydraulic lime:sand mortars when at or below 5°C and falling or below 3•C and rising.

- In thin joint mortar glue when outside the limits set by the mortar manufacturer.

-	Temperature of walling during curing: Above freezing until hardened.

Newly erected walling: Protect at all times from:

-	Rain and snow.

Drying out too rapidly in hot conditions and in drying winds.

F30	ACCESSORIES/ SUNDRY ITEMS FOR BRICK/ BLOCK/ STONE WALLING

To be read with Preliminaries/ General conditions.

###### CAVITIES

110	CONCRETE FILL TO BASE OF CAVITY

-	Concrete: To BS 8500-1:2006+A1:2012 and BS EN 206-1,

Mix: Designated GEN 1. see section E1O . Workability: High.

Extent: Maintain 75 mm between top of fill and external ground level and a minimum of 225 mm between top of fill and ground level dpc.

Placement: Compact to eliminate voids.

120	CLEANLINESS

-	Cavity base and faces, ties, insulation and exposed dpcs: Free from mortar and debris.

- PERPEND JOINT WEEP HOLES Form: Open perpend joint.

Locations: Through outer leaf immediately above base of cavity, at cavity trays, stepped        dpcs and external openings. 75 mm above top of cavity fill at base of cavity.

Provision: At not greater than 1000 mm centers and not less than two over each opening.

- BED JOINT WEEP HOLES

Form: Open 10 mm diameter hole.

Locations: Through outer leaf immediately above base of cavity at cavity trays, stepped dpcs and external openings. 75 mm above top of cavity fill at base of cavity.

Provision: At not greater than 1000 mm centers and not less than two over each opening.

- PARTIAL FILL CAVITY INSULATION Insulation: Foil Faced rigid Foam plastic.

  - Manufacturer: contractor’s choice

Product reference: Kingspan Kooltherm KB, to give 0.023 W/m2K u-value for cavity wall incl. render or equivalent approved.

Face size: 1200mm x 450rnm.

  - Thickness: 75 mm to walls,

- Placement: Secure against face of inner leaf.

-	Residual cavity: Clear and unobstructed.

- Joints between boards, at closures and penetrations: No gaps and free from mortar and debris.

- FIRE RESISTANT CAVITY INSULATION

- Insulation: Rockwool cavity .

  - Manufacturer: contractor's choice.

Product reference: Rockwool cavity, Rockwool Limited.

  - Face size: 1200mm x 455mm .

Thickness: 120 mm.

- Placement: to extend min. 300mm beyond fire breast on all sides to meet Building Regulations with cavity tray flashing over top edge extending 100mm over edge of Rockwool: To be fitted in accordance with Agrement certificate no 94/3079.

Joints between slabs, at closures and penetrations: No gaps and free from mortar and debris.

160		AIR BRICKS IN EXTERNAL WALLING Standard: To BS 493, class 1. Manufacturer: contractor’s choice,

- Product reference: Hepworth Building Products Ltd or equivalent and approved, Louvre Air Bricks :215mm x 140mm, ref. 264 or 264F

- Cavity Liner : 215mm x 140mm x 200mm, ref 44 .

- Installed across cavity, sloping away from inner leaf, bedded fully in mortar to seal cavity, form a stepped dpc cavity tray above duct, extending 150mm on each side and with stop ends

Material/ colour: white .

- Placement: Built in with no gaps at joints.

- CAVITY CLOSER AT WINDOW AND DOOR JAMBS

Proprietary insulated cavity closer — rigid J—section one piece, PVC-U extrusion with flange keys for direct plaster application, projecting fixing flange and rigid thermoset insulation core with ñ-value of 0.022 W/m.K, manufactured with a blowing agent that has zero Ozone Depletion	Potential (OOP) and low Global Warming Potential (GWP).

Include for all PVC-U jointing clips and fixing ties all in accordance with manufacturers recommendations.

- Cavity width: 150mm Manufacturer: contractor’s choice

- Product reference: Kingspan Kooltherm6 Cavity Closer or equivalent and BBA approved

- CAVITY CLOSER AT EAVES AND VERGE Material: Rockwool mineral wool

Product: To give min R value 1.2m2 K/W

- ACOUSTIC PARTY WALL DPC Manufacturer: contractor's choice.

Product reference: ARC T Barrier Vertical Masonry (ARC Building Solutions)

or Rockwool Acoustic Party wall dpc (Rockwool Ltd .) or equivalent and approved

Glass tissue faced, semi-rigid, Rockwool insulation bonded to a black polyethylene DPC. Rockwool insulation: 1200 x 260 mm x 150mm thick

DPC: 13dO mm long, with 100 mm end lap and 40 mm side lap.

Fit vertically as blockwork progresses. To be friction fitted within cavity and compression fitted within party wall care taken that joints are compressed together, and all gaps are sealed.

REINFORCING/ FIXING ACCESSORIES

211	WALL TIES FOR PART-FILL CAVITY WALLS

- Standard: To Eurocode 6, BS EN 845-1, PD6697:2010, CE Marked to BS EN 845-1, Agrement certified.

- Material: austenitic stainless steel

Length: 275mm to suit cavity size of 150mm with 100mm insulation. Manufacturer: contractor’s choice

Product reference: ACS 3000 Range Tie

ACS Stainless, Cross Green Approach, Cross Green Industrial Park, Leeds LS9 OSG tel: +44(0) 1133918200

Fixing: M6 Plug and Screw.

Spacing: as per manufacturer’s instructions and BBA certification. Placement: as per manufacturer’s instructions and BBA certification.

280		LATERAL RESTRAINT TIES FOR MOVEMENT JOINTS IN EXTERNAL LEAF Ties: 200mm x 3mm dia galvanised steel bar

Debonding arrangement: Polythene sheet wrapping to one half length of tie. Fixing centers: alternate courses.

###### FLEXIBLE DAMP PROOF COURSES/ CAVITY TRAYS

320	DAMP PROOF COURSE - POLYETHYLENE Standard: To BS 6515.

Manufacturer: contractor’s choice.

Product reference: Visqueen Polyethyene damp-proof course (embossed) or equivalent and approved.

330	DAMP PROOF COURSE - Bitumen Polymer Manufacturer: contractor’s choice.

Product reference: Extra Load (Andersons Waterproofing), complete with pre-formed corners cavity trays etc. or equivalent and approved.

360	GAS RESISTANT DPCS/ CAVITY TRAYS Manufacturer: contractor's choice

- Product reference: Easi-Load Radon DPC or equivalent. Manufacturer: Necofiex Limited, Radon Control Systems, Unit 3 Orion Business Campus, Northwest Business Pk., Dublin 15. Tel: +353 (0)1 802 3333.

  - Material Type: Virgin blend LDPE

  - Thickness: 500 microns

  - Radon Permeability: 7.9 x 10”’ 2 m*/s Tensile strength >350N/50mm Weight: 460 g/mz

Tear Resistance >200N

  - MVTR : <0.3 g/mz/day

380A	PREFORMEO DPC/ CAVITY TRAY JUNCTION CLOAKS/ CORNERS

Manufacturer: contractor’s choice

Product reference: Easi-Load Radon Corner Assembly or equivalent. Manufacturer: Necoflex Limited, Radon Control Systems, Unit 3 Orion Business Campus, Noi1hwest Business Pk., Dublin 15. Tel: -r353 (0)1 802 3333.

  - Vacuum-formed corner units Material Type: Virgin blend LOPE Minimum Thickness: 500 microns

  - Radon Permeability: 7 x 10-12 m2/s

-	MVTR : 8.28x10-13 kg/(m2.s.Pa)

Placement: To provide a free draining radon resistant and watertight installation. Seal laps with dpcs and / or cavity trays

385	PREFORMED DPC/ CAVITY TRAY JUNCTION CLOAKS/ STOP ENDS

- Manufacturer: to be approved before used on site.

-	Product references and locations: As shown on drawings.

- Placement: Seal laps with dpcs and/ or cavity trays.

390	SITE FORMED DPC/ CAVITY TRAY JUNCTIONS/ STOP ENDS

- Three dimensional changes in shape: Form to provide a free draining and watertight installation. Seal laps.

- Alternative use of preformed junction cloaks/ stop ends: Submit proposals.

###### INSTALLATION OF DPCS/ CAVITY TRAYS

415	HORIZONTAL DPCS

Placement: In continuous lengths on full even bed of fresh mortar, with 100 mm laps at joints and full laps at angles.

Width: At least full width of leaf unless otherwise specified. Edges of dpc not covered with mortar or projecting into cavity.

Overlying construction: Immediately cover with full even bed of moi1ar to receive next masonry course.

Overall finished joint thickness: As close to normal as practicable.

425	GROUND LEVEL DPCS

-	Joint with damp proof membrane: Continuous and effectively sealed.

435	STEPPED DPCS IN EXTERNAL WALLS

External walls on sloping ground: Install dpcs not less than 150 mm above adjoining finished ground level.

445	SILL DPCS

Form and placement. In one piece and turned up at back and fixed to inner leaf when sill is in position.

455	COPING/ CAPPING DPCS

Placement: Bed in one operation to ensure maximum bond between masonry units, mortar and dpc.

Dpcs crossing cavity: Provide rigid support to prevent sagging.

475	SITE FORMED CAVITY TRAYS

Requirements to prevent downward ingress of water:

- Profiles: To match those shown on drawings. Firmly secured.

- Joint treatment: Use un-jointed wherever possible, otherwise lap at least 100 mm and seal to produce a free draining and watertight installation.

- Horizontal cavity trays: Support using cavity closer,

- Sloping cavity trays: Prevent sagging.

- Cleanliness: Free from debris and mortar droppings.

485	CAVITY TRAYS OVER OPENINGS AND OTHER CAVITY BRIDGINGS

-	Length: To extend not less than 150 mm beyond ends of lintels/ bridgings.

490	SEALING DPCS

Overlaps and junctions: Seal with: double-sided radon resisting tape Manufacturer: contractor’s choice

Product reference: Monobond RT or equivalent. Manufacturer: Necofiex Limited, Radon Control Systems, Unit 3 Orion Business Campus, Noi1hwest Business Pk., Dublin 15. Tel:

+353 (0)1 802 3333 Fax: +353 (0)1 803 6060

- Dynamic Shear Adhesion: 12 N/cm2

- Dynamic Tensile Adhesion: 18 N/cm2

- 180 ° Peel Adhesion: 17 N/cm2

- Specific Gravity: 1.6 g/cm3

- MVTR: 0.17 g/m2/day/mm

- Radon Permeability: 10-12 m2/s

Joint treatment: Minimize joints wherever possible, otherwise lap at least 100 mm and seal to form a gas and watertight installation.

Joint with radon damp proof membrane: Overlap dpc / cavity tray not less than 100 mm.

491		PREFORMED GAS RETARDANT DPC AT CORNERS Seal all corners with:

Manufacturer: contractor’s choice

Product reference: Easi-Load Radon Corner Assembly or equivalent; Manufacturer: Necofiex Limited, Radon Control Systems, Unit 3 Orion Business Campus, Northwest Business Pk., Dublin 15. Tel: +353 (0)1 802 3333

Corner Assembly Kit Ref: L 100 / T 100 / L 215 / T 215 Select as Required Corner Assembly Kit components:

L 100: (1 internal section, 1 external section, and 1 ’L’ corner cap to suit 100mm leaf} T 100: (2 internal sections and 1 'T’ corner cap to suit 100mm leaf)

L 215: (1 internal section, 1 external section, and 1 ‘L' corner cap to suit 215mm leaf}

T 215 (2 internal sections and 1 ‘T’ corner cap to suit 215mm leaf} Refer to manufacturer's current guidelines for assembly installation instructions

-	Apply sealant tape to the edge of each assembled corner unit, ready to accept and seal to main radon damp-proof course membrane (Easi-Load Radon DPC).

495	GAS RESISTANT DPCS/ CAVITY TRAYS

DONEGAL COUNTY COUNCIL

Joint treatment: Use un-jointed wherever possible, otherwise lap at least 150 mm and seal to form a gas and watertight installation.

Joint with damp proof membrane: Overlap dpc/ cavity tray not less than 150 mm.

495A	GAS RESISTANT DPCS/ CAVITY TRAYS

Joint treatment: Use un-jointed wherever possible, otherwise lap at least 100 mm and seal to form a gas and watertight installation.

Joint with damp proof membrane: Overlap radon dpc/ cavity tray not less than 100 mm.

535	OPC/CAVITY TRAY LEADING EDGE IN FACEWORK - PRPOJECTING

-	Treatment at face of masonry: Projecting 5 mm from face of wall at the following locations:

560	VERTICAL OPCS GENERALLY

Form: In one piece wherever possible.

-  Joints: Upper part overlapping lower not less than 100 mm.

570	JAMB DPCS AT OPENINGS

- Joint with cavity tray/ lintel at head: Full underlap. Joint with sill/ horizontal dpc at base: Full overlap. Projection into cavity: Not less than 25 mm.

Relationship with frame: In full contact.

580		JAMB DPCS TO BUILT IN TIMBER FRAMES Fixing: Securely fastened to back of frame.

- Fasteners: Galvanized clout nails or staples.

###### JOINTS

610	MOVEMENT JOINTS WITH SEALANT TO EXTERNAL RENDER

Joint preparation and sealant application: As section Z22. Filler: A flexible and compressible closed-cell polyethylene joint filler — Expandafoam, 12 mm thick

- Manufacturer: Consult Fosroc Ltd. (01827262222)

- Placement: to be installed as brickwork is laid with no

projections into cavities and to correct depth to receive sealant system.

- Sealant:

-	Designation: see section Z22 .

Manufacturer: contractor’s choice, Expandite or equivalent and approved. Product reference: Thioflex 600.

Colour: white.

630	UNEXPOSEO CONTRACTION JOINTS

Formation: Close butt as work proceeds.

650	POINTING IN FLASHINGS

Joint preparation: Free of debris and lightly wetted. Pointing mortar: As for adjacent walling.

Placement: Fill joint and finish flush.

660	PINNING UP TO SOFFITS

-	Top joint of load bearing walls: Fill and consolidate with mortar.

PROPRIETARY SILLS/ LINTELS/ COPINGS/ DRESSINGS

720	SILLS

Standard: To BS 5642-1. Material: Pre-cast concrete

Manufacturer: contractors choice.

DONEGAL COUNTY COUNCIL

Product sample to be approved before used on site Dimensions: As shown on drawings. with stooled ends. Finish: smooth.

Mortar for bedding/ jointing: Cement gauged as section Z21.

-	Mix: 1:1:5-6, Group 3 . Joints: Flush.

Bedding one-piece sills: Leave bed joints open except under end bearings. On completion, point to match adjacent work.

730	PRECAST CONCRETE LINTELS

-	Standard: To BS EN 845-2:2013.

Manufacturer: Contractor’s choice.

-	Product sample to be approved before used on site.

Placement: Bed on mortar used for adjacent work with bearing of not less than 150 mm. Use slate packing pieces.

740	PRESTRESSEO CONCRETE LINTELS

Manufacturer: contractor's choice.

-	Product sample to be approved before used on site

Placement: Bed on mortar used for adjacent work with bearing of not less than 150 mm. Prop at not more than 1.2 m centers to prevent displacement during construction. Retain props in position for not less than 14 days or until mortar has matured, whichever is longer.

750	PROPRIETARY PREFABRICATED INSULATEO STEEL CORNER LINTELS

including proprietary galvanized mild steel hollow square section all as per manufacturer’s design/instructions,

- Dimensions: as shown on drawings.

- Standard: To BS EN 845-2:2013, BBA/IAB certified.

- Manufacturer: Keystone Lintels Ltd or equivalent and approved.

-	Product reference: as shown on drawings.

Material/ finish: Z1 Grade galvanised steel to BS EN 10346:2015.

Placement: Bed on mortar used for adjacent work with bearing of not less than 150 mm as per manufacturer's instructions.

760	COPING UNITS

Standard: To BS 5642-2.

-	Material: Pre-cast concrete.

Manufacturer: contractors’ choice.

Product sample to be approved before used on site.

Dimensions: As shown on drawings. Finish: Smooth.

Mortar for bedding/ jointing: Cement gauged as section Z21.

-	Mix: 1:1:5-6, Group 3. Joints: Full and finished flush.

-	Placement: Lay on a full bed of mortar to line and level.

MISCELLANEOUS ITEMS 840	OPENINGS FOR FRAMES

Formation: Use accurate, rigid templates to required size.

850	WALL PLATES

-	Placement: On full bed of mortar to correct horizontal level.

910	CLAY/ CERAMIC FLUE LINING AND FLUE BLOCK CHIMNEY SYSTEM FOR CONNECTION TO STOVE

- Lining standard: CE certified to BS EN 1457-1 & 2. Block standard: To BS EN 1806

  - Class: A1 N1.

  - Section/ internal dimensions: 200mm diameter HETAS approved

DONEGAL COUNTY GOUNGIL

Manufacturer: Schiedel & Co, or equivalent and approved

  - Product reference: Traditional Swift.

Accessories: incl. all necessary accessories such as precast concrete corbel to carry brick/block cladding, expansion plate at coping with chimney pot (F30/940) and as indicated on the drawings. Suppoi1 block and adaptor on lintels for connection to stove and including proprietary chimney tray, please refer to architectural detail.

- Jointing material: Ceramic high temperature sealant as supplied by the flue manufacturer.

- Linings placement: Fully bed with socket or rebate uppermost using correct starters, adaptors, bends etc.

Joints: Flush to provide an unrestricted flueway with smooth interior surfaces.

Void between lining and surrounding work: Fill with flexible insulation as supplied by the flue manufacturer.

Block placement: Finish joints flush to ensure correct alignment with correct stai1er, offset and transfer blocks. Course bonded blocks with adjacent masonry. Support non-bonded blocks.

Core ball test and smoke test to BS 6461-1: On completion, carry out each test. If obstructions or leaks are revealed, submit proposals for making good.

Complete chimney to be built as per manufacturer’s instructions.

930	FIREPLACE COMPONENTS

- Standard: See spec Z11-101.

- Manufacturer: to be approved.

- Product reference: sample to be approved before order placed.

- Cement sand surround and hearth.

940	CHIMNEY POTS

Manufacturer: Schiedel & Co. Ltd. or equivalent and approved. Product reference: 450mm high Roll Top finishing pot.

Coping: pre-cast concrete to detail.

Placement: Bed solid in mortar specified for chimney stack.

###### F31	PRECAST CONCRETE SILLS/ LINTELS/ COPINGS/ FEATURES

To be read with Preliminaries/ General conditions.

###### TYPES OF COMPONENT

110	EXTERNAL PRECAST COPINGS, SILLS, CHIMNEY CAPPING

Concrete grade and cover to reinforcement: To BS 8500-1:2006+A1:2012 AND BS EN 206- 1, grade RC50 or C50 with 30mm minimum nominal cover on visible faces, 20mm on internal or bedded faces, for single mix.

Finish to visible faces:

-	Matching sample: Fine and smooth.

Bedding and jointing mortar: 1:0.5:4 cement:lime :sand..

130	CONCEALED PRECAST LINTELS Standard: To BS EN 845-2:2013.

-	Verification of performance: Submit calculations or test certificates.

140	CONCEALED PRECAST LINTELS

-	Concrete: To BS 8500-1:2006-rA1:2012 and BS EN 206-1.

Mix: Not less than RC30 or C30.

-	Aggregate nominal maximum size: 20 mm.

Reinforcement provision for spans up to 1800 mm: As indicated in the following table:

Clear span

Up to 900 mm wall

900 to

1800 mm

wall

Section	Bearing	Reinforcement

140 mm deep	150 mm at	1 no. 12 mm mild x width of		both ends	steel bar for each 105 mm of

wall thickness.

215 mm deep	225 mm at	1 no. 16 mm mild x width of	both ends	steel bar for

each 105 mm of wall thickness.

-	Cover to reinforcement (nominal): 25 mm minimum.

150	CONCEALED PRECAST TO DOOR BLOCKS

-	Concrete: To BS 8500-1:2006+A1:2012 and BS EN 206-1.

- Mix: Not less than RC30 or C30.

- Aggregate nominal maximum size: 20 mm.

-	Other requirements: to detail.

###### GENERAL REQUIREMENTS

210	MOULDS

-	Permissible fabrication and operating tolerances: Length 0 to +6 mm, other dimensions z3 mm.

220	CONCRETE GENERALLY

-	Production of concrete: To BS 8500-1:2006+A1:2012 and BS EN 206-1.

Chloride ion content of mix constituents: Not to exceed 0.4% of weight of cement. Admixtures containing calcium chloride: Not allowed.

230	AGGREGATES

-	Standard: To BS EN 12620:2002-rA1:2008.

-	Maximum drying shrinkage when tested to BS 812-120? 0.07s”/o

250	REINFORCEMENT

Carbon steel reinforcement: As appropriate to BS 4449:2005+A2:2009, BS 4482:2005 and BS 4483:2005.

-	Cutting and bending: To BS 8666:2005.

Galvanized reinforcement: Galvanized to BS EN ISO 1461:2009 after cutting. Chromate treated.

- Stainless steel reinforcement: To BS 6744.

  - Designation (grade): 1,4301 (304).

- Non-structural reinforcement: Include to resist shrinkage and handling stresses.

Bimetallic corrosion and staining: Prevent by appropriate selection and use of materials. Condition at time of placement: Clean, free of corrosive pitting, loose materials and substances that adversely affect reinforcement, concrete, or bond between the two.

Fixing: Accurate and secure.

  - Method: Wire tying, approved steel clips or tack welding if permitted.

  - Concrete cover: Maintain free of all tying wire or clips.

260	CASTING AND CURING

Placement of concrete: Thoroughly compact.

Immature components: Avoid movement, vibration, overloading, physical shock, rapid  cooling and thermal shock.

Protection from weather: Do not expose panels to direct sunlight and drying winds until at least five days after casting.

Delivery to site: Not until at least 14 days after casting.

262	RECORDS

Records for each type of component: Maintain details including:

- Unique identification number,

- Correlation with records of mixes, including batch numbers.

- Date of each stage of manufacture.

- Dates and results of all tests checks and inspections.

- Dimensions related to specified levels of accuracy.

- Specific location in the finished work.

- Damage and making good.

- Any other pertinent data, e.g. if unit is a production control unit.

-	Availability of records for inspection: On request.

###### FAIR FACED COMPONENTS

310	CONTROL SAMPLES

Required samples: After finalization of design, one each of the following components: Windowsill

Coping stone.

Chimney capping.

Threshold block.

Approval of appearance: Obtain before manufacture of remaining units.

Identification and storage location: Clearly label and retain at factory for comparison with production units.

Final use: can be incorporated in the works.

320	DETAILS OF SAMPLES

Submittals after approval of appearance and before manufacture of production units:

- Aggregates: Confirm type, nominal maximum size, grading and source.

- Mix properties: Evidence of compliance for compressive strength and free-water/ cement ratio.

330	MIXES FOR VISIBLE FACED COMPONENTS

Constituent materials and mix design for each finish type: To remain constant.

Colour and appearance of each finish type: To remain constant.

Aggregates: To BS EN 12620:2002+A1:2008 and consistent colour, free of deleterious materials that may cause popping and staining.

-	Origin: Single source for each finish type, having sufficient quantity for whole contract.

341		CONDITIONS FOR SEPARATE FACING AND BACKING MIXES Difference in cement content: Not greater than 80 kg/m’.

Thickness of facing mix (minimum): 10 mm greater than nominal maximum aggregate size, and not less than 25 mm.

Location of reinforcement: Not less than 20 mm away from the interface between mixes. Compaction of facing and backing mix: Carry out to create monolithic construction.

350	QUALITY OF FINISHES

-	Appearance standard: As established by samples.

- DURABILITY FOR EXTERNAL COMPONENTS

- Concrete grade and cover to reinforcement. The quality of concrete and nominal cover to reinforcement (including secondary reinforcement) on visible faces in the finished work must be related and chosen from the following table:

Minimum acceptable concrete grades to BS 8500-1:2006+A1:2012 and BS EN 206-1:

  - Designated mixes	RC40	RC45	RC50

  - Designed mixes	C40	C45	C50

| Minimum cement content (kg/m°): |  |  |
| --- | --- | --- |
| - With 20 mm aggregate 325 | 350 | 400 |
| - With 10 mm aggregate 365 | 390 | 440 |

Maximum free	0.55	0.50	0.45 water/ cement ratio

| Air entrainment Required | Required | Optional |
| --- | --- | --- |
| Nominal cover (mm) for: - Visible faces: |  |  |
| Single mix 50 40 | 30 |  |
| Different mixes for 50 facing and backing | 50 50 |  |
| - Internal faces (protected) | 30 25 | 20 |

  - Bedded faces: Thickness of nominal cover: 20 mm.

Exposed aggregate faces: Thickness of nominal cover to be exclusive of aggregate projection.

Use of sulfate resisting cement: Thickness of nominal cover on visible faces to be increased by 10 mm.

Use of galvanized or stainless steel reinforcement: Submit proposals for quality of concrete and nominal cover.

Air entrainment: 5% by volume using admixture to BS EN 934-2:2009 + A1:2012, Concrete testing: Sample and cube test every batch.

- DURABILITY FOR EXTERNAL COMPONENTS Concrete grade: RC50 or CSO.

  - Minimum cement content: 400 kg/m’ with 20 mm aggregate or 440 kg/m° with 10 mm aggregate.

  - Maximum free water/ cement ratio: 0.45.

- Nominal cover to reinforcement on visible faces:

  - Single mix: 30 mm minimum,

  - Different mixes for facing and backing work: 50 mm minimum. Nominal cover to reinforcement on internal and bedded faces: 20 mm.

370	COVER ON VISIBLE FACES Spacers: Not permitted.

Proposed method statement: Submit.

380	CONSISTENCY OF PRODUCTION METHODS

Production methods: To remain consistent for each matching type of finish.

-	Finish appearance: To remain within the range of variation indicated by the samples submitted.

Changes to production methods: If variations are proposed for components of the same finish, submit evidence that there will be no difference in appearance.

390	INSPECTION

Give notice: When completed components are ready to be inspected at factory.

400	DAMAGED COMPONENTS Repairs: Submit proposals.

INSTALLATION

420	LAYING

Bedding components: On full bed of moi1ar.

-	Packing: If required use slate.

Removal of marks, stains and extraneous mortar on visible faces: Rubbing not permitted.

430	SUPPORT DF EXISTING WORK OVER NEW LINTELS

-	Joint above lintels: Fully fill and compact with semi-dry mortar.

440	ONE PIECE SILLS/ THRESHOLDS

-	Bed joints: Leave clear of mortar except at end bearings and beneath masonry mullions.

-	On completion: Point with mortar to match adjacent work.

GZO

G20	CARPENTRY/ TIMBER FRAMINGI FIRST FIXING

To be read with Preliminaries/ General conditions.

GENERAL INFORMATION/ REQUIREMENTS

115	CONTRACTOR DESIGNED STRUCTURAL COMPONENTS

Design standard: To BS EN 1995-1-1:2004 and Roof Truss Manufacturers Approval Scheme IS EN 1995-1-1:2004 - List of Registered Companies.

Completion of design: Design and detail members and connections to satisfy loading requirements specified or otherwise calculable from the information given.

Drawing content: Drawings must show:

- Geometry of structure.

- Timber species and grades/ strength classes.

- Target/ Finished sizes of all timber sections.

- Joints not detailed on Engineer’s drawings, their locations and evidence as to their structural adequacy.

Compliance with design standard: Demonstrate by calculation or by prototype load testing in accordance with BS EN 1995-1-1:2004.

Preservation treatment: All timbers to be vacuum/pressure treated with approved

preservative to I.S. EN 14250:2010 desired service life: 60 Years.

150	STRENGTH GRADING OF TIMBER

Grader: NSAI Visual Strength Graded Timber Approval scheme I.S. 127:2002 — List of Registered Companies.

Grading and marking of timber:

- Timber of a target/ finished thickness less than 100 mm and not specified for wet exposure: Graded at an average moisture content not exceeding 20% with no reading being in excess of 24% and clearly marked as ‘DRY’ or 'KD’ (kiln dried).

- Timber graded undried (green) and specified for installation at higher moisture contents: Clearly marked as ’WET’ or ’GRN'.

- Structural timber members cut from large graded sections: Regraded to approval and marked accordingly.

###### TYPES OF TIMBER

210	GRADED SOFTWOOD FOR JOISTS

Grading standard: To IS 127:2002, BS 4978:2007 or BS EN 14081-1:2005 and so marked. Strength class to BS EN 338:2009 C24.minimum

Treatment: Vacuum treated with approved preservative as section Z12.

-	Moisture content at time of erection: As clause 450.

230	GRADED SOFTWOOD FOR TRUSSES AND MAKE-UP TIMBERS IN ROOFS Species and origin: NSAI Approved truss manufacturer.

Grading standard: To IS EN 1995-1-1:2004 and so marked.

-	Grade: To truss manufacturers spec.

- Treatment: Vacuum treated with approved preservative as section Z12.

- Moisture content at time of erection: As clause 450.

235	GRADED SOFTWOOD FOR INTERNAL NON-STRUCTURAL USE

—		To IS 127:2002 and so marked. Species and origin: Whitewood Grade: GS/M50

Surface finish: Sawn generally. Regularized.

Preservative treatment: As section Z12 and British Wood Preserving Association Specification CB

Type/desired service life: CCA40

—	Moisture content at time of erection: As clause 450.

DONEGAL COUNTY COUNCIL

240	UNGRADEO SOFTWOOD FOR STUDS AND GENERAL FRAMING:

—	Studs to consist of minimum 75 x 38mm studs @ 400mm centers

- Free from decay, insect attack (except pinholes borers) and with no knots wider than half the width of the section.

- Surface finish: Regularized as Clause 410

- Preservative treatment: As section Z12 and British Wood Preserving Association Specification C8

Type/desired service life: CCA40

- Moisture content at time of erection: As clause 450.

245	UNGRADED SOFTWOOO FOR SOFFITS AND FASCIAS:

—	Free from decay, insect attack (except pinholes borers) and with no knots wider than half the width of the section.

- Surface finish: Regularized as Clause 410

- Preservative treatment: As section Z12 and British Wood Preserving Association Specification C8

Type/desired service life: CCA40

- Moisture content at time of erection: As clause 450.

- PLYWOOD FOR DOMESTIC FLOORING

Standard: To the relevant national standards and quality control procedures specified in BS EN 1995-1-1:2004, and so marked.

Type: American/Canadian/Finnish/Plywood.

  - Grade: C-O.

  - Nominal thickness/ number of plies: 18.5

  - Finish: Unsanded.

- NONSTRUCTURAL PLYWOOD FOR EXTERNAL SOFFITS

- Thickness: 12mm

Appearance Class: BS.635 II

Band quality to BS.EW. 314 - 2 Class 3

- Finishing coats:

  - Type: Exterior gloss

  - Number of coats: one.

Treatment: N/A

320	PLYWOOD FOR GUTTER LININGS AND EXTERNAL SOFFITS ANO ATTIC WALKWAY Standard: To the relevant national standards and quality control procedures specified in BS EN 1995-1-1:2004, and so marked.

Type: American/Canadian/Finish Plywood. Grade: WBP external grade.

Nominal thickness/ number of plies: 18.5 Finish: Unsanded.

350	TRUSSED RAFTERS

Design and fabrication: To BS EN 1995-1-1:2004 and BS EN 14250:2010. Manufacturer: A firm currently registered under a third-party quality assurance scheme.

###### WORKMANSHIP GENERALLY

- CROSS SECTION DIMENSIONS OF STRUCTURAL SOFTWOOD AND HARDWOOD  TIMBER

- Dimensions: Dimensions in this specification and shown on drawings are target sizes as defined in BS EN 336:2013.

Tolerances: The tolerance indicators (T1) and (T2) specify the maximums permitted deviations from target sizes as stated in BS EN 336:2013, clause 5.3:

  - Tolerance class 1 (T1) for sawn surfaces.

  - Tolerance class 2 (T2) for further processed surfaces.

DONEGAL COUNTY COUNCIL

- CROSS SECTION DIMENSIONS OF NONSTRUCTURAL SOFTWOOD TIMBER Dimensions: dimensions in this specification and shown on drawings are finished sizes. Maximum permitted deviations from finished sizes: As stated in BS EN 1313-1:2010

  - Clause 6 for sawn sections.

  - Clause NA.2 for further processed sections.

- CROSS SECTION DIMENSIONS OF NONSTRUCTURAL HARDWOOD TIMBER Dimensions: Dimensions in this specification and shown on drawings are finished sizes. Maximum permitted deviations from finished sizes: As stated in BS EN 1313-2:1999

Clause 6 for sawn sections.

  - Clause NA.3 for further processed sections.

420	WARPING OF TIMBER

-	Bow, spring, twist and cup: Not greater than the limits set down in BS 4978:2007 or BS EN 14081-1:2005 for softwood, or BS 5756:2007 for hardwood.

430	SELECTION AND USE OF TIMBER

- Timber members damaged, crushed or split beyond the limits permitted by their grading: Do not use.

Notches and holes: Position in relation to knots or other defects such that the strength of members will not be reduced.

- Scarf joints, finger joints and splice plates: Do not use without approval.

435	TREATMENT OF TIMBER

Timber wall plates to be vacuum/ pressure treated with Tanalith E to comply with treatment code TE/BI to give a service life of d0 years.

Following treatment any areas of timber revealed by crosscuts, holes, notches etc. shall be brushed with ENSELE or similar end grain preservative

440	PROCESSING TREATED TIMBER

-	Cutting and machining: As much as possible before treatment.

Extensively processed timber: Retreat timber sawn lengthways, thicknessed, planed, ploughed, etc.

Surfaces exposed by minor cutting and/ or drilling: Treat with two flood coats of a solution recommended by main treatment solution manufacturer.

450	MOISTURE CONTENT

- Moisture content of timber and wood based products at time of installation: Not more than:

  - Covered in generally unheated spaces:	24%.

  - Covered in generally heated spaces: 20%.

  - Internal in continuously heated spaces:	20%.

451	MOISTURE CONTENT TESTING

- Procedure: When instructed, test timber sections with an approved electrical moisture meter.

- Test sample: Test 5% but not less than 10 lengths of each cross-section in the center of the length.

Test results: 90% of values obtained to be within the specified range. Provide records of all tests.

510	PROTECTIDN

Generally: Keep timber dry and do not overstress, distort or disfigure sections or components during transit, storage, lifting, erection or fixing.

Timber and components: Store under cover, clear of the ground and with good ventilation. Support on regularly spaced, level bearers on a dry, firm base. Open pile to ensure free movement of air through the stack.

Trussed rafters: Sleep vertical during handling and storage.

520	EXPOSED END GRAIN

-	Components: Treat and Seal exposed end grain before delivery to site: Using sealer to manufacturers spec.

530	PAINTED FINISHES

-	Structural timber to be painted: Primed as specified before delivery to site.

540	CLEAR FINISHES

Structural timber to be clear finished: Keep clean and apply first coat of specified finish before delivery to site.

550	EXPOSED TIMBER

Planed structural timber exposed to view in completed work: Prevent damage to and marking of surfaces and arises.

###### JOINTING TIMBER

570	JOINTING/ FIXING GENERALLY

-	Generally: Where not specified precisely, select methods of jointing and fixing and types, sizes and spacings of fasteners in compliance with section Z20.

Fixing: Secure using not less than the number of nails recommended by anchor manufacturer.

600	BLACK BOLTS AND NUTS

-	Standard: To BS EN ISO 898-1:2013 and BS EN ISO 898-2:2012

Finish (applied by manufacturer)

610	BLACK CUP AND COUNTERSUNK HEAO BOLTS AND NUTS

Standard: To BS 4932:2010 Finish (applied by manufacturer)

620	WASHERS

- Standard: Plain to BS 4320:1968, spring to BS 4464:1969 Material and finish: To match bolts.

- ‹ Dimensions when seated directly on timber surfaces: Unless specified otherwise:

  - Diameter/ side length: Nat less than 3 times bolt diameter.

  - Thickness: Not less than 0.25 times bolt diameter.

625	STAINLESS STEEL BOLTS, NUTS AND WASHERS

Bolts and nuts:

- for connection of trusses and joists

- Standard: To BS EN ISO 3506:2009, Grade 8.8 Material: stainless steel

- Diameter: 12 mm and 10 mm as requested on details. Washers:

- Material: To match bolts.

- Diameter: 12 mm and 10 mm to match bolts.

626	STAINLESS STEEL BOLTS FOR FIXING JOISTS TO CONCRETE BLOCKWORX

Provide Hilti M12 Anchor Rod with HY50 resin injection with nut and washer. Standard: To BS EN ISO 3506:2009, Grade 8,8

Washers

Material to match bolts Diameter: 12 mm

630	BOLTED JOINTS

Holes for bolts: Located accurately and drilled to diameters as close as practical to the nominal bolt diameter and not more than 2 mm larger.

Washers: Placed under bolt heads and nuts that would otherwise bear directly on timber. Use spring washers in locations which will be hidden or inaccessible in the completed building.

Bolt tightening: So that washers just bite the surface of the timber. Ensure that at least one complete thread protrudes from the nut.

-	Checking: At agreed regular intervals up to Practical Completion. Tighten as necessary.

670	ANTICORROSION FINISHES FOR FASTENERS

Galvanizing: To BS 7371-6:1998, with internal threads tapped and lightly oiled following treatment.

Sherardizing: To BS 7371-8:2011, Class 1.

- Zinc plating: To BS EN ISO 4042:2000 and passivated.

- Treatment not specified: Select from the above to suit service conditions.

ERECTION AND INSTAI.LATION

710	PROPOSALS FOR ERECTING STRUCTURAL TIMBER

- Proposals: Submit details of:

  - Method and sequence of erection.

  - Type of carnage.

  - Temporary guys and bracing proposed for use during erection.

- Latest date for submission: one week before erection starts.

740	PRE-ERECTION CHECKING

Timing: Not less than 10 days before proposed erection start date. Checklist:

Foundations and other structures to which timber structure will be attached: Check for accuracy of setting out.

-	Holding down bolts: Check for position, protruding length, condition and slackness. Inaccuracies and defects: Report without delay.

Erection: Obtain permission to commence.

750	MODIFICATIONS/ REPAIRS

- Defects due to detailing or fabrication errors: Report without delay.

Methods of rectification: Obtain approval of proposals before starting modification or remedial work.

- Defective/ damaged components: Timber members/ components may be rejected if the nature and/ or number of defects would result in an excessive amount of site repair.

760	TEMPORARY BRACING

-	Provision: As necessary to maintain structural timber components in position and to ensure complete stability during construction.

770	AODITIONAL SUPPORTS

Provision: Where not shown on drawings, position and fix additional studs, noggings or battens for appliances, fixtures, edges of sheets, etc.

Material properties: Additional studs, noggings and battens to be of adequate size and have the same treatment, if any, as adjacent timber supports,

775	BEARINGS

- Timber surfaces which are to transmit loads: Finished to ensure close contact over the whole of the designed bearing area.

Packings: Where provided, to cover the whole of the designed bearing area.

  - Crushing strength: Not less than timber being supported.

  - In external locations: Rot and corrosion proof.

780	WALL PLATES

Position and alignment: To give the correct span and level for trusses, joists, etc. Bedding: Fully in fresh moi1ar.

-	Joints: At corners and elsewhere where joints are unavoidable use nailed half lap joints. Do not use short lengths of timber.

784	JOISTS GENERALLY

-	Centers: Equal, and not exceeding designed spacing.

Bowed joists: Installed with positive camber.

End joists: Positioned approximately 50 mm from masonry walls.

786	JOISTS ON HANGERS

Hangers: Bedded directly on and hard against supporting construction. Do not use packs or bed on mortar.

Joists: Cut to leave not more than 6 mm gap between ends of joists and back of hanger. Rebated to lie flush with underside of hangers.

Fixing to hangers: A nail in every hole.

791	JOIST HANGERS FOR FIRST FLOOR TRIMMER

Manufacturer: To BS 845-1:2013. Material/ finish: Galvanised.

-	Size: To suit joist, design load and crushing strength of supporting construction.

795	TRIMMING OPENINGS

Trimmers and trimming joists: When not specified otherwise, not less than 25 mm wider than general joists.

800	TRUSSED RAFTERS

Erection: To BS EN 1995-1-1:2004,

Trusses generally: Do not modify without approval.

-	Damaged trusses: Do not use.

Fixing: Truss clips and bottom chords of standard trusses and rafters of raised tie trusses bearing fully on wall plates.

-		Bottom chords of standard trusses: Do not fix to internal walls until roofing is complete and cisterns are installed and filled.

805	TRUSS CLIPS

Manufacturer: To be confirmed.

-	Fasteners: 32 x 3.5 mm galvanized or sherardized square twisted nails in every hole.

810	PERMANENT BRACING OF TRUSSED RAFTERS

Bracing and binders: Fixed to every rafter, strut or tie with not less than two 75 x 3.35 mm galvanized round wire nails.

Lap joints: Extended over and nailed to at least two truss members.

820	VERTICAL RESTRAINT STRAPS

Manufacturer: Ancon or equal approved. Material/ finish: Mild Steel, galvanised.

- Size:

Cross section: Not less than 30 x 5 mm.

  - Length: 1200mm

- Centers: Not more than 1.2m.

Fixing:

  - To timber members with not less than two 8 mm nails.

  - To masonry with not less than 4 screws evenly spaced.

  - With at least one screw located within 150 mm of the bottom end of each strap.

830	LATERAL RESTRAINT STRAPS

Manufacturer: Ancon or equal approved. Material/ finish: Mild steel galvanised.

- Size: Not less than 30 x 5 mm cross section, 150 mm cranked end.

Fixing: To top of joists/ rafters/ ties at not more than 1.2 m centers and as shown on drawings.

  - Ensure that cranked end is in tight contact with cavity face of wall inner leaf and is not pointing upwards.

- Straps spanning joists/ rafters/ ties running parallel to wall: Fix noggings and packs tightly beneath straps.

  - Size of noggings and packs: Not less than three quarters of joist rafters/ tie depth and not less than 38 mm thick.

|  |  | - Notching: Notch joists so that straps fit flush with surface. Do not notch rafters/ ties. Fasteners: Not less than four 50 mm x 8 gauge sherardized countersunk screws per strap, evenly spread. |
| --- | --- | --- |
| 840 |  | STRUTTING Type: One of the following: - Herringbone strutting: At least 38 x 38 mm softwood, - Solid strutting: At least 38 mm thick softwood and at least three quarters of joist depth. - Fixing: Between joists as follows: - Joist spans of 2.5 to 4.5 m: One row at center span. - Joist spans over 4.5 m: Two rows equally spaced. - Strutting must not project beyond top and bottom edges of joists. Outer joists: Blocked solidly to perimeter walls. |
| 850 - |  | INSPECTION Structural timberwork: Give reasonable notice before covering up. |
| 860 | ' | BOLTED JOINTS Inspection: inspect all accessible bolts at the end of the Defects Liability Period and tighten if necessary. |
| 910 |  | EAVES SOFFIT VENTILATION Proprietary eaves vent units: Fixed to leave a continuous ventilation opening not less than 15 mm wide for full length of eaves. Insect mesh to be incorporated into vent: A 3 to 4 mm mesh screen fixed across the opening to prevent large insect entry. |

###### M20	PLASTERED/ RENDERED/ ROUGHCAST COATINGS

To be read with Works Requirements

###### TYPES OF COATING

110	CEMENT:LIME:SAND RENDER TO CHIMNEY, BELLCAST AND REVEALS

- Background: dense concrete blocks.

  - Preparation: recess joints and apply scud coat of 1:2 cement:sharp sand. Undercoats:

  - Lime:sand: Ready-mixed to BS 4721.

  - Sand: To BS 1199, type A.

  - Mix (cement:lime:sand): 1:1:6 cement : lime : sand.

Other requirements: none.

  - Thickness (excluding dubbing out): 10-16mm. Final coat:

Lime:sand: Ready-mixed to BS 4721. Sand: To BS 1199, type A.

Lime:sand manufacturer: contractors choice. Product reference: to be agreed.

Mix (cement:lime:sand): 1:1;4.5.

Thickness: 8-10mm. Finish: wood float.

- Accessories: angle beads. MESH

160	RENDER TO EXTERNAL WALLS- Refer to Works Requirements

- Background: dense concrete blocks.

-	Preparation: recess joints and apply scud coat of 1:2 cement:sharp sand.

- Undercoats:

Accessories: AR mesh reinforcement must be included in the rendering along the lintel. external arises are formed by using rigid PVC or stainless steel beads Bell cast stops and beads.

413	SAMPLES

-	General: Provide representative samples of the following: Proprietary self coloured render including raised smooth finished bands to openings,

418	CONTROL SAMPLES

Complete sample areas, being part of the finished work, in locations as follows: Rear elevation to one house to be completed and approved before proceeding,

423	UNIFORMITY OF COLOUR AND TEXTURE

-	General: Maintain consistent colour and texture. Obtain materials from one source. Mix different loads if necessary.

438	CEMENTS FOR RENDER MORTARS Cement:

- Standard: To BS EN 197-1.

- Types:	Portland cement, CEM I.

Portland slag cement, CEM II/ B-S. Portland fly ash cement, CEM II/ B-V.

- Strength class: 42.5 or 52.5.

- Sulfate resisting cement:

  - Standard: To BS 4027.

  - Strength class: 42.5 or 52.5. Masonry cement:

  - Standard: To BS 5224.

  - Class: MC 12.5 (with air entraining agent. Certification for all cements: BSI Kitemark scheme.

441	SITE PREPARED LIME:SAND FOR CEMENT GAUGED RENDER MORTARS Permitted use: Where a special colour is not required and in lieu of factory made ready- mixed material.

Lime: Hydrated nonhydraulic lime to BS 890.

Mixing: Thoroughly mixed. Allow to stand, without drying out, for at least 16 hours before using.

444	READY TO USE CEMENT GAUGED RENDER MORTARS

Standard: Generally, to BS 4721, using materials specified in this section.

Time and temperature limitations: Use within limits prescribed by moi1ar manufacturer.

-	Retempering: Restore workability with water only within prescribed time limits.

449	ADMIXTURES FOR CEMENT GAUGED RENDER MORTARS

- Suitable admixtures: Select from:

  - Air entraining (plasticizing) admixtures: To BS 4887-1 and compatible with other mortar constituents.

  - Other admixtures: Submit proposals.

Prohibited admixtures: Calcium chloride and any admixture containing calcium chloride.

452	MIXING

Render mortars (site prepared):

- Batching: By volume. Use clean and accurate gauge boxes or buckets.

- Mix proportions: Based on damp sand. Adjust for dry sand.

Mixes: Of uniform consistence and free from lumps. Do not retemper or reconstitute mixes. Contamination: Prevent. Keep plant and banker boards clean,

466	SCAFFOLDING

-	General: Prevent putlog holes and other breaks in coatings.

474	COLD WEATHER

General: Do not use frozen materials or apply coatings to frozen or frost bound backgrounds.

External work: Avoid when air temperature is at or below 5°C and falling or below 3°C and rising. Maintain temperature of work above freezing until coatings have fully hardened.

Internal work: Take all necessary precautions to enable internal coating work to proceed without damage when air temperature is below 3°C.

###### PREPARING BACKGROUNDS

510	SUITABILITY OF BACKGROUNDS

General: Suitable to receive coatings/ backings.

- Soundness: Free from loose areas and significant cracks and gaps.

- Cutting, chasing, making good, fixing of conduits and services outlets and the like: Completed.

- Tolerances: Permitting specified flatness/ regularity of finished coatings. Cleanliness: Remove dirt, dust, efflorescence and mold, and other contaminants incompatible with coatings.

527	RAKING OUT FOR KEY

Soft joints in existing masonry: Rake out to a depth of 13 mm (minimum).

-	Dust and debris: Remove from joints.

531	ROUGHENING FOR KEY

Backgrounds: Roughen thoroughly and evenly.

-	Depth of surface removal: Minimum necessary to provide an effective key.

536		SPATTERDASH KEYING MIX Materials:

Cement: Portland.

Sand: Clean coarse.

- Admixtures: SBR bonding agent. Mix proportions (cement:sand): 1:1.5-2.

Consistency: Thick slurry kept well stirred.

Application: Throw onto dampened background and leave rough.

- Thickness: 2-3 mm.

-	Curing: Controlled to achieve a firm bond to background.

541	BONDING AGENT

- Manufacturer: Kingfisher Chemicals Ltd. Cooper Lane Bardsea, Ulverston, Cumbria LA12 9RA. Tel. O1229 869100, fax 01229 869101.

-	Product reference: SBR bonding agent.

- Application: Evenly across background to achieve effective bond or plaster/ render coat to background. Protect adjacent joinery and other surfaces.

556	REMOVING DEFECTIVE EXISTING RENDER

Render for removal: Loose, hollow, soft, friable, badly cracked, affected by efflorescence or otherwise damaged.

- Patches: Cut out rectangular areas with straight edges,

  - Horizontal and vertical edges: Square cut or slightly undercut.

  - Bottom edges to external render: Do not undercut.

  - Render with imitation joints: Cut back to joint lines.

Cracks (other than hairline cracks): Cut out to a width of 75 mm (minimum). Dust and loose material: Remove from exposed backgrounds and edges.

###### BACKINGS/ BEADS/ JOINTS

600	AODITIONAL FRAMING SUPPORTS FOR BACKINGS

Fixtures, fittings and service outlets: To suit fixings.

Board edges and perimeters: As recommended by board manufacturer to suit type and performance of board.

605	GYPSUM PLASTERBOARD BACKINGS

Type: To BS 1230, type 1.

-	Core density (minimum): 650 kg/m’.

Exposed surface and edge profiles: Suitable to receive specified plaster finish,

610	FIXING PLASTERBOARD BACKINGS TO TIMBER BACKGROUNDS

Fixings, accessories and installation methods: As recommended by board manufacturer. Fixing: At following centers (maximum):

- Nail fixing: 150 mm.

- Screw fixing to partitions/ walls: 300 mm reduced to 200 mm at external angles.

- Screw fixing to ceilings: 230 mm.

Position of nails/ screws from edges of boards (minimum):

- Bound edges: 10 mm.

- Cut/ unbound edges: 13 mm.

Position of nails/ screws from edges of supports (minimum): 6 mm.

Nail/ screw heads: Set in a depression. Do not break paper or gypsum core.

612	JOINTS IN PLASTERBOARD BACKINGS

Ceilings:

- Bound edges: At right angles to supports and with ends staggered in adjacent rows.

- Two-layer boarding: Stagger joints between layers. Partitions/ walls:

- Vertical joints: Centre on studs. Stagger joints on opposite sides of studs.

Two-layer boarding: Stagger joints between layers.

- Horizontal joints:

Two-layer boarding: Stagger joints between layers by at least 600 mm. Support edges of outer layer.

Joint widths (maximum): 3 mm.

630	BEADS/ STOPS FOR INTERNAL USE

-	Material: Galvanized steel to BS EN 10142 :1991. DX51D + Z275 & DX51D + Z450.

636	BEADS/ STOPS FOR EXTERNAL USE

-	Stainless steel: To BS EN 10088-1: 1995 X5CrNi18-10 / 1.4301 or equal approved

640	BEADS/ STOPS GENERALLY

- Location: External angles and stop ends except where specified otherwise.

Corners: Neat miters at return angles.

- Fixing: Secure, using longest possible lengths, plumb, square and true to line and level, ensuring full contact of wings with background.

-	External render: Fix mechanically.

- Finishing: After coatings have been applied, remove surplus material while still wet, from surfaces of beads/ stops which are exposed to view.

646	CRACK CONTROL AT JUNCTIONS BETWEEN DISSIMILAR SOLID BACKGROUNDS

- Locations: Where defined movement joints are not required. Where dissimilar solid background materials are in same plane and rigidly bonded or tied together.

Crack control materials:

  - Isolating layer: Building paper to BS 1521.

  - Stainless steel Metal lathing:

Installation: Fix lathing over isolating layer at staggered centers along both edges. Width of installation over single junctions:

  - Isolating layer: 150 mm.

  - Lathing: 300 mm.

Width of installation across face of dissimilar background material (column, beam, etc. with face width not greater than 450 mm):

  - Isolating layer: Not less than 25 mm beyond junctions with adjacent background.

  - Lathing: Not less than 100 mm beyond edges of isolating layer.

653	SEALANT MOVEMENT JOINTS WITH STOP BEAD EOGINGS FOR EXTERNAL RENDER

Stop beads: as clause 636.

Installation: Centered over joint in background.

- Joint width: to suit that of structural movement joint in background.

- Fixing: stainless steel screws.

Sealant: as Z22/120.

Colour: to match finish.

- Preparation and application: As section Z22.

659	PLASTERBOARD JOINTS

Joint reinforcement: Applied to joints and angles except where coincident with metal beads.

-	Installation: Continuous lengths of tape pressed well into filled joints, flat and smooth.

673	PLASTERING OVER CONDUITS/ SERVICE CHASES

General: Prevent cracking over conduits and other services.

Services chased into background: Isolate from coating by covering with galvanized metal lathing. Fixed at staggered centers along both edges.

###### PLASTERING

710	APPLICATION GENERALLY

General: Apply coatings firmly and in one continuous operation between angles and joints. Achieve good adhesion.

Appearance of finished surfaces: Even and consistent. Free from rippling, hollows, ridges, cracks and crazing.

-		Accuracy: Finish to a true plane, to correct line and level, with angles and corners to a right angle unless specified otherwise, and with walls and reveals plumb and square.

-	Drying out: Prevent excessively rapid or localized drying out.

715	FLATNESS/ SURFACE REGULARITY

Sudden irregularities: Not permitted.

Deviation of plaster surface: Measure from underside of a straight edge placed anywhere on surface.

Permissible deviation (maximum) for plaster not less than 13 mm thick: 3 mm in any consecutive length of 1800 mm.

718	JUNCTIONS OF NEW PLASTERWORK WITH EXISTING

General: Finish new plasterwork flush with original face of existing plasterwork to form a seamless junction.

720	DUBBING OUT

General: To correct background inaccuracies.

-	Smooth dense concrete and similar surfaces: Dubbing out prohibited unless total plaster thickness is within range recommended by plaster manufacturer.

Thickness of any one coat (maximum): 10 mm. Mix: As undercoat.

Application: Achieve firm bond. Allow each coat to set sufficiently before the next is applied. Cross scratch surface of each coat.

725	UNDERCOATS GENERALLY

General: Rule to an even surface. Cross scratch to provide a key for the next coat. Undercoats on metal lathing: Work well into interstices to obtain maximum key.

Undercoats gauged with Portland cement: Do not apply next coat until drying shrinkage is substantially complete.

742	THIN COAT PLASTER

Preparation for plasters less than 2 mm thick: Fill holes, scratches and voids with finishing plaster.

777	SMOOTH FINISH

-		Appearance: A tight, matt, smooth surface with no hollows, abrupt changes of level or trowel marks. Avoid water brush, excessive troweling and over polishing.

-		Appearance: An even overall texture. Finish with a dry wood float as soon as wet sheen has disappeared.

RENDERING

810	APPLICATION GENERALLY

General: Apply coatings firmly and in one continuous operation between angles and joints. Achieve good adhesion.

- Appearance of finished surfaces: Even and consistent. Free from rippling, hollows, ridges, cracks and crazing.

-		Accuracy: Finish to a true plane, to correct line and level, with angles and corners to a right angle unless specified otherwise, and with walls and reveals plumb and square.

- Drying: Prevent excessively rapid or localized drying out.

815	FLATNESS/ SURFACE REGULARITY OF RENDERING TO RECEIVE CERAMIC TILES

Sudden irregularities: Not permitted.

Deviation of render surface: Measure from underside of a 2 m straight edge placed anywhere on surface.

-	Permissible deviation (maximum): 3 mm.

820	DUBBING OUT FOR RENDERING

General: To correct background inaccuracies. Thickness of any one coat (maximum): 16 mm.

-	Total thickness (maximum): 20 mm. Seek instructions where this will be exceeded. Mix: As undercoat.

Application: Achieve firm bond. Allow each coat to set sufficiently before the next is applied. Comb surface of each coat.

830	ANCHORED MESH REINFORCEMENT

-	Application of first undercoat: Through and round mesh to fully bond with solid background.

840	UNDERCOATS GENERALLY

- General: Rule to an even surface. Comb to provide a key for the next coat. Do not penetrate the coat.

- Undercoats on metal lathing: Work well into interstices to obtain maximum key.

856	FINAL COAT - PLAIN FLOATED FINISH

-	Finish: An even, open texture free from laitance.

861	FINAL COAT - SCRAPED FINISH

-	Finish: Scraped to expose aggregate and achieve an even texture.

880	CURING AND DRYING

General: Prevent premature setting and uneven drying of each coat.

Curing coatings: Keep each coat damp by covering with polyethylene sheet and/ or spraying with water.

- Curing period (minimum):4 days.

- Final coat: Hang sheeting clear of the final coat.

Drying: Allow each coat to dry thoroughly, with drying shrinkage substantially complete before applying next coat.

Protection: Protect from frost and rain.

890	CLEANING ON COMPLETION.

Before final hand over all external rendered surfaces are to be left clean and free from dirt or plant growth. The cleaning methods to be used must be agreed with the OS before use.