CONTRACT SCOPE OF WORKS

It is intended to remove and replace several corners and gables within a list of 26 dwellings in Buncrana. It is intended to strip each outer leaf from the DPC to the barge, replace the horizontal and vertical DPC, and extend radon barriers as necessary.

These external block walls are to be replaced like-for-like. The corners of each gable (not to receive full demolition works) and selected for minor repairs must be replaced for a distance of 0.9m into the front/back elevations and the gable elevation. This allowance of 0.9m is to be used to tie into existing blockwork on the front/back and gable elevations. All new blockworks will receive a new smooth render to match the remainder of the property.  On completion, all property gutters/fascia/soffits will be emptied/cleaned.

1.0	DEMOLITION

Blockwork removal shall be carried out in accordance with HSA and Waste Management Compliance, or any other relevant code of practice, and shall be carried out carefully with regard to the existing surroundings.

The order and method of each removal operation will be submitted to the DCC Engineer for review before commencement. This must include proposed methods for propping, shoring, or protecting adjoining areas and property. Notwithstanding any information in the Contract documents, the onus shall be on the Contractor to visit the site of the works and satisfy him/herself as to the nature and extent of the work and all other matters that might influence the price or time for completion of these works.

The blockwork removal shall be done without damaging the retained property (gardens/walls, sheds, etc.) or adjoining public areas (footpaths and roads). If damage should occur through negligence in the carrying out of this blockwork removal, the contractor shall reinstate and make good at his/her own expense. Any such reinstatement shall be executed with materials and workmanship to match the existing.

1.1	SUPERVISION

All blockwork removal work to be carried out under the supervision of a responsible and competent/experienced person who shall ensure that adequate safety precautions are taken, and that all necessary statutory regulations are adhered to. Before commencement of works, the Contractor is to advise the Client in writing of the name and contact details of the Contractor’s Site Manager.

1.2	SAFETY

The Contractor shall perform the blockwork removal works in accordance with the Safety, Health and Welfare at Work (Construction) Regulations 2013 and shall carry out this work in such a manner as to cause the minimum inconvenience to the public and adjoining property occupants. The contractor shall provide and maintain all necessary temporary fencing, hoardings, fans, planked footways, guard rails, gantries, and the like to protect the public and meet the requirements of the Local Authority. Please allow for the alteration, shifting, and adaptation of these barriers/hoardings as necessary. The Contractor will be held responsible for any claim that may arise from disregarding this clause.

1.3	METHODS OF BLOCKWORK REMOVAL

The Contractor shall indicate in the Method Statement in writing the manner of blockwork removal, i.e., machine, mechanical tools, piece-by-piece hand method, etc., for each area and specifically for adjoining existing buildings and boundaries. It should be noted that blockwork removal must be closely linked with the provision of any temporary supports required to adjoining buildings and facades. All methods used shall ensure an orderly and safe transfer of the loads from the existing to the temporary works and the permanent structure.

1.4	REMOVING CAVITY INSULATION

The contractor is to allow for the collection/disposal of bonded bead/fibre and rigid board insulation that becomes loose from the existing cavities during blockwork removal. This same material is not permitted to pollute adjacent properties/environment. The Contractor shall indicate by Method Statement in writing the manner of cavity insulation collection/disposal removal during blockwork removal, i.e. machine, by hand method, etc.

2.0	CONCRETE BRICKWORK / BLOCKWORK / MORTAR

All concrete blocks/bricks shall be manufactured in accordance with IS EN 771-3 and must have a Declaration of Performance and CE marking to comply with the Construction Products Regulations and shall have a System 2+ level of attestation of conformity. System 2+ requires the manufacturer’s production process to be independently audited and certified by a notified body.

Bricks: To IS EN 771-3:2011+A1:2015. Average compressive strength must be a minimum of 13.0 N/mm². - Work sizes: 215 x 65 x 100mm

Blocks: To IS EN 771-3:2011+A1:2015. Compressive strength (minimum):13.0 N/mm². Work sizes: 440 x 140 x 100mm.

Mortar: Mix: 1:3 masonry cement: sand with lime content

The contractor must provide certification of compressive strengths and confirmation that the blocks/bricks have been tested by the manufacturer for expansive minerals such as pyrites, and that the tests have demonstrated they contain quantities of such minerals below the minimum standard.

2.1	BLOCK LAYERS

Only experienced masons, skilled in their trade, shall be allowed to carry out block laying. Blocks shall be buttered with mortar before being laid, and all joints shall be thoroughly flushed up as the work proceeds. Blockwork shall be brought up uniformly; no portion being raised more than 1.5m above another at any time. All walls shall be kept strictly in horizontal and vertical alignment, and the whole shall be bonded appropriately together and levelled around the top at the completion of blockwork.

2.2 	WALL TIES

The two leaves of the cavity shall be tied together using the existing undamaged wall ties, and if necessary (depending on how the demolition is conducted), new inserted/fixed wall ties of wire or strip construction composed of stainless steel and complying with IS-EN 845-1, ACS 3000 Range tie with insulation retaining clips.

Cavities less than 110mm (TYPICAL SIZE TO BE ENCOUNTERED IN THIS CONTRACT): The tie spacing shall be 600mm horizontal and 450mm vertically staggered. Window and door openings: The tie shall be 225mm vertically and horizontally, 150mm back from the door/ window reveal. Plus, the ties must be resistant to water entering the cavity and must have mounted insulation-retaining clips.

2.3	DAMP PROOF COURSE

POLYETHYLENE: type DPC, which must be Agreement certified.

3.0	Cavity Insulation

When rebuilding the corners/gables / external walls, the contractor must ensure the rigid insulation board is undamaged.

Any windows or door openings must have insulated cavity closers, and the contractor must provide Agreement Certificates and certifications that all insulation to be used meets this specification. Closer to be Kingspan Kooltherm (or similar) rigid thermoset insulation core with ñ-value of 0.022 W/m.K, around windows.

4.0	RENDER TO EXTERNAL WALLS

Contractor to allow smooth rendering to all walls, bellcasts and tie-ins for the front and rear elevations receiving works.

4.1	RENDER 3-COAT SYSTEM

Scud coat must be 1nr layer 5mm thick; Undercoat: maximum 12mm thick, combed finish; Outer coat; maximum 10mm thick.  The new render must be faded to match the existing render at the tie in points on the front and rear elevations and the overall thickness of the new external render must be 25mm.

Contractor will be required to recess all joints before applying scud coat and must supply and install a K Rend Alkali-Resistant Reinforcing Mesh (or similar approved by DCC) at joints between new and existing blockwork on the front/rear and gable elevation tie-ins, around window and door openings, and on lintels. Supply must include all rigid uPVC (white) beads for corners, bell cast, and stop ends.

- SCAFFOLDING / ACCESS

The contractor shall supply (at his/her cost) all general scaffolding, hoists, working platforms, etc. required for the execution of the works. This equipment shall comply with all applicable statutory requirements and codes of practice and be erected by persons holding certification or registration cards in accordance with the Regulations.

The contractor must allow for its erection, adjustment, maintenance, and removal for the duration of the works, and its removal on completion. Only approved users with tickets/training records will be permitted to operate/erect this equipment on site. GA 3 forms will always be required for all scaffolding.

6.0	GENERAL REQUIREMENTS

6.1	HOURS OF WORK

The working hours under this contract are from 8.30 a.m. to 6.00 pm, Monday through Friday. Approval from Donegal County Council (DCC) is required for Saturday and Bank Holiday work. No work will be permitted on Sundays.

6.1	SUB-CONTRACTING AND PIECE WORK

Contractors shall not subcontract any part of the work to piecework without the Council's written approval. This will only be acceptable if the main contractors’ insurance indemnifies DCC against all claims arising from the hiring and use of subcontractors for these works. It will be the responsibility of the PSCS to evaluate and approve the H&S competency of these subcontractors.

7.0         Filling of Cracks / Openings

7.1	APPLICATION of Structural filler

All cracks 3mm or smaller shall be ground out with an abrasive wheel to a width of 5mm (30mm deep) and shall be filled with Fosroc Renderoc FC as per the manufacturer's instructions (others will paint).

As a min:

- All cracks over 5mm shall be filled with mortar and then filled with Fosroc Renderoc FC as per manufacturer's instructions (others will paint).

8.0	COMMENCEMENT OF WORK

The Contractor shall notify the Council 48 hours prior to commencing work on each property. Any works carried out prior to this notification shall not be accepted as work carried out under the terms of this Contract.

8.1	NOTICE TO OCCUPIERS OF BUILDING

The Contractor shall give dwelling occupiers 24 hours’ notice of intention to commence work on the premises. This shall be by visiting the property and using the company ID cards as identification. Any inconvenience to occupants caused by work set out in the contract should be kept to a minimum.

8.2	RESPONSIBILITY & PROTECTION OF BUILDINGS

The Contractor shall give 24 hours’ notice and arrange access to the work with the occupants. Contractor shall allow for bending over, staking, covering, or otherwise protecting all vegetation as may be necessary for the avoidance of damage and to leave the same in a similar condition as when the work commenced. No trimming or pruning of trees, shrubs, plantings, or other vegetation will be permitted without prior arrangement with the Council. The Contractor shall take all possible care to protect surfaces, furnishings, and furniture.

8.3	CLEANING UP

It is essential that upon the cessation of work each day, the property grounds shall be left clean and habitable. Rubbish generated by the work specified must be removed daily. Materials required for use the next day (or within the contract) are to be removed at the end of each day. The exception to this rule will require the contractor to provide adequate storage facilities (at his/her cost), to be agreed in advance with Donegal County Council.

8.4	REPAIRS TO EXTERNAL PLASTER / ROUGHCAST

All repairs to plaster, render, or roughcast that are needed because of preparation works for painting and not associated with the new render works to the gables or tie-in points shall be equal in finish to that of existing work and shall be of a minor nature. Tenderers are to allow for these repairs (up to a maximum of 5 m² per dwelling) in their price.  All such work shall be neatly jointed to the existing surface (including for K-Rend mesh patching) in a true and tradesman-like manner to the approval of the Council. Allow for skim, mesh and finish costs when repairing plaster above.

9.0	CONTRACT DURATION

The contract duration will be 16 weeks. Donegal County Council will require a post-tender meeting, and before appointment, the successful contractor must submit a detailed programme outlining the schedule of works to be covered each week, with weekly progress reports. Please note that the council can terminate this contract upon review of unsatisfactory progress reports, as this work is highly weather-dependent and must be completed during the summer season.

10.0	Contract Resources

The contractor shall employ the necessary resources to complete the contract within the permitted time frame. The evaluation of contractor resources will take place during each progress review.