VOLUME A: WORKS REQUIREMENTS

Knightstown Pontoon Mooring Replacement Phase 2

Kerry County Council

June, 2026

Contents

Preamble to the Specification	1

APPENDIX 0/3: List of Numbered Appendices Referred to in the Specification and Included in the Contract	3

APPENDIX 0/4: List of Drawings Included in the Contract	12

APPENDIX 1/4: Working And Fabrication Drawings	13

APPENDIX 1/5: Testing to be Carried Out by the Contractor	14

APPENDIX 1/7: Site Extent and Limitations of Use	15

APPENDIX 1/9: Control of Noise & Vibration	17

APPENDIX 1/13: Programme for Design and Execution and Completion of the Works	20

APPENDIX 1/14: Monthly Statements	21

APPENDIX 1/16: Privately and Publicly Owned Services and Supplies	22

APPENDIX 1/19: Routing of Vehicles	23

APPENDIX 1/22: Progress Photographs	24

APPENDIX 1/25: Product Certification Schemes	25

APPENDIX 1/70: Health and Safety	26

APPENDIX 1/71: As-Built Records	28

APPENDIX 1/72: Environmental Management and Waste Management	29

APPENDIX 1/73: Marine Works - General	32

APPENDIX 2/1: List Of Buildings, etc., to be Demolished or Partially Demolished	35

APPENDIX 2/3: Retention of Material Arising From Site Clearance	36

APPENDIX 26/5: Marine Fittings	37

| Doc. No. | Rev. | Date | Prepared By | Checked By | Approved By | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 25775-XX-XXX-CC-MWP-CE-6006-P8-S | 0 | 19/06/25 | MOS | EL | MOS | TENDER |
| 25775-XX-XXX-CC-MWP-CE-6006-P8-S | 1 | 23/06/25 | MOS | EL | MOS | TENDER |
| 25775-XX-XXX-CC-MWP-CE-6006-P8-S | 2 | 23/06/26 | MOS | EL | MOS | TENDER |

MWP, Engineering and Environmental Consultants
Address: Park House, Bessboro Road, Blackrock, Cork, T12 X251, Ireland

www.mwp.ie

Disclaimer: This Report, and the information contained in this Report, is Private and Confidential and is intended solely for the use of the individual or entity to which it is addressed (the “Recipient”).  The Report is provided strictly on the basis of the terms and conditions contained within the Appointment between MWP and the Recipient.  If you are not the Recipient you must not disclose, distribute, copy, print or rely on this Report. MWP have prepared this Report for the Recipient using all the reasonable skill and care to be expected of an Engineering and Environmental Consultancy and MWP do not accept any responsibility or liability whatsoever for the use of this Report by any party for any purpose other than that for which the Report has been prepared and provided to the Recipient.

# Preamble to the Specification

- The Specification referred to shall be the 'Specification for Works' (SPW) in TII Publications (Standards) published by TII as a collective group of documents under the Construction and Commissioning activity within the online TII Publications system (http://www.tiipublications.ie/) current on the date 10 working days prior to the tender returns date or, if applicable, the extended tender returns date and incorporating all amendments current on that date and as extended by the following:

- APPENDIX 0/1: Contract-specific Additional, Substitute and Cancelled Clauses, Tables and Figures;

- APPENDIX 0/2: Contract-specific minor alterations to existing Clauses, Tables and Figures;

- The Numbered Appendices listed in APPENDIX 0/3;

- APPENDIX 0/4 containing a list of the drawings referred to in the Specification.

- An Additional Clause, as indicated by a suffix 'AR' in APPENDIX 0/1, is a Contract-specific alteration.

- A Substitute Clause, as indicated by a suffix 'SR' in APPENDIX 0/1, is a Contract-specific alteration.

- A Cancelled Clause indicated by a suffix 'CR' in APPENDIX 0/1 is a Contract-specific alteration.

- Insofar as any of the Numbered Appendices may conflict or be inconsistent with any provision of the Specification for Road Works, the Numbered Appendices shall always prevail.

- Any reference in the Contract to part of the SPW or Appendix shall be deemed to refer to the corresponding Substitute Clause number or Appendix listed in APPENDIX 0/1 or APPENDIX 0/2.

- Where a part of the SPW is altered, any original Table/Figure referred to in the Clause shall apply unless the Table/Figure is also altered. Where a Table/Figure is altered, any reference in a Clause to the original Table/Figure shall apply to the altered Table/Figure.

- Where a part of the SPW relates to work goods or materials that are not required for the Works, it shall be deemed not to apply.

- Any Appendix referred to in the Specification that is not used shall be deemed not to apply.

- References in the SPW to “NRA Road Construction Details” shall be taken to refer to “Standard Construction Details (SCD) in TII Publications (Standards) published by TII as a collective group of documents under the Construction and Commissioning activity within the online TII Publications system (http://www.tiipublications.ie/), current on the date 10 working days prior to the tender returns date or, if applicable, the extended tender returns date.

- Texts shown in grey highlight apply only to Contracts where the applicable Conditions of Contract are the Public Works Contract for Civil Engineering Works Designed by the Employer, the Public Works Contract for Minor Building and Civil Engineering Works Designed by the Employer.

- Texts shown in square brackets immediately following text shown in grey highlight, if any, are deemed to replace the text in grey highlight for Contracts where the applicable Conditions of Contract is the Public Works Contract for Civil Engineering Work Designed by the Contractor.

- Throughout the SPW and Appendices to the SPW, the following interpretations shall apply:

“Base Course” shall mean “Binder Course”

“Roadbase” shall mean “Base”

“Wearing Course” shall mean “Surface Course”

“UK Department of Transport” shall mean “UK Highways Agency”

APPENDIX 0/3: List of Numbered Appendices Referred to in the Specification and Included in the Contract

APPENDIX 0/3 is comprised of a list of the Numbered Appendices referred to in the Specification and included in this Contract.

Responsibility for the compilation/completion of the appendices is indicated by the following symbols:

E 	Employer compiles

C 	Contractor to compile and submit to the Employer’s Representative

E/C 	Employer partially compiles, and Contractor completes and submits to the Employer’s Representative

T/C 	Tenderer compiles and returns with their Tender. Post-award of the Contract, the Contractor reviews and submits to the Employer’s Representative.

(P) 	The symbol ‘P’ indicates the appendix is a national pro-forma, and the format must not be altered.

Where a Contractor is required to compile or complete an Appendix, they shall do so in accordance with the Notes for Guidance on the Specification for Works and providing information similar to that stated in the sample appendices as a minimum.

APPENDIX 0/3 is comprised of two lists, A and B, of Numbered Appendices as follows;

List ‘A’ is a complete list of the Numbered Appendices referred to in the Specification for Works, with those not adopted marked “Not Used”. Those identified by the letters ‘T’ or ‘C’ shall be compiled by the Tenderer or the Contractor, respectively.

List ‘B’ gives a list of Contract-specific Numbered Appendices devised for this Contract.

List A: Numbered Appendices referred to in the Specification for Works.

| Appendix | Title | Complied/ Completed by | Used |
| --- | --- | --- | --- |
| INTRODUCTION |  |  |  |
| 0/1 | Contract-specific Additional, Substitute and Cancelled Clauses and Tables included in the Contract | E |  |
| 0/2 | Contract-specific Minor Alterations to Existing Clauses and Tables included in the Contract | E |  |
| 0/3 | List of Numbered Appendices Referred to in the Specification and Included in the Contract | E |  |
| 0/4 | List of drawings included in the Works Requirements | E |  |
| PRELIMINARIES |  |  |  |
| 1/1 | Accommodation and Equipment for The Employer’s Representative | E |  |
| 1/2 | Vehicles for the Employer’s Personnel | E |  |
| 1/3 | Communications System for the Employer’s Personnel | E |  |
| 1/4 | Working and Fabrication Drawings | E |  |
| 1/5 | Testing to be Carried out by the Contractor | E |  |
| 1/6 | Supply and Delivery of Samples to the Employer’s Representative | E |  |
| 1/7 | Site Extent and Limitations on Use | E |  |
| 1/8 | Operatives for the Employer’s Representative | E |  |
| 1/9 | Control of Noise and Vibration | E |  |
| 1/10 | Principal Structures to be Designed by the Contractor | E |  |
| 1/11 | Structural Elements and Other Features to be Designed by the Contractor | E |  |
| 1/12 | Setting Out and Existing Ground Levels | E |  |
| 1/13 | Programme of Design and Execution and Completion of the Works | E |  |
| 1/14 | Monthly Statements | E |  |
| 1/15 | Accommodations Works | E |  |
| 1/16 | Privately & Publicly Owned Services & Supplies | E |  |
| 1/17 | Traffic Safety & Management | E |  |
| 1/18 | Temporary Diversions for Traffic | E |  |
| 1/19 | Routeing of Vehicles | E |  |
| 1/20 | Recovery Vehicles for Breakdowns | E |  |
| 1/21 | Information Boards | E |  |
| 1/22 | Progress Photographs | E |  |
| 1/23 | Substances Hazardous to Health | E |  |
| 1/24 | Quality Management System | E |  |
| 1/25 | Product Certification Schemes | E |  |
| 1/26 | Irish Agrément Board Roads and Bridges Certificates | E |  |
| SITE CLEARANCE |  |  |  |
| 2/1 | List of Buildings, etc, to be Demolished | E |  |
| 2/2 | Filling of Trenches and Pipes | E |  |
| 2/3 | Retention of Material Arising from Site Clearance | E |  |
| 2/4 | Explosives and Blasting | E |  |
| 2/5 | Hazardous Materials | E |  |
| FENCING AND ENVIRONMENTAL BARRIERS |  |  |  |
| 3/1 | Fencing, Gates and Stiles | E |  |
| 3/2 | Fencing: NRA Road Construction Details | E |  |
| ROAD RESTRAINT SYSTEMS |  |  |  |
| 4/1 | Safety Barriers | E |  |
| 4/2 | Pedestrian Restraint Systems | E |  |
| 4/3 | Safety Barrier Terminals | E |  |
| 4/4 | Safety Barrier Maintenance | E |  |
| 4/5 | Anti-Glare Screens | E |  |
| 4/6 | Safety Barriers: NRA Road Construction Details | E |  |
| 4/7 | Vehicle Parapet Systems | E |  |
| DRAINAGE AND SERVICE DUCTS |  |  |  |
| 5/1 | Drainage Requirements | E |  |
| 5/2 | Service Duct Requirements | E |  |
| 5/3 | Surface Water Channels and Drainage Channel Blocks | E |  |
| 5/4 | Fin Drains and Narrow Filter Drains, and Geotextiles for Filter Drains | E |  |
| 5/5 | Combined Drainage and Kerb Systems | E |  |
| 5/6 | Linear Drainage Channel Systems | E |  |
| 5/7 | Drainage and Service Ducts: NRA Road Construction Details | E |  |
| 5/8 | Thermoplastics Structural Wall Pipes and Fittings | E |  |
| 5/9 | Attenuation | E |  |
| EARTHWORKS |  |  |  |
| 6/1 | Requirements for Acceptability & Testing of Earthworks Materials | E |  |
| 6/2 | Requirements for Dealing with Class U2 Unacceptable Material | E |  |
| 6/3 | Requirements for Excavation, Deposition, Compaction (other than Dynamic Compaction) | E |  |
| 6/4 | Not Used | E |  |
| 6/5 | Geotextiles Used to Separate Earthworks Materials | E |  |
| 6/6 | Fill to Structures & Fill Above Structural Foundations | E |  |
| 6/7 | Sub-formation & Capping & Preparation & Surface Treatment of Formation | E |  |
| 6/8 | Topsoiling, Grass Seeding and Turfing | E |  |
| 6/9 | Earthwork Environmental Bunds, Landscape Areas, Screening Mounds, Strengthened Embankments | E |  |
| 6/10 | Ground Anchorages, Crib Walling and Gabions | E |  |
| 6/11 | Swallow Holes & Other Naturally Occurring Cavities & Disused Mine Workings | E |  |
| 6/12 | Instrumentation & Monitoring | E |  |
| 6/13 | Ground Improvement | E |  |
| ROAD PAVEMENTS GENERAL |  |  |  |
| 7/1 | Permitted Pavement Options | E |  |
| 7/2 | Excavation & Reinstatement of Existing Surfaces | E |  |
| 7/3 | Surface Dressing | E |  |
| 7/4 | Bituminous Sprays | E |  |
| 7/5 | Road Pavements: NRA Road Construction Details | E |  |
| 7/6 | Breaking Up or Perforation of Existing Pavement | E |  |
| 7/7 | Not Used | E |  |
| 7/8 | Not Used | E |  |
| 7/9 | Cold Milling (Planning) of Bituminous Bound Flexible Pavement | E |  |
| KERBS, FOOTWAYS AND PAVED AREAS |  |  |  |
| 11/1 | Kerbs, Footways and Paved Areas | E |  |
| 11/2 | Access Steps | E |  |
| 11/3 | Kerbs, Footways and Paved Areas: NRA Road Construction Details | E |  |
| TRAFFIC SIGNS |  |  |  |
| 12/1 | Traffic Signs: General | E |  |
| 12/2 | Traffic Signs: Reflective Markers | E |  |
| 12/3 | Traffic Signs: Road Markings & Studs | E |  |
| 12/4 | Traffic Signs: Cones, Cylinders, FTD's & Other Traffic Delineators | E |  |
| 12/5 | Traffic Signs: Traffic Signals | E |  |
| 12/6 | Traffic Signs: Special Sign Requirements on Gantries | E |  |
| 12/7 | Traffic Signs: Preparation and Finish of Metal and Other Surfaces | E |  |
| ROAD LIGHTING COLUMNS & BRACKETS |  |  |  |
| 13/1 | Information to be Provided by the Designer for the Road Lighting Works when Specifying Lighting Columns and Brackets | E |  |
| 13/2 | Column & Bracket Data Sheets 1 & 2 | C (P) |  |
| 13/3 | Instructions for Completion of Column and Bracket Data Sheet | (P) |  |
| 13/4 | Certification for Lighting Columns | E/C (P) |  |
| 13/5 | Road Lighting Column and Brackets: NRA Road Construction Details | E |  |
| ELECTRICAL WORK FOR ROAD LIGHTING AND TRAFFIC SIGNS |  |  |  |
| 14/1 | Site Records | E |  |
| 14/2 | Location of Lighting Units and Feeder Pillars | E |  |
| 14/3 | Temporary Lighting | E/C |  |
| 14/4 | Electrical Equipment for Road Lighting | E/C |  |
| 14/5 | Electrical Equipment for Traffic Signs | E |  |
| 14/6 | Preparation and Finish of Metal and Other Surfaces | E |  |
| MOTORWAY COMMUNICATIONS |  |  |  |
| 15/1 | Motorway Communications | E |  |
| PILING AND EMBEDDED RETAINING WALLS |  |  |  |
| 16/1 | General Requirements for Piling and Embedded Retaining Walls | E |  |
| 16/2 | Precast Reinforced and Prestressed Concrete Piles and Precast Reinforced Concrete Segmental Piles | E |  |
| 16/3 | Bored Cast-in-Place Piles | E/C |  |
| 16/4 | Bored Piles Constructed Using Continuous Flight Augers and Concrete or Grout Injection Through Hollow Auger Stems | E/C |  |
| 16/5 | Driven Cast-in-Place Piles | E |  |
| 16/6 | Steel Bearing Piles | E |  |
| 16/7 | Reduction of Friction on Piles | E |  |
| 16/8 | Non-Destructive Methods for Testing Piles | E |  |
| 16/9 | Static Testing of Piles | E |  |
| 16/10 | Diaphragm Walls | E |  |
| 16/11 | Hard/Hard Secant Pile Walls | E |  |
| 16/12 | Hard/Soft Secant Pile Walls | E |  |
| 16/13 | Contiguous Bored Pile Walls | E |  |
| 16/14 | King Post Walls | E |  |
| 16/15 | Steel Sheet Piles | E |  |
| 16/16 | Integrity Testing of Wall Elements | E |  |
| 16/17 | Instrumentation for Piles and Embedded Walls | E |  |
| 16/18 | Support Fluid | E |  |
| STRUCTURAL CONCRETE |  |  |  |
| 17/1 | Concrete – Classification of Mixes | E |  |
| 17/2 | Concrete – Impregnation and Coating Schedule | E |  |
| 17/3 | Concrete – Surface Finishes | E |  |
| 17/4 | Concrete – General | E |  |
| 17/6 | Grouting and Duct Systems for Post-Tensioned Tendons | E |  |
| 17/7 | Precast Concrete Products | E |  |
| STRUCTURAL STEELWORK |  |  |  |
| 18/1 | Requirements for Structural Steelwork | E |  |
| PROTECTION OF STEELWORK AGAINST CORROSION |  |  |  |
| 19/1 | (Specification for Road Works) Sheet No. Form BE/P1 (New Works) Paint System Sheet | E/C (P) |  |
| 19/2 | (New Works) Requirements for Other Work | E/C |  |
| 19/3 | (Specification for Road Works) Form BE/P2 Paint Data Sheet | C (P) |  |
| 19/3** | (Specification for Road Works) Form BE/P3 Paint Sample Despatch List: Sheet 1 | C (P) |  |
| 19/3** | (Specification for Road Works) Form BE/P3 Paint Sample Despatch List: Sheet 2 | C (P) |  |
| 19/4 | (New Works) General Requirements | E |  |
| 19/5 | (Specification for Road Works) Form BE/PE1 (Maintenance) Paint System Sheet 1 | E/C (P) |  |
| 19/6 | (Specification for Road Works) Form BE/P1 (Maintenance) Paint System Sheet 2 | C (P) |  |
| 19/7 | (Maintenance) Requirements for Other Work | E/C |  |
| 19/8 | (Maintenance) General Requirements | E |  |
| WATERPROOFING OF CONCRETE STRUCTURES |  |  |  |
| 20/1 | Form PWS Proprietary Waterproofing System Data Sheet (1991 & Annex 'A') | C (P) |  |
| 20/2 | Waterproofing for Concrete Structures | E |  |
| BRIDGE BEARINGS |  |  |  |
| 21/1 | Bridge Bearing Schedule | E |  |
| BRIDGE EXPANSION JOINTS AND SEALING OF GAPS |  |  |  |
| 23/1 | Bridge Deck Expansion Joint Schedule | E |  |
| 23/2 | Sealing of Gaps Schedule (other than in Bridge Deck Expansion Joints) | E |  |
| BRICKWORK, BLOCKWORK AND STONEWORK |  |  |  |
| 24/1 | Brickwork, Blockwork and Stonework | E |  |
| 24/2 | Brickwork, Blockwork and Stonework: NRA Road Construction Details | E |  |
| SPECIAL STRUCTURES |  |  |  |
| 25/1 | Requirements for Corrugated Steel Buried Structures | E |  |
| 25/2 | Requirements for Reinforced Soil and Anchored Earth Structures | E |  |
| 25/3 | Requirements for Reinforced Clay Brickwork Retaining Walls of Pocket Type and Grouted Cavity Construction Structures | E |  |
| MISCELLANEOUS |  |  |  |
| 26/1 | Ancillary Concrete | E |  |
| 26/2 | Bedding Mortar | E |  |
| 26/3 | Cored Thermoplastic Node Markers | E |  |
| WATERMAINS, UTILITIES AND ACCOMMODATION WORKS |  |  |  |
| 27/1 | Requirements for Watermains | E |  |
| 27/2 | Watermains: NRA Road Construction Details | E |  |
| TRENCHLESS INSTALLATION OF ROAD DRAINAGE & SERVICE DUCTS |  |  |  |
| 28/1 | Trenchless and Minimum Dig Techniques | E |  |
| CCTV SURVEY OF ROAD DRAINAGE SYSTEMS |  |  |  |
| 29/1 | CCTV Survey of Road Drainage Systems | E |  |

List B: List of Contract-Specific Numbered Appendices Devised for the Contract

| Appendix | Title | Complied/ Completed by | Used |
| --- | --- | --- | --- |
| 1/70 | Health and Safety | E |  |
| 1/71 | As-Built Records | E |  |
| 1/72 | Environmental Management and Waste Management | E |  |
| 1/73 | Marine Works - General | E |  |
| 26/5 | Marine Fittings | E |  |

APPENDIX 0/4: List of Drawings Included in the Contract

- Contract Specific Civil/Structural Drawings Supplied to each Tenderer:

| Drawing Number | Drawing Title |
| --- | --- |
| 27335-MWP-XX-XX-DR-C-0001-S2-P03 | Site and Existing Moorings Layout |
| 27335-MWP-XX-XX-DR-C-0002-S2-P01 | Existing Angel Weights |
| 27335-MWP-XX-XX-DR-C-0003-S2-P02 | Nylon Rope Elevations |
| 27335-MWP-XX-XX-DR-C-0004-S2-P01 | Proposed Nylon Rope Details |
| 27335-MWP-XX-XX-DR-C-0005-S2-P01 | Nylon Rope Installation Details |

- Standard Drawings Brought into The Contract by Reference)

The drawings in the SCDs are intended to be used directly in the Contract without reproducing them in the documents and without supplying them to the Contractor.

Standard Construction Details are listed in the Appendices to this Specification. Where listed, these drawings shall be brought into the Contract by Reference.  Unless otherwise stated, the whole drawing is brought into the Contract.

The current versions of the individual details are available for download from the TII publications website: http://www.tiipublications.ie.

APPENDIX 1/4: Working And Fabrication Drawings

- General

- The Contractor shall submit all relevant documents/material submissions to the Employer’s Representative at least two weeks in advance of the date on which the Contractor proposes to procure or commence such works. Some elements of the works require greater periods for review, as tabulated below.

- The Contractor shall supply the following submissions/drawings/details to the Employer’s Representative.

| Series/reference | Description of Work | Minimum Period for review of package for approval by Employer’s Representative |
| --- | --- | --- |
| 100 | Site Compound Layout | 2 weeks |

APPENDIX 1/5: Testing to be Carried Out by the Contractor

- General

- The Contractor is reminded that, at a minimum, where materials are used, they will be sound and reasonably fit for the purpose for which they are required, and where goods are supplied, they will be of merchantable quality. Upon request from the Employer's Representative (E.R.), the Contractor is to supply evidence of the quality of the goods and materials proposed to be used or already incorporated into the permanent works. Associated costs are included in the Contractor’s rates.

APPENDIX 1/7: Site Extent and Limitations of Use

- Extent of the Site

- The Site is any place:

  - Where the Works are to be executed according to the Contract.

  - Or provided by the Employer for the Works.

  - Or where the Contractor is to operate or maintain Employer’s facilities.

  - Or that the Works Requirements identify as part of the Site.

- Overall areas provided by the Employer are identified in the Contract Drawings.

- The Contractor is not entitled to exclusive use of the Site. Limitations on the Contractor’s right to occupy and use the Site are described below.

- Access to Site

- The contractor can access site  via the access point at the head of the hardstanding area.

- Allowable Working Hours

| Allowable Working Hours |  |
| --- | --- |
| Day | Hours |
| Monday to Friday | 0800-1900 |
| Saturday | 0900-1300 |
| Sunday and bank Holidays | No working permitted. |

- Limitations on the Use of the Site

- The Contractor shall include for all necessary phasing of the works to ensure that materials required for the construction of the works can be delivered to the Site and stored at an agreed location until being incorporated into the works.

- The local ferry is operational from April to October (1st Week). Access and passage thrught the site will have to be facilitated by the contractor. Arrangements with local ferry operator and KCC will be required prior to commencement of Works.

- The Contractor’s use of the Site is subject to being superseded at any time by the operational and safety requirements of the KCC. Furthermore, it is the responsibility of the Contractor to constantly liaise with the KCC to ensure the works do not disrupt or impinge on the operations of KCC

- The Contractor is to permit access by the Employer (and their agents / Contractors) at times during the works at the direction of the E.R.. This access may be required for the necessary maintenance and inspection of harbour facilities and all associated infrastructure.

- The Contractor shall establish and remove, operate and maintain suitable hoarding or secure fencing and traffic barriers where possible to separate the Site and Contractor’s Compound from the harbour operations for the duration of the work.

- The Contractor shall be responsible for making all arrangements for providing any land outside the extent of the Site required for offices and other temporary facilities.

- The Contractor shall not allow any plant or material stockpiles to be left on the road carriageway or open quay surface outside of the designated working hours.

- The Contractor shall, at his own expense, make all arrangements for its own supplies required during the execution of the works.

- Other Construction Activities

- There are no know adjacent construction activities.

APPENDIX 1/9: Control of Noise & Vibration

- General

- This  APPENDIX 1/9 includes specific requirements in relation to noise, vibration, control of dust and condition surveys.

- The Contractor shall note that the Employer will carry out independent environmental monitoring at this site for the duration of the project and the Contractor shall allow access and facilitate the Employer at all times in this regard.

- The Contractor shall comply with the recommendations included in BS 5228:2009+A1:2014, Code of Practice for Noise and Vibration Control on Construction and Open Sites (Parts 1 and 2) and the TII Guidelines for the Treatment of Noise & Vibration in National Road Schemes.

- Noise

- As a minimum, all noise levels associated with the Works must be in compliance with the recommended noise threshold limits as specified in TII/NRA Guidelines for the Treatment of Noise and Vibration in National Road Schemes and British Standard BS5228:2009+A1:2014 Noise and Vibration Control on Construction and Open Sites. The TII Guidelines threshold limits are presented in one-hour LAeq and instantaneous maximum values for the various relevant days and times. The BS5228:2009+A1:2014 threshold limits are presented for the various portions of the day.

- Table 1: Maximum Permissible Noise Levels at the “Façade of Dwellings” During Construction

| Day and Times | LAeq (1hr) dB | LpA(max)slow dB | Period of Hours over which Leq, is applicable. |
| --- | --- | --- | --- |
| Monday to Friday 0800-1900 | 70 | 80 | 1 |
| Saturday 0800-1630 | 65 | 75 | 1 |
| Sunday and bank Holidays 0800-1630 | 60* | 65* | 1 |
| Outside normal working hours. | 60* | 65* |  |

* Construction activity at these times. Other than that required in respect of emergency works, will normally require explicit permission

- The onus is on the Contractor to reduce construction noise levels from the construction phase to the lowest possible levels to ensure that no significant noise impact is experienced at the nearest noise-sensitive receptors. The Contractor must comply with all of the guidance included in British Standard BS5228:2009+A1:2014 – Noise and Vibration Control on Construction and Open Sites: Part 1 - Noise in relation to reducing construction noise levels.

- A complaints procedure must be operated by the Contractor throughout the construction phase, and all efforts should be made to address any noise issues at the nearest noise-sensitive properties. The Contractor shall be responsible for all the consequences arising from failure to comply with the contractual obligations relating to noise control.

- All vehicles and mechanical plant used on the works shall be fitted with effective exhaust silencers and shall be maintained in good and efficient working order for the duration of the works in compliance with BS 5228.  Machines in intermittent use shall be shut down in the periods between work or throttled down to a minimum.  The Contractor shall remove from the works any item of plant, which in the opinion of the Employer’s Representative is ineffectively silenced.  All compressors shall be “sound-reduced” models fitted with properly lined and sealed acoustic covers and shall be kept closed whenever the machines are in use.  All ancillary pneumatic percussive tools shall be fitted with mufflers or silencers of the type recommended by the manufacturers.  Pumps and mechanical static plant shall be enclosed by acoustic sheds or screens where directed by the Employer’s Representative.

- Any plant, such as generators and pumps, shall be surrounded by an acoustic enclosure to the approval of the Employer’s Representative, which shall restrict the noise level to not less than 5dB(A) below levels quoted in the schedule to this Appendix.

- The Contractor shall organise his operations with regard to the positioning of plant, etc., so it minimises construction noise to adjacent properties.

- Vibration

- The Contractor shall select and utilise methods of working and items of plant so that the maximum measured ground vibrations do not exceed the following peak particle velocities:

  - 5 mm per second at the location of quay structures and approach bridges.

- In exceptional circumstances, the Contractor may apply to the Employer’s Representative for consent to carry out Works, which the Contractor considers will exceed the specified limit of ground vibration.  The procedure for submitting and processing the application will be as specified for obtaining consent to exceed the specified noise levels.

- Control of Dust

- In addition to the requirements of Clause 132, the Contractor shall provide, use, maintain and keep available plant and equipment necessary to minimise the formation and accumulation of dust arising from the Works, for any and all weather conditions.

- The Contractor shall take all measures necessary to control dust at the site of the Works during the course of the Works, including spraying the affected area with water as required by the Employer’s Representative.

- Training

- Operatives should be trained to employ appropriate techniques to keep Site noise to a minimum and should be effectively supervised to ensure that the best working practice with respect to noise reduction is followed. All employees should be advised regularly of the following as part of their training:

  - Proper use and maintenance of tools and equipment.

  - Positioning of machinery on Site to reduce the emission of noise to the neighbourhood and Site personnel.

  - Avoidance of unnecessary noise when carrying out manual operations and when operating plant and equipment.

  - Protection of persons against noise.

  - Operation of sound measuring equipment (selected personnel).

- Special attention should be given to the use and maintenance of sound-reduction equipment fitted to power tools and machines. Persons issued with ear protection equipment should be instructed on its use, care and maintenance. Education programmes should be provided which draw attention to the harmful effects of noise and make it clear that there are several ways in which employees can help themselves to protect their hearing, for example by:

  - Using and maintaining measures adopted for noise control.

  - Reporting defective noise control equipment to their superiors.

  - Not damaging or misusing ear protectors provided and immediately reporting damage to or loss of such items to their superiors.

- A monitoring programme should be implemented to ensure that condition limits are not exceeded and that all the relevant recommendations are met. Managers and supervisors can help by recognising the need for employees to make proper use of equipment so that noise emission will be minimised and to make proper use of ear protectors when required.

APPENDIX 1/13: Programme for Design and Execution and Completion of the Works

- General

- The Contractor shall submit his programme for the Works in accordance with the Contract Conditions and the following requirements.

- The Contractor shall provide the programme in the form of a Gantt/Bar-chart, produced as a result of a Critical Path Analysis, and must abide by the constraints stated or implied in the Contract. It shall show the level of detail appropriate to each stage of the Works and all activities and restraints, each of which shall be given a short title.  All events shall be numbered and annotated with the earliest and latest event dates.  The Critical Path Analysis referred to above shall also be provided to the Engineer, together with a schedule of outputs and resources to support the activities shown.

- Each activity is to be segmented to indicate activity duration, time risk allowances, float, health and safety requirements and any other milestones or dates required by the Contract within its period.

- Each activity is to show predecessor and successor dependencies.

- Free float, total float, and end float are to be shown in the programme relative to the critical path analysis, planned Completion, and the Completion Date.

- Any revised programmes issued for the E.R.’s acceptance must be accompanied by a narrative identifying the critical path operations and a schedule detailing the changes since the last programme. Any programme issued to the Engineer showing progress should be of the vertical drop line type and re-scheduled to indicate any revised planned Completion.

- Constraints

- Constraints are to be determined by assessment and review of the Contract Documents as a whole or considered prudent by the Contractor following his inspection of the Site and shall be taken into account by him in formulating his programme.

- Schedule of Information Required

- A Schedule of Information Required from the E.R. shall be prepared by the Contractor and submitted to the E.R., which shall be linked to the detailed programme as will enable the E.R. to review dates proposed by the Contractor for receiving any outstanding information required for the timely design (where applicable) execution and completion of the Works and the various parts thereof.

- Submission of Method Statements

- The Contractor’s programme shall allow for the preparation and submission of the RAMS required for the Works. The Contractor’s RAMS shall be in sufficient detail to satisfy the E.R. that the Works will be carried out in accordance with the Contract.

APPENDIX 1/14: Monthly Statements

- The monthly statements submitted to the E.R. in accordance with the Conditions of Contract by the Contractor shall, whenever dealing with matters covered by the Bills of Quantities, be set out under Part and Section headings like those in the Bill of Quantities and shall separately identify each item and specify quantity, unit, rate and value.

- Where required by the Contract, the Contractor shall allow the E.R. to inspect invoices for goods or materials included in the statement as may be required.

- Where the Contractor considers extra payment is due because of extra or varied work, or otherwise, then he shall include for these in his monthly statements in the month of the commencement of any of the varied or additional work, or in which the event arose, and shall update these particulars monthly as the work or event continues.

- Where the Contractor’s monthly statements include items for which the Contractor considers extra payment is due, then reference to Instructions given by the E.R., notices by the Contractor in accordance with the Contract, and relevant records shall be stated.

APPENDIX 1/16: Privately and Publicly Owned Services and Supplies

- General:

- During the execution of the works, the Contractor shall take all reasonable steps to prevent damage to mains, service pipes, sewers, drains, cables, wires, overhead telephone lines, poles, etc., and shall provide for protecting and supporting all existing services to the satisfaction of the Employer’s Representative and the responsible Authorities.

- If any damage is caused, then the Contractor shall give all notices, make good the damage and pay any costs incurred. The Contractor shall satisfy himself by his own enquiries and observations as to the precise position of all existing services. He shall take full responsibility in connection with them and shall hold the Employer indemnified against claims that may arise from any damage caused to such services by his operations.

- The Contractor is to survey the locations of existing manholes, drains and ducting prior to commencing excavation works.

APPENDIX 1/19: Routing of Vehicles

- Permitted access routes to and from the site

- Permitted access routes are via the main road directly to the pier.

- The Contractor, his Sub-Contractors and Suppliers shall use only the Permitted Access Route for all purposes in connection with the works (including the import of acceptable and disposal of unacceptable materials) unless otherwise agreed by the Employer’s Representative.

- Should the Contractor wish to seek the Employer’s Representative permission to use non-permitted routes or to use the permitted routes for purposes other than those allowed under the Contract, it shall be the responsibility of the Contractor to first liaise as necessary with the relevant Roads Authority, and to demonstrate to the Employer’s Representative that agreement has been obtained.

- The Contractor is deemed to have inspected and examined the site and its surroundings at the tender stage and to have satisfied himself as to the nature and means of access to the site.  In the event of the Contractor not being satisfied with the Permitted Access Routes to and from the site, the Contractor is obliged under the Contract to provide for all expenses and charges for temporary wayleaves required by the Contractor in connection with access to the site.

- Without prejudice to the foregoing provisions of Clause 19, the Contractor and his suppliers shall comply with the Employer’s Representative instructions for the routing of their vehicles. The Contractor shall erect and maintain in good condition signs of a type approved by the Employer’s Representative giving effect to these routing requirements.

APPENDIX 1/22: Progress Photographs

- The Contractor shall arrange for ongoing progress photographs as the Works proceed.

- Photographs of structures are to be taken prior to Site Clearance works.

- Photographs are to be taken prior to covering up areas by works.

- Photographs are to be provided in digital format.

- Progress photographs are to be submitted to the E.R. on a weekly basis.

- Videos are also acceptable, but the Contractor should present a set of stills taken from that video in compliance with points 1 to 3 above.

APPENDIX 1/25: Product Certification Schemes

- General

- Where materials are used, they will be sound and reasonably fit for the purpose for which they are required, and where goods are supplied, they will be of merchantable quality.

- The Contractor is to supply evidence of the quality of the goods and materials proposed to be used or already incorporated into the permanent works.

- The Contractor shall submit a list to the E.R. of the suppliers from whom he proposes to purchase the goods and materials necessary for the execution of the Works. No change in the list of suppliers and the Contractor’s proposals shall be made without the agreement of the E.R..

- Bodies involved in the assessment and verification of constancy of performance of goods and materials

- With respect to the function of notified bodies involved in the assessment and verification of constancy of performance for construction products, a distinction shall be made between:

  - Product certification body: a governmental or non-governmental notified body possessing the necessary competence and responsibility to carry out a product certification in accordance with given rules of procedure and management.

  - Factory production control certification body: a notified body, governmental or non-governmental, possessing the necessary competence and responsibility to carry out factory production control certification in accordance with given rules of procedure and management.

  - Testing laboratory: a notified laboratory that measures, examines, tests, calibrates, or otherwise determines the characteristics or performance of materials or construction products.

- Declaration of Performance and CE Marking

- When a construction product is covered by a harmonised standard or conforms to a European Technical Assessment which has been issued for it, the manufacturer shall draw up a declaration of performance when such a product is placed on the market.

- A copy of the declaration of performance of each such product used in the Works shall be supplied by email to the E.R..

APPENDIX 1/70: Health and Safety

- Project Supervisors

- The provisions of S.I. No 291 of the 2013, Safety, Health and Welfare at Work (Construction) Regulations will apply to this contract, and the Contractor will be appointed as Project Supervisor Construction Stage in accordance with the Regulations.

- The Contractor shall appoint a suitably qualified person to carry out the actions of the Project Supervisor for the Construction Stage, who shall liaise with the Project Supervisor for the Design Process as necessary and who shall prepare the Construction Stage Safety and Health Plan. The information provided by the PSCS shall cover the following matters relevant to the construction works and shall include any other details that may affect health and safety in any future construction work, maintenance work and demolition work to this project:-

- Name, address and contract person of all contractors engaged by the main Contractor on the project and any designers engaged by the main Contractor for elements of the permanent works;

- Name, address and contact person of all specialist manufacturers or suppliers, with details of any specialist items installed in the works by the main Contractor;

- Risk assessments and product safety data information relating to all hazardous substances used in the works;

- Details of construction method procedures/techniques adopted during the works which may affect any element of future construction work, maintenance work and demolition work to this project;

- Identify any major changes to the specified design construction methods/techniques adopted during the completion of the works;

- Details of risk assessments to the completed works which may be relevant to any future construction work, maintenance work and demolition work to this project;

- Test and commissioning certificates relating to any mechanical installation, electrical installation, other equipment/plant installation and specialist procedure/process in the completed works;

- Operating and maintenance manuals to any mechanical installation, electrical installation or other equipment/plant installation provided in the completed works, and

- Record details relating to 'As Constructed' information to be provided in drawing format and based on manually amended copies of the Contract Drawings.

- The Project Supervisor Design Stage is MWP, and a Preliminary Safety & Health Plan has been prepared in accordance with the Regulations.

- The Contractor is to have examined the Site and the information supplied regarding the nature and extent of the proposed works, so as to be satisfied as to what is required to be done to fulfil the duties of Project Supervisor for Construction Stage and also what measures are required to be taken to meet the requirements of the Regulations in the execution of the works including compliance with the Health & Safety Plan.  All costs arising in respect of these matters are to be included in the tender figure. The client will not be liable for any such costs which are reasonably foreseeable and which are not so included.

- Safety Meetings

- At an early date after appointment, the Contractor shall make himself available for a minuted meeting with the Employer’s Representative.

- The basic agenda of this meeting shall be for the Contractor to give to the Employer’s Representative and vice versa information regarding hazards being brought to the site in the form of work operations, equipment, chemicals, flammable materials, power, explosives, etc., together with controls in force to counteract these hazards, the name of the person on site responsible for safety and his/her deputy.

- The Contractor and Employer’s Representative shall subsequently pass on this information regarding hazards to their respective staff. The Employer’s Representative shall also agree with the Contractor on the parts of the Site to which the Contractor is confined, access routes, and any fencing needed for the Contractor's operations.

- Accident Reporting

- The Contractor shall report verbally to the Employer’s Representative all accidents, "near misses", loss and damage to any personnel or third party, whether caused by his own employees or by any other person on site on the day that they occur. The Contractor shall, within 2 working days, submit a written account of the incident to the Employer’s Representative. The Contractor shall also report such accidents to the competent authority whenever such report is required by law.

- Such reporting to the Employer’s Representative will not be deemed to absolve the Contractor from his statutory responsibilities on accident reporting to the relevant statutory authorities and/or insurers.

APPENDIX 1/71: As-Built Records

- General

- The Contractor shall be responsible for providing As-built records/drawings in accordance with the requirements of TII Document – Preparation and Delivery Requirements for as-built records. (CC-CMG-04001)

- All drawings submitted for formal review shall be provided in the following format, to be agreed with the Engineer:

- AutoCAD DWG format in the currently available version (2025 at the time of writing)

- Pdf format

- The following standards shall also apply:

- All CAD files shall be translated to an acceptable version of AutoCAD format, along with any font, linestyle, block or other necessary definition.

- Each transfer shall be fully documented to detail items such as level use cross-referenced files, necessary directory structure, etc.

- Full details of the programs used to generate the DWG format and XM format file shall be attached for reference.

- Data transfers shall be readable using equipment currently employed by the Engineer.  For small transfers, this may take the form of cc: Mail attachments; for large transfers e.g. area completion, this may also be on CD format.

- All data to be in x,y,z form to Irish Transverse Mercator and Ordnance Datum (Malin Head).

APPENDIX 1/72: Environmental Management and Waste Management

- Waste and Material Management (General)

- The Contractor shall prepare and submit a detailed Waste Management Plan prior to commencement of works on Site.

- The Waste Management Plan must be developed in strict accordance with the Best Practice Guidelines for the Preparation of Resource & Waste Management Plans for Construction & Demolition Projects published by the Environmental Protection Agency (EPA), 2021. This includes adherence to all principles and recommendations contained in the document to ensure sustainable, effective and compliant wate management practices.

- Key Elements to be included in the Waste Management Plan are:

- Waste Minimisation Measures: Strategies to reduce the volume and hazardous nature of waste produced during all phases of the project.

- Waste Characterisation: Identification and classification of all potential waste types expected on site.

- Segregation Procedures: Methodologies for on site sorting of waste to maximise re-use and recycling thereby minimising disposal to landfill.

- Storage and Handling: Specifications for secure, safe and environmentally appropriate storage of waste materials on site.

- Collection and Transport: Details of licenced waste contractors to be used, including transport routes and compliance with Irish waste regulations.

- Recycling and Recovery Targets: Commitment to meet or exceed national and local recycling targets in line with Circular Economy principles.

- Documentation & Record Keeping: Systems for tracking quantities of waste generated,

- re-used, recycled, recovered and disposed of including waste transfer notes and manifests.

- Contingency Measures: Procedures for managing unexpected waste types or volumes, including hazardous waste.

- The contractor must ensure that all waste management activities comply fully with all relevant Irish and European legislation including but not limited to:

- Waste Management Act 1996 (as amended)

- Waste Management (Collection Permit) Regulations 2007

- European Union Waste Framework Directive (2008/98/EC)

- Kerry County Council Climate Action Plan

- Establishment of a Site Compound

- Prior to works commencing a secure site compound should be established. Bunded areas for storage of fuels and oils should be set up and maintained at this location throughout the period of the works as will the appropriate waste management systems. The compound should be properly secured against unauthorised access or vandalism, and bunded areas should be provided with spill containment according to current best practices.

- The site compound will be used for site offices, plant storage, materials storage, etc. The Contractor shall be responsible for setting up the site compound, maintaining it throughout the duration of the works, and reinstating it to its former condition upon completion of the works. Reinstatement should include re-levelling the land, sewing grass and repairing any other damage, including storage of material from demolished dry stone walls and reinstatement of same to the original location and height.

- A approximate location for the Site Compound is identified in the Drawings. The actual location is to be agreed with POC.

- Storage of Plat and Material

- The storage of materials, containers and waste, however temporary, should follow best practices at all times and be stored at designated areas within the site compound well away from moving plant, machinery and vehicles. Potentially polluting material such as fuels or oils should be stored in bunded areas, on an impermeable base and undercover to prevent damage from the elements.

- The Contractor shall make his own arrangements for the securing of the storage space allocated to him and for the provision of and securing of any other accommodation required by him for the purposes of the works.

- Fuel and Oil Management

- Fuel oils must not, under any circumstances, discharge into the aquatic zone. All site plant and machinery, e.g. excavators, dumpers, etc., should be refuelled in a bunded, designated area. No servicing or repair of plant, machinery or vehicles will be undertaken outside the site compound area. The management of fuel on-site should have regard to the following elements:

- Machinery should be confirmed as being mechanically sound and without fuel or oil leaks and fit for purpose prior to project start ;

- To minimise the requirement for refuelling during operations, the plant should be refuelled prior to the start of each day’s works program;

- No refuelling should be conducted in the foreshore area;

- Fuelling and lubrication of equipment should be carried out in bunded areas;

- Use of biodegradable products where possible, e.g. hydraulic fluid;

- Mobile bowsers, tanks and drums should be stored in a secure, impermeable storage area within the site compound;

- Fuel containers must be stored within a secondary containment system, e.g., a bund for static tanks or a drip tray for mobile stores;

- Ancillary equipment such as hoses and pipes must be contained within the bund;

- Taps, nozzles or valves must be fitted with a lock system;

- Fuel and oil stores, including tanks and drums, will be regularly inspected for leaks and signs of damage;

- Only designated trained operators will be authorised to refuel plant on site, and emergency spill kits will be present at equipment for all refuelling events;

- Procedures and contingency plans should be set up to deal with any emergency accidents or spills;

- An emergency spill kit with oil boom, absorbers, etc., should be kept on-site in the event of an accidental spill;

- Fuels, lubricants and hydraulic fluids for equipment used should be carefully handled to avoid spillage, properly secured against unauthorised access or vandalism and provided with spill containment according to current best practice;

- Procedures and contingency plans should be set up to deal with emergency accidents or spills.

- Plant Management

- Plant and machinery management measures shall be implemented to ensure that plant machinery is appropriately managed for the duration of the works. Operators shall adhere to the following practices:

- Vehicle and equipment operators must have appropriate training and qualifications for proper and safe use of their equipment;

- Operation of all vehicles, vessels and equipment should be carried out in a safe and professional manner;

- To avoid the risk of spills or leakages into the marine environment, continuous care and maintenance of vehicles and equipment should be maintained;

- Preparation of a contingency plan in case of vehicle breakdown in the intertidal zone should be confirmed prior to the commencement of works;

- All filters, oils, lubricants and other related materials should be disposed of properly;

- Plant should be parked in the secure compound when not in use.

APPENDIX 1/73: Marine Works - General

- Passage of ships

- Prior to entering the water the Contractor is to notify the Harbour Master and at all times to adhere to Harbour Masters requirements.

- The Contractor shall at all times have regard to the passage of ships, and he shall remove his craft/equipment to a position that will not endanger navigation if ordered to do so by the Harbour Master or his authorised representative.

- For communication purposes, any marine plant shall be equipped with VHF radios with channels agreed to enable contact to be made with the Harbour Master at all times.

- The contractor shall ensure that his operations do not interfere with the operation of the Harbour under any circumstances.

- The Contractor shall allow for accommodating shipping movements within the harbour, berths, basin, and the area adjacent to the site.

- Certification of Vessels

- All vessels and floating plant for use in the Works must be fully certified by the Marine Survey Office (MSO) and shall hold any other relevant certification.

- Such certification shall be submitted to the Engineer prior to commencement, and no work shall be carried out by the vessel until the Engineer has received and accepted the certification.

- Provision of a Safety Boat

- The Contractor is to provide a Safety Boat in the water at each works location at all times, whether or not the works being carried out are in the water. The boats shall be made available for the duration of the contract.

- The boat shall be an open boat with an outboard motor; it shall be no less than 3 m in length; it shall be of seaworthy condition and acceptable to the Harbour Master.

- All relevant safety equipment, including personal floatation devices and VHF radios, are to be provided with the boat. VHF radios shall enable contact to be made with the Harbour Master at all times. VHF radio channels shall be agreed upon prior to commencing the works.

- Communications During Works at Sea or Near Water

- All floating plant navigating within the Ringaskiddy area must carry a VHF radio capable of working on marine channels and a listening watch must be kept on the appropriate channel.

- For communication purposes the Contractor’s marine plant shall be equipped with V.H.F radio equipment having Channel 12 to enable contact to be maintained with the Harbour Master at all times.

- A 24-hour watch shall be maintained on all marine plant with an English-speaking crew member in attendance.

- The marine plant shall also be fitted with a mobile telephone and the contact numbers furnished to the Engineer and Harbour Master.

- Marine plant shall be required to report to the Engineer and Harbour Master, their movements when passing locations specified by the Harbour Masters for the benefit of shipping movements.

- The Contractor’s Representative and relevant Site staff shall also be provided with mobile telephones and the contact numbers furnished to the Engineer and Harbour Masters, throughout the Works.

- The Harbour Master shall be given a rota showing who is on shift during all working hours. Any changes in contact staff above must be communicated immediately to the Harbour Master.

- Marking and Lighting of Operations

- All floating craft operated by the Contractor shall, at all times, display such lights and/or shapes as are laid down in the "International Regulations for Preventing Collision at Sea" and “International Association of Lighthouse Authorities” (I.A.L.A.) related to such craft or its nature of work.

- In addition, because of the nature of their work, all floating craft shall display the International Code Flag, “R” over “Y”, where they can best be seen.

- The Contractor shall also comply with all relevant requirements for navigation, lighting and the safety of vessels using Ringaskiddy Port.

- Harbour Fees and Dues

- No harbour dues shall be charged on the Contractor’s plant during the period of the Contract.

- Damage to Shipping

- The Contractor shall take every precaution to prevent accident or injury occurring to shipping of every kind using the area during the execution of the Contract and shall forthwith repair, make good and defray any loss, damage, cost, claim, charges or expenses incurred by or in consequent of any such accident or injury which occurs as a result of his operations.

- Salvage – Safety Precautions

- Without prejudice to relevant Statutory Enactment’s, if any, drilling platform, barge or other floating plant or equipment/ material there from, sinks from any cause whatsoever in the harbour or the approaches thereto, it shall be buoyed and lighted by the Contractor.  He shall immediately, on such sinking, take all necessary steps to lift and remove rapidly from the site or from the navigable channels, the said plant or material to a position selected by the Harbour Master or relevant authority in conjunction with the Engineer, notwithstanding that the same may be a total loss (constructive or otherwise) and despite abandonment.  The said plant or material is to be moved in a manner approved by the Harbour Master or relevant authority and under their direction.

- Prior to entering the water or installing any piles or other items within the water, the Contractor shall refer to and adhere to the Harbour Master’s requirements.

- Safety of Personnel

- The Contractor shall comply with all statutory obligations and take all reasonable precautions to ensure the safety of his operatives, particularly those engaged in over-water work.

- If used, the floating plant shall be equipped with property life-saving equipment and a rescue boat shall be immediately available at all times that the platform is manned.

- Clearance of Work on Completion

- On the completion of the Works, the Contractor shall clear away from the site all his floating plant, surplus materials, ground tackle, rubbish and temporary work of every kind and shall leave the whole of the site and works free of all obstructions to the satisfaction of the Engineer.

- Anchors and Moorings

- All anchors and moorings used by the Contractor shall be to the approval of the Harbour Authority.

- Berthing Facilities and Moorings

- All berthing facilities used by the Contractor shall be to the approval of the Harbour Authority.

Sunken Plant

- The Contractor shall, without delay, remove any sunken plant, vessel or other object which may be sunk during the course of the Contract by him or his staff or sub-contractors. This is to be completed under the strict direction of the Harbour Master.

- Pollution

- The Contractor shall not discharge oil, grease, sewage, waste, rubbish or deleterious matter of any kind into the Harbour waters. The Contractor's attention is drawn to the relevant regulations concerning pollution.

APPENDIX 2/1: List Of Buildings, etc., to be Demolished or Partially Demolished

- General Requirements

- With reference to the TII Specification for Roadworks and Method of Measurement for Series 200 Site Clearance:

- Demolition works must comply with all relevant legislation in respect of all operations, including disposal off-site of all contaminated unacceptable materials.

- The contractor is to allow for:

- Breaking up and removal.

- Cutting of steel.

- Disposal of material.

- Disconnecting, removing and sealing services and supplies.

- Treatment of hazardous materials.

- The items for disposal of material shall include the following:

- Haulage and deposition in tips off Site provided by the Contractor.

- Multiple handling of material.

- Special measures for dealing with Class U2 material.

- Reference Drawing

- Refer to Drawings for details relating to structures or structural elements to be demolished and the resulting waste to be disposed off-site.

- List of Items to be Demolished/Taken Up and Reomved Off Site for Disposal

- The following table lists the items identified in the above drawing.

Table 2/1.1 List of Items to Be Demolished/Taken Up and Removed Off-Site for Disposal

| No. | Description | Location | Details |
| --- | --- | --- | --- |
|  | Existing Mooring Lines. | Pontoons | To be taken up and disposed off-site at a suitably licensed waste facility. |

APPENDIX 2/3: Retention of Material Arising From Site Clearance

- List of Items to be Retained

- The following table lists the items to be retained.

Table 2/3.1 List of Items to be Retained

| No. | Description | Location | Details |
| --- | --- | --- | --- |
|  | All angel weights which are suitable for re-use when re installing the ropes (as approved for reuse by the engineer) | Mooring lines. | Store securely and set aside for inspection and re-use. Reinstated following satisfactory inspection. |

APPENDIX 26/5: Marine Fittings

- Middle Nylon Rope:

- 48mm Diameter Nylon Rope; 8 Strand, Gleistein; 486kN Breaking Load

- 25T Galv. Shackle

- Leg B – 27.8m (44 No.) & Leg C – All 31.5m (38No.)

- There shall be a 1 m spice at one end - hard eye to accommodate a galvanised thimble together with a 1 m splice - soft eye at the other end to accommodate the dyneema sling Splice 1: (82Nr. 1m long splices, complete with galv. Thimble)

- Single strand splicing method to be used

- Top hard Eye of 48mm Nylon to be protected in polyester webbing

- Bottom soft Eye of 48mm Nylon to be protected in polyester webbing

- Entire rope to be coated in PU coating

- It is essential that the entire nylon rope is covered throughout with polyureathane and this is to be applied to the rope when there is a 2 tonne pretension on the rope. Refer to Nylon Rope Detail. The mix proportions shall be included with the contract drawings

- Ropes to be coated with PolyUrethane coating

- A 2T pretension is to be maintained on the rope during the application of the PU coating

- Mix Ratio of the PU Coating as per product data spec sheet

- Coating should be applied in a dry and reasonably warm environment

- Mixing can be done by hand or with a metal implement such as a spatula

- Coating can be done by hand or with a brush

- Photographic record of coating setup  to be sent to E.R. prior to installation commences

- It is proposed to commence changing the ropes in Leg A and all changes must be undertaken in pairs in that when a rope form the north side of the leg is replaced the accompanying rope to the south side of the leg must be the next rope to be replaced. When the nylon ropes and associated attachments on leg A have been replaced it is then proposed to move onto Leg D when ferry operations for the season have finished up. Ropes are not to come in contact with other ropes as this will lead to abrasion and a shorter useful life.

- The tide variation at Knightstown is of the order of 3.7m. and pre-tensioning of the mooring system  ropes shall take place to withstand a load of 1.1 tonnes where angel weights are attached and of the order of 0.9 tonne where no angel weight is to be supported.

- Angel Weights

- Connections to be made from 20mm thick mild steel as per drawings.

- Where retrievable angel weight located at the seabed are to be recovered inspected and reused where possible

- Slings –

- 82 No. Slings in total to connect Nylon rope to ground line. 14 mm dyneema sling 5m lengths as per Contract Drawings

- IMPORTANT NOTES:

- 1. All ropes will initially be pretensioned without the angel weights installed.

- Table 2 is only to be referred to after the angel weights have been installed and the pretension on the mooring lines is being checked.

- Tolerance on pretension is 10%, that is for a 0.3T pretension the tolerance is 30kg