Invitation to Tender

National Parks and Wildlife Services

Refurbishment of Cottage No. 16 (Mulligan’s Cottage)

- Introduction

Killarney National Park (KNP) is invites competent building contractors, with experience in restoration of old buildings of historical and architectural merit, to tender for the full architectural/mechanical/electrical refurbishment of Cottage No. 16 at Muckross, on the N71, Killarney Kenmare route

2.0 Cottage Description

Muckoss Cottage No 16 is an existing dormer semi-detached residential property, of architectural and heritage merit, although it is not a listed structure. It is located on the N71, adjacent to the National Park Ranger/Farm and close to the main entrance to Muckross House and Gardens. The building was constructed circa in the mid-1800’s. At some stage in the recent past a single storey flat roof extension was constructed to the rear of the dwelling. There is a stand-alone small storage shed to the rear of the site.

The building is of traditional construction methods. The external walls are of random rubble masonry construction and load bearing. The ground floor is concrete and is ground bearing. The first/attic floor is construction from timber joists overlaid with a T&G softwood board. The main roof is a traditional cut timber rafter supported on timber purlins, in turn supported on timber trusses. The roof is finished with a natural quarry slate. The roof has dormer window at the front.

3.0 Scope of Works

The refurbishment works entrails the complete internal upgrade of the dwelling and the partial demolition and reconstruction of the  flat roof section. An A-section extension to the roof will be required at the rear, to accommodate the reconstructed stairwell.  In addition, some external site works by way of hardstanding for car parking and lowering of the surrounding ground will be necessary. A detailed scope of works is provided in the tender documentation.

Also note that there are two elements to the pricing structure – a Civil BOQ and a M&E pricing document. Both must be include in the final total tender price.

4.0 Contract Award                                                                                                                                                        The contract will be awarded on the basis of the Most Economically Advantageous Tender (MEAT) basis and the award matrix is as follows:

| Most Economically Advantageous Tender | Marks |
| --- | --- |
| Price | 75% |
| Previous Experience: |  |
| Project Management | 5% |
| Conservation Approach | 10% |
| Company Resources | 10% |
| Total | 100% |