Appendix 1

COMPLIANCE TO SPECIFICATION FORM

    - Specification for a 55kW (75 H.P.) Remote Controlled, Track Mounted Slope Mower:

- PLEASE NOTE: If you are offering different MACHINES, you must fill out a separate compliance form for each MACHINE offered.

Company tendering:

| Description | Please complete in respect of your offer | Confirm your level of compliance with any additional remarks/notes necessary | For OPW use only |
| --- | --- | --- | --- |
| # General: A 55kW (75 HP) remote controlled, track mounted slope mower for general and heavy vegetation maintenance on sloped terrain, capable of operating various attachments and implements as required. (From this point on the remote controlled, track mounted slope mower will be referred to as “slope mower” for simplicity). The Contractor must supply the quantity actually ordered during the currency of this contract to meet the requirements of the OPW at the prices stated in their tender, whether they be more or less in any degree than the quantity stated in the schedule. Minor variances to the specifications may be taken into account and considered at the competition assessment stage. This must be solely at the discretion of the OPW. Note: All equipment must be supplied with Manufacturer approval and compliance. State make and model offered: | Confirm acceptance: Confirm agreement: Make: Model: |  |  |
| - Gradeability: The machine must be capable of operating on terrain with a 55° slope. (i.e. 55° frontal/lateral gradeability). | Confirm & State Specification: |  |  |
| # Skid unit size/Type: The overall length must be in the range (rear frame to front tool-carrier) 2300-2700mm. Overall width must be in the range 1400-1600mm (tracks retracted). Overall height must be in the range 1000-1450mm. The skid unit must have frame mounted, rated lifting point(s) for loading/emergency recovery and ≥4 tie down points on the chassis for transport. | Confirm: Confirm: Confirm: Confirm: |  |  |
| # Engine: 4-stroke Common Rail Diesel. 4 Cylinder. The engine must develop a minimum of 55 kW (75 HP) at a rated speed of 1300 - 1800 RPM (SAE Gross) and a corresponding minimum torque of 280 Nm at an engine speed of 1500 RPM. The engine must comply with Stage V emission regulations or better. | Confirm: Confirm: Confirm: |  |  |
| # Undercarriage: # The undercarriage must be of robust fabricated construction. The slope mower must be fitted with hydraulically tensioned rubber tracks with aggressive agricultural tread pattern. The undercarriage width must be extendable by 400mm ± 10% and operate in the range 1400mm (Minimum) to 2000mm (Maximum) width to enhance the stability of the machine when traversing steep slopes. The tracks must incorporate lower track guides or similar to reduce derailment of the tracks when traversing steep slopes. Minimum Requirements: Undercarriage length (overall length, measured at the outer extremities of the tracks) must be in the range 1950-2200mm. Track Pad Width 380mm ± 10% | Confirm: Confirm: Confirm: Confirm: Confirm: |  |  |
| # Fuel: The slope mower must be capable of running on low sulphur diesel. | Confirm: |  |  |
| # Fuel Tank: The fuel tank must have a capacity not less than 50 litres. | Confirm: |  |  |
| # Transmission: The slope mower must be fitted with a hydrostatic transmission incorporating hydraulic proportional control and be capable of providing a maximum speed of ≥ 6 km/h. | Confirm: |  |  |
| # Hydraulics: The slope mower must be fitted with a hydraulic system with a minimum auxiliary line displacement of ≥ 95 litres per minute @ 320 bar. 1 Nr. high flow/pressure attachment line with flow regulation must be fitted for operating flail attachment motor. 3 Nr. electro hydraulically operated, flow adjustable double acting remote hydraulic valves /connections for operating front mounted equipment. A ‘monobloc’ type quick attach coupler for hydraulic attachments must be included. The quick attach coupler must include a side shift facility with a lateral travel of ≥ 350mm to allow implement use close to obstacles. The flail must be operated via electro hydraulic remote joystick for lift/lower and side shift functions. The slope mower must be fitted with an electro hydraulic self-recovery winch operated by remote control. The winch must be capable of recovering the entire machine and attachment mass, hence must have a capacity in the range 2500-3000kg. The slope mower must be fitted with all required electrical and hydraulic services for the operation of a hydraulic reach ‘side arm’ attachment with flail head. | Confirm: Confirm: Confirm: Confirm: Confirm: Confirm: Confirm: Confirm: Confirm |  |  |
| # Fan: The slope mower must be fitted with a reversible engine-cooling fan to reduce the build of debris while working in adverse conditions. | Confirm: |  |  |
| # Radio Remote Control: The design of the radio remote control must be ergonomic and made of robust materials. A dual joystick, proportional control design complete with operator harness is required. A single joystick must be dedicated to the travel of the machine with the second used to control attachments. A spare battery must be provided with the machine. A sealed compartment that houses a battery charger for the remote control should be housed on the machine allowing work to continue while charging. The remote control must have an operating range of ≥ 100 metres. The remote control must be supplied with a second battery with on-board charging facility. The machine must be equipped with an emergency shut down system should the remote control fail in operation. | Confirm: Confirm: Confirm: Confirm: Confirm: Confirm: |  |  |
| # Hitch: A rear mounted clevis and/or ball type tow hitch must be provided. | Confirm: |  |  |
| # Lighting: The slope mower must have at least two forward facing and two rear facing LED work lights. Flashing strobes/beacons, 2 Nr. front and 2 Nr. rear. | Confirm: Confirm: |  |  |
| # Reversing/Motion Alarm(s): The slope mower must be fitted with a travel alarm, which must sound when reverse gear is engaged. The skid unit must be fitted with a gradeability alarm, which sounds when the maximum safe incline is reached/exceeded. | Confirm: Confirm: |  |  |
| # Roll cage/protection kit The slope mower must be provided with additional protection in the form of a removable roll cage or a protection kit to protect the machine from falling branches and debris. The roll cage/protection kit must be made high strength material while maintaining a lightweight design. | Confirm: Confirm & State Specification: |  |  |
| # Flail Specification: - Forestry flail, swinging hammer type, capable of mulching scrub and small trees in the range 50-100mm diameter. - Working width of 1500mm ± 5% - Working weight of flail must be in the range 400-500kg. - Quick coupling attachment. - Monobloc type quick attach coupler for hydraulics. - Hood opening for cleaning and maintenance. - Skids adjustable in height. - Height adjustable, rear mounted roller. - Single lever control from factory fitted joystick. - Front and rear flying object protection. - Must be supplied with parking stand. | Confirm all & State Specification: |  |  |

| # Supplier Aftersales Service & Support: The slope mower supplied under this contract must be reliable in use and have reasonable servicing, maintenance and inspection requirements for its period in the OPW fleet. Where a slope mower requires additional time out of service due to a high incidence of mechanical or body defects, this will be brought to the attention of the contractor who will arrange an appropriate detailed inspection and full rectification of all faults with the unit. In the event of persistent failures within the first 36 months from delivery of the slope mower, the client reserves the right to reject it and receive in its place a suitable replacement slope mower of similar type and specification and of similar age and hours accumulated. The contractor will be responsible to the OPW for ensuring that all appointed dealers work to the highest standards of technical expertise for all maintenance and repair work carried out to the slope mower delivered under this contract. The contractor must have access to fully functioning and resourced workshop, dedicated breakdown response vehicle and proved evidence of mechanical staff competency when requested by the Contacting Authority. All work undertaken at an appointed dealer will be charged based on the stated time established in the workshop manual for the work done, or such lesser time as may be possible. The OPW will not pay for time over the stated time established in the workshop manual. Similarly, the contractor will be responsible for the suitability, availability, quality and warranty of all service items and replacement parts fitted to the tracked slope mower by an appointed dealer. Please provide a brief description of support capability and coverage for OPW. Please provide a brief description of proposed service level agreement for OPW. | Confirm all: |  |  |
| --- | --- | --- | --- |
| # Warranty: Minimum of 24 months parts and labour on tracked slope mower and attachments. | Confirm: |  |  |
| # Literature: The following literature must be supplied: - - 2 x Operator’s Manuals, 1 hard copy and 1 in electronic printable format. - 1 x Maintenance Manual in both electronic (printable) and hard copy format. - 1 x Spare Parts Manual in both electronic (printable) and hard copy format. | Confirm: Confirm: Confirm: |  |  |
| # Compliance: The mower must comply with all National and EU Regulations and Legislation pertaining to its Construction and its use. | Confirm: |  |  |
| # Finish: The slope mower must have a high visibility (i.e. bright yellow/orange/green) durable painted finish. | Confirm: |  |  |
| # Training: Comprehensive operator familiarisation training must be provided by a dedicated operator trainer and must be provided on the slope mower at an OPW site for up to 15 persons. The session must cover in-depth training focusing on efficient operation and safety related systems – the session content to be recorded in writing and signed by the person giving the session and passed on to the OPW with the signed attendance sheet. Comprehensive service and maintenance training for up to 10 OPW mechanical personnel The session must cover in-depth training focusing on safe and efficient service and repair, electrical/electronic & hydraulic troubleshooting and reading clearing fault codes – the session content to be recorded in writing and signed by the person giving the session and passed on to the OPW with the signed attendance sheet. A short product training video must be provided covering all aspects of the familiarisation training (MP.4 format). | Confirm: Confirm: |  |  |
| # Delivery: Delivery to OPW Depot located in Templemungret House, Mungret, Co. Limerick. | Confirm: |  |  |