*** All equipment stated must come with training for instructors. Warranty and Maintenance guarantee.  Software updates and replacement parts when needed. Technical support and hardware improvements as they come out.  Live demonstrations on all equipment when installed.***

AT001 - NCT Style wheel alignment in ground Ramp

four post inground lift, with runway, lifts cars and vans of up to 6.5 t. The runway lift type with electro-hydraulic SQUARE lifting, fitted rails, an axle jack for wheel-free lifting Optional: Conversion to vehicle reception, wheel alignment or inspection lift with axle play detector and 3.5 t wheel-free jack, as well as various types of installation (flush-mounted runway, with or without floor compensation). The torsion-resistant SQUARE posts.

NCT style centre ramp section that can independently life a vehicle for inspection

The lift must be compatible with ADAS calibration systems and wheels alignment

Example: MAHA VS SQUARE II 6.5 MC or Equivalent

VP 252286

AT002 - Hydraulic power press

Press with pressure capability of up to 75ton with holding capacity for installing or removing bearings, shafts. Solid steel frame and precision gauges

AT003 - Wheel Balancing machine

Modern wheel balancer, laser and weight fitment computer guided system with led colour display capable of balancing wheels up to 22inch with setting for both modern alloy wheels and steel wheels

AT004 - Tyre changing machine

Modern tyre changing machine capable of fitting run flat tyres and low-profile wheels up to 22 inches. Fully lockable stock head and interchangeable duck bill for various alloy wheels and steel wheels

AT005 - Wheel alignment system

- Database system

- Clear selection of the different vehicles. database contains most vehicles and is updated continuously basis.

- Motorized camera beam

- The 3D camera support beam is motorized and manually operated. The camera beam position is suited to optimize the available space.

- Dedicated electronics

- The system features an industrial type computer, Windows 10 based, solid-state SSD Sata, USB interface. Direct Shut-Down technology.

- Efficient and user-friendly software

- Very clear and user-friendly graphics. The software system, entirely developed by CEMB, guides the user through all the alignment operations.

- AUDIT is the new feature that helps operators identify suspension issue

- Ability to perform single wheel lifted run-out in case of restricted space or lifts with steps.

- Measurement of all required alignment specifications

- Targets with no electronics and batteries

- Movable camera boom

- PC computer and storage cabinet

- Windows based software

- Brake pedal depressor and steering lock

- TFT monitor, printer, keyboard, mouse

- Four 4-point clamps 12"-24"* with universal L=45mm grips and targets

- Complete car database with possible customization

- Simple and user-friendly software

AT006 - Laser Alignment tracking system

- Fits rims from 12" - 23"

- Easy mount clamps

- Alloy friendly clamp contact

- Automatic head charge on docking station

- Continual operation up to 8 hours

- Turning discs included

- Steering wheel clamp included

- Brake pedal depressor included

- Extensions for vans, 4x4's and lowered vehicles

- Optional electronic camber / caster gauge available

- Total toe - 20mm

- Toe in - 30mm

- Individual toe - 10mm

- Setback up to 60mm

- Rim diameter - 10" - 23"

- Max tyre diameter - 35"

- Wheelbase - 1500 - 4000mm

- Track width - 1000 - 2500mm

- Power - 230V

- Battery type - Nickel cadmium 4.8V 600mA

- Recharge lift - 1000 cycles min

- Usage - 10 hours continuous

- Laser - 635nm 1mw maximum output, class 2

AT007 - Headlamp Beam Setter

- Modern headlight beam setter with Suitability for LED and XENON lights

- Halogen headlight brightness control

- Plexiglas optical lens

- Digital luxometer and level control

- Mirror visor

- Data collection by photodiodes

- Autonomous power supply

- Operating height 240 mm / 1260 mm

- 3-piece sliding post

- Laser guild

AT008 - Vehicle weight scale system 4 corner

Measures & displays: total weight, cross weight, front axle weight, rear axle weight, RH/LH weight, individual corner weights

• Essential to set up your race car effectively

• Max load, 2000kg

AT009 - Mobile Engine Crane

Frame

- Material: Heavy-duty steel

- Finish: Powder-coated or painted for corrosion resistance

- Design: Welded construction with reinforced joints for durability and stability

Base

- Configuration: Wide, stable base with two fixed legs and two swivel legs

- Dimensions: Approx. 1000 mm width for stability and workshop manoeuvrability

- Wheels: Four casters (minimum two with locking brakes)

- Wheel Material: Nylon or rubber for smooth movement and floor protection

Boom

- Type: Telescoping adjustable boom

- Length Range: Approximately 800 mm to 1200 mm

- Load Rating: Clearly marked with maximum load capacity at each extension point

- Hook: Safety latch hook for secure engine attachment

Lifting Mechanism

- Type: Manual hydraulic pump and ram

- Stroke Length: Minimum 500 mm lifting height

- Operation: Lever-operated with smooth, controlled lift and descent

- Safety: Overload protection or clear maximum load indication

Hydraulic Pump

- Capacity: Suitable for lifting loads up to 1000 kg

- Control: Manual lever with ergonomic handle

- Seals and valves: Durable, leak-proof, and serviceable

Safety Features

- Wheel locks on at least two casters to prevent movement during lifting

- Safety latch on lifting hook to prevent load slippage

- Load capacity chart visibly attached to the boom or frame

- Compliance with relevant safety standards (e.g., EN 14985 or equivalent)

Additional Accessories

- Engine sling or lifting chains compatible with hook

- Foldable or collapsible sections for ease of storage (optional)

AT010 - Transmission stand (with mounting adaptors)

Frame

- Material: Heavy-duty steel tubing and plates

- Finish: Powder-coated or painted for corrosion resistance

- Design: Welded for strength and stability

Base

- Configuration: Wide, stable square base (approx. 1000 mm x 1000 mm)

- Mobility: Four swivel casters, at least two with locking brakes

- Wheel Material: Nylon or rubber for smooth movement and floor protection

Mounting Plate

- Rotation: 360° rotatable for easy access

- Tilt: Adjustable tilt mechanism for multiple working angles

- Locking: Secure locking mechanisms for rotation and tilt

Mounting Adaptors

- Types: Interchangeable adaptors to fit various transmission bell housings and cases

- Materials: Steel or high-strength alloy

- Securing: Clamps, brackets, and pins for firm attachment

Height Adjustment

- Range: Approximately 1000 mm to 1300 mm for ergonomic use

- Mechanism: Manual height adjustment with locking feature

Safety Features

- Locking mechanisms on all adjustable parts

- Load capacity markings clearly visible

- Compliance with relevant safety standards

AT011 - Bench unit engine/gearbox stand mounted

Various types of bench mounted engine unit stand. Timing belt and chain training stands. Head removal and torque training stands. Gearbox cut away

AT012 – Diesel Injector Tester

Test capability diesel injector types from major manufacturers:

Delphi, Denso, and Bosch solenoid.

Denso, Bosch, and Continental Piezo.

Bosch CRIN and Denso Heavy Duty adapters

Rapid test cycles

Compatible with solenoid Common Rail injectors from Delphi, Denso and Bosch as well as piezo equivalents from Denso, Bosch and Continental.

AT013 – Oscilloscope/Diagnostic Equipment

communication module with the most advanced ECU diagnosis for Ethernet-based (DoIP – Diagnostics over Internet Protocol) vehicle interfaces with features

access to the latest measuring technology, and full online data base information.

A Toughbook style laptop arrangement with updates and full comprehensive vehicle interface list with live data and 4 channel capability

e.g. Bosch KTS and a launch X431 pro

AT014 – Emissions Tester

Standalone rolling petrol and diesel emissions tester with monitor and printer. Capable of real time emissions information. Calibration function.

5 gas measuring modules (CO, CO², HC, O², NO)

Vehicle exhaust emission analysis target database

Irish NCT and Uk Mot and euro spec preprogrammed settings.

Capable of updating to new emission standards as they role out.

AT015 – TPMS Activation Tool

Activate All Known TPMS Sensors

Read Sensor ID, Temperature, Pressure

Read Sensor Battery Condition

On-Tool OE Sensor Information

Program MX-Sensor With 3 Programming Options

Batch Program 1-16 MX-Sensors

Advanced Mode Service:

Same TPMS Features as Quick Mode

TPMS Status Screen – View System Faults

Read and clear TPMS DTCs

On-Tool DTC Description Program MX-Sensors by Copying ECU Sensor IDOn-Tool Relearn Procedure Guide Widest Vehicle Coverage for OBD Relearn Tire Pressure Placard Value Reset

This should come with a supply of TPMS sensors that can be used and replaced on a regular basis

AT016 – Alternator Voltage Regulator Tester

diagnostics of alternators with a nominal voltage of 12/24V

diagnostics of Stop-start alternators with a nominal voltage of 12V

diagnostics of voltage regulators with a nominal voltage of 12/24V separately from alternators

database of voltage regulators

reading and saving the data transmitted via the LIN-bus

diagnostics of automotive starters using cable MS-33503

Free software update.

Supply voltage during alternator testing: 12-48V – either battery or test bench supply

Supply voltage during voltage regulator testing: power unit with «Quick Charge 2.0» (Output 9V-2A) function

Touch screen: IPs Capacitive touch

Ingress protection

Nominal voltage of tested alternators, V: 12, 24

Types of tested alternators, 12V: Lam, SIG, RLO, RVC, C KOREA, P/D, COM, LIN, BSS, C JAPAN, VALEO I-STARS,

Types of tested alternators, 24V: Lamp, COM, LIN, PWM

Tested parameters: Stabilizing voltage, Control lamp, FR (voltage regulator response to the load on alternator). For COM voltage regulators: ID, protocol type, data exchange rate, voltage regulator type, type of LIN protocol, voltage regulator self-diagnosis faults.

Voltage measurement accuracy, V: ±0,1

Software updates

Voltage regulator databases

Optional cable for tester for testing starters and starter circuits should be included on this testing kit

AT017 – ADAS Calibration System

New European approval standards, automatic emergency braking (AEB) and lane keeping assistance (LKA).

Additional components could be required to suit manufacturer.

This system must also and not exclusively have the following calibration systems

ACC – ADAPTIVE CRUISE CONTROL

ANV – AUTOMOTIVE NIGHT VISION

AEB – AUTONOMOUS EMERGENCY BRAKING

BSD – BLIND SPOT DETECTION, BSM – BLIND SPOT MONITORING, BSW – BLIND SPOT WARNING

CDW – COLLISION DETECTION WARNING

DDW – DROWSY DRIVER WARNING, DFW – DRIVER FATIGUE WARNING, DDD – DRIVER DROWSINESS DETECTION,

DMS – DRIVER MONITORING SYSTEM

FCW – FORWARD COLLISION WARNING, FCWS – FORWARD COLLISION WARNING SYSTEM, FCA – FORWARD COLLISION AVOIDANCE

LDW – LANE DEPARTURE WARNING, LDWS – LANE DEPARTURE WARNING SYSTEM

LKA – LANE KEEPING ASSIST

PD – PEDESTRIAN DETECTION, PDS – PEDESTRIAN DETECTION SYSTEM

TLR – TRAFFIC LIGHT RECOGNITION

AT018 – Electronic Brake Control (EBC) test/cutaway unit

Electronic Parking Brake Functional Cutaway Model

Educational electromechanical parking brake (EPB) model based on original Audi/VW brake components. The cutaway of the multi-stage gear mechanism of electromechanical brake made for an easier explanation and demonstrate the principle of operations.

Teaching aid on hydraulic brakes and electromechanical parking brake mechanism, the structure, modes of operation and maintenance methods, technical information, spec. tools and their uses

Electromechanical brake model demonstration does not need any additional equipment or tools, without having to connect with diagnostic device it is possible to perform activation of the electromechanical parking brake. Educational electromechanical parking brake mechanism powered by 12V/220V.

Two electrical buttons for press and release the brake pads and the brake disc.

It is possible to demonstrate the operation of the parking brake and simply to explain the electronic systems such as “AUTOHOLD” or “Dynamic drive-off assistant” or brake pad replacement procedure. The electromechanical brake model is not filled with the brake fluid, there is no pressure inside. Activation is performed by pressing the buttons to press or release the brake disc mechanically.

AT019 – Degreasing Unit with Light Fixtures

New degreasing unit with light fixtures. Fluid to be changed at intervals according to manufacturers

AT020 – EV/Hybrid Safety Kits

1. Scope of Requirement

Supply of Heavy-Duty Electric Vehicle (EV) and Hybrid Electric Vehicle (HEV) Educational Safety and Isolation Kits. The equipment will be utilised in a high-turnover apprenticeship Training Centre environment. Equipment must be engineered for high durability to withstand repetitive daily use by multiple apprentices, with specific design features that assist course instructors in visual compliance tracking, safety verification, and asset control.

2. Training Centre Component Specifications

The tenderer must supply a comprehensive safety kit outlined below.

Module A: Personal Protective Equipment (PPE) – Multi-Size Student Deployment

- High-Voltage Insulated Gloves (Class 0): Three (3) pairs of rubber dielectric gloves rated for 1000V AC. Tenderer must supply a range of sizes to accommodate different students (e.g., Size 8/Medium, Size 9/Large, Size 10/Extra Large). Each pair must include leather over-gloves, inner cotton liners (for hygiene), and a breathable storage bag.

- Pneumatic Glove Testers: Two (2) manual glove pump tester per kit to train apprentices on the mandatory pre-use air-leak safety check before live-work simulation.

- Arc Flash Face Shields: Two (2) clear, anti-fog face shields certified to GS-ET-29 Class 1. Visors must feature scratch-resistant coatings tailored for heavy apprentice use and an easily adjustable ratchet headband for fast switching between users.

Module B: Classroom Area Demarcation

- High-Visibility Training Bay Barriers: A minimum of four (4) heavy-duty, self-retracting belt stanchions or weighted folding barriers. Belts/chains must feature high-contrast warning text (e.g., "TRAINING IN PROGRESS - DO NOT ENTER").

- Magnetic/Hook-and-Loop Vehicle Signs: A minimum of four (4) signs reading "DANGER: Apprentice Training Live HV Battery Pack" and "CAUTION: Vehicle De-Energised", allowing instructors to change the training scenario status dynamically.

Module C: Master Lockout-Tagout (LOTO) & Training Pedagogy

- Multi-Lock Hasps: Minimum of two (2) non-conductive, dielectric lockout hasps allowing up to six (6) apprentice padlocks simultaneously. This ensures a vehicle cannot be re-energized until both the instructor and all assigned students remove their individual locks.

- Apprentice & Instructor Padlocks:

- One (1) red "Instructor Master Lock" (keyed uniquely).

- Four (4) blue "Apprentice Locks" (keyed differently).

- Reusable Write-On LOTO Tags: A pack of twenty (20) heavy-duty, dry-erase or wet-erase plastic tags to allow apprentices to repeatedly practice filling out safety logs, dates, and signatures without wasting consumables.

Module D: Educational & Emergency Rescue

- Wall-Mountable Insulated Rescue Pole: Three (3) 45kV fiberglass rescue hook. Must include heavy-duty wall-mounting brackets labeled for high visibility in an emergency training scenario.

- Classroom Dielectric Flooring: Minimum 1 metre x 2 metres Class 0 insulating rubber runner matting to span the entire working side of the training vehicle or simulator rig.

Module E: Added Requirement – Student Verification Multimeter

- Digital Multimeter (CAT IV 600V / CAT III 1000V): One (1) ruggedized, drop-tested digital multimeter compliant with IEC 61010-1. Must feature a "Zero-Energy" testing verification mode to teach apprentices how to prove a high-voltage system is safely isolated before touching components.

3. Storage, Inventory Management & Asset Control

In order to maintain security of EV safety kit items, the storage configuration must meet strict inventory tracking rules:

- Mobile Shadow Cabinets: All kit components must be housed in a lockable, heavy-duty wheeled tool cabinet with ball bearing roller drawers.

- Two-Tone Shadow Foam Inlays: Every tool, glove, lock, and meter must sit in a custom laser-cut, two-tone (e.g., bright red base with black top) high-density foam insert. This allows instructors to verify that 100% of the safety equipment is returned as each training day ends.

Additional Equipment:

10 x Pairs of Dielectric Boots

10 x Flash Helmet

2 x Warning Tape Reel

2 x Fire Containment Blanket with Mobile Case and Gloves

AT021 – Glue Pull Repair Kit

1. Scope of Supply

The supplier shall provide a complete glue pull dent repair system suitable for professional bodyshop use on:

- Mild steel exterior panels

- Aluminum exterior panels

- EV and hybrid vehicle body structures (non-invasive repair)

The system must include:

- Glue pull tools

- Pulling devices

- A comprehensive range of pull tabs

- Consumables

- Heating equipment

- Storage and organisation system

- Operator lighting system

- Training and documentation

2. Standards & Performance Requirements

The system must meet or exceed the following:

- Compatible with Vehicle manufacturers OEM-approved Paintless Dent Repair (PDR) and Glue Pull Repair (GPR) methods

- Suitable for both cold glue and hot glue applications

- Must not damage OEM paint when used correctly

- Must be safe for use on aluminum panels, including EV battery-adjacent structures

- Glue must be bodyshop-safe, low-VOC, and fast-curing

- All metal components must be corrosion-resistant

- All plastic components must be impact-resistant and heat-stable

3. Glue Pull Tools & Devices

The system shall include, at minimum:

Pulling Devices

- Slide hammer (light and heavy versions)

- Mini lifter with adjustable feet

- K-Bar or equivalent leverage bar

- K-Beam or equivalent bridge puller

- Precision pull tower (optional but preferred)

- Controlled tension pulling device for large panel corrections

Features Required

- Adjustable pulling height and angle

- Non-marring feet for painted surfaces

- Quick-release tab connection

- Fine-control tension capability

- Suitable for both small and large dents (essential)

4.  Pull Tabs – Comprehensive Set

A full range of high-strength glue tabs must be supplied, including:

Tab Types

- Round tabs (various diameters)

- Oval tabs

- Crease tabs (short, medium, long)

- Hail tabs

- Heavy-duty tabs for structural pulls

- Flexible tabs for aluminum

- Cold glue tabs (rubber-based)

Requirements

- Minimum 80–120 tabs total

- Tabs must be compatible with Keco-style pulling heads

- Tabs must be colour-coded or size-coded

- Tabs must withstand repeated heating cycles

- Tabs must be suitable for both hot glue and cold glue systems

5.  Glue & Heating Equipment

Hot Glue

- High-strength PDR glue sticks

- Fast-curing, high-tack formula

- Suitable for aluminium and steel

- Minimum 20–40 sticks supplied

Cold Glue

- High-tack cold glue pads

- Compatible with GPR cold glue systems

Heating Equipment

- Industrial-grade glue gun (100–200 W)

- Adjustable temperature control

- Optional: Panel warming gun or induction heater (preferred for aluminium)

6. ️ Knockdown & Finishing Tools

The system must include:

- Blending hammers

- Knockdown punches (plastic, nylon, and polished tips)

- Tap-down set with interchangeable heads

- Soft-face hammers

- Precision finishing tools for aluminium

7. ️ Storage & Organisation

A storage solution must be provided:

- Heavy-duty wheeled modular storage box

- Foam-cut inlays for tool organisation

- Chemical-resistant compartments for glue

- Rolling trolley or workstation (preferred)

- Ball bearing roller drawers

- Extended duty cycle

8. Documentation & Training

The supplier must provide:

- Full inventory list

- User manuals for all tools

- Manufactures Safety data sheets for glue heating equipment

- Manufactures Technical data sheets

- Recommended repair procedures

- On-site training session and technical support to course instructors

AT022 - Two-Post Electric Vehicle Lift — Technical Specification

1. General Requirements

The lift shall be a commercial-grade, asymmetric or symmetric two-post lifting system engineered specifically to support the increased weight and altered centre-of-gravity characteristics of modern (EV) electric vehicles. The system must comply with EN 1493:2010 (Vehicle Lifts) and all applicable Irish and EU safety regulations.

- Minimum rated lifting capacity: 4,000 kg (preferred 4,500–5,000 kg for EV suitability)

- Lift type: Electro-hydraulic or electro-mechanical two-post configurations

- Installation type: Surface-mounted with anchor-bolt floor fixation

- Duty cycle: Minimum 30% continuous operation rating

2. Structural Design

- Posts: Heavy-duty rolled steel columns with reinforced base plates

- Arms:

  - Telescopic, multi-stage arms suitable for EV lifting points

  - Minimum arm reach range: 600–1,200 mm

  - Arm locking: Automatic mechanical arm-locking system

- Crossbeam / overhead:

  - Either overhead cable routing or base-plate configuration

  - Overhead models must include an adjustable height limit switch

- Floor requirements:

  - Minimum concrete thickness: 200 mm C35/45 reinforced

  - Supplier must provide anchor bolt specifications and pull-out test data

  - Supplier must provide risk and method statement for installation

3. EV-Specific Requirements

- High-capacity lifting pads:

  - Low-profile, high-durometer rubber pads suitable for EV battery-tray clearance

  - Optional height-extension adapters (50 mm, 100 mm)

- Load distribution:

  - Arms must support offset load conditions typical of EV battery mass distribution

- Ground clearance:

  - Minimum lowered height: ≤ 95 mm to accommodate low-profile EVs

- Safety for battery packs:

  - No under-vehicle obstructions

  - Smooth arm movement to avoid impact with battery trays

4. Lifting Performance

- Lifting height: Minimum 1,900 mm

- Lifting time: ≤ 55 seconds (full rise)

- Lowering time: Controlled descent ≤ 45 seconds

- Motor rating:

  - 2.2–3.0 kW electric motor

  - 380 V0LT three-phase supply

- Hydraulic system (if applicable):

  - Dual-circuit hydraulic cylinders

  - Burst-proof hydraulic hoses

  - Pressure relief valve

5. Safety Systems

- Mechanical safety locks:

- Automatic engagement at multiple locking positions

- Redundant locking system required

- Synchronization:

- Electronic or mechanical synchronization to maintain level lifting

- Emergency features:

- Emergency stop button

- Manual lowering valve (hydraulic)

- Automatic overload protection

- Arm restraints:

- Automatic arm-locking when lift is raised

- Acoustic warnings:

- Audible alarm during final lowering phase

- Electrical compliance:

- IP54 or higher control cabinet

- CE-marked components

6. Control System

- Control type:

  - Single-sided or dual-sided control panel

  - Push-button or soft-touch controls

- Safety interlocks:

  - Lock-release interlock

  - Height limit switch

- Diagnostics:

  - Fault indicator lights or digital display

7. Documentation & Certification

The supplier must provide:

- CE Declaration of Conformity

- EN 1493 compliance certificate

- Structural load test report

- Installation drawings and foundation requirements

- Operation & maintenance manual

- Spare parts list (minimum 10-year availability commitment)

8. Installation & Commissioning

- Full installation by certified technicians

- Load testing on completion

- Operator training and technical support for Instructors

- Commissioning certificate issued on completion

9. Warranty & Support

- Minimum 24-month full warranty

- Optional extended warranty (36–60 months)

- Guaranteed response time for service calls: ≤ 48 hours

- Availability of technical support (on site visit) and preventive maintenance plans

AT023 - Electric Board Class Trainer Set

Scope of Supply

- The contractor shall supply, deliver, install, and commission a fully functional, open-architecture Electric Vehicle (EV) Educational Training Stand based on original equipment manufacturer (OEM) components. The system must be designed for safe, hands-on technical training, fault diagnosis, and maintenance procedures in an educational laboratory environment.

System Architecture & Physical Construction

- Frame Material: Heavy-duty, lightweight extruded anodized aluminium profile frame.

- Mobility: Equipped with high-capacity industrial castors with locking brakes for 360-degree manoeuvrability.

- Visibility: Open-chassis layout with no vehicle body panels to ensure unhindered visual access to all internal components and high-voltage wiring.

- Dimensions: Approximate footprint of 2500 mm (L) x 1000 mm (W) x 1500 mm (H).

- Weight: Approximate total weight of 700 kg.

Core Technical & Powertrain Components

- Base Vehicle Platform: Synchronous powertrain system based on the e.g Nissan Leaf EV platform.

- Electric Motor: Integrated genuine OEM electric traction motor with a minimum output rating of 80 kW and 280 Nm torque.

- Inverter/Converter: Genuine OEM power inverter unit for precise motor management and three-phase AC conversion.

- Climate Control: Functional electric AC compressor unit integrated to demonstrate thermal management and climate systems in modern EVs.

Electrical & Energy Storage Specifications

- High-Voltage Battery Pack: Genuine OEM lithium-ion traction battery pack with a minimum capacity of 24 kWh and nominal operating voltage of ~400V.

- Low-Voltage System: Integrated 12V DC auxiliary battery system for control units and ECU logic power.

- Mains Input Power: 230V AC, 50 Hz single-phase electricity network connection for auxiliary charging and operation.

- High-Voltage Cabling: Industry-standard, genuine high-visibility "orange" shielded high-voltage cables equipped with OEM sealed connectors.

Diagnostics, Measurement & Fault Simulation

- Diagnostic Interface: Standard OBD II 16-pin diagnostic socket wired directly to the vehicle Electronic Control Units (ECUs).

- ECU Capabilities: Full compatibility with standard automotive diagnostic tools for ECU identification, real-time live data parameter monitoring, and reading/clearing Diagnostic Trouble Codes (DTCs).

- Measurement Interfaces: Integrated measuring box featuring open electrical contacts connected directly to key sensor and actuator circuits for safe multimeter or oscilloscope probing.

- Schematic Integration: Permanently affixed, high-definition principal electric wiring diagram displaying component markings, pin numbers, and data transmission lines (CAN bus).

- Fault Simulation System: Integrated physical or digital fault insertion panel allowing the instructor to simulate at least ten (10) independent low-voltage control system and sensor faults. High-voltage circuits must remain uninterrupted by the fault panel for safety.

Safety & Protective Features

- Physical Isolation: Transparent, impact-resistant plexiglass/polycarbonate protective shields covering all high-voltage connections, terminals, and moving components.

- Service Disconnect: Accessible, manual High-Voltage Disconnect Plug/Fuse to safely isolate the traction battery pack during maintenance training.

- Enclosed Wiring: All internal low-voltage wiring and control modules housed within closed rear panels to prevent accidental impact or user tampering.

- Mandatory Safety Kit: Delivery must include a dedicated High-Voltage Protective Tools Set (comparable to EHVS01) containing certified insulated hand tools and personal protective equipment (PPE) rated for a minimum of 1000V AC / 1500V DC.

Deliverables & Documentation

- Manuals: Comprehensive instructor training and continuous technical support, apprentice workbooks, and step-by-step practical training procedures.

- Visual Aids: High-resolution instructional imagery and complete electrical wiring schematics.

- Software/Firmware: Pre-configured system control software optimised for rapid default reset between classroom lessons.

- CE Marking and Approval. Letter of Conformity

AT024 - Insulated Tools & Necessary Safety Equipment

Hybrid/EV Cabinet Comprehensive Kit

Complete set of insulated high voltage hand tools and test instruments designed for use on hybrid/EV vehicles, with heavy duty drawer lockable tool cabinet made from cold rolled steel with a durable powder coat finish, the drawers should have roller bearing runners and 2 fixed and 2 swivel/brake wheels for easy manoeuvrability around the workshop.

Comprehensive approved hybrid and EV insulated tool & safety equipment set containing:

Workshop Storage & Mobility

- Hybrid/EV Roller Cabinet: Heavy duty, cold rolled steel, powder coat finish, lockable drawers with roller bearing runners, 2 fixed wheels, 2 swivel/brake wheels.

Personal Safety Equipment (PPE)

- Insulated & Leather Gloves Pack: Large size, 1000V rated, includes cotton under liner gloves. (requirement: 20 pairs total)

- Protective Arc Flash Face Shield: 1000V rated.

Workshop & Vehicle Safety Equipment

- High Voltage Floor Matting: 1-metre length, 1000V safe working. (requirement: 5 mats total / 1 standard + 4 additional)

- Restricted Area Mark-out Kit: 4 posts and bases with barrier chain set. (requirement: 3 kits total / 1 standard + 2 additional)

- Transparent Insulating Shrouding: 1000V rated with insulated safety clamps.

- Insulating Rescue Pole

- Disconnect Shut Down Timer

- Steering Wheel Lockout Cover: 500mm

Lockout / Tagout (LOTO) Equipment

- Nylon Lockout Hasp (requirement: 3 hasps total / 1 standard + 2 additional)

- Lockout Tags: 10pc pack. (requirement: 3 packs total / 1 standard + 2 additional packs)

- Long Shackle Safety Lockout Padlock: Nylon body.

Workshop Signage & Posters

- High Voltage Signs & Warning Signs: Includes Hybrid/EV warning signs, double-sided HV signs, and textless danger signs. (requirement: 10 additional signs added to the standard set)

- Electric Shock First Aid Poster

Test Instruments, Probes & Leads

- Automotive Digital Multimeter Kit: CAT IV safety testing compliant. (requirement: 4 kits total / 1 standard + 3 additional)

- High Voltage Insulation Tester for Evs/ Meggermetre : CAT III compliant. (requirement: 2 testers total / 1 standard + 1 additional)

- Voltage Safety Tester: CAT IV 1000V. (requirement: 3 testers total / 1 standard + 2 additional)

- Current Clamp for Electric Vehicle:  (requirement: 2 clamps total / 1 standard + 1 additional)

- Milliohm Measuring Instrument : (requirement: 2 instruments total / 1 standard + 1 additional)

- Electrical Test Leads & Slim-Fit Test Probe Kit

Insulated Hand Tools (1000V / IEC60900 Rated)

(Note: 5 complete sets total of each tool style below / 1 standard + 4 additional)

- Insulated Drive Socket Kits: 3/8"D sets (includes both 22-piece and 50-piece configurations).

- Insulated 'Slipper' Torque Wrench: 3/8"D, 5 - 25Nm range.

- Insulated Spanner Set: Open-ended type.

- Insulated Screwdriver Sets: Includes standard slotted, star (Torx), and precision electrical profiles.

- Insulated Nut Driver Set

- Insulated Hex Key Set: Extra-long length.

- Insulated Star Key Set: Extra-long length.

- Insulated Pliers Set: Includes combination pliers (180mm), bent nose pliers (200mm), and wire stripping pliers (160mm).

- Insulated Magnetic Pick-Up Tool

- 1000V Insulated Inspection Mirror: 50mm diameter.

- Insulated Trim Removal Tool Kit

- Hybrid and EV Insulation & Multimeter Testers: 2 units required.

- Measuring Device for Coolant Solutions: 1 unit required.

- Electrical Insulation Caps / Terminal Covers: 10 units required.

- Charging Station Test Adapter: 1 unit required.

AT025 – HVAC Station Lot

Automatic recovery, recycling, vacuum and charging unit with touch screen and printer

Features Required

- HYBRID-READY: AUTO-CLEAN function for hybrid and electric vehicles

- Automatic Recovery /Recycling.

- Automatic Vacuum / Vacuum test.

- Automatic oil discharge (with scale).

- Automatic PAG, POE and UV oil charge.

- Automatic refrigerant charge.

- Automatic personalized cycle (recovery-vacuum-charge).

- Flushing cycle.

- Database upgradeable with USB memory.

- Automatic alarm for service: the unit advises when you have to replace the filter and the oil in the vacuum pump.

- Automatic bottle pressure control.

- Periodic auto-test: the unit checks the internal circuit tightness.

- Auto test of the non-condensable gases: the unit checks the bottle.

- Pressure test with refrigerant before the charging phase.

- Service connection on low pressure hose (I.C.S.).

- Distillation with automatic control.

- Integrated room temperature probe.

- Ability to connect to wifi required

Technical Specifications:

- Refrigerants: R134a(R456a)

- Recovery rate: 300 g/min

- Recovery accuracy: +/-15g

- Recovery efficiency: >95%

- Pump swept volume: 100l/min

- Bottle capacity: 10 kg

- Refrigerant scale resolution: 5 g

- Oil/UV scale resolution: 1 g

- Storage temperature: -10°C/+50°C

- Working ambient temperature: 0/+40°C

- Power supply: 230/1/50

Additional supports:

- Manufacturers onsite training to Course Instructors

- Continuous technical support

- Software updates

AT10001 – 2 Post Electro Hydraulic Baseless Lift (4-5 Tonne) with Adapter Kit

- 2 Post Electro-Hydraulic Lift with Wireless Control Transmission

- Suitable for Cars, Vans and Electric Vehicles up to LWB 5 Ton

- Extra Long Arms 1800 x 1800

- 5000kg Lifting capacity

- Electrical Safety Lock Release

- 24 V low voltage safety control

- Single phase Electrics

- c/w 90mm pad height extensions

- Arms – 1 set of 4M arms special angle 45 suitable for electric cars

AT10002 – Mobile Electric Hydraulic Lift suitable for Battery Packs for Electric Vehicles

- Mobile Electric hydraulic lifting platform capacity 1200Kg

- Lifting height 800-1810 Max

- Extendable platform length 1756-2066mm, width 900mm

- Mechanical locking system with electric release

- Suitable for use with battery packs for electric vehicles

- Single phase electrics 24V safety system

AT10003 – Lifting Frame for EV Battery Handling

- This lifting to be used during safe handling of batteries or other components, with a maximum total load capacity of 1500kg.

- Required to lift battery’s to and from their transport crate and to lift on and off battery lifting tables

- Equipped with 4 gripping points, equally distributed; which guarantee the stability of the load during movement.

- The equipment consists of a series of hooks and lifting belts for gripping the tools on the battery.

- The two side beams can slide along the main beam with a stroke of 300 mm each.

- The ends of the main beam are equipped with limit stops to prevent the crossbeams from slipping out if the instrument is used incorrectly.

- The gripper forks can slide along the side crossbars for a stroke of 200 mm each