- General

This chassis specification shall be read in conjunction with Appendix 1B – Surface Water Operations Jet Vac Superstructure Specification. All chassis components (engine, PTOs, cooling systems, electrical systems, axle capacities, frame ratings and auxiliary drives) must be fully compatible with, and sufficient for, the continuous parallel operation of all superstructure systems specified in Appendix 1B, without derating or operational limitation.

The Complete Vehicle Must Comply with All EU And Irish Type Approval Standards, Including Safety, Health & Welfare Legislation - Confirm Vehicle Will Comply

General Requirement: One new 6x2, 26 Tonne Jet-Vac Recycler Combination Unit, Including Superstructure and Chassis Cab.

- Vehicle - Chassis and Cab

Proposed Make/Model Must Be Suitable for Mounting a Jet-Vac Recycling Combination Unit

Full Description of Vehicle (State Exact Derivative)

Cab - Right Hand Drive

Hydraulically Operated Cab Tilt Function

Intermediate Day Cab: To Include Storage for Crew Personal Items, Minimum Cab Length 1800 Mm Long

Grab Handles Provided to All Doors for Safe Entry/Egress

Colour:  Cab Painted Factory Finished White

Remote Central Locking with Remote Control Unit on Keys. Two Complete Sets of Keys and Remote Fobs to be provided.

Radio/CD Player, USB Connection and Bluetooth Capability for Hands-Free Phone

Engine Immobilizer

Digital Tachograph and Speed Limiter (Both Calibrated)

Metric Speedometer

Electrical System:   24 Volt Lighting and Starting System.

Large, Well Anchored, Easily Adjusted External Mirrors. To Include L/R Main Mirrors And L/R Wide Angle Mirrors

Electrically Heated and Adjustable Rear View Main Mirrors, Heated Wide Angle Mirrors

Front Cyclops Mirror:

Electric Operated Windows

Air Conditioning

A Central Lubrication/Automatic Greaser System for All Fixed-Point Locations is to Be Provided on the Superstructure (This is Included in Superstructure Section Below). Lubricaton points on the Chassis are also to be conneted to this system (If Applicable)

Cab Heater / De-Mister:

Heated Front Windscreen or Windscreen Demister

Windscreen Glass Laminated;

Sun Blinds on Front Windscreen

External Sun Visor

Anderson Battery Booster Connector Wired to Battery Terminals Fitted Externally, Used for Starting the Vehicle in the Event of a Flat Battery. To Include a Cover to Protect from the Elements

Battery Isolation Switch

2 X Roof Mounted Air Horns

Driver's Seat:  Air Suspension Seat Fully Adjustable with Lumbar Support

Standard Fixed Single Passenger Seat

Seat Covers & Arm Rests Vinyl

Head Rests on All Seats

Adjustable Seat Belts with Safety Belt Warning Buzzer

Charging Points:   12 Volt & 24Volt DC Auxiliary Power Outlet Charging Points Provided to Cater for a Range of Electronic Equipment, Including Tablet Pc, Gas Detection Monitors and Large Torches.

USB Enabled Charging Port:

Internal Cab Lights:

Instrumentation:   Fitted With Speedometer, Engine Temperature Gauge, Ad-Blue Gauge and All Necessary Gauges, Suitably Lighted on the Dashboard.  Satellite Navigation System Also to Be Provided.

Any superstructure control screens or displays may be separate from the OEM chassis display systems, provided they do not obstruct driver visibility, airbag deployment zones, or OEM safety systems.

Control Switches/Buttons:    All Control Switched and Buttons to Be Clearly Labelled for Ease of Operator Use.

Storage Box in Centre of Cab Between Driver and Co-Driver Seat

Rubber Or Vinyl Studded Floor in Cab Area

Tailored Fit Anti Slip Rubber Floor Mats On Driver and Passenger Side

Handbrake Warning Buzzer

Chassis Constructed of Pressed High-Tensile Galvanised or Treated Steel Sections or Suitably Painted with Associated Warranty.  All Cross Members Welded, Riveted or Bolted.  Side Guards and Under Run Protection Installed.

Tank Frame and Associated Sub-Chassis Components of Galvanised Steel or Suitably Painted with Associated Warranty.

Suspension: High Deflection, Road Friendly/Air System Suspension.  Anti-Roll Bar Provided All Round. Air Suspension on Rear Axle

Vertical Exhaust System (To Extend Above Cab):

Rear Lamps to be LED Lamps.

Front And Rear Fog Lights:

End Outline Marker Lights, Side Marker Lights and Reflectors Fitted to Current Regulations

Individual Black Plastic Mudwings with Reflective Safety Mudflaps Fitted to All Wheels (Anti-Spray)

The vehicle electrical system shall support integration with fleet management and telematics systems (e.g. GPS tracking, camera systems) without impacting vehicle systems or warranty.

- Dimensions And Weights

State - Length of Cab

State - Height of Cab

State - Overall Vehicle Length Including Superstructure in Its Transport Position

State - Overall Vehicle Height, Including Superstructure, Unladen.

State - Projected Payload

State - Gross Vehicle Mass (Required 26,000Kg)

Chassis & Superstructure Drawings: To Be Provided, Indicating key Dimensions and Projected Individual Axle Loads Per Axle Unladen and Fully Laden.

Drawings and axle load calculations must demonstrate compliance at full specified tank volumes and full equipment fit-out, with no reduction in tank capacity or system functionality.

State - Turning Circle Diameter - Wall to Wall (State)

State - Max Water Height (Manufacturer to State) For When Wading Through Water

- Drive Train

Engine: Direct Injection Diesel.

Minimum Engine Horsepower Of 450BHP, At 1,900 RPM

State Any Alternatively Fuel Options: Hybrid, Or Dual-Fuel Diesel/Cng Engine, Capable of Delivering Required Power Output (Include Pricing Details Also)

Emissions Euro Stage - Minimum Euro Stage +6D

Adblue - If Required - Minimum Tank Size Of 50L

Gearbox:  Fully Automatic or Automated Manual (If Automated Manual Gearbox, Must Have a Min 10 Forward & 2 Reverse Gears)

All Auxiliary Equipment Powered from the Main Engine Via PTO

Vehicle Of Three Axle Type with Rear Steer Tag Axle

Front Axle Capacity - Minimum 9,000Kg

Middle And Rear Axles 11,500Kg and 8,250Kg, Respectively, Although This May Be Amended to Suit Manufacturers Requirements.  Please State Axle Loadings.

All Axels Must Be Heavy Duty.

Fuel Tank Fitted with Fuel Gauge and Locking Cap   Min.Capacity 400  Litres

Air Filter Housing to Include High Air Intake

Electronically Controlled Drive Axle Differential Lock

- Wheels And Tyres

Single On Front Axle, Dual on Middle and Single Rear Steer Wheels, With Heavy Duty Tyres to Suit Vehicle Offered.  Super Singles 385/65 on Front and Rear Steer. And Tooling Equipment to Change Wheel

State Tyre Size on Middle Driving Axle

Premium Tyre Brand to Be Supplied: - State Make

Yellow Loose Wheel Nut Indicators to Be Fitted to All Wheel Nuts

- Safety

Forward Collision Protection System - As Per Regulations

CCTV1: Integrated Reversing Display Unit & Camera: Minimum Requirement.  A Colour Reversing Camera Fitted, With Audio Capability.  The Output from the Camera Displayed on a Visual Display Unit on or Near Dashboard in Cab.

CCTV2: 360 Degree Recording Camera System. To Include 4 Cameras and Digital Video Recorder (DVR) Hard Drive - Minimum 1Tb Storage. To Include "Bird'S Eye View" Display. All Cameras to Include High Resolution Imaging, Durable Anti Corrosion Waterproof Housing and Infra-Red Led Night Vision Assist

Vehicle Axle Weigh System C/W In Cab Display. Load Cells Weighing System (Minimum 6 Cell) - Chassis Mounted Weighing Systems, Installed to Warn Driver If Over Legal Axel Weights, With In-Cab Weight Display

Warning Triangle, Fire Extinguisher & First Aid Kit, to be provided

Rear Underrun Guard

Audible White Noise Reverse Warning Alarm - C/W Night Silent Mode

Blind Spot Elimination Sensor System (Side & Rear Scan Proximity Sensors, With Interior Visual and Audio, & Exterior Audio Warning)

Two Large "Blind Spot Take Care" Warning Sign Boards. One Fitted to Each Side of Vehicle (3mm Aluminium Dibond or Similar, Approx. 600mm Long X 400mm High) (or according to available space)

Two High-Quality Heavy-Duty Wheel Chocks

Reflective Red Strips on the Inside of Driver Door, Passenger Doors and Any Other Swinging Doors on Superstructure

Front Daytime Running Lamps

Two Surface Mounted LED Amber Flashers on Front Grille of Truck Cab – Mid Level (high intensity, double/triple flashing pattern)

Three Surface Mounted LED Amber Flashers on Each Side of Vehicle - High Level, Must Be Flush and Fitting Not Protruding.  These May Be Mounted on the Tank If Necessary. (high intensity, double/triple flashing pattern)

One Long Bar Beacon Using Flashing Light Emitting Diodes (LEDs) Mounted on the Cab Roof.  (Approx. 1000Mm In Length)

Provision of Wiring from Cab to Chassis (Including Necessary Relays, Cab Switches Etc.) For The Switching On/Off of All LED Flashing Beacons Fitted to the Superstructure.  Cab Switch to Illuminate When Lights Turned On

All lighting and warning equipment requirements in this specification are cumulative with those specified in Appendix 1B, unless explicitly stated otherwise.

Provision of Wiring from Cab to Chassis (Including Necessary Relays, Cab Switches Etc.) For The Switching On/Off of All LED Worklights And Strip Lighting Fitted to Superstructure.  Cab Switch to Illuminate When Lights Turned On

Reflective, Florescent Markings as Per Legal Requirements (RSA)

Two Additional High Output LED Reverse Lamps on Rear Under Run (With On/Off Switch in Cab)

Two Rear Facing High Output LED Lamps Fitted Towards Front Sides of Chassis, To Assist When Reversing. Pre Wired To Reverse Lamp System with Functional On/Off Switch for Daytime Use.

- Brakes (Must Comply to EU Standards)

Disc Brakes on All Axles

Electronic Braking System (EBS)

Anti-Lock Braking System: (ABS)

ASR (Anti Spin Regulator)

ESS (Emergency Brake Signal)

ESP (Electronic Stability Program)

Emergency Brake / Hand Brake:     Hand And/Or Emergency Brake Power Assisted to Operate on Middle and Front Wheels.

Additional Engine Brake or Exhaust Retarder to Work in Conjunction with Service Brake - Include Details

- Chassis Cab - Maintenance and Warranty

Chassis Cab Recommended Service Intervals (State in Kilometres / time period)

Provide A Copy of Chassis Cab Manufacturer's Recommended Service Schedule Documentation (Submit with Tender Submission)

Chassis Cab Warranty Duration (State Number of Years And/or Kilometres/Hours) - Minimum Requirement: 2 Years. Tenderers shall clearly identify any warranty exclusions and provide details and pricing of any optional extended warranty available beyond the minimum requirement.

Proivide A Copy of Chassis Cab Manufacturer's Standard Warranty Terms and Conditions (Submit with Tender Submission)

Breakdown Assist for duration of the chassis warranty period. Tenderers shall provide details of breakdown assistance arrangements, including response times, coverage and exclusions.

- Steering

Power Assisted Steering

Adjustable Steering Column

- Tools, Manuals, Branding, And Delivery

Jack & Tool Kit

Warning Triangle

Operators Manual (Min 2 Hard Copies + Soft Copy)

Workshop Manual (Min 2 Hard Copies + Soft Copy)

Full technical documentation shall be provided, including electrical schematics, hydraulic diagrams, spare parts lists and as-installed system layouts.

Vehicle To Be Delivered to Final Destination with Full Tank of Fuel (And A Full Tank of Adblue - If Vehicle Uses Adblue)

Local Authority Branding Logos 1 Front, 1 On Each Side of Cab, And One Each Side of Superstructure

Set Of Safety Decals; (Samples Can Be Provided At Time Of Order)
Exterior: Payload; Tyre Pressure; Driver Daily Safety Check; Blind Spot; Do Not Pass on Inside. Interior: Seatbelt; No Smoking; No Drugs; Think Bike; Max Height; Driving Licence; Max Water Height (Manufacturer to State) For When Wading Through Water

All Delivery, Transportation and Storage Costs, Including Final Delivery to Local Authority Depot, is the Responsibility of, and Paid by The Supplier.  Vehicle To Be Onsite at Local Authority Depot One Day Before Official Handover (Driver/Operator Vehicle Familiarisation Must Be Provided by the Supplier on the Handover Day)

Crew Handover Familiarisation: To Be Carried Out by The Manufacturer/ Equipment Supplier at The Local Authority Site, On Day of Handover (Unless Agreed Otherwise).