COMPLIANCE TO SPECIFICATION

Lot 1 Drainage Pipes – 630mm – 635mm (External width) / 590mm – 600mm (internal width)

Company tendering: 	         (insert name of company)

| Description | Please complete in respect of your offer | Confirm your level of compliance with any additional remarks/notes if necessary | For OPW use only |
| --- | --- | --- | --- |
| # General: The pipes must be manufactured from MOPVC or UPVC plastic. The pipes must have a smooth interior and exterior walls. Corrugated pipes are not suitable. The pipes must be suitable for drainage of water. Note: All equipment must be supplied with manufacturer approval and compliance. | Confirm: Confirm: Confirm: |  |  |
| # Pressure Rating The Pipes must have a pressure rating of PN20. | Confirm: |  |  |
| # Pipe Diameter: The pipes must have an external diameter in the range of 630 – 635mm. The pipes must have an internal diameter of 590 – 600mm. | Confirm: Confirm: |  |  |
| # Pipe length: The pipes must be supplied in a range 5.95m to 6m lengths. | Confirm: |  |  |
| # Connection Type: The pipes must incorporate spigot and socket connections, formed as part of the pipe, and suitable watertight seal. | Confirm: |  |  |
| # Wall Thickness: The pipes must have a wall thickness not less than 15mm and not greater than 20mm. | Confirm: |  |  |
| # Weight: The pipes must weigh no less than 40kg/m and no greater than 60kg/m. | Confirm: |  |  |
| # Delivery: Delivery to OPW, Templemungret House, Mungret, Co. Limerick, V94 EK07. Delivery not later than 7th of December 2026 for any order placed in 2026. Tenders must state the best available lead delivery time on the pricing schedule. | Confirm: Confirm |  |  |