Generated: 01/07/2026

OY-R401-B-020

Bridge

Contents

Figure

Table

# Bridge Summary

Number (calc.): OY-R401-B-020

Name (alias): Boyne Bridge

Bridge Type: Arch

Maintenance Inspection Date: 14/10/2025

Maintenance Inspection Rating: Warning

Engineering Inspection Date: 14/10/2025

Engineering Inspection Rating: Significant Damage

# Bridge Detail

## Bridge

Bridge Type: Arch

Principle Function: Public Road

Number (calc.): OY-R401-B-020

Number (plate): OY-R401-001.00

Name (alias): Boyne Bridge

Road Number: R401

Is in Skew? Yes

Skew Angle: 10

Is Flood Relief? Yes

Number of Openings: 2

Height Restriction: N/A

Is Signage Present? Yes

Height Restriction (on signage): 150

Weight Restriction? No

Weight Restriction (on signage): N/A

Bridge Over: Watercourse

Bridge Under: N/A

Services Evident? No

## Dimensions

Length: 9

Width: 8.65

Height: 5

Number of Spans: 1

Total Span: 4.78

Max. Span: 4.78

Min. Span: 4.78

## Materials

Materials: Masonry

## Access Hazards

Access Hazards: Deep Water, Steep Banks, Vegetation, Very Tall Structure

## Location

Location Description: N/A

Townland: EDENDERRY

Municipal District: Municipal District of Edenderry

River Basin District: Eastern

## Additional Info

Ownership: N/A

Protected Status: N/A

Ecological Status: N/A

Year of Construction (approx.): N/A

# Bridge Attachments

Bridge Attachments:

File Name : 2025_10_14_14_49_10_8159.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_49_19_7602.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_50_06_4070.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_50_21_0668.jpg Name : N/A Description : N/A

# Maintenance Inspections

Inspection made on: 14/10/2025

Surveyor: E Daly Overall Rating: Warning Parapets: Warning External Walls: Warning Access and Egress: Alert Abutments and Piers: Warning Vegetation: Warning Deck or Arch: Good Comments: Significant settlement of the embankments was noted during inspection.

Inspection Attachments:

File Name : 2025_10_14_14_54_56_3644.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_55_06_6305.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_55_20_1873.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_55_31_7439.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_55_38_2027.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_55_54_0880.jpg Name : N/A Description : N/A

# Engineering Inspections

Inspection made on: 14/10/2025 (Completed)

Surveyor: E Daly Overall Rating: Significant Damage Surface: 2 Surface Comment: Surface generally in good condition with upgrade works having taken place circa 2021 - poor lateral support present at road edges leaving edge of pavement vulnerable Footpath: 2 Footpath Comment: Verges present, rubbing strips should be considered to prevent water egress through Bridge structure and direct traffic away from parapit walls Parapets: 2 Parapets Comment: Some minor damages evident on parapit walls, resetting on stones and minor repointing required Embankment and Revertments: 3 Embankment and Revertments Comment: Embankments have settled significantly and gaps forming between the road edge and embankements. The angle of the embankements are severe and poor lateral support to the road structure is evident. Significant works are required to reconstruction the embankents Wingwalls: 3 Wingwalls Comment: Spndrals in good condition with minor mortar loss, no wing walls or retaining walls present, consideration should be given to provide wing walls Abutments: 2 Abutments Comment: Abutments appear to be in good condition, however, at time of survey, access to the riverbed was prevented due to excessive overgrowth, a further assessment of the abutments is required upon access being available Piers: 1 Piers Comment: No piers present Spandrels: 2 Spandrels Comment: minor repointing works required Arch Barrels: 1 Arch Barrels Comment: The arch barreell appears to be in good condition, however, at time of survey, access to the riverbed was prevented due to excessive overgrowth, a further assessment of the arch barrell is required when access is available is required upon access being available Beams Girders: 1 Beams Girders Comment: N/A Slab Deck: 1 Slab Deck Comment: N/A Riverbed: 2 Riverbed Comment: N/A Bearings and Expansion Joint: 1 Bearings and Expansion Joint Comment: N/A Other Elements: N/A Comments: Significant settlement is present on the embankements, the embankents are steep with poor lateral support in place. Embankment require reconstruction to ensure no further deterioration to the road and structure.

Inspection Attachments:

File Name : 2025_10_14_14_59_07_6676.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_59_24_1147.jpg Name : N/A Description : N/A

File Name : 2025_10_14_14_59_36_2345.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_00_06_2357.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_00_14_4406.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_00_19_3781.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_00_31_7637.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_00_58_2466.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_01_10_7287.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_01_20_9742.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_01_42_1384.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_01_47_8154.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_02_25_8800.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_02_32_8476.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_03_10_7177.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_03_24_7850.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_03_40_8166.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_04_48_0407.jpg Name : N/A Description : N/A

File Name : 2025_10_14_15_04_59_3356.jpg Name : N/A Description : N/A