Generated: 26/06/2026

OY-R400-B-030

Bridge

Contents

Bridge Summary	0

Bridge Detail	0

Bridge Attachments	0

Maintenance Inspections	0

Engineering Inspections	0

Figure

No table of figures entries found.

Table

No table of figures entries found.

# Bridge Summary

Number (calc.): OY-R400-B-030

Name (alias): Raheenkeernan Bridge 2

Bridge Type: Slab

Maintenance Inspection Date: 08/10/2024

Maintenance Inspection Rating: Alert

Engineering Inspection Date: 08/10/2024

Engineering Inspection Rating: Significant Damage

# Bridge Detail

## Bridge

Bridge Type: Slab

Principle Function: Public Road

Number (calc.): OY-R400-B-030

Number (plate): Raheenkeernan Bridge 2

Name (alias): Raheenkeernan Bridge 2

Road Number: R400

Is in Skew? No

Skew Angle: N/A

Is Flood Relief? No

Number of Openings: N/A

Height Restriction: N/A

Is Signage Present? No

Height Restriction (on signage): N/A

Weight Restriction? No

Weight Restriction (on signage): N/A

Bridge Over: Watercourse

Bridge Under: N/A

Services Evident? Yes

## Dimensions

Length: 3.1

Width: 9.1

Height: 1.7

Number of Spans: 1

Total Span: 1.1

Max. Span: 1.1

Min. Span: 1.1

## Materials

Materials: Concrete

## Access Hazards

Access Hazards: None

## Location

Location Description: Raheenkeernan Bridge 2 is a single span composite structure of concrete slab deck above block work and concrete abutments

Townland: RAHEENAKEERAN

Municipal District: Municipal District of Edenderry

River Basin District: South Eastern

## Additional Info

Ownership: N/A

Protected Status: N/A

Ecological Status: N/A

Year of Construction (approx.): N/A

# Bridge Attachments

Bridge Attachments:

File Name : 2024_10_08_10_12_26_4983.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_13_02_5262.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_13_18_1606.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_14_14_0092.jpg Name : N/A Description : N/A

# Maintenance Inspections

Inspection made on: 08/10/2024

Surveyor: E Daly Overall Rating: Alert Parapets: Warning External Walls: Alert Access and Egress: Good Abutments and Piers: Alert Vegetation: Good Deck or Arch: Warning Comments: Sever settlement of the upstream side of structure is evident, the abutment on the LHS upstream side has lost position and has settled on one side, Structure is no longer founded appropriately.

Inspection Attachments:

File Name : 2024_10_08_10_27_15_8374.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_27_30_0002.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_27_41_1972.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_28_39_0513.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_29_21_9283.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_29_34_9226.jpg Name : N/A Description : N/A

# Engineering Inspections

Inspection made on: 08/10/2024 (Completed)

Surveyor: E Daly Overall Rating: Significant Damage Surface: 1 Surface Comment: The Bridge surface is in good condition with a change of surfacing texture noted on deck. Footpath: 1 Footpath Comment: Verges are present on both sides of the structure Parapets: 2 Parapets Comment: The parapits are of blockwork construction with capping stones. The upstream parapit has settled on one side which attests to the settlement of the structure in its entirety on the upstream LHS Embankment and Revertments: 3 Embankment and Revertments Comment: Revetments are in poor condition with underpinning or replacement required very soon Wingwalls: 2 Wingwalls Comment: Wingwalls are generally not visible on the structure, the parapits span for 3.1m in their entirety. There is a visible section of wingwall on the upstream RHS which appears to be structurally OK. Abutments: 4 Abutments Comment: The abutments are in extremely poor condition with loss of bearing evident throughout. Water flow has scoured below the abutments in many areas, The upstream abutment on the LHS has lost position with a repair carried out previously. However, the repair did not address the structural issue with the structure nowhaving inadequate foundations/ abutments Piers: 1 Piers Comment: No piers present Spandrels: 1 Spandrels Comment: N/A Arch Barrels: 1 Arch Barrels Comment: N/A Beams Girders: 1 Beams Girders Comment: N/A Slab Deck: 2 Slab Deck Comment: Concrete slabs form the deck, they appear to be in good condition and may have been replaced at a time previous Riverbed: 2 Riverbed Comment: The bed is inconsistent with pockets formed along its length, on the upstream side, a channel appears to have been formed at a time unkown Bearings and Expansion Joint: 2 Bearings and Expansion Joint Comment: The present bearing is concrete slab directly onto clock The current bearing relationship is concrete slab onto blockwork which should be addressed during remedial works Other Elements: A wrought iron structure is built off the upstream side of the RHS wingwall. it is not known what the purpose of this structure was, further vegetation removal is required to assess Comments: Raheenkeernan Bridge 2 has been assigned a rating of 3. The foundations of this structure are in very poor condition and underpinning/ new foundations will be required very soon

Inspection Attachments:

File Name : 2024_10_08_10_53_54_2378.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_54_10_2942.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_54_21_0116.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_54_27_5964.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_54_45_9777.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_55_14_5193.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_55_57_1495.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_56_05_5834.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_56_12_4339.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_56_29_4734.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_56_45_4296.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_56_51_1304.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_57_05_2907.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_57_35_6126.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_58_05_8838.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_58_18_4686.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_58_25_9408.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_58_48_8415.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_59_11_1864.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_59_17_8597.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_59_30_6035.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_59_50_0314.jpg Name : N/A Description : N/A

File Name : 2024_10_08_10_59_55_8674.jpg Name : N/A Description : N/A

File Name : 2024_10_08_11_00_20_1114.jpg Name : N/A Description : N/A