Appendix 1 –  Detailed Specification of Requirements

Requirements per Lot

Contents

LOT 1 – Cardiovascular Equipment	1

Lot 1 – Cardiovascular Equipment Specification	1

LOT 2 – Plate Loaded & Strength	7

Lot 2 - Plate Loaded & Strength Specification	7

LOT 3 - Functional Zone	14

Lot 3 - Functional Zone Specifications	15

LOT 4 - Premium Performance, Recovery & Rehab	22

Lot 4 – Premium Performance, Rehab & Recovery Specification	22

Lot 5 – Selectorised	24

Lot 5 - Sectorised Equipment Specification	25

Design & Décor	27

# LOT 1 – Cardiovascular Equipment

| Equipment Name | Quantity |
| --- | --- |
| Treadmill | 26 |
| Performance Treadmill | 3 |
| Upright Bike | 12 |
| Recumbent Bike | 7 |
| Elliptical Trainer | 14 |
| Step mills | 10 |
| Rower | 8 |
| Ski Erg | 8 |
| Functional Zone- Digital Solution with display for Group Training Solution | 1 |
| Digital Solution with display for self-led & group Training | 2 |

## Lot 1 – Cardiovascular Equipment Specification

| Treadmills |  |  |
| --- | --- | --- |
| Design |  | Base units should be identical in colour and design > 22" Touchscreen option |
| Colour |  | TBC |
| Max User Weight |  | > 200KG |
| Running Surface (L x W) |  | Minimum 155 x 56cm |
| Speed Range |  | 0.8 - 23km/hr |
| Step-up height (height of deck off floor) |  | Maximum step up 9.5" |
| HR Monitoring |  | Via Hand Sensors |
| LED Console |  | Must have quick start option and additionally a minimum of 8 built in workouts including but not limited to HR Driven, Goal Driven, Hills & Cross Training |
| Touchscreen Console |  | >22" Touchscreen Required |
|  |  | Apple Health/Smart Watch connectivity |
|  |  | Live events/races included in on screen content |
|  |  | On screen interactive workouts |
|  |  | Fitness Testing Options including Gerkin Protocol & VO2 Max |
|  |  | Media shelf/storage slot with inbuilt charging |
|  |  | Apps & Internet Browsing Available |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Closed Captions in on-screen workouts |
|  |  | Please detail all additional accessible design features |
| Performance Treadmill |  |  |
| Design |  | Designed for high performance and HIIT training |
| Colour |  | TBC |
| Max User Weight |  | >160KG |
| Running Surface |  | Slatted Belt |
| Speed Range |  | 0.2 - 25km/hr |
| Console |  | >21" Touchscreen |
|  |  | Smart Watch connectivity |
|  |  | On Screen interactive workouts. |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Please detail all accessible design features |
| Upright Bike |  |  |
| Design |  | Base units should be identical in colour and design with a LED and a 22" Touchscreen option |
| Colour |  | TBC |
| Max User Weight |  | >175KG |
| HR Monitoring |  | Via Hand Sensors |
| LED Console |  | A minimum of 8 built in workouts including but not limited to HR Driven, Goal Driven, Hills & Cross Training |
| Touchscreen Console |  | >22" Touchscreen Required |
|  |  | Apple Health/Smart Watch connectivity |
|  |  | Live events/races included in on screen content |
|  |  | On screen interactive workouts |
|  |  | Fitness Testing Options including Gerkin Protocol |
|  |  | Apps & Internet Browsing Available |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Clear Step through (or low console to step over) |
|  |  | Please detail all additional accessible design features |
| Recumbent Bike |  |  |
| Design |  | Base units should be identical in colour and design with a LED and a 16" Touchscreen option |
| Colour |  | TBC |
| Max User Weight |  | >200KG |
| HR Monitoring |  | Via Hand Sensors |
| LED Console |  | A minimum of 5 built in workouts including but not limited to HR Driven, Goal Driven, Hills & Cross Training |
| Touchscreen Console |  | >16" Touchscreen Required |
|  |  | Apple Health/Smart Watch connectivity |
|  |  | Live events/races included in on screen content |
|  |  | On screen interactive workouts |
|  |  | Fitness Testing Options including Gerkin Protocol |
|  |  | Apps & Internet Browsing Available |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Clear Step through (or low console to step over) |
|  |  | Adjustable backrest |
|  |  | Easy access optional controls on handles |
|  |  | Please detail all additional accessible design features |
| Elliptical |  |  |
| Design |  | Base units should be identical in colour and design with a LED and a 22" Touchscreen option |
| Colour |  | TBC |
| Max User Weight |  | >175KG |
| Stride |  | 26" + Stride |
| Incline/Decline |  | Ramp adjustment to increase the user range of movement |
| HR Monitoring |  | Via Hand Sensors |
| LED Console |  | A minimum of 5 built in workouts including but not limited to HR Driven, Goal Driven, Hills & Cross Training |
| Touchscreen Console |  | 22" Touchscreen Required |
|  |  | Apple Health/Smart Watch connectivity |
|  |  | Live events/races included in on screen content |
|  |  | On screen interactive workouts |
|  |  | Fitness Testing Options including Gerkin Protocol |
|  |  | Apps & Internet Browsing Available |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Please detail all accessible design features |
| Step mills |  |  |
| Design |  | Base units should be identical in colour and design |
| Colour |  | TBC |
| Max User Weight |  | >175KG |
| Speed |  | Up to 165 SPM |
| Step-up height (height of first step off floor) |  | Max 10.5" |
| HR Monitoring |  | Via Hand Sensors |
| LED Console |  | Must have a minimum of 8 built in workouts including but not limited to HR Driven, Goal Driven, Hills & Cross Training |
| Touchscreen Console |  | 10" Touchscreen Required |
|  |  | Apple Health/Smart Watch connectivity |
|  |  | Live events/races included in on screen content |
|  |  | On screen interactive workouts |
|  |  | Fitness Testing Options including Gerkin Protocol |
|  |  | Apps & Internet Browsing Available |
|  |  | Open API capability for integration with Leisure Management Systems |
|  |  | Capability for campaign messaging & diagnostics equipment usage management |
| Accessibility |  | Lateral Foot Rests |
|  |  | Please detail all additional accessible design features |
| Rower |  |  |
| Design |  | Commercial-grade rowing machine designed for high-performance and general population use Aluminium Frame- air and magnetic resistance |
| Colour |  | TBC |
| Max User Weight |  | >160KG |
| Chain |  | Nickel-Plated Steel Chain |
| Monitor |  | 7-10" Bluetooth and connectivity features for seamless user tracking and Heart rate monitoring |
|  |  | Must include preset workouts including but not limited to goal workouts, HR workouts and interval training |
|  |  | ANT+ and Bluetooth Smart wireless heart rate monitoring with compatible equipment |
| Programmes |  | Optional workout selection |
| Extra |  | Phone Holder- accommodates devices of different sizes. |
|  |  | Please provide details on how the equipment and associated software can support both general users and athletes in enhancing rowing performance, including access to advanced training data, performance tracking, and coaching insights. |
| Monitor Arm |  | Must allow adjustment of monitor height |
| Accessibility |  | Backlit display |
|  |  | Please detail all additional accessible design features |
| Upper Body Ergometer - Ski Cardio/Strength Combo |  |  |
| Design |  | Functional Design with air resistance (cardio) and magnetic resistance (Strength) |
| Colour |  | TBC |
| Console |  | Must show time/distance, calories burned, watts, cadence and average pact. |
| On Demand workouts |  | Via on unit screen or via device and app connectivity |
| Bluetooth Connectivity |  | Required |
| Base |  | Must be freestanding to include secure base |
| Accessibility |  | Backlit display |
|  |  | Please detail all additional accessible design features |
| Functional Zone- Digital Solution for Group Training Solution |  | A digital group training solution that delivers structured workouts through large-screen visual guidance, including exercises, timers, and session flow. To enable coaches to run efficient, engaging classes while reducing reliance on manual planning tools |
| Design |  | Provides real-time visual prompts including exercise demonstrations, work/rest timers, and session flow, which includes |
| Functionality |  | Enables delivery of multiple training formats, Suitable for all ability levels, from beginner to advanced. Enhances community engagement through shared workout experiences and post-session performance summaries. |
| Support |  | Enables delivery of multiple training formats, Suitable for all ability levels, from beginner to advanced. Enhances community engagement through shared workout experiences and post-session performance summaries Visual instructions to support correct exercise technique. Integration capability with fitness tracking platforms |
| Additional |  | Customizable branding and content to align with facility identity |
| Digital Solution for self-led & group Training |  | A digital touchpoint solution to enhance both self-led exercise and group training sessions within the facility. The system should support member engagement, coaching delivery, and real-time performance feedback. |
| Design |  | Commercial-grade digital display units, Robust and durable for high-traffic use suitable for gym environments Wall-mounted and/or free-standing options available |
| Functionality |  | Touchscreen interface for ease of use, Ability for users to select workouts based on: Goal (strength, conditioning, mobility) Ability level (beginner to advanced) Visual demonstrations of exercises (video or animation-based) Timer and interval functionality for independent training Supports self-led workouts, including guided exercise programmes |
| Accessibility |  | Clear, easy-to-follow visual instructions, Suitable for all user levels. Multi-language capability desirable |
| Training |  | Staff training, and ongoing technical support and software updates |
| Additional |  | Capability for campaign messaging & diagnostics equipment usage management |

# LOT 2 – Plate Loaded & Strength

| Equipment Name | Quantity |
| --- | --- |
| PL Shoulder Press | 1 |
| PL Seated Row | 1 |
| PL Bench Press | 1 |
| PL Wide Lat Pull down | 1 |
| PL Glute hip extension | 1 |
| PL Hack Squat | 1 |
| PL Leg Extension | 1 |
| PL Leg Press | 2 |
| PL Leg Curl | 1 |
| PL Multi Squat | 1 |
| PL Upper body Fly | 1 |
| PL Nordic Hamstring | 1 |

## Lot 2 - Plate Loaded & Strength Specification

| It is expected that racks will be designed to a superior level suitable for heavy commercial use. Please supply detail/evidence for each element below: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Strong & Robust Design |  | 11 Guage steel with rectangular tubing is required. CE Certifications should be supplied. |  |  |
| Colour & Uniformity |  | Black Frames, powder coat finish for durability |  |  |
| Half Racks |  |  |  |  |
|  |  | Space conscious design with 5 pairs of weight pins for storage and bar holders to rear |  |  |
|  |  | Monkey Bar to front for pullups to include textured grip |  |  |
|  |  | J-Hooks to include urethane protection for bars |  |  |
|  |  | Safety bar catches to include urethane protection for bars |  |  |
| Double Side half racks |  |  |  |  |
|  |  | Space conscious back to back half rack design with shallow depth and 4 pairs of weight pins for storage. |  |  |
|  |  | Monkey Bar to front for pullups to include textured grip |  |  |
|  |  | J-Hooks to include urethane protection for bars |  |  |
|  |  | Safety bar catches to include urethane protection for bars |  |  |
| Smith Machine |  |  |  |  |
|  |  | Bar path @ 7 degree angle |  |  |
|  |  | 11+ Adjustable heights |  |  |
|  |  | 4 pairs of Weight Pins |  |  |
| Olympic Bench Press |  |  |  |  |
|  |  | Adjustable Olympic Bench Press with weight storage integrated |  |  |
| Paralympic Bench Press |  |  |  |  |
|  |  | Must meet current Paralympic WPPO (World Para Powerlifting Organization) specification for competition bench press |  |  |
|  |  | Should include adjustable stands |  |  |
|  |  | Frame should have powder coat finish |  |  |
|  |  | Height 45cm |  |  |
| Plate Loaded Machines |  | It is expected that all plate loaded equipment will be designed to a superior level suitable for heavy commercial use. Please supply detail/evidence for each element below: |  |  |
| Strong & Robust Design |  | 11 Guage steel, rectangular tubing & powder coat finish is required. CE Certifications should be supplied. |  |  |
| Colour & Uniformity |  | Frames, Upholstery TBD |  |  |
| Instruction/Guidance Placards |  | With visual guidance cues |  |  |
| Footprint |  | We are seeking space conscious premium design as a priority. Please detail any space saving features within proposed units. |  |  |
| PL Shoulder Press |  |  |  |  |
|  |  | Iso-Lateral Design |  |  |
|  |  | Adjustable Seat |  |  |
|  |  | Ergonomic Handles |  |  |
|  |  | Starting Resistance < 1KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Plate Capacity - 80KG per Arm |  |  |
|  |  | 6+ Weight Pins for storage |  |  |
| PL Seated Row |  |  |  |  |
|  |  | Iso-Lateral Design |  |  |
|  |  | Adjustable Seat |  |  |
|  |  | Ergonomic Handles |  |  |
|  |  | Starting Resistance = 5KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Plate Capacity - 100KG per Arm |  |  |
| PL Bench Press |  |  |  |  |
|  |  | Iso-Lateral Design |  |  |
|  |  | Adjustable Seat |  |  |
|  |  | Ergonomic Handles |  |  |
|  |  | Starting Resistance < 1KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Plate Capacity - 100KG per Arm |  |  |
|  |  | 6+ Weight Pins for storage |  |  |
| PL Wide Lat Pull Down |  |  |  |  |
|  |  | Iso-Lateral Design |  |  |
|  |  | Adjustable Seat |  |  |
|  |  | Ergonomic Handles |  |  |
|  |  | Starting Resistance < 1KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Plate Capacity - 100KG per Arm |  |  |
|  |  | 6+ Weight Pins for storage |  |  |
| PL Glute Drive |  |  |  |  |
|  |  | Durable back pads |  |  |
|  |  | Non slip foot platform angled 15 Degrees |  |  |
|  |  | Band Pegs |  |  |
|  |  | Plate Capacity - Minimum 200kg |  |  |
|  |  | 4 Weight Pins |  |  |
| PL Hack Squat |  |  |  |  |
|  |  | Adjustable ROM |  |  |
|  |  | 4+ Weight Pins for storage |  |  |
|  |  | Large, non slip foot platform |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Plate Capacity - 320kg |  |  |
| Leg Extension |  |  |  |  |
|  |  | Iso Lateral |  |  |
|  |  | Should include a range limiter for ROM control |  |  |
|  |  | Plate Capacity - 60KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
| Linear Leg Press |  |  |  |  |
|  |  | 45-Degree Angle |  |  |
|  |  | Adjustable Back Pad |  |  |
|  |  | Large, non slip foot platform |  |  |
|  |  | Adjustable ROM |  |  |
|  |  | Visual Placards included |  |  |
|  |  | Load rating > 800KG |  |  |
|  |  | 4+ Weight Pins for storage |  |  |
| Leg Curl |  |  |  |  |
|  |  | Seated |  |  |
|  |  | Iso-Lateral Design |  |  |
|  |  | Load rating > 90kg |  |  |
|  |  | Visual Placards included |  |  |
|  |  | 4+ Weight Pins for storage |  |  |
| Multi Squat |  |  |  |  |
|  |  | Suitable for squats, deadlifts, RDLs, singe leg squats and lunges |  |  |
|  |  | Should support Bilateral and Unilateral training |  |  |
|  |  | Plate capacity - 80KG ( 4 x 20KG Plates) per rod |  |  |
|  |  | Visual Placards included |  |  |
|  |  | 2+ Weight Pins for storage |  |  |
| Upper Body Fly |  |  |  |  |
|  |  | Should include flat to decline adjustment |  |  |
|  |  | Plate Capacity - 60KG per Arm |  |  |
|  |  | Visual Placards included |  |  |
|  |  | 4+ Weight Pins for storage |  |  |
| Nordic Hamstring |  |  |  |  |
|  |  | Counterbalance Design |  |  |
|  |  | Should include single upper body pad adjustment |  |  |
|  |  | Visual Placards included |  |  |
|  |  | 2+ Weight Pins for storage |  |  |
| Grip Plates |  |  |  |  |
|  |  | Tri Grip Ergonomic design |  |  |
|  |  | 50mm Hole Diameter |  |  |
|  |  | Custom Logo Included |  |  |
|  |  | Urethane Discs |  |  |
| Free-weights |  |  |  |  |
| 3 Tier Horizontal racks |  | to store the above 15 pairs per rack. Should include saddles for correct dumbbell position. Black in colour* |  |  |
|  |  | PLEASE NOTE Dumbbells will not be co-located so racks must suitably situate sets as outlined above |  |  |
| Round Urethane Dumbbells |  | Chrome knurled handle for easy grip |  |  |
|  |  | Easily identifiable denomination |  |  |
|  |  | Custom logo included |  |  |
|  |  | 2kg-36kg in 2kg increments ( 3 sets) |  |  |
|  |  | 38-50kg in 2kg increments - ( 2 sets) |  |  |
| Benches |  | It is expected that benches will be designed to a superior level suitable for heavy commercial use. Please supply detail/evidence for each element below: |  |  |
| Strong & Robust Design |  | 11 Guage steel, rectangular tubing & powder coat finish is required with strong durable upholstery. |  |  |
| Colour & Uniformity |  | Black Frames, Black Upholstery with powder coat finish for durability |  |  |
| Adjustable Benches |  |  |  |  |
|  |  | Seat adjustment should provide 0, 15 & 30 Degrees |  |  |
|  |  | Back pad adjustment should provide 0, 15,30, 45, 60 & 75 Degrees |  |  |
|  |  | Should include wheels and a handle for easy manoeuvring |  |  |
| Preacher Curl |  |  |  |  |
|  |  | Adjustable seat |  |  |
|  |  | Wide Arm Pad |  |  |
| Back Extension |  |  |  |  |
|  |  | 45 Degrees with adjustable angled pad |  |  |
|  |  | Lower roller pads |  |  |
|  |  | Anti-slip footplates |  |  |
| Kettlebells |  | 6/8/10/10/12/16/20/24/28/32KG |  |  |
|  |  | Chrome Grip |  |  |
|  |  | Colour coded for easy identification |  |  |
|  |  | Rubber Bell |  |  |
| Kettlebells Stand |  |  |  |  |
|  |  | To store up to 10 individual kettlebells across 1 or 2 tiers |  |  |
|  |  | Horizontal stand |  |  |
| Cable Quad Towers |  |  |  |  |
|  |  | 4 Bay Freestanding tower to include: |  |  |
|  |  | 2 x Adjustable Pulley with 2:1 cable ratio, adjusts from 7" - 76" |  |  |
|  |  | 1 x Pull Down with 1:1 cable ratio, adjustable knee pads, standard Lat bar |  |  |
|  |  | 1 x Row with 1:1 cable ratio, angled footpads , standard row handle |  |  |
|  |  | Should include options to interconnect to enhance design such as crossover bars etc. Please supply details and indicate additional pricing for info only. |  |  |
|  |  | Required each weight stack on each side should have at least 60kg |  |  |
| Cable Accessories |  | Must be rated for commercial use |  |  |
| Bars |  |  |  |  |
| 7ft Olympic Bar |  | Competition standard durable bar |  |  |
|  |  | Tested to 750KG or more |  |  |
|  |  | 20KG |  |  |
|  |  | 28mm Grip |  |  |
| 15KG Bar |  | IWF competition lifting standard |  |  |
|  |  | Tested to 450KG or more |  |  |
|  |  | 15KG |  |  |
|  |  | 25mm Grip |  |  |
| Safety Squat Bar |  | Handles and neck pad integrated |  |  |
|  |  | 21KG |  |  |
| Trap Bar |  | Brushed Steel Bar |  |  |
|  |  | Tested to 300KG or more |  |  |
|  |  | 30 - 35KG |  |  |
| Trap Bar |  | Open ended trap bar with double handles |  |  |
|  |  | Rubber feet for ease of loading |  |  |
| EZ Curl Bar |  | 10KG |  |  |
|  |  | Diameter 50mm |  |  |
|  |  | Length 1200mm |  |  |
| Quick Lock Collars |  | 40 Pairs |  |  |
| High Grade Coloured Rubber Bumper Plates |  | Dead bounce construction |  |  |
|  |  | Tested to 10,000 drops |  |  |
|  |  | Reinforced steel sleeve |  |  |
|  |  | Custom Logo Included |  |  |
| Metal Plyometric Platforms |  |  |  |  |
|  |  | Stackable design |  |  |
|  |  | Heavy duty frame with no slip platform |  |  |
| Air Bikes |  |  |  |  |
| Design |  | Belt Drive design |  |  |
| Colour |  | Black or Dark Grey |  |  |
| Max User Weight |  | 150KG+ |  |  |
| Console |  | Console readings to include RPM, time, speed, calories and Watts. |  |  |
|  |  | Should include workout programmes such as Interval training and target based workouts for time, calories and HR. |  |  |
|  |  | Adjustable seat |  |  |
|  |  | Should include an air guard |  |  |
|  |  | Should include transport wheels for easy manoeuvring |  |  |

| Grip Plates | Quantity |
| --- | --- |
| 1.5kg | 10 |
| 2.5kg | 16 |
| 5kg | 16 |
| 10kg | 42 |
| 15kg | 36 |
| 20kg | 56 |
| 25kg | 30 |
| Dumbbells & Storage | Quantity |
| 3 Tier Dumbbell Rack | 5 |
| Dumbbells - as per specification | 3 sets |
| Dumbbells - as per specification | 2 sets |
| Bench- adjustable | 12 |
| Preacher Curl | 3 |
| 45-degree Back Extension | 2 |
| Kettle Bells | 1 set of various weight as per spec |
| Kettle bell holder /Rack | 1 |
| Cable Machines | Quantity |
| Cable Stations | 4 |
| Accessories |  |
| Tricep rope - Single | 6 |
| Tricep rope -Double | 4 |
| Double handle | 4 |
| Tricep bar | 4 |
| Lat pulldown | 2 sets |
| Straight bar | 6 |
| Band Handles | 6 |
| Ankle straps | 6 |
| Accessory holder | 3 |
| BARS | Quantity |
| Olympic Bar 7ft – 20kg | 20 |
| Olympic Bar 15kg | 2 |
| Safety Squat Bar 21kg | 2 |
| EZ curls 10kg | 5 |
| Trap Bar 30 - 35kg | 3 |
| Trap Bar Open sided-20-30kg- space saving | 3 |
| Trapbar holder | 1 |
| Barbell/Ez holder | 1 |
| Quick Lock Collars | 40 sets |
| Tree Holder for Grip Plates | 2 |
| Bumper Plates-KG | Quantity |
| 5 | 40 |
| 10 | 40 |
| 15 | 60 |
| 20 | 40 |
| 25 | 40 |
| Plyoboxes 12” | 4 |
| 18” | 4 |
| Air Bikes | 4 |
| Racks | Quantity |
| Half Racks | 2 |
| Double Side half racks | 4 |
| Smith Machine | 1 |
| Olympic Bench Press | 5 |
| Paralympic Bench Press | 1 |

# LOT 3 - Functional Zone

| Equipment Name | Quantity |
| --- | --- |
| 12m Stand-alone Wall Training 1 | 1 |
| 7-10m Stand-alone Wall Training 2 | 1 |
| Cable station/¦Tower | 1 |
| Dual Adjustable Pulley | 3 |
| Storage Options to contain |  |
| Accessory dips handles | 2 |
| Accessory split squat roller | 2 |
| Kettle Bells- as per spec | 4 sets |
| Sandbags or equivalent | 2 set |
| Resistance bands | 6 sets |
| Medicine Balls | 4 sets |
| Slam Ball | 4 sets |
| Plyobox 8 | 2 |
| Plyobox 12 | 2 |
| Plyobox 18 | 2 |
| Plyobox 24 | 2 |
| BOSU balance trainer or equivalent | 5 |
| Heavy duty Boxing Bag | 5 |
| Battle ropes | 5 |
| Suspension straps | 6 |
| Olympic Ring attachments | 2 |
| Sleds | 3 |
| Weighted plates for sleds | 3 |
| Cable Accessories |  |
| Tricep rope - Single | 8 |
| Tricep rope -Double | 8 |
| Lat pulldown | 5 sets |
| Straight bar | 4 |
| Band Handles | 10 pairs |
| Fitness all in one training bench | 5 |
| 45 Degree Back Extension | 4 |
| Benches | 22 |
| Dumbbell racks 99in | 6 |
| Dumbbells as per spec | 6 sets |
| Vertical Rack tower | 4 |
| Sprint Track |  |
| 13 m X 3 lanes | 1 |
| 11m X 1 Lane | 1 |
| Boxing Studio |  |
| Boxing/suspension rig | 1 |
| Heavy duty Boxing Bags- Long | 5 |
| Heavy duty Boxing Bags- Short | 5 |
| Heavy Duty Teardrop Bag | 3 |

## Lot 3 - Functional Zone Specifications

| It is expected that racks will be designed to a superior level suitable heavy commercial use. Please supply detail / evidence for each element below. |  |  |
| --- | --- | --- |
| Stand alone Wall Training 1 | 1 | 12 Metres- Floor fixed in one or two units |
|  |  | Wall Modular solutions for Versatile/ Functional training |
| Stand alone Wall Training 2 | 1 | 7-10 Metres Wall or floor fixed in one or two units |
|  |  | Wall Modular solutions for Versatile/ Functional training |
| Cable station/¦Tower | 1 | 4 Cable bay to include 2 Adjustable pulley, Lat pull, Row |
| Dual Adjustable Pulley | 3 | >50kg per Column |
| Storage to contain |  |  |
| Accessory dips handles | 2 | Strong durable commercial grade |
| Accessory split squat roller | 2 | Strong durable commercial grade |
| Kettle Bells | 4 sets | 4-20kg |
| Sand Bags or equivalent | 2 set | 10/15/20kg |
| Resistance bands | 6 sets | Light to Hard with Handles |
| Medicine Balls | 4 sets | Leather Over size ( 4/6/9kg ) |
| Slam Ball | 4 sets | High impact slam balls, durable Rubber, bounce resistance 22-24cm in diameters 5/8kg |
| Plyobox 8 | 2 | Stackable/velco |
| Plyobox 12 | 2 | Stackable/velco |
| Plyobox 18 | 2 | Stackable/velco |
| Plyobox 24 | 2 | Stackable/velco |
| BOSU balance trainer or equivalent | 5 | Both sides up, Durable, Thick PVC cut resistance, nonslip, 58cm 150kg+ load |
| Heavy duty Boxing Bag | 5 | 30-32kg Bag |
| Battle ropes | 5 | Strong durable commercial grade |
| Suspension straps | 6 | Strong durable commercial grade |
| Olympic Ring attachments | 2 | Pair |
| Sleds | 3 | Strong durable commercial grade |
| Weighted plates for sleds | 3 | 3x 20kg +10kg x4 |
| Cable Accessories |  |  |
| Tricep rope | 8 | Double arm |
| Tricep rope | 8 | Single Arm |
| Lat pulldown | 5 set | Variety of sizes - nonslip complete rubber grip Narrow, Medium, Wide |
| Straight bar | 4 |  |
| Band Handles | 10 pairs | Strong Nylon, heavy duty |
| Fitness all in one training bench | 5 | Full kit out- Dumbbells, Kettlebell, Med ball, resistance bands |
| Back Extension | 4 | 45 degree |
| Benches | 22 | adjustable |
| Dumbbell racks 99in | 6 | 2-3 tier, 10 pair rack (holds up to 60 pairs) |
| Dumbbells | 6 sets | Polyutherane 12kg-30kg - 2 kg increments ( 60pairs) |
| Vertical Rack tower | 4 | 1-10kg in 1 kg increments with Dumbbells- Polyutherane |
| Sprint Track | 1 | 13metre by 3 lanes Integrated in original black Matting – Logo TBC |
|  | 1 | 11 metre by 1 lane Integrated in Black matting - Logo TBC |
| Standalone Functional Wall Modular Training |  | It is expected that Modular frames will be designed to a superior level suitable for heavy commercial use. Please supply detail/evidence for each element below: |
| Strong & Robust Design |  | Powder-coated steel with high-density composite panels |
| Footprint |  | It is accepted premium commercial units will have a larger footprint than lighter semi-commercial units but we are seeking space conscious premium design as a priority. Please detail any space saving features within proposed units. |
| Sustainability |  | Sustainable procurement is important and key factors considered include sustainable packaging, energy saving/reduction gym equipment and reduced carbon footprint. Please outline how your products deliver sustainability. |
| Accessibility |  | Proposed items should include accessible design and features suitable for a diverse range of users of different sizes and abilities while also being inclusive to those with disabilities. |
| Multi-User Capability |  | Supports individual, small group, and coached sessions. |
| Stand alone multi functional rigs |  | Unit 1 should be no more than 12m long, 3m wide & 2.5m high in one or two units |
|  |  | Should include 2 x DAP, Boxing bag x 3, Suspension Trainer x 3 attachments |
|  |  | Storage holder, for Kettlebells, medicine & slam balls, Battle ropes, foam rollers etc. |
|  |  | Cable accessory holder |
|  |  | Stretch bar and monkey/ pull up bars |
|  |  | Anchor points for battle ropes |
|  |  | Accessory support for Dips, Split squat roller |
|  |  | Option for Support bar to hold screen/ TV for advanced training solutions |
|  |  | Unit 2 should be no more than 8m long, 2m wide & 2.5m high in one or two units |
|  |  | Should include 2 x DAP, Boxing bag x 2, Suspension Trainer x 2 attachments |
|  |  | Storage holder, for Kettlebells, medicine & slam balls, Battle ropes, foam rollers etc. |
|  |  | Cable accessory holder |
|  |  | Stretch bar and monkey/ pull up bars to be included |
|  |  | Include anchor points for battle ropes |
|  |  | Accessory support for Dips, Split squat roller |
| Dual Adjustable Pulley |  |  |
|  |  | Commercial grade |
|  |  | Weighted stack < 50kg per Column |
|  |  | Single-hand adjustment mechanism |
|  |  | Anti-snap cable rating |
|  |  | Built in cable accessory storage |
|  |  | Pull up bar. |
| Cable station/¦Tower |  |  |
|  |  | 4 Bay Freestanding tower to include: |
|  |  | 2 x Adjustable Pulley |
|  |  | 1 x Pull Down with 1:1 cable ratio, adjustable knee pads, standard Lat bar |
|  |  | 1 x Row with 1:1 cable ratio, angled footpads , standard row handle |
| Accessory support- Roller & dip |  |  |
|  |  | Frame Material Powder-coated steel |
|  |  | Roller Material-High-density EVA / PU composite |
|  |  | Removable part from frame |
|  |  | Roller- to be used for single leg work |
|  |  | Dip - Handle material for advanced grip |
| Kettlebells |  |  |
|  |  | Chrome Grip |
|  |  | Rubber Bell |
|  |  | Colour coded for easy identification |
| Sand Bags |  |  |
|  |  | Heavy Duty Design with wipe clean design |
|  |  | Multi Grip handles including top handle |
| Resistance Bands |  |  |
|  |  | Commercial-grade resistance bands designed for strength training, mobility work, rehabilitation, and functional conditioning. |
|  |  | Bands are manufactured using high-elasticity latex or synthetic elastomer |
|  |  | Power/Strength Bands (thicker loop format) |
| Medicine Balls |  | Rubber weighted balls |
|  |  | Premium rubber surface providing good grip |
|  |  | Colour coded for easy identification |
| Slam Ball |  |  |
|  |  | High impact slam balls, durable Rubber, bounce resistance 22-24cm in diameters 5/8kg |
|  |  | Durable Rubber, bounce resistance 22-24cm in diameters |
| Stackable Soft Plyometric Boxes |  |  |
|  |  | High density foam construction with non-slip vinyl cover |
| BOSU balance trainer or equivalent |  |  |
|  |  | Must be rated for commercial use |
|  |  | Anti-skid base that will not mark floors |
|  |  | Both sides up, Durable, Thick PVC cut resistance, non slip, 58cm 150kg+ load |
| Heavy Duty Boxing Bags |  |  |
|  |  | Leather Punch Bag |
|  |  | Between 30 & 32KG |
|  |  | 4-6 hook chain |
|  |  | Must be rated for commercial use |
| Battle ropes |  |  |
|  |  | Heat-sealed ends |
|  |  | 100% Poly Dacron (commercial standard) |
|  |  | 12–15 m (40–50 ft) length |
|  |  | 10–16 kg in weight |
| TRX Pro Suspension Trainers |  |  |
|  |  | Must be rated for commercial use |
|  |  | Adjustable straps |
|  |  | Anchor clip for fixed positioning |
| Olympic Rings |  |  |
|  |  | Adjustable Length |
|  |  | Plastic durable wipe clean handle |
|  |  | Must be rated for commercial use |
| Sled |  |  |
|  |  | Heavy-duty steel frame |
|  |  | Powder-coated finish (matte black standard) |
|  |  | Welded structural joints |
|  |  | Reinforced vertical plate post |
|  |  | Low-friction glide surface |
| Cable Accessories |  |  |
|  |  | Must be rated for heavy duty commercial use |
| Studio / HIIT Benches |  |  |
|  |  | Adjustable (to 80 degrees) soft top |
|  |  | Black frame & upholstery with lockable wheels for transport. |
|  |  | Integrated dumbbell storage (5 pairs), kettlebell storage (2 units) and medicine/slam ball (1) as well as small unit storage for skipping ropes and bands |
|  |  | Should include 5 x Polyurethane Dumbbell Pairs as follows: 2.5KG, 5KG, 7.5KG, 10KG & 12.5KG |
|  |  | Should include a battle rope attachment point |
| Back Extension |  |  |
|  |  | 45 Degrees with adjustable angled pad |
|  |  | Lower roller pads |
|  |  | Anti-slip footplates |
| Adjustable Benches |  |  |
|  |  | Seat adjustment should provide 0, 15 & 30 Degrees |
|  |  | Back pad adjustment should provide 0, 15,30, 45, 60 & 75 Degrees |
|  |  | Should include wheels and a handle for easy manoeuvring |
| Free weight/Dumbbells |  |  |
|  |  | Chrome knurled handle for easy grip |
|  |  | Easily identifiable denomination |
|  |  | Custom logo included |
|  |  | Polyutherane |
| Dumbbell Racks |  |  |
|  |  | To store the above 10 pairs per rack. Should include saddles for correct dumbbell position. Black in colour* |
|  |  | PLEASE NOTE Dumbbells will not be co-located so racks must suitably situate sets as outlined above |
| Vertical Rack & Dumbbells |  |  |
|  |  | 1KG Increment Dumbbell Sets, 1KG - 10KG & Vertical Racks to store 10 pairs |
| Sprint Track |  |  |
|  |  | Suitable for sprinting, sled push/pull, strength & conditioning work. Shock-absorbent backing and durable under high foot traffic. |
|  |  | Turf colour * TBC. Contrasting inlaid line markings. Mardyke Arena logo incorporated |
|  |  | Non-slip, high-grip surface suitable for change-of-direction work. Low maintenance. Minimum 5-year manufacturer warranty. |
|  |  | Fully bonded indoor installation within floor matting, with professional seam finishing and flush transition edges. |
| Boxing, & suspension training Rig Frame |  | Bespoke ceiling beam suspended floating rig fixed to structural beams, or Free-standing structural steel rig where ceiling fixing is not suitable. Site survey with structural assessment, load testing and certification |
|  |  | The preferred solution is a space-efficient modular system incorporating up to 8-10 bags with: Adjustable pulley/track system to allow bags to retract or tidy away when not in use Quick and easy conversion between boxing bags and suspension training attachments Modular layout allowing future expansion or reconfiguration Positive locking system to secure bags during use Commercial durability for high-frequency class environments Powder coated, corrosion resistant fittings and commercial rated pulleys/bearings. The studio footprint ranges from 94 m² to 154 m² with flexible layouts required |
| Heavy duty Long Bag |  | Heavy-duty engineered synthetic leather or equivalent commercial-grade outer shell Reinforced stitched seams throughout Reinforced hanging points with minimum 4-point connection Swivel attachment to reduce twisting during use Compatible with retractable rig systems Bag weights 30-50kg Length 1500-1800mm |
| Heavy duty short Bag |  | Heavy-duty engineered synthetic leather or equivalent commercial-grade outer shell Reinforced stitched seams throughout Reinforced hanging points with minimum 4-point connection Swivel attachment to reduce twisting during use Compatible with retractable rig systems Bag weights 30-50kg Length 900-1200mm |
| Uppercut/tear bag |  | Heavy-duty engineered synthetic leather or equivalent commercial-grade outer shell Reinforced stitched seams throughout Reinforced hanging points to include swivel attachment to reduce twisting and multidirectional striking Compatible with retractable rig systems Bag weights 25-45kg Length 700-1000mm |

# LOT 4 - Premium Performance, Recovery & Rehab

| Equipment Name | Quantity |
| --- | --- |
| Treadmill | 6 |
| Recumbent Bike | 1 |
| Upright Bike | 4 |
| Elliptical Trainer | 4 |
| Fascia Stimulator | 1 |
| Glute Stretch frame | 1 |
| Hamstring and Hip Flexor stretch | 1 |
| Multi stretch cage ladder | 1 |

## Lot 4 – Premium Performance, Rehab & Recovery Specification

| Treadmill |  |  |
| --- | --- | --- |
|  |  | Advanced biomechanics and personalization. |
|  |  | allows users to adjust deck firmness for comfort or performance |
|  |  | advanced performance metrics such as cadence, stride length, and left/right balance. |
|  |  | Precision sensors capture real-time gait data, on-demand coaching, entertainment, and immersive experiences. |
|  |  | < 24” HD console delivers on-demand coaching, entertainment, and immersive experiences. |
|  |  | Technology capability – Open API to integrate with Leisure Management systems |
| Upright Bike |  |  |
|  |  | Advanced biomechanics and personalization. |
|  |  | Hybrid upright and indoor cycling bike combining performance riding with immersive digital content. |
|  |  | Supports both in-saddle and out-of-saddle riding, with high adjustability for handlebars and console positioning |
|  |  | <24” HD console delivers on-demand coaching, entertainment, and immersive experiences. |
| Recumbent Bike |  |  |
|  |  | Advanced biomechanics support for a comfortable yet engaging workout experience |
|  |  | low step-through design |
|  |  | assisted reclining seat for ease of use and premium comfort. |
|  |  | <24” HD console delivers on-demand coaching, entertainment, and immersive experiences. |
| Elliptical trainer |  |  |
|  |  | Focused on natural movement and muscle engagement. |
|  |  | Features to include interactive muscle mapping, |
|  |  | Incline mechanism maintains a consistent ellipse across all incline levels |
|  |  | <24” HD console delivers on-demand coaching, entertainment, and immersive experiences. |
| Stretch - Fascia stimulator or equivalent approved |  | Commercial-grade fascia stimulation machine suitable for high-usage gym environment, |
|  |  | Adjustable intensity settings, programmable modes, digital controls, and quiet operation. |
|  |  | Clear user operating instructions displayed on unit |
|  |  | Maximum user weight capacity to meet commercial gym standards |
| Glute Stretch frame or equivalent approved |  | Commercial-grade glute stretch machine designed to improve hip mobility, flexibility, and lower-body recovery within a high-usage gym environment. |
|  |  | Heavy-duty steel frame, smooth adjustable positioning mechanism, ergonomic padding |
|  |  | suitable for a wide range of user heights and abilities. |
|  |  | Appropriate maximum user weight capacity for commercial use. |
|  |  | Clear user operating instructions displayed on unit |
| Hamstring and Hip Flexor stretch or equivalent approved |  | Commercial grade stretch machine designed to target hamstrings and hip flexors, |
|  |  | Supporting mobility, flexibility, and injury prevention within a high-usage gym environment. |
|  |  | Heavy-duty steel frame, smooth and controlled adjustment mechanism, ergonomic padding, and adjustable settings. stable non-slip base |
|  |  | Clear user instruction signage, |
|  |  | Suitable for a wide range of user heights and flexibility levels. |
|  |  | Appropriate maximum user weight capacity for commercial use. |
| Multi Stretch Unit or equivalent approved |  | Commercial-grade multi-stretch unit designed to target multiple muscle groups (e.g., lower back, glutes, hamstrings, hip flexors) |
|  |  | suitable for high-usage gym environments. |
|  |  | Heavy-duty steel frame, smooth adjustable mechanisms, ergonomic padding, and adaptable settings to accommodate a wide range of users |
|  |  | Clear user instruction signage |

# Lot 5 – Selectorised

| Equipment Name | Quantity |
| --- | --- |
| Leg Curl 95kg+ Weight Stack | 2 |
| Prone Curl 65kg+ | 1 |
| Leg Extension 95kg+ | 2 |
| Leg Press 190kg+ | 2 |
| Glute/Hip Extension 95kg+ | 1 |
| Rear Deltoid/Fly Dual 65kg+ | 1 |
| Seated Row 95kg + weight stack , Independent Arms, Multiple Grips | 2 |
| Chest press 100kg+ wight stack, Ind Arms & Multiple grips | 2 |
| Shoulder Press 65kg+ | 2 |
| Lat Pull down 100kg+ Weight stack, Roller pads for stability, Straight bar design | 2 |
| Assisted Pull/Chin 80kg + Adjustable for kneeling pad with assistance or body weight, multi angled handles | 2 |
| Adduction 70kg+ | 1 |
| Abduction 70kg | 1 |
| Add/Abduction Dual 75kg+ | 1 |
| Back Extension 65kg | 2 |
| Ab Crunch 95kg | 2 |
| Torso Rotation 65kg | 2 |
| Leg Curl- advanced biomechanical | 1 |
| Leg Extention - advanced biomechanical | 1 |
| Seated Row - advanced biomechanical | 1 |
| Chest Press - advanced biomechanical | 1 |
| Shoulder press -advanced biomechanical | 1 |

## Lot 5 - Sectorised Equipment Specification

|  | It is expected all selectorised equipment will have the following features, please supply details of how proposed units match these, highlighting any unit specific omissions. |
| --- | --- |
| Strong & Robust Design | Premium commercial line to be proposed, suitable for a heavy use facility. CE Certifications should be supplied. |
| Colour & Uniformity | Frames, Upholstery TBD (with hi visibility adjuster handles/knobs etc) supplying continuity of brand/user experience across strength and cardiovascular units. |
| Instruction/Guidance Placards | With visual guidance cues |
| Instruction QR Codes | Link to exercise guidance and video tutorials |
| Weight Stacks | Should be enclosed with a 2.5kg incremental weight plate included for gradual loading. |
| Technology | Integrated coaching via on unit screen to prescribe and show workout programmes, provide workout feedback and adjustments. Open API to integrate with Leisure Management Systems |
| 1 Rep Max Testing | The equipment should have a built in 1RM testing feature |
| Data Insights | The integrated screen should provide usage insights for management |
| Accessories Storage | To prevent towels, water bottles, phones and keys being left on the gym floor |
| Accessibility | High Visibility adjustors and weight stack pins |
|  | Ergonomic adjustments available from workout position |
|  | Ergonomic back and seat pads with adjustable roller pads where included on a unit |
|  | Please detail any additional accessible design features |
| Requirements | Comments |
| Leg Curl | 95KG+ weight Stack |
| Prone Curl | 65kg + |
| Leg Extension | 95kg + |
| Leg Press | 190kg+ |
| Glute/Hip Extension | 95kg + |
| Rear Deltoid/Fly Dual | 65kg+ |
| Seated Row | 95kg + weight stack , Independent Arms, Multiple Grips |
| Chest press | 100kg+ weight stack , Independent Arms, Multiple Grips |
| Shoulder Press | 65kg + |
| Lat Pulldown | 100kg+ Weight stack, Roller pads for stability, Straight bar design |
| Assisted Pull/Chin | 80kg + Adjustable for kneeling pad with assistance or body weight, multi angled handles |
| Adduction | 70kg+ |
| Abduction | 70kg + |
| Add/Abduction Dual | 75kg + |
| Back Extension | 65kg+ |
| Ab Crunch | 95KG + |
| Torso Rotation | 65kg+ |
| Leg Curl | - AI-driven or adaptive resistance technology - Visual or digital prompts to guide correct technique - Equipment shall include or support integrated digital technology for user interaction - Ability to provide variable or adaptive resistance (preferred) - Equipment shall provide natural, biomechanically correct movement patterns - Independent or converging/diverging movement arms where appropriate - Adjustable seating, pads, and range of motion to suit a wide user demographic - Compatibility with external fitness management platforms or apps |

# Design & Décor

Requirements from Specification

Concept design proposals and mood boards

Detailed layout drawings (CAD)

Project programme and delivery timeline

Full cost breakdown (design, supply, installation)

Evidence of relevant project experience

Equipment specifications and technical datasheets