BEAUMONT HOSPITAL FALLS GROUP

GUIDANCE ON

BUILD DESIGN

FOR

CONSIDERATION TO ASSIST IN FALLS PREVENTION MANAGEMENT

February 2016

These guidelines are for reference purposes only and a working party made up of the relevant stakeholders should be created prior to building work or upgrades of clinical areas, including wards and other patient treatment areas in order to effectively make design plans appropriate for the patient group and involving relevant staff members.

CONTENTS

Page

- Purpose								3

- Scope									3

- Involvement of relevant stakeholders			             3

- Approach/Entrances							4

- Internal Environments					             4

- Ward Areas								5

- Bathrooms								6

- Visual Considerations						             6

- Patient Equipment Considerations                                                          7

10.0       References			                                 			8

- Purpose

To provide simple guidelines to inform relevant stakeholders regarding key design considerations when redesigning or refurbishing of all areas.

- Scope

These guidelines apply to all area upgrades, in Beaumont Hospital, RCNU or St Joseph’s rehabilitation unit and day hospital areas.

- Involvement of relevant stakeholders:

3.1 A working party including the following stakeholders for advice in the planning stages of design considerations should include:

- Nominee from the Head of IQS with Health and Safety Specialist

- Directorate Nurse Manager/Clinical Nurse Manager

- Technical Services Department Representative

- Infection Prevention & Control Representative

- Occupational Therapist

- Physiotherapist

- Healthcare Assistant

- Portering

- Security

Figure 1 Factors that influence space layout design in healthcare facilities.

(Zhao, Mourshed &Wright, 2009)

4.0 Approaches/Entrances:

4.1 Convenient dedicated parking located close to entrance

4.2 Dished kerbs and ramped access with appropriate surfaces i.e. flat, smooth and slip resistant e.g. ribbed concrete or tarmacadam

4.3 Surfaces should also be without gravel/uneven paving with avoidance of use of bollards, litter bins and thorny plants

4.4 Whereby flat access is not possible, there should be gradual changes in level and advance tactile warnings of change in gradient. General recommendation for gradient as 1:20 with an absolute maximum of 1:15

4.5 If steps are required, consider provision of different texture surfaces top and bottom and clear visual identification of each step

4.6 Adequate lighting should be sufficient to cover the area travelled from outdoors to indoors

5.0 Internal Environment:

5.1 Entrances should be clearly identifiable.  Reception facilities should be approachable and clearly marked and where not permanently staffed, clear instructions to visitors should be provided, with an explanation of how to access further assistance.

5.2 All entrances, corridors and common activity areas should be accessible.

5.3 Corridors should be wide enough for two wheelchairs to pass with ease.

5.4 Widths and turning circle requirements for wheelchair users are given in HBN 40 – ‘Common activity spaces’ Volume 4 ‘Circulation areas’, and in BS 8300.

The lobby area should have adequate space for turning and transfer. Provision of storage space for a wheelchair would be beneficial

5.5 Doors should open outwards.  Where this is not possible, as the door may cause a hazard, mechanisms may be fitted which allow staff to open the door outwards in an emergency.  All door locking mechanisms should allow staff to open the door from the outside in an emergency.

5.6 Clear opening width of a door should be 900mm or 750mm at an absolute minimum

5.7 Thresholds should be flush, avoid door saddles

5.8 Ensure coloured marking strips are provided on a glass door

5.9 Non-ground-floor facilities require lifts. Lifts, directions and controls should all be accessible. Visual and audible information is a requirement for all lifts serving more than three storeys.

6.0 Ward areas

6.1 Radiators should be clearly visible and be either covered or offer only low surface temperatures to avoid burns.

6.2 Facilities should be located at the nursing station to allow location of patients using alarm cord/button systems. Call points should be available in all places where patients are alone, including WCs, bathrooms and single rooms.

6.3 Staff call points should also be considered in spaces where staff consult with patients e.g. out patient department, therapy treatment areas

6.4 Further guidance is found in HTM 2015 – ‘Bedhead services’. Some facilities may also require fast-response “bleep” facilities or personal alarm calls.

6.5 The use of assistive technology, such as a fall detector attached to clothes or a mobile call unit to alert staff in the event of a fall, may be considered appropriate for certain patient groups, see FAST.

6.6 Consideration should be given to storage facilities for specific items potentially requiring additional space in the ward bay area e.g. space for electric wheelchairs (which require more space than manual wheelchairs for storage and recharging batteries) and space for motorised buggies that may require dedicated parking/recharging bays.

7.0 Bathrooms

7.1 Grab rails should be provided in all WCs – see BS 8300; consider coloured grab rails for visual aid.

7.2 As patients may have individual requirements e.g. loss of use of one side of the body, toilet roll holders and/or grab rails need to be provided to both sides of the WC.

7.3 Contrasting colour, between sanitary ware and wall and floor finishes is essential to aid with visual discrimination. This should be limited to two colour choices.

7.4 Thermostatic controlled showers and taps are recommended

7.5 Floor surfaces should be slip resistant

7.6 Turning circles for all moving equipment:

Manual wheelchair: 1,500mm

Powered wheelchair: 1,800mm

Hoist-mobile: 2,000mm

7.7 A level access shower is the preferred type of showering unit. It is area is an area free of a threshold , with a larger than standard drainage point. The gradient to the drainage point must not exceed 1 : 40. The drain can be sited in the centre or corner, depending on what seating aids are planned to be used.

7.8 Grab rails (consider coloured) should also be provided alongside baths, WCs, beds and in other suitable locations.

7.9 Unobserved areas, such as en-suite bathrooms, should be fitted with suspended alarm cords to allow users to call for help in the event of a fall.

7.10 Space alongside accessible baths should be large enough for patients to be offered assistance from both sides and for hoists to be positioned.

7.11 All taps should be set to non-scalding temperatures.

7.12 Grab rails should be fitted in bathrooms and shower areas.  Grab rails should be located adjacent to shower seats.

8.0 Visual Considerations:

8.1 Effective signage should be employed, preferably with the word and picture associated.

8.2 Warm, calming colours should be chosen for all areas. Softening features such as indoor plants are popular, but local infection control officers should be consulted prior to use as these may represent an infection risk.

8.3 Floors should be single-level and made from smooth, easily cleanable, non-slip material.  Where changes in floor level are necessary, grab rails should be provided and a clear demarcation indicated on the floor.

8.4 Excessive patterning on floors should be avoided as it may cause downward visual focus and precipitate forward falls. People with visual impairment may also interpret “busy” patterns as obstacles.  Shiny floors should also be avoided, as they may be seen as hazardous and reduce confidence. Non-reflective finishes are therefore generally preferable. See HTM 61 – ‘Flooring’.

8.5 Stairs and corridors require easy-to-grip handrails on both walls.

8.6 Doorways and lift entrances should have smooth and congruent joints to avoid tripping. Access to lifts should be clearly marked. HBN 40.

- Patient Equipment Considerations

9.1 Availability of appropriate footwear i.e. soles with adequate grip, appropriately secured with laces/velcro. Footwear with heels or backless shoes should be avoided.

9.2 Assessment for and availability of appropriate walking aids at ward level should be considered for all patients and kept within reach of the patient.

- References

Association of Occupational Therapists of Ireland, Housing Design Guidelines for Occupational Therapists, November 2007, Abbeyset Print and Design Ltd.

Building Note 00-01, General Design Guidance for Healthcare Buildings, March 2014, Department of Health UK (This document is available from the website at www.gov.uk/government/collections/ health-building-notes-core-elements)

Calkins, M.P, Biddle, S. & Biesan O. , Contribution of the Designed Environment to Fall Risk in Hospitals, 2012, IDEAS Institute

HBN 37, In-patient facilities for older people, NHS, TSO, 2005.

Zhao, Y., Mourshed, M. and Wright, J. (2009) Factors influencing the design of spatial layouts in healthcare buildings. In: Dainty, A. (Ed) Procs 25th Annual ARCOM Conference, 7-9 September 2009, Nottingham, UK, Association of Researchers in Construction Management, 1041-49.

Useful Documents / Links

Slips and Trips advise from NHS:

http://www.hse.gov.uk/slips/

http://www.hse.gov.uk/pubns/ck4.pdf