Works Requirement: Preliminaries for Pyrite Remediation Works at Notional Project in North Dublin for Housing and Sustainable Communities Agency (The Housing Agency)

|  | # PARTIES 1.01 NAMES, ADDRESSES AND TELEPHONE NUMBERS: EMPLOYER: Housing and Sustainable Communities Agency (Housing Agency), 53 Mount Street Upper Dublin 2. Telephone: +353 (0)1 6564100 For the attention of: Martin Lynch, Programme Manager EMPLOYERS REPRESENTATIVE: Design Professional To be appointed by Employer STRUCTURAL/CIVIL ENGINEER: To be appointed by Design Professional MECHANICAL AND ELECTRICAL: To be appointed by ENGINEER: Design Professional PSDP: To be appointed by Design Professional QUANTITY SURVEYOR: To be appointed by Design Professional - DESCRIPTION OF THE SITE - LOCATION OF THE SITE: This notional Project of 17 Units is located in North County Dublin. - BOUNDARIES OF THE WORKS: are as shown on the /Site Plan The Contractor should Assume that Adjacent Car Park within 50m of Works can be used for Site Compound, storage. - ACCESS TO THE SITE: Access to the site is by Public Road without restrictions. - WORKING AREAS: for the Contractor will be generally confined to those as indicated on Site Plan Drawings and Site Compound - SEQUENCING OF THE WORK: the sequencing of the works Must be in accordance with IS 398 – 2013. However, the sequencing of houses to be completed must be in consultation with the Design Professional with consideration to prevention of creating unrestrained retaining substructures or the like - EXISTING SERVICES: Existing underground services – there are standard Underground services entering each house, including: Gas, Water, Electricity, Telephone, TV, Drainage Foul and Storm. - ADJACENT BUILDINGS: The Contractor must protect adjacent properties from damage and/or interference due to the works. - SITE VISIT: As this is a Notional Project the Contractor should presume they have visited the site and found conditions to be normative. - SITE SECURITY: The contractor is advised to take into account the possibility of children and general public in the locality and therefore protect the site accordingly. - MAINTENACE OF THE ROADS: The Contractor is to ensure that vehicular and pedestrian access is maintained along all access roads. This is subject to requirements that may be imposed by the Gardaí and the Local Authority. The Contractor must ensure that roads surrounding the site are kept clear, to allow traffic to move freely, that all roads and footpath are swept frequently and no mud, debris or the like is allowed to accumulate at any time. The Contractor shall provide for cleaning all roads surrounding the site, to the satisfaction of the ER, for the duration of the contract. ## DESCRIPTION OF THE WORKS - THE WORKS COMPRISE the Pyrite Remediation Works to Existing Occupied Houses, Notional Project Tender Includes 17 2 storey houses in 2 Blocks. Works as per attached Works Requirements - TIME FOR COMPLETION: the works are to be completed in accordance with the programme, No House Shall be Vacated for more than 12 Weeks calendar weeks. Anticipated Forty Weeks from the date of Letter of Acceptance. Phased Handover is envisaged for each House as they become completed, see Schedule 1 D. - FORM OF CONTRACT - The form of agreement and conditions of contract will be the “Public Works Contract for Minor Building and Civil Engineering Works Designed by the Employer”, published by the Department of Public Expenditure, Infrastructure, Public Service Reform and Digitalisation, dated 25th November 2025 PW-CF5 v2.9 4.02 SCHEDULE: PART 1 The completed schedule: Part 1 of the contract is set out in the attached tender documentation. Provide for all costs in complying in all respects with the Schedule - DRAWINGS - CONTRACT DRAWINGS are as included with the Works Requirements. - PRICING DOCUMENT DRAWINGS: the general drawings from which the Pricing Document has been prepared are as included with the Works Requirements. - SPECIFICATION/PREAMBLES TO TENDER DOCUMENTS - APPROVAL and works derived therefrom, means the approval in writing of the Employers Representative. - ABBREVIATIONS: the following abbreviations have been used in the description of items in the Specification. I.S. for "Irish Standard Specification" B.S. for "British Standard Specification" B.S.C.P. for "British Standard Code of Practice". Reference to Irish and British Standards and Codes of Practice do not give year of issue or date of amendment slips. The published version current at designated date will apply. The Specification shall take precedence over the Standards and Codes of Practice which are referred to where those documents are at variance. - OR EQUIVALENT: means that commodities of different manufacture may be substituted if prior approval has been obtained. The rates or prices will be held to be based on the commodity or commodities specified. - PROPRIETARY NAMES: the phrase ‘or other approved' shall be deemed included whenever commodities are described by proprietary name. In every case in the Specification where materials, appliances or fittings of special design, manufacturers or agents are given, it should be clearly understood that such references are inserted solely for the guidance of tendering contractors with respect to the nature, performance and quality of the articles or services required. - MANUFACTURER'S RECOMMENDATIONS: means the manufacturers recommendations or instructions, printed or in writing and current at the designated date. Note - AND THE LIKE: means analogous work to that described. Note - SINGULAR/PLURAL: Words imparting the singular only also include the plural and vice versa when the context requires. GENERAL - QUALIFICATIONS OF THE RULES OF THE A.R.M.5 The Department of Public Expenditure, Infrastructure, Public Service Reform and Digitalisation approved use of Agreed Rules of Measurement 5 for use with Public Works Contracts. NOTE PRICING DOCUMENT NOT IN ACCORDANCE WITH ARM 6.09 PRICING DOCUMENT METHOD OF MEASUREMENT: The Pricing Document comprises of this Preliminaries Summary, General Summary, Schedule of Works for Typical House Type A, Typical House Type B, Typical House Type C. The tendering contractor must provide within one weeks notice a detailed breakdown of the works priced in the summary, together with description, quantity, rates and totals for the works. - STATUTORY/GENERAL OBLIGATIONS GENERALLY - STATUTORY OBLIGATIONS: comply with statutory obligations and the regulations of any local authority public service or statutory undertaking relating to the execution of the works. - WASTE MANAGEMENT: comply with statutory obligations and regulations concerning waste management, having particular regard to the Waste Management (Amended) Act, 2001 or as amended, current at the date of tender - FEES: the cost of fees payable to Utility Providers for electrical, connections or the like will be paid directly by the Employer. and will not form part of the building contract. - FEES AND CHARGES: Include here for all customs and excise duties and import licence fees and all other duties payable at the designated date. - WORKING RULE AGREEMENT: allow for all payments in connection with obligations arising under work rule agreements. Item - GARDA SIOCHANA REGULATIONS: ascertain and comply with Garda Siochana regulations affecting the execution of the works. Item - DISCOVERIES: all fossils, coins, articles, structures and other remains or things Discovered on the site of the Works shall be the absolute property of the House Owner and the Contractor shall take all precautions to prevent his workmen or any other persons from removing or damaging any such articles or things and shall immediately upon discovery thereof and before removal acquaint the ER of such discovery and except insofar as is provided under his contract for the removal and disposal of rubble, rubbish, demolished material etc., - SAFETY, HEALTH AND WELFARE: the Contractor shall include for complying in full with enactments, regulations and working rules relating to the safety, health and welfare of work people. Item In particular the Contractor shall include for complying in full with the requirements of the Safety, Health and Welfare at Work Act 2005 Safety, Health and Welfare at Work [Construction] Regulations: 2013 and any proposed changes or amendments to the construction regulations during the project The Contractor’s attention is particularly drawn to the requirements under the Safety, Health and Welfare at Work [Construction] Regulations: 2013 The Contractor shall include for complying in full, with the requirements of the Safety, Health and Welfare at Work [Construction] Regulations: 2006. The term "temporary works" shall mean all temporary works of every kind required in or about the construction, completion, and maintenance of the works; it shall include temporary works to support and ensure the stability of the partially completed permanent works and of adjoining structures and lands, as well as such matters as trench shoring, scaffolding, propping, working platforms, gangways, access stairs and landings. The Main Contractor shall (for Complex Projects BUT NOT For THIS PROJECT) appoint an approved, competent Temporary Works Designer who shall take overall design responsibility for the stability of the partially completed permanent works and of adjoining structures and lands during the envisaged construction process and, further, the main Contractor shall employ a suitably qualified site engineer; both shall liaise with each other and with the Project Supervisors, all in accordance with the HSA Guidelines to Safety, Health and Welfare at Work (Construction) Regulations 2013. Subject to the ERs consent, (a) on small routine projects the site engineer need not be permanently on site; and/or (b) it may be permissible for one person to fulfil both of these roles. The Contractor shall in consideration of the acceptance of his tender and by way of Collateral Agreement to the Main Contract, (See Model Form MF 1.10 – Form of Appointment of Project Supervisor Construction Stage only) provide the services of Project Supervisor Construction Stage within the meaning of the Safety, Health and Welfare at Work (Construction) Regulations 2013. The cost of the provision of this service shall be deemed to be included in the Contract Sum. The Contractor shall in respect of all person and site activities provide for and ensure compliance with all Safety, Health and Welfare Legislation in connection with the works. The Contractor shall ensure that safe practices and methods of work are adopted at all times. The contractor in that capacity or in his capacity as Project Supervisor Construction Stage shall notify the Architect in writing prior to the submission of his tender if he considers any requirement relating to the time for carrying out the works or any part of the works or the design of the work or specification of materials render work methods or procedures necessarily hazardous or in any way prevent the works from being safely carried out. The Contractor shall bear all costs arising out of his failure to give notice of foreseeable matters in this regard. The Contractor shall include for procuring that the insurance cover required to be obtained and maintained by the Contractor pursuant to Clause 3 of the main contract shall provide indemnity in respect of any claim for bodily injury or property damage which the contractor may incur by reason of the performance of the functions and duties of the Project Supervisor for the Construction Stage of the project. The Main Contractor shall coordinate arrangements between all contractors to ensure the provision, in writing, of such information as is necessary for the Project Supervisor Design Process to complete the safety file, and shall monitor the implementation of the arrangements and take any necessary corrective action as set out in the Regulations. The Main Contractor shall provide in writing to the Project Supervisor Design Process all relevant information necessary for the Project Supervisor Design Process to complete the safety file. All contractors must promptly provide the Project Supervisor Construction Stage, in writing, with all relevant information necessary to prepare the safety file. NOTE - If not stated elsewhere the minimum information required from the Main Contractor for the completion of the Safety File includes: (a) Details of services, plant, and the project equipment, which comprise part of the structure, from specialist supply and installation contractors, as well as from statutory bodies and local authorities where appropriate. (b) Manuals, and where appropriate certificates, produced by the specialist contractors and suppliers which outline operating and maintenance procedures and schedules for all plant and equipment and (c) Accurate site survey drawings of the as- constructed location and layout of all utilities and services, including drainage, water mains. The Contractor shall include for a competent person [nominated by the Contractor and to whom the Employer does not object] being appointed by the Employer to act as and carry out the duties of Project Supervisor for the duration of the Construction Stage as set out in the Safety, Health and Welfare at Work [Construction] Regulations: 2013. The Contractor shall include for all costs for the provision of the services of Project Supervisor for the Construction Stage pursuant to Clauses 16 to 23 of the Regulations. The Contractor shall be appointed by the client under a Collateral Agreement to the main contract as Project Supervisor for Construction Stage in respect of the project pursuant to the Safety, Health and Welfare at Work [Construction] Regulations: 2013 The Contractor shall include for procuring that the insurance cover required to be obtained and maintained by the Contractor pursuant to Clause 21 of the main contract shall provide indemnity in respect of any claim for bodily injury or property damage which the contractor may incur by reason of the performance of the functions and duties of the Project Supervisor for the Construction Stage of the project. The Contractor shall include for providing assistance to the appropriate member of the design team [and to relevant nominated sub-contractors] in preparing ‘As Built’ drawings for inclusion in the Safety File. The Contractor shall include for providing all necessary information as requested by the Project Supervisor for the Construction Stage to enable the preparation and completion of the Safety File excluding drawings and specifications which shall be provided by the appropriate member of the design team and relevant sub-contractors and specialists. The Contractor shall include for the Project Supervisor for the Construction Stage giving notice to the National Authority for Occupational Safety and Health prior to commencement of work on site in accordance with the First Schedule of the Safety, Health and Welfare at Work [Construction] Regulations: 2013 The Contractor shall include for the Project Supervisor for the Construction Stage preparing a Safety File, making any necessary adjustment and on completion of the contract delivering the Safety File to the Employer, in accordance with the requirements of the Safety, Health and Welfare at Work [Construction] Regulations: 2013 The Contractor shall note the Project Supervisor Design Stage for this Contract under the Health, Safety and Welfare at Work [Construction] Regulations 2013. The Contractor shall also include for carrying out all functions required of the Project Supervisor for the Construction Stage under the Health, Safety and Welfare at Work [Construction] Regulations 2013 - SAFETY, HEALTH AND WELFARE AT WORK: Provide for all costs in connection complying with the provisions of the Safety, Health and Welfare at Work provisions, including the following: - the operation of the requirements of the Safe Pass and Construction Skills Certificates Scheme - Safety Officers and all safety supervisors, first aiders, inspectors and certifiers required - Tests and certificates on plant and equipment - Safety signs and signals - Provision of Personal Protective Clothing (PPC) and Personal Protective Equipment (PPE) - Provision and maintenance of emergency routes and exits and designated personnel responsible for emergency procedures - Energy distribution installations - Temporary fire detection and fire fighting systems - Maintenance of appropriate working conditions and work stations - Provision and maintenance of on site traffic routes - Provision of First Aid equipment and First Aid rooms, as appropriate - USE OF SITE: do not use the site for any purpose other than carrying out the works. - ADVERTISING: do not display or permit advertisements to be displayed on the site without consent of the Employer’s Representative. Note PREVENTION OF ACCIDENT OR NUISANCE - USE OF EXPLOSIVES: will not be permitted - NOISE CONTROL: comply with statutory requirements relating to control of noise levels on site. Item - FIRE PRECAUTIONS: take all reasonable precautions to prevent loss or damage from fire. - NUISANCE: take all necessary precautions to prevent nuisance from water, smoke, dust, rubbish and other causes. - INCONVENIENCE: execute any work which, in the Employers Representative’s opinion, is likely to cause annoyance or inconvenience to occupants of adjoining property at such time during normal working hours as the Employers Representative may direct. PREVENTION OF DAMAGE OR LOSS - PUBLIC AND PRIVATE SERVICES: Notify all service authorities of the proposed works before commencing site operations. So far as reasonable check location of known services and check for any other services in the vicinity of the works. Should services not shown on drawings be encountered, immediately notify the ER of their nature and location. Protect, uphold and maintain all services. Do not interfere with their operation without consent of the Service Authorities, or private owners or the ER as appropriate. If any damage results from the execution of the Works, immediately notify the ER and the appropriate service authority and make arrangements for the work to be made good, without delay, to the satisfaction of the service authority or private owners as appropriate. - MAINTENANCE OF ROADS: ensure that no damage beyond fair wear and tear is caused to public roads and footpaths by site traffic. Keep approaches to the site clear of mud and debris. The Contractor must ensure that roads surrounding the site are kept clear, to allow traffic to move freely, that all roads and footpaths are swept frequently and no mud, debris etc., is allowed to accumulate at any time. - TREES AND SHRUBS: protect and preserve all trees and shrubs within the boundary, except those to be removed. Replace to approval any trees or shrubs damaged or removed, without prior approval. Item - EXISTING FEATURES: protect existing fences, gates, walls and any other site features which are to remain in position during the execution of the works - ADJOINING PROPERTY: consult the convenience of anybody who may be affected by the work with regard to users of any rights of way and the approach to adjoining property, suitable times for making deliveries and all other matters affecting them. Take all precautions to prevent anyone concerned with the building operations from entering on the adjoining property. Provide everything that may be necessary for the protection and safety of adjoining property while carrying out the works. The Contractor shall alter, adapt and maintain as necessary any temporary works required in this connection and finally clear away and make good - The Contractor is to ensure that his work on this Contract does not interfere in any way with Contractors engaged on adjoining sites. - MANAGEMENT/ADMINISTRATION PROCEDURES PROGRESS/SUPERVISION - CONTRACTOR’S SUPERVISOR: obtain Employer’s Representative’s approval prior to appointment of a suitably qualified and experienced Supervisor for the contract. Such approval will not relieve the Contractor of any obligations under the contract ## CONTRACTOR’S REPRESENTATIVE: employ and keep available at all reasonable times in their offices or on the site, a competent Contractors Representative and any instructions given to him by the Employer’s Representative, shall be deemed to be given to the Contractor. The Contractor’s Representative shall take responsibility for the accurate transmission of the Employer’s Representative’s instructions to the site, stockyard or workshop as appropriate, and to their expeditious implementation. - WORKPEOPLE: None but fully qualified and competent Workpeople shall be employed on the Works and the Contractor shall furnish such evidence as the Employer’s Representative may require as to the qualifications and competence of any workpeople. Ensure that all work people employed on the Works (including Sub-contractors staff) are registered for PAYE and PRSI and are registered under the CIF Pension Scheme - ORDER OF WORK: carry out the work in an order approved by the Employer’s Representative. 8.05 PROGRAMME: prepare (a) a detailed job programme and (b) projected cash flow for the whole of the Works. Submit within one week of being requested so to do. The programme will be viewed as an essential part of the overall tender documentation. Upon the signing of the contract this programme will become a part thereof, and must be rigidly adhered to. The programme submitted must be a detailed programme containing intended sequence of operations and must also allow for all testing and all commissioning of services within the relevant period for completion stated in the Conditions of Contract and for any work to be carried out by Statutory Authorities or the like concurrent with the Contract. The Programme submitted by the Contractor should include date from issue of letter of acceptance to start on site and contingency as provided for in the contract, (Schedule 1 K) The Cash Flow is to be based on the detailed programme and is to show expenditure requirements for the course of the project. The programme shall show the dates by which any significant items of information will be required from the Employer’s Representative. The Employer’s Representative will supply the information as required in accordance with the stated contract duration. The Contractor shall submit all requests for information so as to allow the Employer’s Representative respond in sufficient time, without adversely affecting the programme. Where submissions for approval are specified, the Contractor shall prepare and submit with his programme a schedule of such submissions, co-ordinated with the programme, and including agreed time allowances for submissions and approval before the related work commences. The agreed time allowance shall be sufficient so as to allow the Employer’s Representative properly review and examine all submissions before approval. - SUBMIT: four copies of programme to Employer’s Representative and keep one copy in the site office. - SUBMISSION OF: programme will not relieve the Contractor of his obligations to act in accordance with procedures set down under clauses 4.9 and 4.10 of the Contract - MONITORING: record progress on a copy of the chart kept on site. Update, and redraft if any circumstances arise which affect the progress of the works, and submit copies of all revisions to the Employer’s Representative or his nominee at monthly intervals. Inform Employer’s Representative or his nominee at each site meeting the extent of time to which the work is ahead or behind the programme time. - SITE MEETING: hold site meetings when required by the Employer’s Representative. Attend such meetings and inform Specialists when their presence is required. - MINUTES: the Employer’s Representative or his nominee will take minutes of site meetings and distribute them to those concerned. 8.11 SUBMIT: to the Employer’s Representative a Cash Flow Forecast. The Cash Flow is to be based on the detailed programme and is to show expenditure requirements for the course of the project. - RECORDS, MEASUREMENTS AND VALUATIONS - STATUTORY RECORDS: provide for all costs in connection with the keeping of all statutory records - MEASUREMENTS: give reasonable notice to the Employer’s Representative before covering up work which he requires to be measured or check. - LABOUR RETURN: provide each week for the Employer’s Representative a daily distribution return showing the number and description of Tradesmen and General Operatives employed on the works, including those employed by Sub-Contractors listed separately. Maintain proper wages books and time sheets showing the wages paid and the time worked by all work people in execution of the contract. Such wages books should show the RSI number of each work person. Produce such wages books and time sheets whenever required for the inspection of any person authorised by the Employer. - MATERIAL RETURN: provide each week for the Employer’s Representative a daily return showing the quantity, description and source of materials brought on or off site. - PLANT RETURNS: provide each week for the Employer’s Representative a daily return of the number type and capacity of all plant excluding hand tools and scaffolding, currently operating on the works. - INTERIM PAYMENTS: submit when applicable, up to date detailed statements of claim including all details required under Clause 11 of the Contract - WORKING HOURS: carry out the work during normal working hours – 8am – 5pm, Monday – Friday, no work on Bank Holidays or weekends without consent of Employer’s Representative. Where this is considered impractical by the Contractor he may apply to the Employer’s Representative for permission to work outside the normal working hours. Where permission is given the additional costs of non-productive time shall be borne by the Contractor - EXISTING LEVELS: signify in writing agreement to the correctness of the existing ground levels, as indicated on the drawings, before the commencement of the works and before the existing ground is disturbed. - CHANGE ORDER: The Employer’s Representative may issue instructions to the Contractor on any matter connected with the Works. Where the Contractor considers any instruction or direction to be a change order the Contractor shall comply fully with the provisions of Clause 4 of the Contract. 10 COMPLETION 10.01 MAKING GOOD DEFECTS: make arrangements with the Employer’s Representative and give reasonable notice of the precise dates and locations for access to the various parts of the works for purposes of making good defects. 10.02 MAINTENANCE INSTRUCTIONS: obtain and hand over to Employer’s Representative House Owner at Partial substantial completion any maintenance instructions provided by manufacturers, suppliers or sub- contractors. 10.03 The appointed PSCS will be required to assist prepare a Safety File in accordance with the details laid down in the PSDP Preliminary Plan. The contractor is to allow for the cost in assisting complying this file and obtaining the required information from all contractors. Collation of the file should commence at the start of the project and be complete within 2 weeks of the project completing, Separate files will be required for each House. 11 RESOURCES/TEMPORARY WORKS AND SERVICES GENERALLY 11.01 LOCATIONS: inform the Employer’s Representative and obtain approval of the intended siting of all spoil heaps, temporary works and services. 11.02 RATES: pay all rates in respect of temporary buildings 11.03 MAINTAIN, alter adapt and move temporary works and services as necessary. Clear away when no longer required and make good. 12 TEMPORARY WORKS - WELFARE FACILITIES: Welfare facilities for all work people engaged on the works shall be in compliance with the fourth schedule of the Safety, Health and Welfare at Work (Construction) Regulations; 2013 - BUILDINGS: provide as necessary temporary sheds, offices, mess rooms, sanitary accommodation and other temporary buildings required for your own use and for use by sub- contractors. The use of sanitary facilities within the houses is not permitted. Comply with the requirements of the Factories Act. - EMPLOYER’S REPRESENTATIVE'S SITE OFFICE: provide and erect on site where directed temporary offices for the use of the Employer’s Representative with total overall floor area not less than 10 square metres, with floor to ceiling height not less than 2.40 metres, together with all necessary heat, light, power, furniture, first aid and fire extinguishers. The office is to be of sound construction, the floors are to be insulated from damp and cold and shall have a covering which is easily maintained. Walls and roof are to be weather proof and sufficiently insulated to maintain a room temperature of 65°F with an outside temperature of 30°F. The external walls and any internal partitions are to be smooth lined with suitable sheet material painted in light colours. Windows providing natural light and ventilation are to be of an area not less than 10% of the floor area and a lockable door, is to be provided. - SANITARY ACCOMMODATION: provide and maintain in a clean condition separate sanitary accommodation for the Employer's Representatives. Provide W.C. wash hand basin, clean towel, soap and water. - HOARDINGS AND GANTRIES: provide temporary fencing, hoardings, fans, planked footways, guard rails, gantries and the like as may be necessary for protecting the public and others, for the proper execution of the works and for meeting the requirements of any local or other authority. - GENERAL SCAFFOLDING: Provide all scaffolding required for the execution of the works. Item - SCAFFOLDING: The Contractor is to include here for: (a) bringing plant to site and removing on completion (b) maintaining plant on site for the duration of the works 12.08 NAME BOARDS: obtain approval for and provide for the full period of the Works a temporary name board displaying title of project in 150mm high lettering, name of Employer, Consultants name in 75mm high lettering, name of Contractor in 125mm high lettering and their respective logo’s, and if required names of Sub- Contractors. - TEMPORARY PROTECTION: The Contractor shall provide [as a minimum requirement] appropriate temporary fencing, hoardings, gates, doors, screens, fans, planked footways, guardrails, gantries, floor, stair and roof edge protection and protection of openings, safety nets and similar items. In particular the contractor shall note the requirement to fully protect the occupants of privately owned dwellings which will be occupied for the duration of the contract. All necessary scaffolding, boarding and screening to allow these occupiers full protected access to their dwellings for the duration of the works is to allowed for. 13 TEMPORARY SERVICES - TELEPHONES: provide temporary telephone facilities for your Contractors own use and pay all charges including cost of calls. 13.04 COMPUTER FACILITY: The contractors representative should have access to computer services with e-mail account, for transmission of information and printing. 13.05 WATER: provide clean, fresh water for the works and make temporary arrangements for storing and distributing about the site and pay all charges. 13.05 LIGHTING AND POWER: provide electricity and all equipment for lighting and power for the works and temporary arrangements for distribution about the site. Provide lighting for any temporary hoarding and the like. Pay all charges. 14 CONTRACTOR’S EQUIPMENT 14.01 EQUIPMENT COSTS: Provide all equipment and vehicles required for the execution of the works. 14.02 EQUIPMENT: The Contractor is to include here for: (a) bringing equipment to site and removing on completion (b) maintaining equipment on site for the duration of the works This clause applies to all equipment required to properly carry out the works. Include here for all costs associated with complying with this clause. 15 NOVATED FIRMS/UTILITY PROVIDER 15.01 SUPERVISION: accept responsibility for the supervision and administration of all novated firms and arrange and monitor a programme with each novated firm. Arrange and monitor a programme with each public and private utility provider or other organisation engaged upon work concurrent with the contract. The E.S.B., Eircom or other similar companies will be carrying out works on or adjacent to the site. The Contractor shall consult with these bodies and allow in his programme for any interference these works may have on his operations. He shall agree a programme with all parties and keep them fully informed as to his own programme and progress of works. The Contractor shall give adequate notice (say 14 days) to any of the above parties when he requires them to carry out specific works on site. The Contractor shall be deemed to have included in his price for any expense incurred in complying with this requirement. 15.02 ATTENDANCE: ascertain the requirements of novated specialists and/or local authorities or public and private utility providers engaged upon the works. Give adequate notice, in writing, to all concerned, when their work requires the use of scaffolding, light, water or special attendance in order that such work may be completed before operatives, scaffolding or plant required are transferred. Item 15.03 GENERAL ATTENDANCE: In addition to the facilities referred to in Clause 6.10 of this Specification all items of general attendance include arranging with novated specialists as to time for commencement of their work on the site or manufacture and delivery of their goods and materials as appropriate, obtaining from them particulars of holes, mortices, chases, recesses, fixings and the like and supplying them with all dimensions and other information required for the proper execution of the works. 16 QUALITY/TESTING/APPROVALS 16.01 COMMODITIES: to be new and in all cases the best quality of the kinds specified, unless otherwise stated. Handle, store and fix commodities with care to ensure that they are in perfect condition when incorporated into the work. 16.02 MANUFACTURER'S RECOMMENDATIONS: handle, store and fix each commodity in accordance with manufacturer's recommendations. Inform Employer’s Representative if these conflict with any other specified requirement. Submit copies of manufacturer's recommendations to Employer’s Representative when requested. 16.03 SINGLE SOURCES: where a choice of manufacturer is allowed for any particular commodity, obtain the whole quantity required to complete the work from one manufacturer or obtain approval of any change in source of supply. Produce written evidence of sources of supply when requested by Employer’s Representative. 16.04 SAMPLES: where approval of commodities is specified submit samples. Do not confirm orders for commodities until approval has been obtained. Retain approved samples on site for comparison with commodities used in the works. Remove when no longer required. Where samples of finished work are specified obtain approval before proceeding with the works. Remove when no longer required. A MINIMUM OF 8 DIFFERENT NEW FLOORING AND WALLING SAMPLES MUST BE PRESENTED TO EACH HOMEOWNER, FOR EACH FLOOR AND WALL FINISH THAT REQUIRES REPLACEMENT. - Provide the following: Samples required External - External finish – render - External finish – painted render Internal - Plasterboard – ceilings and boxing outs - Sealing tapes, - Floor – Marmoleum - Floor – tiles/ Wood flooring - Wall – tiles - Kitchen units – - Internal doors - Internal finish - paint - Skirting - Limestone for fireplace heart - Sanitary equipment dwellings: WC, Obtain approval of colour texture before starting work. Work to be the same standard as the approved sample. 16.05 TESTING PROGRAMME: The Contractor shall be aware of the Employer’s intentions relating to testing generally and of materials in particular REF: IS 398-2 In order to ensure that all materials fully comply with the Specification, a rigorous testing programme will be carried out in conjunction with NSAI or other approved agency. An official order from the Contractor is to accompany the samples sent to the Standards. Contractor to arrange with NSAI to supply original reports of all tests to Employer’s Representative 16.06 FEES FOR TESTING: Provide for paying fees to Testing Authorities and NSAI for testing materials as detailed below In accordance with the Conditions of Contract, the Contractor is bound to supply materials that comply with the terms of the Specification and he will, therefore, be held liable for all testing fees including additional cost arising from any materials failing to pass the tests. Provide for the carrying out and reporting results of the following tests of materials and / or processes: Engineering The contractor is to carry out photographic and video survey of each house prior to commencement and on completion. to monitor where necessary all existing structural cracks during the course of construction on site. Samples to be taken of all hardcore material and approved by Design Professional before use on site. 1 No. of concrete cubes are to be taken and tested for strength and consistency as per specification per house. Foul sewers to be pressure tested as per the specification where affected. Watermains to be pressure tested as per the specification where affected. All foul and storm sewers to have cctv survey carried out prior to handover of the completed project where specified. Allow for all testing in accordance with Drawings and Specification Allow for the provision of the following data sheets and certifications Insulation: data sheets for each type Radon barrier: data sheet on type proposed for use Plasterboard: data sheet from manufacturer Gypsum plaster: data sheets from manufacturer Plastering accessories: data sheets from manufacturer Sanitary ware: data sheets from manufacturer where necessary Floor finishes: data sheets from manufacturer Sand for plastering: evidence of conformity with specification Internal doors: certification of doorsets required for fire doors and details of manufacturer and type of doors for all other types 16.07 NOTICES: give early notices to the Employer’s Representative before placing foundations, laying drains, filling trenches, pouring reinforced concrete and otherwise proceeding with work that will cover and conceal work, to afford him an opportunity of investigating, inspecting or testing the work. Approvals given will not relieve the Contractor from full responsibility for the quality of the work throughout. 17 ACCURACY 17.01 SET OUT the works accurately from information provided by the Employer’s Representative. Inform Employer’s Representative when overall setting out is complete and before commencing construction. The checking of any setting out, line or level by Employer's Representative shall not relieve the Contractor of his responsibility for the correctness thereof. 17.02 TOLERANCES: on all setting out dimensions of a tolerance of plus or minus 2.5mm will be allowed. A tolerance of plus 2.5mm will be permitted on the cross-sectional dimensions of structural members unless otherwise required by the drawings. The Contractor will be responsible for the cost of all corrective measures required by the Employer’s Representative to rectify work which is not constructed within the tolerances set out above. 17.03 INSTRUMENTS: provide all necessary surveying instruments, chains, tapes and other instruments for use of site supervisory staff. The equipment shall be maintained in proper adjustment. Provide all necessary staffmen and attendance necessary for checking the setting out or measuring the work at the request of the Employers supervisory staff. 17.04 CHECK ALL DIMENSIONS: both on drawings and site, particularly the correlation between components and the work in place. 18 PROTECTION - PUBLIC AND VISITORS: provide for all costs in connection with protecting the public from any site operations and putting in place any arrangements for the protection of third parties and visitors to the site in accordance with all statutory requirements - SAFEGUARD the site, the works, materials and plant from damage and theft. Take all precautions to prevent unauthorised access to the site, the works and adjoining property. Make good any damage caused by ineffective protection. - PROTECT the works from damage by inclement weather overloading, impact or any other cause. Provide waterproof and frostproof coverings, temporary shelters, temporary drainage and any other necessary protection for this purpose. Make good any damage caused by ineffective protection. - WATCHING AND LIGHTING: provide watching and lighting necessary for the protection of the public and the works and to prevent trespass. Item - INCLEMENT WEATHER: use all reasonable and approved building aids and methods to prevent, or minimise delays during cold and inclement weather. Item - CLEANLINESS: keep the site and works clean and tidy. Item - FINISHED SURFACES: keep finished surfaces clean as work proceeds and protect from dirt, staining and physical damage. - POLLUTION: do not deposit deleterious material into surface water channels, or lakes. Comply with the requirements of the Rivers Pollution Act, the Public Health (Ireland) Acts and the Fisheries Acts. Item - STORAGE OF MATERIALS: store materials in a tidy manner, separate different materials and protect them from dirt, physical damage, reaction with other materials, marking or staining, micro organisms, mildew, fungus and vermin. - STORAGE OF MATERIALS, PRECAUTIONS: do not store materials in ways which may cause damage to the works or to the material or endanger the safety of people or property. - PROTECTION GENERALLY: The Contractor is to price here for any costs in connection with protecting all trades in these Pricing Documents. Item 19 DRYING THE WORKS 19.01 FUEL, EQUIPMENT AND ATTENDANCE: provide everything necessary for drying and controlling the humidity of the works and pay all costs in connection therewith. 19.02 CONTROL the drying and humidity of the works and the application of heat to prevent blistering and failure of adhesion, damage due to trapped moisture and excessive movement. 20 WORK AT COMPLETION 20.01 CLEAN the work thoroughly inside and out with all reasonable speed at completion of each house and from time to time removing all splashes, deposits, rubbish and surplus material. Clean all roads and pavements at handover stage. 20.02 REMOVE all temporary markings, coverings and protective wrappings unless otherwise instructed. Item 20.03 CLEANING materials and methods to be as recommended by manufacturer of commodity being cleaned. In the absence of such recommendations cleaning materials and methods to be approved by the Employer’s Representative. 20.05 PAINTED SURFACES: touch up minor faults in painted surfaces carefully matching colour and brushing out edges. Item 20.06 MOVING PARTS: adjust, ease and lubricate all moving parts of doors, windows, drawers, ironmongery, appliances, controls and other moving parts as necessary to ensure easy and efficient operation. Item 20.07 SECURITY AT COMPLETION: leave the works secure at completion with all accesses locked. Account for all keys and hand over in accordance with the Works Requirements. 20.09 SUBSTANTIAL COMPLETION: Notify the Employer’s Representative in writing the date on which it is considered Substantial Completion has been achieved and have a date agreed and certified in writing by the Employer’s Representative. 20.10 REINSTATEMENT: The Contractor will be required to reinstate areas provided for compound, temporary construction traffic road access routes and spoil storage, all to the entire satisfaction of the Employer’s Representative. The following is a brief description of work to be carried out: - reinstate generally. Include for removal of all building materials and all traces of builders operations; restore ground. 20.11 WORK AT END OF MAINTENANCE PERIOD: A thorough inspection of the Works will be made by the Statutory Authority at the end of the maintenance period. The Contractor is to include here for any costs involved in general reinstatement of roads, kerbs, grass areas (including weeding etc.,) which may be necessary; also a general grass cutting is to be provided for. 21 LABOUR ON-COSTS, INSURANCE AND BOND LABOUR ON COSTS 21.01 LABOUR: provide for all costs in respect of labour including costs of the following and any other Acts affecting labour at present in force (unless allowed in the rates). - Social Welfare Acts 2. Conditions of Employment Acts. 3. Insurance (Intermittent Employment) Act. 4. Holiday (Employees) Act 5. Redundancy Payments Acts 6. Factories Act 7. The Industrial Training Act 8. Minimum Notice and Terms of Employment Act - Anti-Discrimination (PAY) Act 10. Protection of Young Persons (Employment) Act 11. Protection of Employment Act 12. Unfair Dismissals Act. 13. Employment Equality Act 14. Travelling time expenses, Fares, Transport. 15. Guaranteed time. 16. Non-Productive overtime and other expenses in connection with overtime. - Incentive and bonus payments. - Sick Pay Disbursements. 19. Country Money. 20. Pensions. 21. Safety at Work Act Item INSURANCE 21.02 EMPLOYERS LIABILITY, PUBLIC LIABILITY AND CONTRACTORS ALL RISKS: the successful tenderer will be required to produce the following policies before commencing work and particular attention is directed to the following: - 1. Employer's Liability (a) Cover should apply to all employees on the contract, including gang labourers and sub-contractors and PSCS. (b) The policy should include an indemnity to the Employer as principal. (c) Minimum cover should be € 13,000,000 for any one accident. (d) Any restrictions on the policy regarding certain types of work should not relate to this contract. (e) The policy should cover the contractor's liability assumed by him under this particular contract. 2. Public Liability (a) Cover should apply to the work mentioned in the contract. (b) The policy should include an indemnity to the Employer as principal. (c) Minimum cover should be € 6,500,000 for any one accident. (d) The following risks should be covered: (i) Fire and explosion (ii) The negligence of sub-contractors and their employees (iii) Lifting plant used by the Contractor (iv) Loss or damage to property caused by vibration or by the removal or weakening of supports. (v) Loading and unloading of vehicles beyond the limits of any carriageway or thoroughfare. (vi) Sub-Contractors. (e) The policy should indemnify the Contractor in respect of any liability assumed by him under his contract with the Employer. (f) Any restriction regarding certain types of works should not relate to this contract. (g) The policy should include risks arising from hazardous work. - Professional Indemnity Insurance (NOT REQUIRED) 4. Contractors All Risks (a) Any restriction in this policy should not relate to this contract. (b) Cover under this policy should include permanent and temporary works, site buildings and constructional plant. (c) The policy should be in the joint names of the Employer and the Contractor. (d) Extent of cover to be for the full value of the works and Ancillary Items from time to time including an "escalator clause", the percentage for professional fees and the cost of site clearance stated in the appendix. PERFORMANCE BOND 21.03 PERFORMANCE BOND: obtain from an approved insurance company a Performance Bond for the due and proper completion of the works and pay all premiums and fees required including costs of solicitor to the employers for negotiating the bond. The amount of the performance bond shall be 12.5% of the initial Contract Sum up to certification of Substantial Completion of the Works, and 6.5% of the initial Contract Sum for the subsequent period stated in the form of bond in the Works Requirements. AMOUNT FOR PRELIMINARIES € _________________________________ FORWARD TO GENERAL SUMMARY | Note Note Note Note Note Note Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item item Item Item Item Item Item Item Item Item Item Item Item Item item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item tem Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item Item | __________ __________ __________ __________ __________ __________ __________ __________ _________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |  |
| --- | --- | --- | --- | --- |