1

2026

Solar PV design, supply, installation and all associated works for 4no. Buildings at Sport Ireland Campus, Snugborough Road

Tender Brief

Solar PV design, supply, installation and all associated works for 4no. Buildings at Sport Ireland Campus, Snugborough Road

Tender Brief

Document Control Sheet

| Client: | Sport Ireland |
| --- | --- |
| Document No: | Tender Brief |

| Revision | Status | Author: | Reviewed by: | Approved By: | Issue Date |
| --- | --- | --- | --- | --- | --- |
| P01 | S2 | CM | AD | DF | 08/05/2026 |
| P02 | S2 | CM | AD | DF | 27/05/2026 |
| P03 | S2 | CM | AD | DF | 21/07/2026 |
| P04 | S2 | CM | AD | DF | 07/08/2026 |

Table of Contents

1	General Information	3

1.1	Site Location	3

1.1.1	National Indoor Arena	4

1.1.2	Sport HQ	4

1.1.3	Pavilion	5

1.1.4	Campus Conference Centre	5

1.2	Site Inspection	6

1.3	Permanent Roof Protection & Maintenance Walkways	6

2	Tender Submission	6

2.1	Validity of Tenders	7

2.2	Omissions Made after Tenders Are Received	7

2.3	Tender Evaluation	7

3	General Conditions	7

3.1	Standards of Workmanship and Works Items	7

3.2	Contractor’s Design Responsibility	7

3.2.1	Role of the Employer and Employer's Representative	8

3.3	Insurances	8

3.4	Building Regulations	8

3.5	SEAI Registered	8

3.6	Materials	8

3.7	Site Access	9

4	Preliminaries	9

4.1	Builders Work and Attendances – General	9

4.2	Electronic Leak Detection (Spark Testing)	9

4.2.1	Contingency Mechanism for Pre-Existing Roof Defects	10

4.3	Method Statements	10

4.4	Protection and Reinstatement of Working Areas	10

4.5	Safeguarding the Works	10

4.6	Business as Usual	11

4.7	Out of Hours Work	11

4.8	Craneage and Materials Handling	11

4.9	Existing Toilet and Canteen Facilities	11

4.10	Cleaning	11

4.11	Avoidance of Dust	11

4.12	Activation of Fire Alarm Systems	12

4.13	Making Good	12

4.14	Removal and Disposal of Obsolete Services	12

4.15	SEAI Non-Domestic Microgen Grant	12

5	Contract Supervision	13

5.1	Site Management Team	13

5.2	Qualified Trades Personnel	13

5.3	Compliance with the Specification	14

5.4	Client Contractor Meetings and Work Permits	14

6	Safety, Health and Welfare at Work	14

6.1	Project Supervisors for the Design Process (PSDP)	14

6.2	Preliminary Safety and Health Plan	14

6.3	Appointment of Main Contractor as PSCS	14

6.4	Solas Safe Pass	15

6.5	Freedom of Information Act	15

7	Programme for Completion of Works	15

8	Scope of Works	15

# General Information

Following an energy audit of the Sport Ireland Campus, the four buildings included in this project have been identified as suitable candidates for the installation of rooftop PV panels. The goal of the project is to contribute towards Sport Ireland meeting its Climate Action Target of a 51% reduction in carbon emissions by 2030.

## Site Location

The works will take place on four buildings in the Sport Ireland Campus, Snugborough Road, Blanchardstown, Dublin 15.

### National Indoor Arena

The National Indoor Arena is a multi-sport arena which is used by high-performance athletes and the general public. The flat roof section highlighted below will be the location of the PV array with 211 kWp proposed.

Location of PV Installation

### Sport HQ

Sport HQ is an office building occupied by various national sporting bodies. The flat roof section highlighted below will be the location of the PV array with 72 kWp proposed.

Location of PV Installation

### Pavilion

The Pavilion building contains the reception and changing rooms for the multi-sport pitches located to its rear. It is a curved standing seam roof and a 56 kWp system is proposed in conjunction with 60kWh of BESS to cover the floodlighting load at the facility.

Location of PV Installation

### Campus Conference Centre

The Campus Conference Centre contains an auditorium and meeting spaces. The flat roof section highlighted below will be the location of the PV array with 18 kWp proposed.

Location of PV Installation

## Site Inspection

A site inspection will be mandatory before completing the tender response. The Design Intent documents do not specify exact routes or distances for AC/DC cable runs, so it is expected that tenderes ascertain these details during the site visit.

Tenderers will also be expected to survey and inspect the site in order to get familiar with the local conditions including access to the site, the conditions affecting labour, delivery of materials and any other matters affecting the execution of the works.

Site visits will be conducted on 13th August at 10:00am, where all potential tenderers will be invited to attend at the same time. Representatives from Sport Ireland Facilities and the Engineer will be present during the site visit.

No claims for variations, alterations or additional payment will be considered or admitted on the grounds of lack of knowledge of the works, lack of information, deficiency of description or occasioned by any default of inspection on the part of the Contractor or by any contingency arising which could be ascertained as a result of such inspection.

## Permanent Roof Protection & Maintenance Walkways

The Contractor shall supply and install a permanent, heavy-duty protective walkway membrane system across all three flat roofs (National Indoor Arena, Sport HQ, Campus Conference Centre). The layout of the walkway network must provide dedicated, continuous routes from the roof access points to all solar panel arrays, inverters, and key maintenance zones. This protective layer must be fully installed prior to the positioning of any solar ballasts or frames to safeguard the primary waterproofing membrane against mechanical puncture or damage during solar construction. Following project completion, the walkways will remain as a permanent feature for ongoing maintenance foot traffic.

# Tender Submission

Tenders must be completed in the format described herein and shall contain all the required information including the following:

- Completed Pricing Schedule included as part of this tender information documents

- Completed Form of Tender & Schedule document issued as part of tender package

- Completed Suitability Assessment Questionnaire (QW2 – Parts 1&2)

- Completed Suitability Questionnaire Supplements 3.4.1, 3.4.2 and 3.4.3

- Completed SAQ Appendices (as relevant)

- Completed Model Forms (as relevant)

- Declarations on company headed paper where requested.

- Any other submission requirements as indicated in the SAQ

Failure to complete and submit all of the above documents will result in disqualification from the tender process.

Please note, the successful tenderer will be required to submit further documentation if successful e.g. proof of insurances, detailed programme etc.

## Validity of Tenders

Tenders shall be valid for acceptance by the Client for a period of ninety (90) days from the closing date for receipt of tenders. The closing date will be as specified in the covering letter accompanying this Invitation.

## Omissions Made after Tenders Are Received

No compensation for loss of profit or otherwise shall be allowed to the Contractor for omissions made to the Contract after Tenders are received. The Client reserves the right to omit any portion of the work and any claim for loss of profits due to such omissions shall not be considered.

## Tender Evaluation

This Tender competition is following the Open Procedure meaning Tenderers will initially be evaluated on a pass/fail basis based on their SAQ (Suitability Assessment Questionnaire) and associated SAQ Supplements and SAQ Appendices. Then qualifying tenderers will be evaluated and tender awarded based on lowest price once all other requirements and queries are satisfied.

# General Conditions

This section outlines the General Conditions and shall be read and construed in conjunction with all other relevant sections of the specification.

## Standards of Workmanship and Works Items

The Contractor shall ensure all of the following:

- that the Works are designed, executed and completed

(i) in accordance with all the requirements in, and reasonably inferred from, the Contract [including, where so required by the Contract, in accordance with Contractor’s Documents that have been submitted to the Employer’s Representative] and

(ii) in a proper and workmanlike manner and using good practice

- that all Works Items [whether or not the Contractor is required to select them]

(i) comply with the Contract and the Legal Requirements and

(ii) are of good quality and, unless the Contract provides otherwise, new

- that all materials and goods that are Works Items are fit for their intended purpose in the Works

- that the completed Works are fit for their intended purpose as stated in or to be inferred from the Works Requirements.

## Contractor’s Design Responsibility

The Contractor assumes full responsibility for the detailed design, engineering, integration, layout, optimisation, and performance of the solar PV system, including all civil, electrical, structural, and grid connection elements forming part of the Works. The Contractor warrants that the design shall be carried out with the skill, care, and diligence to be expected of a fully qualified, experienced, and competent solar PV design contractor, as defined in Documents Describing the Works (Volume A).

The Contractor warrants and guarantees that the completed design and installation shall be fully fit for the purpose intended, as detailed in these Documents describing the Works (Volume A), including meeting all statutory requirements, Building Regulations, ESB Networks grid connection conditions, and targeted system performance outputs.

### Role of the Employer and Employer's Representative

Employer's Representative Review and Absence of Liability

Any review, comment, approval, check, attendance, instruction, or failure to object by the Employer, the Employer’s Representative, or their technical advisors regarding any design, calculation, drawing, material, or submittal provided by the Contractor shall be solely for monitoring compliance with the Contract principles.

Non-Relief of Contractor Obligations

No action or omission by the Employer or the Employer’s Representative shall relieve the Contractor of any of its obligations, liabilities, or design responsibilities under this Contract, nor shall it constitute acceptance of a design defect or shift any portion of design liability to the Employer or the Employer’s Representative.

Employer Information as Background Only

Any outline concepts, preliminary capacity layouts, site information, or roof drawings provided by the Employer or the Employer's Representative prior to or during tender are supplied for background information only. The Contractor shall independently verify all site constraints, roof load capacities, cable routes, and electrical connection points. The Contractor shall not be entitled to rely upon the accuracy or completeness of such Employer-provided information for design purposes.

## Insurances

The Contractor shall include for all necessary insurances, including Employers Liability, Public Liability, Third Party, Workman’s Compensation, and Contractors All Risks, and all other statutory requirements applicable. Collateral warranties will be required from any 3rd party contractors being brought in as part of the works.

## Building Regulations

This Contractor shall be deemed to have a full understanding of the current Building Regulations, and with the contents of all technical guidance documents to these Regulations. He shall ensure that all works are undertaken and completed in accordance with these Regulations.

## SEAI Registered

All works must be completed by a registered Company and their associated Installer listed on the SEAI Non-Domestic Microgen List of Registered Companies. All systems installed must comply with the requirements set out in the Code of Practice as published on the SEAI website.

## Materials

Materials and goods shall be of the best quality of their respective kinds and shall comply with the appropriate European, Irish or British Standard, unless otherwise stated. Where materials or components are specified to comply with a Standard Specification, the relevant standard number shall be marked on the material itself, or on a delivery note accompanying all materials.

Samples of materials and works shall be submitted to the client for their approval, before inclusion in the works and articles not approved shall be removed, if directed. Approval of any materials or work will not relieve the Contractor from responsibility for defects subsequently arising. All unfixed materials shall be carefully stored in accordance with manufacturer’s recommendations in a suitable location on site.

## Site Access

All access to the premises shall be by agreement with the client. Means and timing of access shall be the sole prerogative of the client, and the Contractor shall allow for conforming is all respects to these requirements.

Names of contracting personnel are to be provided where possible before attendance on site.

# Preliminaries

Under this Section the Contractor shall include for all necessary “Preliminaries” associated with the contract works including but not limited to those described in the following paragraphs.

## Builders Work and Attendances – General

The contractor shall include for all necessary builders work and attendances associated with the electrical services installation that would normally be provided by a main building contractor.

Such builders work and attendances shall include but not be limited to the following:

  - Office and storage accommodation.

  - Temporary lighting for the works.

  - Temporary power for the works.

  - Temporary Water for the works.

  - Welfare facilities – Canteen and Toilets.

  - Cleaning and disposal of rubbish.

  - Protecting completed works.

  - Replacement of damaged ceiling tiles.

  - Forming openings in walls.

  - Intumescent fire stopping of services through floors and walls.

  - Installation of fire proofing materials as required

## Electronic Leak Detection (Spark Testing)

The Contractor shall include within their fixed price lump sum all provisions for mandatory electronic leak detection (spark testing) across all four specified roof structures. Testing must be carried out by an independent, accredited specialist with a minimum of 3 years of verifiable experience, using calibrated methods suited to the respective flat and curved roof geometries (e.g., ASTM D7877 / ASTM D7007 or equivalent).

To isolate liability for membrane integrity under Clause 3 of the PW-CF6 Contract Conditions, spark testing must be executed at two distinct milestones:

- Pre-Installation: Directly prior to the placement of any solar panel ballasts, frames, or anchors. No solar installation works shall commence on any roof until a leak-free baseline is formally certified

- Post-Installation: Immediately following the complete installation of all solar panels, mounting structures and electrical cabling, prior to final Completion.

Digital mapping test reports must be submitted to the Employer’s Representative within 48 hours of each inspection. Any leaks, pinholes, or membrane damage identified during the Post-Installation Phase shall be deemed the fault of the Contractor. Such defects must be remediated immediately by an approved roofing specialist, with all remediation and subsequent re-testing executed entirely at the Contractor’s expense before final payment can proceed.

### Contingency Mechanism for Pre-Existing Roof Defects

If the Pre-Installation Phase test identifies pre-existing membrane defects or leaks that are not the fault of the Contractor, such remediation work shall be managed as an Employer Change under Clause 4 of the Contract Conditions.

To facilitate pricing for any such unforeseen repairs, the Tenderer must complete the Pricing Document included in Volume C of this tender package. The Employer reserves the right to either:

- Instruct the Contractor to execute the necessary patching or repairs using these pre-agreed competitive rates via a formal Change Order.

- Appoint an independent, third-party roofing contractor to repair the defects prior to the solar installation commencing.

No claims for extensions of time or disruption costs will be considered for delays resulting from pre-existing roof defects, provided the Employer instructs a resolution within five (5) working days of receiving the Contractor's Pre-Installation digital mapping test report.

## Method Statements

The contractor will be required to carry produce method statements and risk assessments for all hazardous operations to be undertaken by him which pose a risk to the client’s business activities.

These shall be in the form described in the “Preliminary Health and Safety Plan” appended to this specification. They shall comprise a written risk assessment and a system of procedures which must be undertaken in order to safeguard workers, passers-by, buildings, plant and equipment when undertaking hazardous operations.

## Protection and Reinstatement of Working Areas

It is the Contractors responsibility to protect all working areas as required, both internally and externally and return or reinstate them to their original condition.

Prior to the commencement of works the Contractor shall carry out a photographic survey of all areas to establish a record of the condition of finishes. In particular they shall note any damages to the fabric of the building so that there is no subsequent dispute over the origin of these damages. Any indications of damage or repairs required in advance of works commencing should be highlighted to the contracting authority and until an action is recommended works are not to proceed. No responsibility will be taken for damage or injury as a result of failing to highlight any actionable repairs in advance of works starting.

## Safeguarding the Works

The contractor shall provide everything necessary for the protection of the works and materials and all things on the site until completion of the works. This includes protecting, securing, lighting and watching all places in or around the works. No access equipment or plant shall be left accessible for unauthorised persons to enter the site.

## Business as Usual

The contractor should note that the buildings, paths and roadways in the Sport Ireland Campus will be occupied at all times and no disruption to their operation will be allowed. Safe access/passage must be maintained except for those sections and phases as agreed with the contract administrator. The contractor is responsible for ensuring no loss of services occurs to adjoining buildings.

When required the contractor is to liaise with Sport Ireland Facilities, service/utility providers and the Health & Safety Authority to ensure that minimum disruption is caused to the occupant’s area, members of the public and surrounding neighbours. The contractor is to provide all necessary phased or night working required, traffic control and calming measures, temporary traffic lights, special hoardings, plant, labour etc. for the duration of the contract to ensure minimum disruption.

## Out of Hours Work

The final connection, testing, and go-live of the solar PV systems may need to take place outside of standard business operational hours. The Contractor must perform this work during evening shutdown windows to ensure zero business disruption. Allowance should be made for this in Volume C – Pricing Document.

## Craneage and Materials Handling

The contractor shall make provision for all craneage, hoisting and other materials handling equipment necessary for the completion of the works.

## Existing Toilet and Canteen Facilities

The contractor shall note that client agrees to the contractor’s use of the existing toilet and canteen facilities subject to the contractor ensuring that the facilities are cleaned and kept to the existing standard. Otherwise, the contractor shall be required to provide Portaloo’s and external canteen facilities at their own expense.

## Cleaning

The Contractor shall be responsible for keeping the site in a tidy condition at all times and shall remove all rubbish to skips to be supplied as part of the contract.

Each work area must be left in a tidy and safe condition at the end of each working day or when the works are completed in the area.

All dust and dirt shall be removed by mean of an industrial type vacuum cleaner.

Removal of rubbish shall not impede access roadways, car park areas etc. Free passage to fire exit escape routes shall be maintained at all times.

The contractor shall provide for clearing away all rubbish from the site. They shall include for all necessary skips and the like as required.

## Avoidance of Dust

Where drilling is required, the Contractor shall use proprietary dust cup attachments on the drill along with industrial vacuum cleaning equipment.

Where a contractor is identified as having generated dust and dirt in a vulnerable area such as a computer room, the client shall invoice the contractor for all necessary specialised cleaning of equipment.

## Activation of Fire Alarm Systems

All necessary steps shall be taken by the Contractor to avoid accidental activation of either Fire Alarm or Fire Suppression Systems. The Contractor is responsible for fully familiarising themselves with these systems. The client’s personnel will make themselves available upon request to advise what procedures are required.

Any Fire Brigade call out charges resulting from accidental Fire Alarm activation’s caused by the Contractor shall be borne by the Contractor.

## Making Good

The contractor, upon completion of the works, shall ensure that all areas are thoroughly vacuumed and that all marks are wiped clean from walls and ceilings. They shall make good all structural, fabric and service elements of the buildings: These works shall include but not be limited to the following:

    - Structural

    - Block work and Brickwork

    - Partitioning

    - Suspended ceiling and grid

    - Raised Access Floor Tiles

    - Fire barriers

    - Furnishings and Decor

    - Lighting circuitry and fittings

Should any existing lighting protection be damaged, broken or disconnected in the course of any works associated with this project this should be fixed, reinstated and re-certified by suitable persons qualified and recognised to do so by the relevant professional bodies.

## Removal and Disposal of Obsolete Services

The contractor shall include for the disconnection, taking down and disposal of all obsolete electrical services related to the contractor’s discipline and scope.

The extent of the existing services is not fully indicated on the drawings. The contractor shall be deemed to have inspected the site and made an assessment of the work involved.

It is highly recommended that the Contractor inspects the site before completing the tender submission. The client will entertain no claims for extras arising out of lack of knowledge of site conditions.

All electrical equipment shall be disposed of in accordance with the “Waste Electrical and Electronic Equipment WEEE Directive”. Any hazardous materials shall be disposed of in accordance with manufacturer’s health and safety recommendations. The contractor is deemed to be familiar with these regulations and should include for full compliance within their tender submission.

## SEAI Non-Domestic Microgen Grant

The contractor shall include for completing SEAI Declaration of Works, Test and Commissioning Certificate and Safe Electric (RECI) copy of Certificate to enable the client to meet the criteria of the SEAI NDMG grant.

# Contract Supervision

Under this Section the Contractor shall include for all costs associated with management and supervision of the works including coordination with other trades, development of the design, attendance at meetings, liaison with client’s representative etc.

## Site Management Team

The Contractor must identify the following personnel proposed for site works in the tender documents and must provide CVs of these specific personnel showing suitability for the position:

- Contracts Director

- Contract Manager

- Main Site Foreman

- Nominated Sub-Foreman

- PSCS – Health and Safety Representative

The preferred format for the CVs is as follows:

- Name of candidate

- Primary Trade: Example: Electrician

- Year of qualification.

- Number of years with your company as an “employee”.

- Provide list of similar projects which the candidate has project managed.

- Experience acting as PSCS in accordance with Safety Health & Welfare at work cert.

- Specialist Qualifications: - Health and Safety Training

- Specialist Qualifications in other areas

These will be subject to approval as part of the tender submission. Once approved, these key personnel will be fixed and no changes can be made without prior approval in writing from the engineer or client. They must be available to start on site, once the contract works commence.

The Contract Manager shall be responsible for liaison with the client, and their duties shall include the following:

  - Programming the works

  - Procurement of plant and materials

  - Submittal technical submittals for approval

  - Maintenance of the RFI Register

  - Submission of monthly payment claims

  - Prepare site reports and query lists etc. as required.

They shall have been in full time employment with the contractor in a Contract Management role for at least three years and must have relevant previous experience in a similar work environment.

## Qualified Trades Personnel

Only fully qualified and competent trades personnel, together with their necessary labourers or helpers shall be employed by the Contractor on the works. This Contract shall, if requested, provide all necessary evidence to confirm the qualifications and competence, of any workman.

## Compliance with the Specification

Prior to commencing the works the contract supervisor foreman, purchaser and any other senior personnel who may become involved in the project must read the specification.

Any ambiguities will be clarified with the client and agreed with the contractor

The contractor shall note that the client’s representative will be visiting site occasionally to inspect the works. Any works installed in a manner which is deemed not to be in accordance with the specification will be taken down and reinstalled in the specified manner at the contractor’s own expense.

## Client Contractor Meetings and Work Permits

The contractor shall include for site meeting each day with the client’s representative and their staff to outline the work to be undertaken and review the necessary method statements. Similarly, an update meeting shall be arranged at the end of each working day to review the works undertaken that day.

# Safety, Health and Welfare at Work

## Project Supervisors for the Design Process (PSDP)

The Safety, Health and Welfare at Work Act 2005 requires designers to ensure that the project is capable of being constructed and maintained in a safe manner and that systems should comply with all relevant health and safety legislation.

The Client has separately appointed a Project Supervisor for the Design Process (PSDP) of the project to carry out the duties set out in regulations 11 to 14 of the 2013 Regulations.

## Preliminary Safety and Health Plan

A Preliminary Safety and Health Plan has been prepared by the PSDP in accordance with regulation 12 of, the Safety, Health and Welfare at Work (Construction) Regulations, S.I.504 of 2013. This is provided in Documents Describing the Works (Volume A).

## Appointment of Main Contractor as PSCS

The contractor will be required, with the prior written consent of their Insurance Company, in consideration of acceptance of this tender to act as the PSCS under the Safety, Health and Welfare (Construction) Regulations SI 504 of 2013.

The contractor shall be appointed by the client under a collateral agreement to the main contract as PSCS which shall be separate to and independent of the contract for the construction works.

The Contractor will be required to comply with the requirements on all safety related issues including the provision of all necessary information in their capacity as Project Supervisor (Construction) for the preparation of the Safety File.

The Contractor shall be entirely responsible for the safe working practices of the Contractor’s own employees and of the employees of any of the Contractor’s suppliers and subcontractors. All current regulations of the Health & Safety at Work Act and other current legislation shall be strictly adhered to by the Contractor, the Contractor’s suppliers, employees and all the Contractor’s subcontractors and their employees.

The contractor shall include all costs for the provision of the services of the PSCS. They shall provide indemnity in respect of any claim for bodily injury or property damage which the contractor may incur by reason of the performance of the functions and duties of PSCS.

## Solas Safe Pass

Before commencing work on the site all construction workers must receive Solas Safe Pass safety awareness training and be in possession of registration cards. All workers undertaking specified safety critical duties must have received training approved by FAS under the Construction Skills Certification Scheme (CSCS) and be in possession of CSCS registration cards.

The PSCS must co-ordinate arrangements to ensure that workers are in possession of appropriate registration cards and keep a record of the names of those at work on the site to whom the registration requirements apply.

## Freedom of Information Act

Sport Ireland is subject to the requirements of the Freedom of Information Act, 1997. If a candidate considers that any of the information supplied in response to this tender is either commercially sensitive or confidential, this should be highlighted and reasons for its sensitivity given. The relevant material will, in response to a request under the Act, be examined in the light of exemptions provided for in the Act.

# Programme for Completion of Works

A full programme of works is to be agreed between the successful tenderers Contracts Manager, Sport Ireland Project Manager and Sport Ireland Facilities Manager following awarding of the final tender. The contract works shall be carried out in accordance with this programme.

The Contractor shall programme works and those of nominated subcontractors in accordance with the agreed requirements. The contractor shall note that the buildings will remain occupied during the construction period. The contractor shall prepare and submit a master programme clearly showing the proposed sequence and timing of the works. The programme should include DSO applications, procurement, construction test and commissioning and handover with all key elements identified with a clear critical path.

The programme of works should facilitate the commencement of installation works on Sport HQ, Pavilion and Campus Conference Centre buildings by December 2026. Installation works at the National Indoor Arena may commence in early 2027.

It has been identified that the DSO applications may be an area of delay for the project. It is required that the Contractor’s programme of works prioritises DSO applications which should be submitted without delay.

# Scope of Works

The contractor is responsible for carrying out the design and installation of solar photovoltaic systems at the following buildings within the Sport Ireland Campus:

- National Indoor Arena

- Sport HQ

- Pavilion

- Campus Conference Centre

The contractor shall provide a full set of detailed construction drawings for the proposed system and carry out the installation of the panels throughout the roof of each building, along with related works, in accordance with the project's guidance documents. Please refer to the “Specification Document” contained within Documents Describing the Works (Volume A) for further details and requirements of each system.

In addition to the photovoltaic system specifications outlined in the previously mentioned report, the contractor shall ensure compliance with a series of requirements, including demonstrating competence in the design and installation of solar PV systems with a proven track record.

Related to the system specifications, the design and layout shall be in accordance with the Design Intent documents contained within Supplementary Information of Documents Describing the Works (Volume A).