CLONMULLEN HALL 2026

VOLUME C

PRICING DOCUMENT

PART 1

CLONMULLEN HALL 2026

| Document Control Sheet |  |
| --- | --- |
| Document Reference | Vol C |
| Report Status | Tender Issue |
| Report Date | May 26’ |
| Current Revision | D01 |

VOLUME A – WORKS REQUIREMENTS

Roads Design Section Offaly County Council Áras an Chontae, Charleville Rd, Kilcruttin, ore, Co. Offaly, R35 F893 Tel: + 353 57 9357415

| Revision | Description | Author: | Date | Reviewed By: | Date | Authorised by: | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D01 | Tender Issue | TF | May 26’ | BmC | May 26’ | JM | May 26’ |
| Disclaimer This Document is Copyright of Offaly County Council. This document and its contents have been prepared for the sole use of Offaly County Council. No liability is accepted by Offaly County Council for the use of this report, or its contents for any other use than for which it was prepared. |  |  |  |  |  |  |  |

Table of Contents

1.	PREAMBLES TO BILL OF QUANTITIES	4

1.1 General Directions	4

1.2 Measurement	4

1.3 Pricing of Items	5

1.4 Privately and Publicly Owned Services or Supplies	5

1.5 Roadworks Overall Requirements	5

1.6 Work within and below any Water Condition	5

1.7 Dealing with Flow	5

1.8 Site Limitations and Constraints	5

1.9 Hard Material	5

1.10 Equivalent Products and Materials	6

1.11 Permanent Works Designed by the Contractor	6

1.12 Amendments to the Requirements for Measuring and Pricing	6

1.13 Remeasurement	6

2. AMENDMENTS TO THE REQUIREMENTS FOR MEASUREMENT AND	7

PRICING	7

3. BILL OF QUANTITIES	15

# PREAMBLES TO BILL OF QUANTITIES

## 1.1 General Directions

- The Requirements for Measuring and Pricing (RMP) in TII Publications (Standards) is published as a collective group of documents under the Construction and Commissioning activity within the TII Publications system. The Bill of Quantities has been prepared in accordance with the RMP.

- In the Bill of Quantities the sub-headings and item descriptions identify the work covered by the respective items, read in conjunction with the matters listed against the relevant marginal headings “Item Coverage” in the different parts of the RMP, these Preambles and the amendments to the RMP immediately following these Preambles. The nature and extent of the work is to be ascertained by reference to the Works Requirements and Contract Conditions. The rates and prices entered in the Bill of Quantities shall be deemed to be the full inclusive value of the work covered by the several items including the following, unless expressly stated otherwise.

- Labour and costs in connection therewith.

- The supply of materials, goods, storage and costs in connection therewith including delivery to Site. Taking delivery of materials and goods supplied by others, unloading, storage, and costs in connection therewith.

- Plant and costs in connection therewith.

- Fixing, erecting and installing or placing of materials and goods in position.

- Temporary Works.

- The effect on the phasing of the Works or any element of the Works to the extent set forth or reasonably implied in the documents on which the tender is based.

- General obligations, liabilities and risks involved in the execution of the Works set forth or reasonably implied in the documents on which the tender is based.

- Establishment charges, overheads and profit.

- Waste.

- Attendance and transport for sampling and testing carried out by the Employer’s Representative, tests carried out by the Contractor in accordance with the particular requirements of Appendix 1/5, supplying results of tests carried out by the Contractor and providing test certificates.

- Supply and delivery of samples to the Employers Representative in accordance with the particular requirements of Appendix 1/6.

- Complying with Quality Assurance schemes and providing certificates of conformity.

- Preparation and supply of detailed working drawings.

- Awaiting approvals and or consents.

- Complying with all Legal Requirements and ensure that the Contractor’s Personnel comply with all Legal Requirements, including but not limited to those environmental, archaeological and health and safety requirements as set out in the Works Requirements and Contract Conditions.

## 1.2 Measurement

- The measurement of work shall be computed net from the dimensions stated in the Contract unless otherwise stated in the RMP.

Where the tender documents provide for the Contractor to select the type of buried structure (where the structure is not within Designated Outlines) to be constructed then the measurement of all work in each area so affected shall be based upon the thinnest pavement construction and surfacing over structures permitted by the Contract in that area for the particular type of buried structure the Contractor has elected to construct.

## 1.3 Pricing of Items

4

      - Each individual item shall have a rate or price entered against it. Rates and prices shall be expressed to two decimal places.

Reserved Sums and Contract Sums for Specialists will be provided by the Contracting Authority.

Prices entered for attendances and management of specialist works will remain as tendered, even after the Reserved Sums are replaced with actual sums following tendering of the specialist works. These prices may not be less than zero.

## 1.4 Privately and Publicly Owned Services or Supplies

- The information in the Contract as to the whereabouts of existing services and mains is believed to be correct but the Contractor shall not be relieved thereby of his obligations under the Contract. The Contractor shall include in his rates and prices for locating and taking measures for the support and full protection of pipes, cables and other apparatus during the progress of the Works, obtaining the written consent of the appropriate authority to interrupt the service or supply and for keeping the Employer’s Representative informed of all arrangements he makes with the owners of privately owned services or supplies, Statutory Undertakers and Public Authorities as appropriate.

## 1.5 Roadworks Overall Requirements

- The Contractor shall allow in his rates and prices for complying with requirements in respect of pavement construction, horizontal alignments, surface levels and surface regularity of pavement courses, cold weather working, use of surfaces by traffic and construction plant, and general requirements for sub-bases and bases.

## 1.6 Work within and below any Water Condition

- The Contractor shall allow in his prices for any items of structures designed by the Contractor provided under CC-RMP-02500, for taking measures required to execute work in any condition where there is a presence of any type of water.

## 1.7 Dealing with Flow

- The Contractor shall allow in his rates and prices for taking measures to deal with the existing flow of water, sewage and the like.

## 1.8 Site Limitations and Constraints

- The Contractor shall allow in his rates and prices for complying with any limitations and constraints on the use of the Site.

## 1.9 Hard Material

- For the purposes of the Contract the following are designated as Hard Material in accordance with Section 2 Definitions, paragraph 1 (h):

  - Large Boulders in excess of 0.20 cubic metres;

  - Those deposits designated by limits shown on the Drawings;

  - Existing pavements, footways, paved areas (but excluding unbound materials) and foundations in masses in excess of 0.20 cubic metres.

## 1.10 Equivalent Products and Materials

14

      - Where the Contractor offers an equivalent product or material in place of the one identified or specified, which is accepted for incorporation into the Works by the Employer’s Representative, then the rates and prices in the Bill of Quantities shall be deemed to include for all the obligations and costs associated with the incorporation of the equivalent into the Works, including design, provision of data and drawings, certificates, awaiting approvals, resubmissions and modifications and amendments to the Works.

      - Unless specifically stated to the contrary in the Contract the measurement of the Works affected by the incorporation of the equivalent products and materials shall be based on the Tender documents and not on the Works as amended and completed to incorporate the equivalent products and materials.

## 1.11 Permanent Works Designed by the Contractor

- Where the Contract requires part(s) of the Works to be designed by the Contractor, the rates and prices in the Bill of Quantities shall include for all the obligations and costs associated with the incorporation of the Contractor’s design into the Works, including design, provision of data and drawings, certificates, awaiting approvals, resubmissions and modifications and amendments to the Works.

## 1.12 Amendments to the Requirements for Measuring and Pricing

- Amendments to the Requirements for Measuring and Pricing are included in Section 2 of this Volume C.

## 1.13 Remeasurement

- The quantities set out in this Pricing Document shall be subjected to remeasurement. As such the Contractor shall only be paid the initial Contract Sum as defined in article 3 of the Agreement subject to any adjustments (if any) for reasons other than in respect of final quantities as permitted by the Contract.

# 2. AMENDMENTS TO THE REQUIREMENTS FOR MEASUREMENT AND

# PRICING

CC-RMP-00100: Preliminaries

Traffic Safety and Management

Paragraph 30 add sub-clause(s):

Item Coverage

(t)	manual traffic control;

(u)	provision, installation, maintenance and subsequent removal of Variable Message Signs;

(v)	publicity in accordance with Appendix 1/17;

Facilities for Local Authority, Public or Private Utility Undertakers and Other Contractors

Paragraph 58 add sub-clause(s):

Item Coverage

(n)	Security measures required in relation to temporarily exposed services / utilities.

Add the following after paragraph 81:

Paragraphs 82 - 92 are added:

Construction Management

Units  		82	The unit of measurement shall be:

(i)	Construction Management	(item).

Measurement	83	Construction Management shall be measured once only.

Itemisation 	84	One item shall be provided for Construction Management in accordance with Section 3 of CC-RMP-00010 paragraphs 3 and 4 and the following:

Construction Management

85	The items for Construction Management shall in accordance with the Preambles to Bill of Quantities General Directions include for:

Item Coverage

(a)	All items in Appendix 1/74 of the Works Requirements (including preparation, implementing and maintaining Construction Management Plan, Environmental Operating Plan, Waste Management Plan and Quality Plan).

Noise, Dust Deposition and Vibration Monitoring

Units		86	The unit of measurement shall be:

(i)  	Noise, dust deposition and vibration monitoring (item).

Measurement	87	Noise, dust deposition and vibration monitoring shall be measured once only.

Itemisation		88	One item shall be provided for noise, dust deposition and vibration monitoring in accordance with Section 3 of CC-RMP-00010 paragraphs 3 and 4 and the following:

Noise, Dust Deposition and Vibration Monitoring

89	The item for noise, dust deposition and vibration monitoring shall in accordance with the Preambles to the Bill of Quantities General Directions include for:

Item Coverage	(a)	installing,	commissioning,	calibrating	and maintaining monitoring equipment;

(b)	taking readings, measurements and observations;

(c)	copies of reports and results supplied to the Employers Representative;

(d)	removing equipment on completion.

Expose, Lower and Excavation of Existing Services

Paragraphs 85 – 88 are added:

Units	85

The unit of measurement shall be:

(i)	Expose existing service (No.)

(ii)	Expose and lower existing service (m)

Measurement	86	(i)	The measurement of expose existing service shall be the complete excavation to expose all service pipes or ducts a locations shown in the Contract. (This in in addition to the general obligations in the Contract for location of existing services).

(ii)	The measurement of expose and lower existing service shall be the summation of their lengths measured along centre lines of the pipe or duct between the positions of terminations shown in the Contract.

Itemisation	87	Separate items shall be provided for the expose, lowering and excavation of existing services in accordance with Section 3 of CC-RMP-00010 paragraphs 3 and 4 and the following:

| Group | Feature |
| --- | --- |
| I 1 2 | Expose existing service Expose and lower existing service |
| II 1 | Different service |
| III 1 2 | Depths to invert not exceeding 2 metres. The average depth to invert to be stated to the nearest 25 mm. Depths to invert exceeding 2 metres but not exceeding 4 metres and so on in steps of 2 metres. The average depth to invert to be stated to the nearest 25 mm. |
| IV 1 | Construction to trench |
| V 1 2 3 | Ducts Pipe internal diameter not exceeding 300mm Pipe internal diameter exceeding 300mm but not exceeding 450mm and so on in steps of 150mm. |

Expose, Lower and Excavation of Existing Services

88	The items for expose, lowering and excavation of existing services shall in accordance with the Preambles to Bill of Quantities General Directions include for:

Item Coverage

(a)	excavation of acceptable material (CC-RMP-00600 paragraphs 17 and 18;

(b)	excavation of unacceptable material (CC-RMP-00600 paragraph 19);

(c)	excavation by hand;

(d)	backfilling and compaction;

(e)	disposal of material (CC-RMP-00600 paragraph 39);

(f)	variations in average depth to invert level of up to 500mm (increase or decrease);

(g)	lower the existing ducts to new position;

(h)	bedding, haunching and surrounding;

(i)	movement joints to beds, surround and the like;

(j)	repair and replace damaged ducts;

(k)	split ducts.

CC-RMP-00600: Earthworks:

Disposal of Material

Paragraph 39 sub-clause (f) - (g) are added:

Item Coverage

(f)	obtaining all necessary approvals, permissions, consents, permits, licences and the like;

(g)	payment of all fees, charges, levies and the like;

(h)	comply with Appendix 0/1 176.1 AR.

Add the following new series for Landscaping and Environmental:

CC-RMP-03000: Landscaping and Environmental

Paragraphs 1 – 11 are added:

Planting:

Units		1. The unit of measurement shall be:

(i) Woodland mix, wetland mix, hedgerow mix and trees (no.)

Itemisation	2 Separate items shall be provided for planting in accordance with Section 3 of CC-RMP-00010 paragraphs 3 and 4 and the following:

Group		Feature

I	1	Woodland mix

2	Wetland mix

3	Hedgerow mix

4	Low shrub tree mix

5	Woodland tree

6	Hedgerow tree

II	1	Species and type

III	1	Different Sizes

IV	1	In Pits

2	In trenches

3	In beds

4	Bare-root

5	In or adjacent to water areas

6	In grassed areas.

Planting	3	The items planting shall in accordance with the Preambles to Bill of Quantities General Directions include for:

Item Coverage			(a)	notices, inspections, provenance and reporting;

(b)	transportation, packing and unpacking;

(c)	storage, handling and distribution;

(d)	making arrangements for selection;

(e)	treatment of nursery stock;

(f)	protection;

(g)	stakes, labelling, ties, strapping, spacers, anchors, timber frames, root barriers, cables and the like;

(h)	marker posts;

(i)	tubes, guards and ties;

(j)	excavation, breaking up subsoil, backfilling, compaction and firming;

(k)	clearing, excavation, backfilling, compaction and firming trenches for hedgerow planting;

(l)	topsoil;

(m)	reinstatement;

(n)	multiple handling;

(o)	compost, fertiliser and mixing;

(p)	cultivation;

(q)	sprays, dips and additives;

(r)	drainage layers;

(s)	ameliorants;

(t)	root spreading and other measures;

(u)	measures to avoid root disturbance of adjacent plants;

(v)	support;

(w)	pruning roots;

(x)	planting through turf and mulch mats;

(y)	disposal of material (CC-RMP-600 paragraph 39);

(z)	anchors;

(aa)	irrigation pipes;

(bb)	measures for planting in or adjacent to water areas;

(cc)	protective posts;

(dd)	additional ground preparation for wildflower planting;

(ee)	establishment maintenance of new planting;

(ff)	marking out and awaiting approval;

(gg)	anti-desiccant measures;

(hh)	notch planting;

(ii)	watering including water supply;

(jj)	weed control (as per Appendix 30/70);

(kk)	pruning shrubs and climbers (as per Appendix 30/70);

(ll)	scrub control;

(mm)	hedge cutting, hedge laying (as per Appendix 30/70);

(nn)	maintenance of trees (as per Appendix 30/70)

(oo)	maintenance of grassed areas (as per Appendix 30/70)

(pp)	maintenance of wildflower areas (as per Appendix 30/70);

(qq)	management of waterbodies (as per Appendix 30/70);

(ss)	Spreading of slow release fertiliser.

(tt)	Weed control mats

# 3. BILL OF QUANTITIES

| Bill Part | Construction Heading |
| --- | --- |
| Part 1 | Preliminaries |
| Part 2 | Roadworks |
| Part 3 | Accommodation Works |

Roads Design Section

Offaly County Council

Áras an Chontae,

Charleville Rd,

Kilcruttin,

Tullamore,

Co. Offaly,

R35 F893

Tel: + 353 57 9357415