| 1. Description of Project/Works | The re-construction of all 260m of the road network within Clonmullen Hall housing estate, Edenderry, Co Offaly. The Works will also include local repairs to underground services, footpaths, kerbs and accommodation works within the housing estate. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2. Location of Project/Works | Clonmullen Hall, Edenderry, Co Offaly |  |  |  |  |
| 3. Road Type (If applicable. & Roadwork’s type) | Carriageway Type; Single Carriageway; Multi Lane/ Dual and or Motorway. Speed Limit; ≤30Km; 40km; 50km; 60km; 80km; 100km; ≤60km; 80km; ≥100km. Road Level; 1(i); 1(ii); 1(iii); 1(iv); 2(i); 2(ii); 1(iv); 3(i); 3(ii). Yes/ N/A; YES. Roadworks Type. ( ≥ 12 hours) Type A. |
| 4. Introduction: | A Preliminary Safety & Health Plan must be completed by the PSDP for all projects if work is planned to last longer than 30 days or exceed 500 person days or if the works involve a particular risk as per section 15 |  |  |  |  |
| 5. Objectives: | To ensure the safety, health and welfare of persons connected with this work and that of any person who could be affected by the work and in particular members of the public. |  |  |  |  |
| 6. Name of Client: | Offaly County Council, Áras an Chontae, Charleville Road, Tullamore, Co. Offaly. |  |  |  |  |
| 7. Commencement/Time Details: (Insert expected start date and expected completion date) | Start Date: TBA Expected Completion Date: TBA |  |  |  |  |
| 8. Provide names and contact details of relevant organizations /persons as necessary |  |  |  |  |  |
| Project Supervisor Design Process (PSDP): | Offaly County Council, Áras an Chontae, Charleville Road, Tullamore, Co. Offaly. |  |  |  |  |
| Designers: | Offaly County Council, Áras an Chontae, Charleville Road, Tullamore, Co. Offaly. |  |  |  |  |
| Project Supervisor Construction Stage (PSCS) if known: | As Appointed |  |  |  |  |
| Temporary Traffic Management Designer: | As nominated by appointed contractor |  |  |  |  |
| Contractors: | As Appointed |  |  |  |  |
| Contractors: | As Appointed |  |  |  |  |
| Details of consultants: (where applicable) | N/A |  |  |  |  |
| 9. Notification to HSA | YES Notification to the HSA is required if work is planned to last longer than 30 days or 500 person days. (See section 15 on the next page) The Client must use Form AF1 to notify the HSA as soon as possible. |  |  |  |  |
| 10. Existing Drawings or Specifications: | - Volume A1 – Works Requirements - OCC Road Network and Schedule for TTMD - Utility Information |  |  |  |  |
| 11. Detail arrangements for communication with PSDP,PSCS, Contractors as appropriate e.g. pre-start meeting/schedule for ongoing meetings: | Prestart Meeting: Contractor to Liaise with Roads Section &/or Area Engineer &/or MD Engineer ahead of & during works Ongoing Project Meetings: Daily □ Weekly □ Monthly □ TBA As Arranged End of Project Meeting: date TBA |  |  |  |  |
| 12. Details for Monitoring and Review of the Project/Works | Site Visits and H&S Inspections by Roads Engineer &/or MD Staff (Engineer, Tech or GSS) Regular inspections and audits by the Contractor / PSCS. |  |  |  |  |
| 13. Emergency Arrangements(give name and contact details) | Hospital: |  | 999/112 |  |  |
|  | Doctor: |  | 999/112 |  |  |
|  | Gardai: |  | 999/112 |  |  |
|  | First Aider (If you are aware) |  |  |  |  |
| 14. Existing Environment: (please note this is a non-exhaustive list add further information where required) | Item |  |  | Y/N,NA | Comments |
|  | Known Existing Services - Water Mains - Underground Electrical services - Overhead Electrical Services - Gas Supply - Telephone Supply - Broadband Supply - Sewage Lines |  |  | Y | Contractor to scan and check for all existing services prior to carrying out any works on site. |
|  | Area Access or Egress Issues |  |  | Y | Most works will be on public roads in a rural area. |
|  | Poor Site Visibility |  |  | Y | To be assessed by Contractor at each site / works location. |
|  | Poor ground conditions/Contaminated Land |  |  | Y | Soft ground conditions &/or poor drainage conditions will be present. |
|  | Other activities in area? (schools/crèches) |  |  | Y | Activities in the vicinity of proposed works location to be assessed prior to works. |
|  | Adjacent land use |  |  | Y | In general, Houses, Farms, Quarries, Bord na Mona, etc. |
|  | Existing Structures/Hazardous Materials |  |  | Y | To be assessed on site, material around broken Foul Water pipes to be removed to a licenced waste facility. |
|  | Restrictions on Working Hours |  |  | N | Not anticipated as works will require a full road closure |
| 15. Particular Risks to Safety and Health: | Item |  |  | Y/N,NA | Comments |
|  | 01 | Falls from Height |  | Y | Risk of falling into excavations. Contractor to appropriately slope excavations. Contractor to provide appropriate protection at the edge of excavations. |
|  | 02 | Risk of burial under earth falls |  | Y | All excavations/trenches to be assessed by contractor on a continual basis. Works shall be scheduled to avoid periods of heavy rainfall. |
|  | 03 | Engulfment in swampland |  | Y | Bogs/soft ground located within and adjacent to the works area. Contractor to assess excavations on a continual basis. Works shall be scheduled to avoid periods of heavy rainfall. Contractor shall appropriately slope excavations and provide appropriate protection within excavations. |
|  | 04 | Risk from chemical or biological substance |  | Y | Contractor to access works with hot bitumen, cement, sealant products, fuel and oils, etc. Contractor to implement control of hazardous substance register and procedures. |
|  | 05 | Work with ionizing radiation |  | Y | If equipment used/assess levels. |
|  | 06 | Work near High Voltage Lines |  | Y | Details of known utilities provided in information pack. Contractor to scan and check for all existing services prior to carrying out any works on site. |
|  | 07 | Risk of drowning |  | Y | Works near culverts, drains, etc – Contractor to assess site and provide appropriate protection where required. |
|  | 08 | Work on wells, underground earthworks or tunnels |  | N |  |
|  | 09 | Work carried out by divers having a system of air supply |  | N |  |
|  | 10 | Work in a caisson with a compressed air atmosphere |  | N |  |
|  | 11 | Work involving use of explosives |  | N |  |
|  | 12 | Assembly or dismantling of heavy prefabricated components |  | Y | Assess if involves lifting PCC units (headwalls or culverts) into final position; load and method of lift to be contractor assessed |
|  | 13 | Other (list) |  |  |  |

Client

Offaly County Council

16. Project Structure

PSDP

Offaly County Council

Temporary Traffic Management Designer

PSCS

(TBC) Contractor

Site Management Details

H&S Officer

Dennis Gibbons, H&S Officer, O.C.C.

PSDP Health and Safety Co-ordinator

Thomas Fitzpatrick – EE - OCC

PSCS Health and Safety Co-ordinator

To be appointed by Contractor

External Contractor

Unknown

Supervisor/Overseer

To be Appointed

| Hazards and risks associated with the works along with hazards and risk associated with maintenance works on completion of the project must be considered. The list below is non-exhaustive |  |  |
| --- | --- | --- |
| ### Hazard Identification/Risk Assessment | Risk Rating | Possible Control Measures for PSCS to consider |
| ### Hazards (Please identify site hazards) | H/M/L & N/A |  |
| Live Traffic / High Speed Traffic Road Layout Poor Weather Conditions | L | -Site to be investigated ( PPE to be worn as applicable) -Traffic management plan to be designed in complaince with Chapter 8 of the Traffic Signs Manual and all accosicated guidance documents -Temporary Traffic management signage to be erected/maintained/modified and removed as per the Temporary Traffic Management plan either taken from the OCC tempoary traffic management Manual or a site specific plan designed by the Temporary Traffic Management Designer (TTMD) -Where a designed site specific plan requires modification the TTMD must be contacted to approve any alteration to the plan to ensure they comply with chapter 8. -Modifications to any traffic management plans must be docuemented. -Temporary traffic mnagement sigange to be erected and removed under the suprvision of a Temporary Traffic Operations Supervisor (TTOS / 3 DAY SLG cardholder). -Laise with Gardai if required -Erect signage as per traffic management plan/phases. -Roadworks ahead sign to be first sign to be put in position. -Staff erecting traffic management to face oncoming traffic at all times and be vigilant -In adverse weather conditions (fog, heavy rain, snow, ice) adjust/increase warning signage according -Remove / cut back road side growth where required (low tree branches) -Ensure all signage/guard rails erected to be maintained in good condition & clean - Ensure traffic management for night works and Type A considers sufficent lighting for those working and the public |
| Working close to the Public (Pedestrians, Cyclists, Children) | H | -Liaise with house owners & business etc where required -Ensure that pedestrians / cyclists / vehicles movements are considered & controls implemented on site.Traffic management plan to be in place if required -Install Fencing,hoarding & barriers where required -Allow for lighting of routes where required -Ensure control of build up of muck/dust -Ensure tools & equipment are secure when not in use -Ensure work site & equipment is left safe/secure at end of working shift (exposed wiring, excavations/holes, reconnect water supply,housekeeping) |
| Lifting Operations | H | -Use certified lifting appliance (check valves) & lifting gear (chains/slings etc) -Check equipment before use. -Plan lift. Slinger/signaller to be used.Use guide ropes where required -Exclusion zone under lift area. -Weather conditions to be taken into account |
| Plant & Machinery | H | -Drivers of mobile plant to carry out pre-start checks on mobile plant -Flashing beacons, mirrors, reversing sirens and reversing cameras to be in operation on applicable plant. Roll over protection where required. - Provide protection for ground workers, where possible from live traffic and construction traffic. Plan work activities and communicate controls. -Working close to mobile plant. Ground workers not to approach vehicles from the rear. Approach from the side and signal to the driver, wait for the vehicle to stop and the driver to indicate for you to come forward. - Competent drivers to operate mobile plant and where required drivers to have CSCS tickets. -Seatbelts to be worn -All keys to be removed from plant / vehicles when not in operation -Plant Refueling to be undertaken only when the plant engine is turned off. -No naked flames in vicinity of refueling -Gas bottles to be secure in position and have flash back arrestors fitted. -All rotating shafts, augers, PTOs etc to be adequately guarded. -Locking pins to be in use on quick release attachments where applicable -Transporter Trailers to be hitched to towing vehicles when loading and unloading -Bitumen boilers (when in operation) to be supervised at all times. Thermal cut out to be fitted to boiler. Bitumen Boiler to be returned to depot at close of working day -Always park plant safely & secure especially if left on site overnight |

| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| --- | --- | --- |
| Working near overhead services/ Underground services | M | Working near Overhead & underground services - Overhead services to be identified & marked with paint/signage erected//highlighted with goalposts where required. -No tip zones of 3 metres horizontal distance from overhead powerlines to be marked on the roadway (No Tip Zone). -Banksman to be available where required (e.g. reversing trucks , overhead power lines, tipping trucks). -Electrical Wires Crossing Road:All tipper trucks etc to lower trailor before entering tip zone.Trailors must not be raised until trailor has exited tip zone.No vehicle in “No Tip Zone” to exceed 4.2m in height. -Electrical Wires parallel to Road: Minimium clearance required: Low Voltage/1m, 10,20,38kV/3m 110kV/4.5m 220Kv/6m 400Kv/8m Underground Services -Under ground services to be identified & marked with paint/signage. -Contact service provider if required to divert/shut off. -Barriers to be installed around excavation -Hand digging to be carried out to expose underground service - Trained personnel to identify underground services prior to commencement of works. (CSCS Trained for Locating Underground Services) - Ensure compliance with Code of Practice for Avoiding danger for Overhead electricity lines |
| Dust | M | -Keep control of dust exposure by dampening the environment -Ensure adequate ventilation where required. Dust extraction unit to be in place where adequate ventilation cannot be obtained -Water suppression when using consaw - Equipment operators to wear appropriate respiratory protection. -Waste timber, shavings and sawdust should not be allowed to accumulate. |
| Rodent Infestation/Animals | M | -Check for evidence of rodent infestation during site risk assessment. If available, check existing monitoring points for activity -Decontamination of infected area if required -Appropriate PPE to be worn at all times in infestation area (overalls, gloves, mask) -Open cuts/scratches to be covered up at all times -No eating/smoking in vicinity of infested area -Contact local Authority Dog Warden for assistance with dogs if required |
| Working with Electricity | M | -Refer to Building safety file/drawings for details on utilities -Only trained, competent persons to carry out work with electricity -Contact service provider if required to divert/shut off. -Isolate power by lockout/Tagout system (see relevant SOP) -Barriers and warning signs to be installed around electrical appliances under repair |
| Chemical Handling / Biological Hazards (e.g., Diesel, Petrol, cement, bitumen) | M | -Ensure substances/chemicals are stored safely & labelled -Safety data sheets to be available -Staff to wear correct PPE when handling substances/chemicals -Welfare facilities to be available -Vaccinations to be offered to staff (where deemed necessary) |
| Asbestos | L | -Carry out site survey & identify asbestos source -Carry out site risk assessment -Draw up & implement removal & handling procedure -Ensure safe disposal of waste -Ensure compliance with the H.S.A Asbestos-containing Materials (ACMs) in Workplaces Guidelines |
| Confined Spaces | L | -Carry out site assessment & identify possible confined spaces -Carry out site risk assessment -Draw up & implement confined spaces procedure -Ensure rescue procedures in place -Ensure compliance with the H.S.A Code of Practice for working in Confined spaces |

| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| --- | --- | --- |
| Working with Hand Tools & Power tools | M | -Always select the correct tool for the relevant job -Handtools / Powertools to be in good condition. Guards to be in place -Staff member to be trained/informed in use e.g. consaw training -Only 110volt appliances allowed to be used on site if required -Vibration controls when using Jackhammers. (alternating work) -Anti-whip lines to be installed on air compressor connections -Dust suppression to be in place where required. Correct PPE to be worn - Ensure protective guards are present on power tools where required. E.g Angle Grinders |
| Excavations/Openings/Deep Drains | H | - Trained personnel to identify underground services prior to commencement of works. (CSCS Trained for Locating Underground Services) -All trenches/openings to be identified during site risk assessment. -Excavations to be appropriately fenced off and signage erected. -Batter back side of trenches -Where below 1.25m the excavation must be adequately shored/trench box (may also be required for excavations less than 1.25m depending on work activity-assess) -Ladders to be provided for access/egress -Spoil to be stored safely back from excavation -Wheel stops to be in place at tipping locations -No undermining of structures (e.g. walls) -Excavation to be inspected visually daily & recorded weekly on AF3 form |
| Working at Height/Falling Objects | H | -Correct work equipment for relevant job -Ladders to be used for short duration work only -Consider using other equipment (MEWP, Scaffolding, Nets, Bean bags etc) -All Machines/Equipment to be certified, serviceable, checked prior to use -Weather conditions to be taken into account -Secure materials stored at a height (risk of blown down by wind) -Ensure warning signage is in place -Exclusion zone to be in place for safety of persons on ground -Ensure edge protection is in place -Be vigilant for overhead powerlines -Where required, staff to be trained in use of Fall Arrest/Fall Restraint equipment and any other equipment. E.g M.E.W.P. -Excavations to be appropriately fenced off and signed. |
| Working with Services/Utilities | M | -Refer to safety file/drawings for details on utilities -Contact service provider if required to divert/shut off. -Mark appropiately as required -Barriers and warning signs to be installed around electrical appliances under repair -No naked flames in vicinity of gas lines -Report damaged services to the appropriate Service provider |
| Working close to water | L | -Install edge protection where required -Life jackets to be worn where required & Ringbuoyto be available for rescuce -Fall arrest equipment to be worn where required -Dive plan required if diving in operation -Rescue boat may be required (e.g fast flowing rivers etc) - Avoid entering water |

| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| --- | --- | --- |
| Roof Work | N/A | -Unless necessary, work on a roof should not take place -Where possible works should be conducted by preventing the need fora person to go onto a roof -When working on a roof, appropriate access equipment such as crawl boards, roof laders, walkways and planks must be used -An exclusion zone must be marked ou tbelow the works area -Perminter to have full edge protection -Ensure an adequate system of access to the roof area is put in place -Suitable means of getting materials to the roof level should be provided. -Redundant materials and debris must not be thrown off the roof, a chute should be used -The roof and equipment should be checked each day before work to ensure that they are in a safe condition -Fall Arrest/Restraint equipment should be used where required -Weather conditions should be considered prior to commencement of work -Additional precautions for Fragile Roofs -Unless absoutely necessary, work on a fragile roof should not take place -Walking is not permitted on any fragile roofs except where crawl boards, walkways or roof ladders are used. These must be secured against movement. -Fragile roof signage must be fixed to the walls of buildings where access can be made to a fragile roof |
| Working with Scaffolds | N/A | -Ensure all Scaffold is erected and installed by a qualified CSCS Scaffold card holder in accordance with the Construction Regulations 2013. -Ensure scaffold is tagged “unsafe” until erection is complete. -During erection, ensure that all scaffolds have handrails at 1.0m or safety harness and inertia reels are worn. -Adhere to SWL (Safe Working Load) of scaffold. Do not overload with materials or personnel. -Ensure work platforms are clean prior to dismantle of scaffold. Do not remove lower ties until scaffold is dismantled down to that level. -A visual inspection of the scaffold must take place daily -Ensure weekly inspections are carried out on all scaffold and results of inspection are recorded on a GA3 form -Ensure compliance with the H.S.A Code of Practice for Access and working scaffolds |
| Fire & Emergency | M | -Assembly point to be clearly located/agreed -Appropriate fire extinguisher & first aid kit to be available with crew |
| Manual Handling | H | -Risk assessment is carried out -Mechanical aids are provided where necessary -All staff to be trained in manual handling -Work to be organised to reduce manual handling where possible -Routs for transporting loads to be kept clear of obstructions. |
| Noise/Vibration | M | -Carry out site assessment & identify possible sources -Remove/reduce source of noise/vibration where possible -Inform site personnel of hazards -Restrict entry into location with high noise and vibrations levels -Provide PPE where required -Equipment to be maintained in line with the manufactures instruction |
| Lack of PPE | H | -Appropriate PPE for the task to be worn at all times. -Where employee are unsure of the PPE requirments they must contact their line manager. -Class 3 for working on Roads -Ensure PPE fit correctly, adjust fitting or replace to get the stated amount of protection. -Report damaged PPE to you line manager |
| Visibility (E.g. Fog, Night-time hours) | M | -Weather conditions to be consider prior to commencement of work -Adequate lighting is provided -General housekeeping is maintained. - Addtional Signage maybe needed. - At the end of the working day, the site left safe for public use. |
| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| Structures (E.g Overhanging low tree branches?, Low Bridge,) | L | -Sites to be assess prior to the works. -Consideration regarding the plant used on site/area -Low hanging trees and vegation may be removed prior to the works -Consideration to be givrn to the effects of vibrations on structure where applicable - Incorporate work measure to protect/work around sturctures |
| Tree Cutting (Removal of trees, or overhanging branches, or verge trimming) | N/A | -All employees to be briefed before works starts on the task -All operatives to be suitably trained in the equipment they are operating. - Plan work at height and provide protections for employees -Check equipment before use. -Ensure emergency stops and guards are fitted to all equipment where required. -All employees to wear appropriate Personal protective equipment -Ensure that the area, where the tree could fall into is closed off and that no employees or members of the public are in the vicinity. -Traffic management maybe required. - Ensure that the work is supervised at all times. Park cutting equipment away from the public, where possible. -Keep others clear of the cutting equipment and do not leave equipment unattended. -Suitable traffic management erected to warn other road users of the works taking place - Check area is clear, prior to using verge trimmer (Stones, structures, services etc) |
| Use of Chainsaw & Tree felling operations | M | -Chainsaw operators must receive appropriate training to operate a chainsaw safely. -Chainsaws should only be operated in line with training and instructions given. -The chainsaw must be fitted with a safety device and a chain brake to stop the chain and prevent kickback. -Maintain a full throttle setting while cutting to maximise productivity and to reduce fatigue. -Never run the engine slowly at the start or during cutting as this will increase the risk of kickback. -To reduce the risk of injury in the event of loss of control of the chainsaw cut while standing slightly to the side and out of the plane of the cutting chain and guide bar. -Do not overreach during cutting, or operate the chainsaw above shoulder height. -Never begin cutting with the upper half of the nose of the blade. While cutting, watch out from branches, logs or other material that could come into contact with the danger zone and case kickback. -Turn the guide bar daily to equalize wear on the bar. Never use the chainsaw if it has excessive wear on the guide bar. -The throttle trigger lockout must be properly functioning and locked in the idling position when the lockout is not engaged by the proper hand grip. -Do not operate the chainsaw with a loose chain – tension them immediately. -Set the depth regulator as per the manufacturers’ instructions. -Do not operate the chainsaw if the on/off switch is faulty. -Always apply the chain brake to an idle chainsaw. -Maintain a safe operating distance from others. -Maintain a secure footing during chainsaw operation. -The appropriate PPE must be worn when operating the chainsaw (see picture opposite for a guide to the PPE to be used) – chainsaw gloves, leg protection should incorporate a clogging material and safety boots must give a good grip. -Refuel chainsaws are per training given. Clean up any spillage and do not smoke. Start the chain saw slightly away from where you re-filled it. -After contact with petrol / oils wash your hands. -Never leave a chainsaw unattended, particularly in public areas. -Never carry a chainsaw while it is turned on. -Maintain chainsaws in good working order and in accordance with training given. -The chainsaw should be stored in a secure storage area when not in use. -The chainsaw should be adequately sharpened and free from defect. The correct file and file holder should be used to sharpen the chain. -Chainsaws must never be operated when working alone. -Provide a first aid kit in the working area – ensure it contains large wound dressings. -Where there is significant exposure to vibration from long periods of use gloves will be worn. Workers will swap to tasks without exposure to vibration for periods to reduce exposure to vibration. -Tree shedders must be used in line with manufactures guidelines. Operators must load trees with caution into the shedder and let go of the tree once the feeder takes hold. Keep others clear during this operation. Where eye and hearing protection. |
| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| Use of M.E.W.P Use of M.E.W.P | N/A | This document is for all types of boom (articulated and telescopic) mobile elevating work platforms (MEWP’s) commonly known as ‘cherry pickers’ including ones that are vehicle mounted, self propelled and trailer-mounted. This document does not cover risks associated with work near overhead power lines (Refer to RA/SS on overhead power lines) The risk of falling from the MEWP is from sudden movements caused by an impact, ground movement, overreaching or the failure of a critical stability part. The wearing of fall protection (normally fall restraint) provides protection against falling or being thrown out of the carrier Safe Plant -Select the correct MEWP for the job (consider ground conditions, working height, the task and the anticipated load). A MEWP must not be used as a crane. -Ensure the MEWP has a thorough examination by a competent person at least once every six months (Form GA1 to be completed). It must be visaully inspected before each use, as outlined to you during training and must be documented weekly using a GA3 Form. Keep this in the safety file. -Ensure those carrying out maintenance on the MEWP are competent to do so -When an MEWP arrives to site, ensure it is the make and model required. This is important where there are possible ground bearing capacities as control systems can vary leading to operator errors. MEWP’s designed to be used on firm level slabs should not be used elsewhere. Safe Site -Segregate other site traffic from the work area. The area underneath tthe basket should also be marked out of bounds to all other in the area. -Ensure parts of the MEWP cannot protrude into roads or other transport routes -Check the work area for localised features e.g manholes, service ducts, potholes etc as these can cause an overturn. Also check overhead for protruding objects and services. -Check temporary covers are strong enough to withstand the applied pressure, inspect temporary and permanant covers at regular intervals where an MEWP is operating on them -Establish the load bearing capacity of the MEWP -Check for overhead crushing or contact hazards regularlay and the works progresses. -Have agreed communication systems between the operator and the groundman -Ensure weather conditions have not altered ground conditions e.g heavy or prolonged rain -Ensure weather conditions are acceptable for use of the MEWP e.g wind conditions. If unsure then stop the process and contact a memebr of management. -Ensure there is a rescue plan in place in the event of an emergency. Ensure there are sufficient people and equipment to implement the rescue plan -Assess alternative work methods before operating beside a steep slope or edge. Safe Operator -When the MEWP is delivered and removed from the site, instruction on the operation of the MEWP must be given to the operator, this is important as MEWP’s have different operating mechanism e.g some do not have breaking on all wheels -Operator must be trained and familiar with the performance and controls of the MEWP they are going to use e.g the operator should understand the types of ground/slope it can operate on or when outriggers require packing -Ensure operators have any task-specific training e.g use of chainsaw -Where a fault is identified on a MEWP, stop use and have the fault repaired. Contact a memebr of management. -Keep the plaform tidy and free from uneeded material, which could possibilty fall to ground level. -Operators are not to climb out of the basket or to over reach while working in the platform. Use of Fall Protection (All quipment to be inspected prior to use and every six months. Document inspections) -If there is a risk of impact or persons falling, then the use of fall protection equipment should be considered e.g. -When working next to live traffic -When travelling with the carrier in the raised position where it may strike fixed objects in its path -When travelling with the carrier in the raised position over uneven ground -The MEWP must be suitable for travelling with the carrier in the rasied position -There are two types of fall protection that a person can use in the carrier: Work restraint system (also known as fall restraint and incorrectly referred to as work positioning) – this stops a person falling from the carrier in the first place (unless it is a MEWP overturn). Fall arrest system - this stops a person after they have fallen from the carrier (unless it is a MEWP overturn). In the event of a fall, a rescue plan must be in place with sufficient people and materials to conduct the rescue. If an employee falls (or in the event of breakdown) from a M.E.W.P then a second employee can lower the M.E.W.P, if it’s not possible to get the employee back inside the basket. Lanyard length (of both fixed length and retractable systems) should be carefully selected and matched to the carrier of the specific MEWP that is going to be used. They must be set short enough to prevent a person reaching a position where they could fall. Working Near Water When working next to water, a life jacket should be worn along with the fall arrest system When working directly over water the system may need to be re-looked at. In relation to tie off points. Rescues systems in the event of anyone entering water. (Rope lines, rescue boat, life buoys etc) |
| Use of Tree Shredder | M | -Tree shredding shall only be undertaken by trained staff with appropriate training and in accordance with relevant legislation and standards -Warning signs erected prior to works commencing -SSWP to be completed by all personnel involved -Work to be supervised and advise other personnel to keep clear -Shredder to be turned off and isolated in the event of removal of blockage -Operatives to exercise caution during loading of shredder & wear suitable ppe requirements - Operator to wear visor and hearing protection and to be mindful of kickback from branches when loading the wood chipper. Stand clear once the material is feed into the mouth of chipper PPE Requirements: (When cutting) - Chainsaw safety boots and protective trousers - Hearing and full face protection (e.g. combination helmets with ear muffs and face shields) - Protective mitts or gloves for use with chainsaw - Non-snag outer clothing - Additional – visor and hearing protection when operating tree shedder. -Tree shredder must be positioned: - Away from the area where trees/branches will be dropped to the ground; -To minimise the potential for woodchip and sawdust to impact on adjacent work areas or the public (e.g.: when on roadways). ~ If this is not possible in the area of tree felling works, shredding should be undertaken elsewhere. |
| Landscaping (Tree pruning, tree/shrub planting, weeding, general maintenance) | N/A | -Always select the correct tool for the relevant job SSWP/Risk Assessment to be communicated to all staff. -Hand tools / Power tools (Including hedge trimmers, leaf blowers, strimmers etc) to be in good condition. Guards to be in place -Staff member to be trained/informed in use - Correct PPE to be worn - Ensure protective guards are present on power tools where required. E.g chainsaws - Wear appropriate PPE when operating tools, may include eye projections, hearing protection, gloves - Assess manual handling tasks (Consider the Task, Load & Environment) -Mechanical aids are provided where necessary -All staff to be trained in manual handling -Work to be organised to reduce manual handling where possible. - Consider rotating employees to avoid employees working at the same tasks all day. -When planting ensure openings are closed as soon as possible to avoid others stepping into holes/openings in a public area. - Ensure all debri are collected as soon as possible to avoid tri hazards, - Ensure others stand clear when cutting back vegetation so they are not struck by branches or other objects. Be careful to avoid branches or others catching your eyes. Consider wearing protective glasses - All equipment to be returned to vehicles/stores when no longer needed. -Consider the use of barriers or similar to keep the public clear from the work area. - If required take rest breaks when weeding, cutting back edges of flowerbeds/footpaths. - Use mechanical aids where possible when planting. (Trolleys, machines etc) - -Where appropriate use MEWPs to create a safe working platform if work is at height. |

| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| --- | --- | --- |
| Litter Picking | N/A | -Use gloves when emptying bins and littler picking. -Do not put hands where you cannot see. -Mechanical aid (tongs etc) also to be used when litter picking. -When handling bags, first visually check for protruding / sharp objects before lifting. Grab bag by the rim, never at sides or bottom, and hold it away from the body. -Employees must only remove discarded needles through the use of suitable personal protective equipment e.g. litter pickers, disposable tongs, long nose pliers, sharps gloves, etc. -Never re-sheath a needle or separate needle from the syringe -Ensure safe disposal of all recovered syringes and needles in an appropriately designed sharps container (Available from Department/Section) -Ensure the sharps container is never more than ¾ full -Employee to wash hands prior to eat or smoking |
| Mowing Grass / Strimming / Spraying pesticides | N/A | Grass Cutting – Staff must be trained. SSWP/Risk Assessment Communicated -Wear hearing protection and high visibility clothing and safety footwear. -Safe transport to the location – secure. -Safe unloading from vehicle. Use of ramps where needed -Check mowers before use, ensure guards are in place, never tie guards open or bypass any safety devices, stop engine when fuelling, keep feet clear of mowers when starting, check area before cutting for stones or other material. -Disengage blades when traveling and push mowers must be turned off, -Don’t run engines indoors. -Ride on mowers not to be left running indoors outside of entry and exit point. -Ensure area is ventilated. Stop engine if anyone approaches. -Wear seatbelts where fitted and ensure roll over bars are used where fitted. - -Do not turn on slopes. -If you are unsure then stop and report concerns to your line manager. Stay clear of embankments and edges such as rivers. -Clean down mowers when engine turned off, keys removed, and keys placed in your pocket. -Do not put your hand near any blades. Store mowers safely away after use. --Report any defects to your line manager. |
| Strimming operations | N/A | Strimmer – Staff must be trained. SSWP/Risk Assessment Communicated -Wear hearing and eye protection along with high visibility clothing and safety footwear. -Safe transport to the location-secure. -If more than open operator, then agree areas and keep a safe distance from each other. -Use approved blades and strimming cord. Ensure it is fitted as per your training. -Check blade/cord at re-fuelling time. -Only refuel when the engine is cooled down. -Never put your hand or leg near the cutting blade/cord. -Ensure engine is off and blade has come to a complete stop before working on blade/cord. Do not use for extended periods without a break. -Stay clear of embankments and edges such as rivers. Allow device to cool before storage or transport -Report any defects to your line manager. |
| Spraying of Herbicides/ Pesticides/ Insecticides Spraying of Herbicides/ Pesticides/ Insecticides | N/A | Spraying of Herbicides/ Pesticides/ Insecticides - Staff must be trained. SSWP/Risk Assessment Communicated -Wear face shield protection, gloves, and suitable masks along with overalls and safety footwear. -Safe transport to the location-secure. -Safety Data Sheet (SDS) and instruction on container to be referred -Wear the necessary PPE and protective clothing appropriate to the chemical being used. -Heed all warnings and labels. -Always wear face shield to protect against splashes when opening containers or pouring concentrate. -Do not carry out spraying in windy or gusty conditions. -Do not overfill the container. -Place on your back using the safe manual handling principles. Take rest breaks if needed and remove the container from your back. -Report any defects such as leaks, issues with pump or spryer to your line manager. -Wash gloves and overalls at end of each days spraying. -Store chemicals away from eating areas. -Do not eat, drink or smoke during spraying work. -Remove PPE before eating. Wash hands before eating. Use hand wipes if water not available. -Shower at the end of each day’s work. -All chemicals should be handled and stored in accordance with manufacturer’s instructions. -All chemicals should always be stored in their original containers which should be labelled and placed in a secure storage point. Empty containers must be returned to the yard for disposal. -Similarly, unused chemicals must be returned to the yard, not discharged into watercourses. Mix only the amount you will use on each occasion. -Do not attempt to use chemicals unless you are trained and competent in their use and check that the chemical can be used in the location firstly with your line manager (So as not to contaminate water in and around water boreholes, pumping station, treatment location etc) |
| Lack of site security | M | -Vehicles locked when not in use -Fencing around work area where practicable -Signage used to warn public -Reception office (if relevant) -Arrangements made to secure site overnight/during holidays. |
| Management of Covid-19 | M | -Implement current HSE Covid-19 guidelines and consider CIF Covid-19 standard operating procedure guidance document. -Review your work practices where required in relation to the guidance documents. -Update your safety documents. -Communicate new work practices with staff. |
| Compliance with Chapter 8 of the Traffic Signs Manual and all associated Guidance Documents | H | - Temporary Traffic Management Designer must be assigned to the project by the PSCS. (Training records to be provided / managed by PSCS) - The PSCS is responsible for all aspects of Traffic Management up and until the contract closeout. - All works must be underpinned by a Risk Assessment and in particular this assessment must consider diversion routes designated for HGV traffic and must include engagement and comment from all statutory bodies affected by the works especially the Municipal District Offices where the works are to be executed. - Temporary Traffic Operations Supervisor (SLG Cardholder) assigned to the project. (Training records to be provided / managed by PSCS) - PDF copy of the roads schedule included Yes - A map of the county showing road numbers included Yes |
| Surface Dressing Surface Dressing | H | - Temporary Traffic Management Designer must be assigned to the project by the PSCS. (Training records to be provided / managed by PSCS) - The PSCS is responsible for all aspects of Traffic Management up and until the contract closeout. - Make arrangements for traffic control, to identify any special precautions required in relation to overhead cables and to accommodate local access. - Mobile plant arriving to the location is to be un-loaded safely in designated area as outlined by the works supervisor. - All vehicles to park away from the works area until required (Sweeper, Trucks, vans etc) Carrying out the surface dressing; - All work to be carried out in daylight to reduce risks, due to traffic and plant movements. - All surface dressing vehicles to be fitted with audio and visual auxiliary devices as required and All plant to be operated by trained and experienced drivers and maintained. - Protective clothing to be worn to prevent skin contact with the binder and to be aware of the essential first aid procedure for dealing with bitumen burns. - Access points to be provided on plant, so that employees can get access safely when needed. (Tankers, Phonenix Gritter) - Specified person to be appointed to control and guide the movement of reversing trucks and the attaching/detaching of these trucks to the Phoenix Gritter. - No operator shall pass between truck and Phoenix gritter as truck is reversing into position - The tailboard of the trailer are never to be opened, closed or adjusted while the lorry is in motion. Stand to the side as they are opened. Close them again if needed is a safe area away from other plant and ensure the vehicle is parked safely. - All traffic management signs (RWA, loose chipping, Cautionary speed limits etc) shall remain in place until the road is swept in accordance with the Traffic Management Plan for the works. The traffic Management Plan for the works must account for all phases of the project up an until road markings have been completed. - In the event of anyone getting bitumen on their skin seek first aid treatment as soon as possible. Remove employee from the area. Remove contaminated clothing if possible. Cool with water. Cover area with a clean cloth or cling film and get the injured party to the local emergency unit. Do not try to remove the bitumen. |
| Depots Management (Include 3rd Party) Depots Management (Include 3rd Party) | H | - Access to the site must be protected at all times from members of the public - Warning signage to be displayed at entrance to depot/yard - Clear understanding and demarcation of works areas must be agreed in situations where the Depot/Yard is used by more than one department. - Movement of traffic on site to be managed. If site conditions allow, then one-way traffic management systems should be in place. Designate an area for parking of private cars and visitor vehicles. Consideration must also be giving to segregate pedestrian routes from traffic where possible. - Good housekeeping must be maintained at all times at the depot (yard/storage sheds/canteen/welfare facilities). The sheds must be kept clean and tidy. Store away material and tools safely. Keep access pathways clear in the sheds. High standard of housekeeping to be maintained in the canteen (where provided). - The depot must have adequate lighting for work activities - All lifting equipment must be stored in a suitable location, certified and maintained as required. - All battery charging must take place in a well ventilated area - Gas bottles will be stored in secured, ventilated and protected area. - No smoking in the buildings/vehicles - Vehicles shall be serviced in accordance will manufacturer’s recommendations and certified with D.O.E requirements. - Machine guards shall be in place on equipment (e.g bench grinder etc) & kept in a serviceable condition - As minimum, high visibility clothing and safety footwear to be worn by employees /contractors/visitor whilst on site at depots - Onsite pressure vessels for compressors etc must be certify and compliant. - All Services onsite must be checked and certified where applicable e.g gas heating systems. Chemicals - All chemicals must be stored in an allocated storage area or disposed of if not in use. Sheds should be locked. - All chemical containers must be labelled with the name of the chemical and the hazard label associated with the chemical must be visible. Safety Data Sheets to be available for chemicals in use. - All chemicals must be segregated appropriately. Flammable chemicals must be stored in a flame proof cabinet. Corrosives must be segregated from other chemicals Plant & Equipment - All plant and equipment kept at the depot must be maintained, secured and stored in a safe manner (keys to be removed from plant when not in use) - Plant operators to be trained - Persons not to approach a vehicle (e,g machine/truck) until you have made eye contact from a distance with the driver and the driver has come to a complete stop and signalled to you. Never approach any machine from behind without the driver been first being aware. Emergency Equipment - Fire Extinguishers must be maintained and serviced annually. - Stocked First Aid Kit must be available. - Spill kit should be held in the area. If a spill occurs, it must be contained as soon as possible and the material used to contain it. The contaminated material must be disposed of in a suitable manner. Fuel Tanks for storage of fuel for plant/equipment - Warning & information signage must be posted on all fuel tanks (E.g Flammable, No Smoking) - No smoking or naked flames in the vicinity of fuel storage tanks and during refuelling - Mobile phone use is prohibited in the refuelling area - Firefighting equipment should be stored within the vicinity of the fuel tank - Area around filling point must be kept clean of spillages (including rags) at all times. - Fuel tank must be protected from being struck by vehicles (Ensure protective measures are in place, bollards, lego blocks, water barriers etc) - Drip trays should be used underneath the tank and nozzle to avoid contaminating the ground where possible. Offloading/loading material; - Material to be unloaded by vehicle driver into an area in an area agreed with local supervisor/ line management. Do not unload material against a building wall/structure/obstruct traffic routes/mix material with traffic signage etc - Vehicle driver to ensure the vehicle is inline/straight to the area to be unloaded upon i.e must ensure that the truck and body are in line before tipping. - Stand to the side when opening the tail gate (so as to avoid being hit by any material as you open the tailgate). Lorry should be on firm level ground. It is up to the driver to stop the process of tipping, in the event of any issues (material sticking to body, high winds etc). Ensure when raising the body, that no employee/contractor/visitor are standing in the vicinity of the trailer (in the event of the truck over turning all should be outside this range) During loading of vehicle with material: - Loading shovel to have visual aids and warning devices fitted (mirrors/reversing siren/flashing beacons/camera) - Driver to remain in vehicle with handbrake applied (except for dumpers with no cab. Driver to apply handbrake, shut down and exit dumper before loading commences. Return when loading complete) - Where applicable, lower the vehicle body before driving out of the location. Use of Yards/ Depots - From time to time a 3rd party maybe given access to an Offlay Local Authority Depot. This would generally occur where Civils works are taking place locally and the location maybe used as a base to park vehicles and for the storage of materials and equipment. - The use of the location by third party must be agreed with the relevant area office. - See Form SF-YD-01 within RASS-D-00 Depots. This must be completed by the person agreeing to the shared area. |
| M.Yard management (Visitors and Contractor Rules) M.Yard management (Visitors and Contractor Rules) | L | - Visitor and Contractor Site rules to be followed by all on site. - M.Yard management to monitor compliance Visitor Safety Rules – - On arrival at the location/site visitors must make themselves known to the appropriate person or report immediately to their County Council contact person. - All visitors should be accompanied by the person that facilitates their visit - The visitor is the responsibility of the employee who has facilitated their visit to the County Council once they have made themselves know to the staff member. - Only persons authorised are permitted entry onto the premises/property. - Visitors must comply with the instructions and directions given by reception/host person. - Safety notices and warning signs must be obeyed - Visitor must ensure that any children accompanying them are supervised closely by an adult, when on Council premises /property. - Visitors must proceed directly and without undue delay to the designated areas as directed by the reception/host person. - Relevant personal protective equipment (where required) must be worn at all times by the visitor. - Visitors are not permitted to operate, interfere with or wilfully damage any property within the council premises/sites, i.e. vehicles equipment plant, machinery, tools and materials etc. - Pay due attention to the movements of mobile vehicles e.g. trucks, vans etc. - On hearing the fire alarm, evacuate the building to the assembly point and make yourself known to the return promptly to the designated assembly area. - The Council will not accept liability for loss or damage to any person’s property brought onto the company premises by visitors/customers. - Use handrails when ascending or descending the stairs. Avoid the use of mobile devices or stopping to talk to people on the stairs as you may obstruct others - All walkways /stairwells must be kept free from obstructions at all times. - Spills on floors/stairs to be cleaned up immediately. Report any spillages or issues you notice Contractor Site Rules - All contractors that wish to work at Offaly County Council must comply with the Offaly County Council tendering requirements on contractors and complete the appropriate prequalification questionnaire - All contractors that wish to work at Offaly County Council must comply with section 20 of the Safety, Health & Welfare at Work Act 2005 (regarding Safety Statements) and have valid insurance before commencing any work. - All contractors must prepare site specific risk assessments and/or safe systems of work plans before commencing work. - All site rules, safety notices, warning signs and instructions given by Offaly County Council must be adhered to by the contractor. - Contractors must ensure that only trained and competent personnel undertake work on site. Contractors must provide adequate supervision of their employees and activities. - Contractors must inform Offaly County Council contact person of any intention to bring subcontractor personnel onto site. - Contractor must not use any equipment or the service of personnel belonging to or engaged by the Offaly County Council without prior approval being granted by a County Council employee. - Contractors must ensure that all equipment brought on site is in safe condition and has valid statutory certification, where necessary. - Contractors must comply with the permit-to-work system (if relevant). This may be required for high-risk work as deemed necessary. - Contractors must take care for their own health and safety and also the safety of others affected by the work they are doing, and must observe all statutory safety and health precautions. - Ensure that any substances brought onto the premises / sites are accompanied by the relevant Safety Data Sheets (S.D.S.). Ensure that 'substances are stored and used safely whilst on the premises / site. Ensure, on completion of work, that all substances are removed from the premises / site. - Contractors must report all accidents and dangerous occurrences immediately to the Offaly County Council contact person without delay and must co-operate in any investigation into the circumstances of the incident. - Contractors must provide and wear PPE appropriate to the area/task where they are working. - All contractors working for Offaly County Council site shall receive health and safety induction. This involves the communication of the hazards associated with the site/works and site safety requirements. This can be conducted in a number of ways e.g. - Through the communication of the SSWP - Through the completion of the site induction record - Through a Safety Discussion - Through a safety meeting - Contractors in may also be assessed for the role of PSDP/PSCS and be appointed |
| Sweeper Operations Sweeper Operations | L | -Driver to hold the appropriate license for the vehicle to be driven. Staff members who are employed as drivers must complete driver certificate of professional competence (CPC) training. -Any driving offences must be notified to the Line Manager by the driver. A copy of the driver’s License should be obtained and retained on file. - Traffic Management for this operation is as follows: -Urban Areas/Towns/Villages (within 50kph limits)- On rear of Sweeper “Man at Work” Sign with supplementary plate “Road Sweeping” & “Keep Right” Arrow. Note: If the operation requires gulley cleaning then a semi-static lane closure traffic management shall be implemented - Urban Areas/Towns/Villages (outside 50kph limits)- Semi-static lane closure traffic management -The vehicle must be fit for purpose and maintained in accordance with the manufacturers recommendations -The vehicle must be clearly visible to all road users. Flashing Beacon must be fitted and in operation at all times while the vehicle is mobile carrying out sweeping duties. Mirrors and reversing alarm must also be fitted. Fire extinguisher to be available in the event of fire. -Driver must carry out visual daily check on the vehicle. These visual checks will be documented weekly on a vehicle safety check sheet and submitted to the Machinery Yard for attention. Defects to be reported to Line Manager also. -Under no circumstances shall hand held mobiles be used while driving. Park safely before taking any calls. -Cleanliness and appearance of vehicles is the responsibility of the driver. -Driving speed should be appropriate to the weather conditions -Driver to park safely before exiting the truck. Check exit route first, let traffic pass before exiting and ensure vehicle handbrake is on. Remove keys. Face the vehicle when getting down and maintain three points of contact. Driver to avoid having to park on an incline -Drivers must wear high visibility clothing when carrying out any work outside the vehicle -Driver not to work underneath the body of the sweeper truck. Machinery yard maintenance staff to carry out repairs in place in line with manufactures guidelines -The driver must unload the material collected in an agreed area, ensure that the area is clear behind before tipping. -Where possible work in urban areas should be carried out in periods of light traffic flow -Park the vehicle in a safe location and look out for approaching traffic before opening doors of the road sweeper and alighting – also give due consideration to pedestrians -All accidents/incidents must be reported to the Line Manager as soon as possible -Follow safe manual handling techniques when connecting up and removing water connections. Ensure pipes are left neatly and do not cause a trip hazard to others. Ensure water source is turned off before leaving the area. |
| Footpath repairs | N/A | -Traffic controls set up in accordance with Chapter 8 of the Traffic Signs Manual. - A site specific Traffic management plan must be developed by a TTM designer for any footpath works -Safe System of Work Plan (SSWP) to be completed by the crew. -Signage must be erected so that it is visible to the road user at all times and in accordance with the traffic management plan. -Work to be protected off from public access. -All traffic management signage to be installed, modified and removed by an SLG cardholder -All staff to have received manual handling training. -Mechanical aids to be used for lifting activates where practicable. -Check for overhead cables es prior to work -Check area for underground cables prior to works, this must be carried oout by a LUGS cscs cardholder with a certified cable detector. -All employees to wear high visibility clothing (Class 3) -Gloves to be worn when contact with wet concrete become a possibility -Banks man to be used where require for lifting, digging and vehicle movement where required. -All equipment operators on site must have the appropriate training for the equipment they are using -All lifting equipment to have an in date statutory inspection and to be check before use, trained Slinger signaller to shackle loads -Exclusion zone to be set up around lifting area to exclude people from working under lifted loads -Dust suppression to be used where required -Dust mask to be worn when operating consaws -Eye protect to be worn if using Kango or jack hammer to remove footpath. -Store equipment in the works areas |
| Pothole repairs Pothole repairs | N/A | - Traffic controls set up in accordance with Chapter 8 of the Traffic Signs Manual. - Traffic Management Plans form the OCC temporary traffic management manual can be used for pothole repair where applicable where required a site specific plan must be developed by a TTM designer. - All traffic management signage to be installed, modified and removed by an SLG cardholder. - Safe System of Work Plan (SSWP) to be completed by the crew. - Signage must be erected so that it is visible to the road user at all times and in accordance with the traffic management plan. - All employees to wear high visibility clothing (Class 3) - Patching Unit and associated vehicles to be serviced and maintained as required - The vehicle must be clearly visible to all road users. Flashing Beacon must be fitted and in operation at all times while the vehicle is operating. Mirrors and reversing alarm must also be fitted. - Certified fire extinguisher to be available in the event of fire. - Driver must carry out visual daily check on the vehicle. - All personal to wear seat belts when the vehicle is commuting. - Before leaving the vehicle, ensure you are wearing the correct PPE. This will include hi visibility clothing and safety footwear. Depending on the task you may also require Heat Resistant Gloves/Gauntlets, Face/Eye Protection as required (E.g when using the lance) - When exiting the vehicle, do not jump from the cab. Use hand hold provided and face the vehicle, same applies when using the steps onto the truck body. Where practicable try an exit the vehicle on the side away from live traffic - When leaving the vehicle the driver must apply the handbrake and remove the keys from the ignition - Avoid reversing where possible - The vehicles used for patching should move forward during normal operations. Always wait for the employees to stand clear before reversing - The whacker/compactor plate should be used to compact the material, stand to the side while the lifting plate is lowered to ground level. Do not use the wheel of the vehicle to compact the material in the pothole. - Tar lance to be stored correctly on the vehicle when moving between sites, Tar lance pipe not to be dragged along the road surface between stops as this will causing damaged and eventually lead to a burst pipe. Tar lance and pipe should be regularly checked for damage. ---Always wash through the tar lance at the end of the day’s work. - Repairs to machinery and vehicle only to be carried out by Machinery yard operatives or competent sub-contractors - Where sub-contractor are used for pothole repairs then they must be assessed and approved prior to use - All accidents incidents to be reported to your line manager. |
| Public Lighting Operations | N/A | -Public Lighting Contractor s must complete, at tendering stage, the SF-CC-01Pre Qualification Questionnaire Form. -If competent, then public lighting contractors can be appointed as PSDP and PSCS. -Public Lighting Contractor must comply with the site health & safety requirements /legislation. Works on Public Lighting – must be managed and conducted in a safe manner. The appointed contractor must consider, - The pre-planning stage, - Design of lighting columns, - The safe erections of the columns, - The safe removal/repair of columns, - The risk assessment and controls required to ensure the works is carried out safely, - The training of staff require and the safe plant & equipment needed. -If a staff member or contractor notice any issues on the public lighting network, then contact the MD office or the roads office, so arrangements can be made to correct the issue. If needed close off the area to the public in the interim |
| Winter Maintenance Operations Winter Maintenance Operations | N/A | - All those involved in the winter maintenance must be briefed/ inducted on their role prior to starting the winter maintenance program. - Plant to be inspected before use and documented by the driver. Any issues which are identified should be communicated to the line manager - Employees must have a form of communication in the event of any difficulty in the yard or out on the roadways. (Mobile phone. Ensure that the device is charged.). - Employee not to carry out any maintenance work on plant, while working alone. - All trucks and loaders to be fitted with a reversing audible device. - All personnel to remain vigilant and alert at all times. Salt Barn; - Park Safely. Leave room for salt to be unloaded safely into the storage area. - All to stand clear while salt is being unloaded into the shed or being loaded for winter gritting - Salt stockpile to be stored and maintained in a safe condition. Knock down any faces. - Lights must be working in the area. Report any issues with them. - High Visibility Clothing must be worn while carrying out winter service maintenance. Jackets and trousers must be worn. Appropriate footwear with good soles must be worn at all times. - The teleporter operated by a trained employee, will be used to load salt into the brine saturator, all other employees to stand clear while this operation is taking place. - The platform or podium ladder will be used to check the contents of the brine tanks and when opening the bags of salt before they are emptied into the tanks. - Care should be taken when coupling and un-coupling from the brine unit to the salt speader unit. Employee to remain in the vicinity while the brine is being pumped over and then to leave the hose back in the resting position. Mounting the spreader unit; See RASS-MY-07 Winter Gritting Loading the spreader; - Personnel to remain a safe distance from working plant when loading - All personal using the teleporter/ JCB/ Loading shovel or any other plant must be trained - Park the spreader on level ground. Ensure the handbrake is engaged. - Use three points of contact and face the vehicle as you make your exit. - Load the unit if required with the brine from the storage tanks. Connect pipes between the unit and the brine storage tank. Once this is completed position the lever in the on position. Stay in the area in order to ensure that the brine does not overflow and start to spill. Once the tank on the truck is full. Put the lever in the off position and disconnect the pipe from the truck. - Turn on the flashing amber lights on the spreader before leaving the yard. - Lock the gates of the yard as required Use of Snow plough See RASS-MY-07 Winter Gritting Driving of vehicles on roadways/ spreading of salt - All persons driving in the course of their duties are subject to the Road Traffic Acts and all the Regulations made there under. Staff are responsible for the maintenance, upkeep & appearance of their own vehicle. - Under no circumstances shall hand held mobiles be used while driving. Park safely first. - Cleanliness and appearance of vehicles is the responsibility of the driver. - Driving speed should be appropriate to the weather conditions. Leave extra space than normal from the vehicle in front of you during the spreading of salt on the roadways. - Beware of black ice (road looks polished or glossy it could be Black Ice) - Vehicles should be in a roadworthy condition & kept maintained. - A winter maintenance roster will form part of the Winter Maintenance plan. Vehicle breakdown - In the event that winter service equipment breaks down, The Machinery Supervisor is to be notified immediately and the unit is to be returned to the Machinery Yard or particular depot for repair works if it is safe to do. - If for any reason a blockage occurs and renders the spreader inoperable the driver is to immediately notify the machinery supervisor and wait for assistance. - If the vehicle breaks down on route contact is to be made with the Supervisor to arrange repairs. Warning triangle or signs are to be placed on the road, to warn other road users of the danger. The Emergency Lights are to be flashing at all times during the breakdown. (Vehicle to be equipment with emergency kit in the event of a breakdown). If you have to get out of the vehicle, ensure to wear your high visibility clothing. Washing Down the vehicle; - When cleaning vehicle and equipment, appropriate Personal Protection Equipment is to be worn. PPE is to include gloves, boiler suit and protective eye equipment. - The cleaning of equipment is to be carried out at a location that will prevent other employees or passers-by from being sprayed by salt laden moisture or droplets. - Equipment must be immobilised before washing commences. - Equipment is not to be entered into, during the wash down process. - The water jet is to be used to remove the salt. - The operative must stand at a safe distance away from the machine during the operation. |
| Use of Sub-contractors. | M | - All sub-contractor on Offaly County Council project are to be assessed and approved prior to use. - Subcontractors must be appointed prior to the works to role of PSDP/PSCS/TTMD where an appointment applies. - Sub-Contractor to be supervised by OCC staff to ensure compliance with OCC work practices and procedures. - Sub-contractor to sign on to the site SSWP where applicable - Sub Contactor to follow the instructing to the works sponsors in OCC - Sub contactor to follow OCC site Rules. - Sub-Contractor to wear PPE as instructed by OCC - Failure of subcontractor to abide by site rules and follow direction may lead to dismissal from site. - Sub-Contractors to remove equipment from site to carry out repair unless approved by OCC |
| Aggressive Animals | L | - Be vigilant for aggressive animals. - Check any lands for animal prior to entering, Do not enter land containing bulls. - When entering a property that you are aware three is an animal which may pose a risk to your safely ask the owner to restrain/secure the animal prior to entering the property. - Do not enter dwellings containing animals which you suspect may pose a significant risk to your safety. - Where an owner fails to secure/restrain an animal do not enter the premises, contact your line manager. - Take care when disturbing material that is in situ for a period of time as insect/animal may be present - If you disturb a wasp/bees nest remove yourself from the area quickly |
| ### Hazards (Please identify site hazards) | H/M/L & N/A | Possible Control Measures for PSCS to consider |
| Others Site Specific Hazards-Please specify (Add rows as required) (This section may be added to at any stage of the project as hazard are identified.) |  |  |

| ### Design Hazards related to construction of project and future maintenance of the structure (Please identify design hazards) | H/M/L/ NA | Possible Control Measures for Design and PSDP to consider and implement (Designer and PSDP to consider and implement general principles of prevention when considering controls- See overleaf) |
| --- | --- | --- |
| Example Future maintenance of glass windows on roof of proposed new structure |  | Example -Consider redesigning to remove glass windows from roof and no equipment to be located on roof (e.g. air handling units, communication equipment) -If access for roof required then design for installation of roof access ladder from internal of building -Design for installation of parapet wall around roof to allow safe access onto roof if required -Design for installation of fall arrest system to be installed if no parapet wall in place. Fall arrest system to be installed from roof ladder access to all required access points on roof. -Other? |
| Future maintenance of High-level Lighting Units in proposed new structure atrium |  | - Example -Consider redesigning to replace high level lighting with low level lighting up/down light that can be easily accessed using standard manual working platform access platform -If high level lighting must be used then consider use of long life/low maintenance units - If high level lighting must be used then ensure lights can be accessed by current standard M.E.W.P. -If high level lighting must be used then ensure atrium access doors are designed to allow standard M.E.W.P access/egress. -If high level lighting must be used then ensure atrium floor surface is designed to accommodate weight of M.E.W.P to allow access/egress. -Other? |
| Other? Future Access required onto a flat roof to service equipment |  | Example - Consider if equipment needs to be placed on the roof, could it be placed at ground level? -If access for roof required then design for installation of roof access ladder from internal of building -Design for installation of parapet wall around roof to allow safe access onto roof if required -Design for installation of fall arrest system to be installed if no parapet wall in place. Fall arrest system to be installed from roof ladder access to all required access points on roof. |
| Other possible design hazards? Servicing of equipment, access points, cleaning of material, access to fittings, inspections of statutory items. – see general principles below |  |  |

# General Principles of Prevention

Schedule 3 (Safety Health & Welfare at Work Act 2005)

General Principles of Prevention

          - The avoidance of risks.

          - The evaluation of unavoidable risks.

          - The combating of risks at source.

          - The adaptation of work to the individual, especially as regards the design of place of work, the choice of work equipment and the choice of systems of work, with a view, in particular, to alleviating monotonous work and work at a predetermined work rate and to reducing the effect of this work on health.

          - The adaptation of the place of work to technical progress.

          - The replacement of dangerous articles, substances or systems of work by safe or less dangerous articles, substances or systems of work.

          - The giving of priority to collective protective measures over individual protective measures.

          - The development of an adequate prevention policy in relation to safety, health and welfare at work, which takes account of technology, organisation of work, working conditions, social factors and the influence of factors related to the working environment.

          - The giving of appropriate training and instructions to employees.

| Roadwork’s to be managed on site, depending on the type of road, where the works is being undertaken. Work to template Drawing and tailor to site needs in line with Chapter 8. Preliminary Traffic Information (To be considered & developed into site specific Traffic Management Plan(s) by Temporary Traffic Management Designer) |  |  |
| --- | --- | --- |
|  | Items | Details |
|  | Location of Project & Road No | Clonmullen Hall, Edenderry, Co Offaly |
|  | Description of the Project: | The re-construction of all 260m of the road network within Clonmullen Hall housing estate, Edenderry, Co Offaly. The Works will also include local repairs to underground services, footpaths, kerbs and accommodation works within the housing estate. |
|  | Type of Roadwork’s: (Please circle) | ( ≥ 12 hours) Type A; (≤ 12 Hours) Type B; (≤ 15 Minutes) Type C. |
|  | Details of adjoining Roads that could be considered for Diversion |  |
|  | Type of Traffic: (Please circle) Traffic Volumes (AADT): | Heavy goods/ Light Commercials / Cars/ Mixed |
|  | Prevailing Traffic Speeds: (Estimate) | ___________Km/h |
|  | Speed Limit Zone: (Please Tick) |  |
|  | Particular Considerations on site? (Schools, crèche, shops, church, school bus run, milk deliveries etc) |  |
|  | Works expected to interfere Cyclists/Pedestrian Facilities on site? |  |
|  | Overhead Services (Please circle) | ESB Telecom Public Lighting |
|  | Underground Services (Please circle) | Water Sewer Gas ESB Telecom Broadband |
|  | Is Roadwork’s speed Limit Order Required? | Yes No (Please circle) |
|  | Is a Road Closure necessary/recommended? | Yes No (Please circle) |
|  | Speed Limit Restriction for Works: (Special/Advisory) | 50km/h 30km/hr Other________ (Please circle) (Specify) |
|  | Total Road Width (in metres): | ___________metres |
|  | Works Area Width | ___________metres |
|  | Lateral Safety Zone (sideways) |  |
|  | Unobstructed Road Width | ___________metres |
|  | Principal Features of traffic Management Plan (Please Tick) To be considered by the Traffic Management Designer | Tick; Tick. Closed; Lane Closure. Closed (Local Access Only); Signage/Cones. Closed (Local Access) with Diversion; Speed Control Vehicle “Follow me Truck”. Traffic Light System (Garda Notification); Statutory (e.g.80/50 /30km/hr) Speed Limit. Stop/Go System (Garda Notification); Advisory Speed Limit (e.g.55/45/35/25km/hr). All Stop (Garda Notification). |
|  | Any Other Comments |  |
|  | Attach Template Traffic Management Plan(S) | Internal O.C.C Roadwork’s: Type B &Type C Works attach template TMP TM Plan Reference: |
|  |  | Internal O.C.C Roadwork’s: Type A or B Works attach site specific TMP Site Specific TM Plan Reference: |
|  |  | External Works: Temporary Traffic Management Designer (TTMD) to produce Traffic Management Plans |

Definitions

Road Classifications

TTM Roadworks Types Table 8.2.1.2 “ Chapter 8 Traffic Signs Manual (Page 13 Section 8.2.1.3)” ;

*Type A – Full-time roadwork’s that remain in operation in all traffic flows and all visibility conditions. This type of works typically includes temporary traffic measures that will remain in position for a duration in excess of 12hrs. PLEASE NOTE: A qualified Temporary Traffic Management Designer must be appointed to design the Traffic Management Plans for any and all Type A works.

Type B – Part-time roadwork’s that remain in operation when the expected traffic flow is less than the available carriageway capacity. This type of works typically includes temporary traffic measures at off-peak times. With this type of works, the roadwork’s are such that they can be removed if necessary to minimise potential traffic delays. Works duration of less than 12hrs.

PLEASE NOTE Internally: A qualified Temporary Traffic Management Designer must be appointed to design the Traffic Management Plans for any and all Type B works in situations where the application of Offaly County Councils Temporary Traffic Management Manual does not apply.

Type C – Roadwork’s that are of a short duration (Max 15 minutes) and involve the use of one or two vehicles, typically maintenance to utilities or street furniture, in all visibility conditions, when the expected traffic flow is less than the available carriageway capacity and the works do not reduce the carriageway width significantly. PLEASE NOTE: A qualified Temporary Traffic Management Designer must be appointed to design the Traffic Management Plans for any and all Type C works in situations where the application of Offaly County Councils Temporary Traffic Management Manual does not apply.

Attach Traffic Management Plan Drawing

- External Works: Temporary Traffic Management Designer (TTMD) to produce Traffic Management Plans

(To be considered & developed into a site specific Traffic Management Plan(s)

by Temporary Traffic Designer

Note; Where unforeseen circumstances arise, the safety and health issues arising are to be submitted to the Project Supervisor (Design Stage) as soon as possible after the event.

The Construction Stage Health and Safety Plan will include the above arrangements and will be revised as necessary by the PSCS.

The information contained in this Preliminary Safety and Health Plan has been prepared prior to the commencement of the work site.

It does not take account of any matters or information that may come to light after that time.