| # | Requirements | Bidder |
| --- | --- | --- |
| 1 | Dún Laoghaire-Rathdown County Council (DLRCC) is seeking tenders for the supply and delivery of a 4WD Out Front Mower or equivalent, suitable for parks and open space maintenance, roadside verge cutting, sports grounds, amenity areas, and general municipal grounds maintenance operations. Minor variances to these specifications may be considered at the competition assessment stage, solely at the discretion of Dún Laoghaire-Rathdown County Council. Only vehicles that are brand new will be accepted. |  |
| 2 | The mower shall be powered by a water-cooled diesel engine with a minimum output of 36 hp. The engine shall be a 4-cylinder configuration and fully compliant with EU Stage V emissions standards. The machine shall be equipped with an electric starting system, a heavy-duty dual-element air cleaner, and a fuel tank with a minimum capacity of 40 litres for extended operation. The mower shall be capable of a maximum travel speed of not less than 20 km/h. The mower shall be equipped with an effective radiator cleaning system designed to prevent debris build-up, reduce the risk of overheating, and maintain optimal engine cooling performance. Fuel compatibility: HVO fuel to EN 15940 |  |
| 3 | The mower shall be equipped with a hydrostatic transmission system providing smooth, continuous operation without the need for gear changes. The drive system shall be permanent four-wheel drive (4WD) and shall include a standard differential lock to ensure optimum traction under varying ground conditions. The machine shall be fitted with hydrostatic power steering for ease of operation and manoeuvrability. Forward and reverse travel speeds shall be infinitely variable, allowing precise control during mowing and transport operations. The transmission and drive system shall maintain effective traction and performance on wet grass, slopes, and uneven terrain |  |
| 3.1 | The mower shall be equipped with a front-mounted rotary cutting deck with a minimum cutting width of 1.5 metres. The cutting height shall be adjustable over a minimum range of 25 mm to 127 mm through a tool-free or rapid-adjustment system. The mower shall feature a rear discharge and/or mulching system and shall be equipped with a triple-blade cutting configuration to deliver efficient mowing performance and a high-quality finish across a wide range of grass conditions. |  |
| 3.2 | The mower shall be equipped with a hydraulic system incorporating operator-controlled deck raise and lower functions, a minimum of one auxiliary hydraulic circuit, and a hydraulic pump delivering not less than 25 litres/minute at a minimum operating pressure of 150 bar. It should be equipped with a hydraulic lift system for the cutting deck and attachments. Hydraulic connections shall be of the quick-coupler type, and the hydraulic reservoir shall be suitable for continuous daily operation. |  |
| 3.3 | The mower’s overall length shall be minimum 3 meters. The width, including the cutting deck, shall be within 1.65 meters. Ground clearance shall be at least 150 mm. The total operating weight shall not exceed 1700 kg. The machine shall be compact enough to operate in confined urban environments while maintaining stability on uneven ground and gradients. |  |
| 4 | The mower shall be equipped with a heavy-duty 12-volt electrical charging system designed to support all lighting, control, and auxiliary electrical functions. The lighting system shall have full head/dip lights, spotlights front & rear, tail & brake lights, front, rear & side indicator lights A high-visibility amber flashing beacon must be fitted on the machine for safety and compliance with municipal regulations. |  |
| 5 | The vehicles must be supplied with full side and rear reflectorized and fluorescent marker strips. The Tenderer may be required to fit additional high visibility markings to the vehicle as per the requirements of Dun Laoghaire Rathdown Council. |  |
| 6 | Two training sessions (for up to 5 personnel total) must be included upon vehicle delivery, covering safe operation. Operator manual to be provided in both digital and printed form. |  |
| 7 | DLRCC will need to fit a GPS system for the vehicles to get real-time location tracking for fleet management and security purposes. The mower shall be able to accommodate this. |  |