SECTION 3

Technical Appendices for

UHF Antennas at

Woodcock Hill and Aranmore

Tender Doc. 2RN2604-03

# Woodcock Hill

1.1	Summary

Replacement spine on top of the mast with a single UHF antenna system. The existing spine hosts one 4H x 2R antenna.

| Site Name | Woodcock Hill |
| --- | --- |
| Coordinates | 52.71977, -8.692273 |
| Site Height | 254m |
| Support Structure | 640mm square spine on Tower |
| Structure Outline Drawing. | As per attached drawing |
| Total Tower Height | 56m (including existing spine) |
| Design Wind Speeds Vz (m/s) | 81 m/s (Factored gust wind speed) |

1.2	Existing Transmitting System Details

| Existing Antenna | 4H x 2R |
| --- | --- |
| Existing Channels in use | 44, 47 |
| Antenna Height | 50-56m |
| Main Feeders Number | 2 |
| Feeder Type | 1 5/8” |

1.3	New Antenna Requirements (Tendered Antenna Systems)

| Antenna Title | UHF1 |
| --- | --- |
| Estimated Feeder Length | 90m |
| Number of Feeders | 2 |
| Estimated Feeder Requirements | 1 5/8” |
| Fault Conditions | Full power into half stack. |
| Available Aperture | 50-56m |
| Configuration (suggested) | 4 x 2 |
| Channel Range | 21 - 48 |
| Optimised Channels | Priority 1 – 44, 47 Priority 2 – 39, 41, 42, 45 Priority 3 – all others from 21 to 37 |
| Polarization | Horizontal |
| Max ERP | 10kW or more |
| Number of services (at full power) | 2 |
| Tx Output Power per service | 1 kW |
| In building losses | 1dB |
| Beam tilt | 1.5 degrees |

1.4	Radiation Pattern Template

| Azimuth | Normalised Gain (dB) | Azimuth | Normalised Gain (dB) | Azimuth | Normalised Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 0 | -20 | 120 | 0 | 240 | -4 |
| 10 | -20 | 130 | 0 | 250 | -7 |
| 20 | -20 | 140 | 0 | 260 | -12 |
| 30 | -20 | 150 | 0 | 270 | -20 |
| 40 | -20 | 160 | 0 | 280 | -20 |
| 50 | -12 | 170 | 0 | 290 | -20 |
| 60 | -7 | 180 | 0 | 300 | -20 |
| 70 | -4 | 190 | 0 | 310 | -20 |
| 80 | -2 | 200 | 0 | 320 | -20 |
| 90 | -1 | 210 | 0 | 330 | -20 |
| 100 | 0 | 220 | -1 | 340 | -20 |
| 110 | 0 | 230 | -2 | 350 | -20 |

1.5	Tower Drawing – existing spine with existing 2R x 4H antenna

# Aranmore

2.1	Summary

Replacement UHF antenna system mounted on a triangular mast. The existing mast hosts one 8H x 2R antenna.

| Site Name | Aranmore |
| --- | --- |
| Coordinates | 54.983287, -8.508962 |
| Site Height | 120m |
| Support Structure | Stayed Mast |
| Structure Outline Drawing. | As per attached drawing |
| Total Mast Height | 50m |
| Design Wind Speeds Vz (m/s) | 70 m/s (Factored gust wind speed) |

2.2	Existing Transmitting System Details

| Existing Antenna | 4H x 2R |
| --- | --- |
| Existing Channels in use | 44, 47 |
| Antenna Height | 38m – 47m |
| Main Feeders Number | 2 |
| Feeder Type | 1 5/8” |

2.3	New Antenna Requirements (Tendered Antenna Systems)

| Antenna Title | UHF1 |
| --- | --- |
| Estimated Feeder Length | 70m |
| Number of Feeders | 2 |
| Estimated Feeder Requirements | 1 5/8” |
| Fault Conditions | Full power into half stack. |
| Available Aperture | ~ 38m – 47m |
| Configuration (suggested) | 8 x 2 |
| Channel Range | 21 - 48 |
| Optimised Channels | Priority 1 – 44, 47 Priority 2 – 32, 34, 35, 40, 41 Priority 3 – all others from 21 to 37 |
| Polarization | Vertical |
| Max ERP | 5.5kW |
| Number of services (at full power) | 2 |
| Tx Output Power per service | 250 W |
| In building losses | 1dB |
| Beam tilt | 2.5 degrees – with heavy null fill |

2.4	Radiation Pattern Template

| Azimuth | Normalised Gain (dB) | Azimuth | Normalised Gain (dB) | Azimuth | Normalised Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 0 | -12 | 120 | 0 | 240 | -20 |
| 10 | -8 | 130 | 0 | 250 | -20 |
| 20 | -5 | 140 | 0 | 260 | -20 |
| 30 | -3 | 150 | 0 | 270 | -20 |
| 40 | -1 | 160 | 0 | 280 | -20 |
| 50 | 0 | 170 | -1 | 290 | -20 |
| 60 | 0 | 180 | -3 | 300 | -20 |
| 70 | 0 | 190 | -5 | 310 | -20 |
| 80 | 0 | 200 | -8 | 320 | -20 |
| 90 | 0 | 210 | -12 | 330 | -20 |
| 100 | 0 | 220 | -20 | 340 | -20 |
| 110 | 0 | 230 | -20 | 350 | -20 |

2.5	Mast Drawing – existing triangular mast (700mm face width)