Fish Passage Note:

Doneraile Weir 1, Awbeg River, Awbeg Catchment

Doneraile Weir 1 is one of two weirs nestled within the Doneraile Estate in Doneraile, County Cork. Doneraile Weir 1 is located on the Awbeg river within the Awbeg[Buttevant]_020 subcatchment which forms part of the Blackwater[Munster] catchment and the Blackwater River SAC. Doneraile Weir and Estate is owned and managed by the Office of Public Works (OPW) and monitored by the EPA. This stretch of the Awbeg river is currently At Risk due to nutrient and organic matter pollution and currently holds Poor WFD Status 2019-2024.

Location of Doneraile Weir 1 on the Awbeg River, tributary to the Blackwater(Munster) River. Indicated by red arrow.

Doneraile Weir 1 was surveyed using the SNIFFER WFD111 (2a) Coarse resolution rapid-assessment methodology to assess obstacles to fish migration on the 24th May 2022 while the river was at summer low levels.

Doneraile Weir 1 on the day of assessment. Note: Shallow depth of water across the face of the weir. Lip present at foot of the slope.

On the day of assessment Doneraile Weir 1 was determined to be a complete barrier to upstream fish passage for Adult and Juvenile Salmonids, Cyprinids and Lamprey, and a high impact barrier to Juvenile Eels. During high flows, this weir remains a high impact barrier to all species other than Juvenile Eels. The main factors affecting upstream fish passage at this site are the presence of a lip at the foot of the weir, the 45% slope and the depth of water across the face of the weir.