Fish Passage Note:

Doneraile Weir 2, Awbeg River, Awbeg Catchment

Doneraile Weir 2 is one of two weirs nestled within the Doneraile Estate in Doneraile, County Cork. Doneraile Weir 2 is ~200m upstream of Doneraile Weir 1. Doneraile Weir 2 is located on the Awbeg river within the Awbeg[Buttevant]_020 subcatchment which forms part of the Blackwater[Munster] catchment and the Blackwater River SAC. Doneraile Weir and Estate is owned and managed by the Office of Public Works (OPW) and monitored by the EPA. This stretch of the Awbeg river is currently At Risk due to nutrient and organic matter pollution and currently holds Poor WFD Status 2019-2024.

Location of Doneraile Weir 2 on the Awbeg River, tributary to the Blackwater(Munster) River. Indicated by red arrow.

Doneraile Weir 2 was surveyed using the SNIFFER WFD111 (2a) Coarse resolution rapid-assessment methodology to assess obstacles to fish migration on the 24th May 2022 while the river was at summer low levels.

Doneraile weir had two options for fish passage (TS1 and TS2 indicated above). Note: Shallow water depth across the crest of TS1 and large hydraulic head height (0.9m). There was some debris blocking downstream passage. High water velocities at TS2. Debris was blocking downstream of both TS1 and TS2, and blocking upstream at TS2.

On the day of assessment Doneraile Weir 2 was determined to be a complete barrier to the migration of Cyprinids, Lamprey and Juvenile Salmonids, and a high impact barrier to Adult Salmonids. In high flows, the weir continues to be a high impact barrier to Cyprinids, Lamprey and Juvenile Salmonids. The main issues affecting upstream migration are the high levels of turbulence at TS2, the height of the hydraulic drop, shallow water depths and the presence of debris blocking the structure.